| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0051 |
| --- | --- |

| Title: | Enhanced Filtering and Interpolation Methods for Video Signals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Yeping Su, David Singer Apple, Inc. 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple, Inc. |  |  |

_____________________________

# Abstract

New adaptive filtering mechanisms for the 4:4:4 to 4:2:0 and 4:2:0 to 4:4:4 conversion processes are introduced in HDRTools. Two classes of filters are introduced. A parallel multi-filter scheme that tries to determine the “best” filter, given a criterion for each column/row/segment, and schemes that adaptively select the length of the filters based on local colour variations/edges.

# Introduction

Currently, the JCT-VC HDR/WCG activity employs relatively simplistic filtering mechanisms when converting 4:4:4 video signals to 4:2:0 and vice versa. In particular, a 3 tap linear filter, with coefficients [1 6 1]/8, is employed for the down-conversion process, while for up-conversion the Lanczos2 4 tap filter with coefficients [-1 9 9 -1]/16 is used [2]. Both of these filters and the down-conversion and up-conversion processes, were applied on fixed precision data.

These filters, and especially the filter used for down-conversion were selected primarily since they appeared “safe” and could avoid, under most situations and in combination with the luma adjustment method [1], certain color bleeding artifacts that were observed primarily when converting HDR material that utilize the HDR10 representation. The HDR10 representation includes the use of Non-Constant Luminance YCbCr representation, using the ST.2084 transfer function (PQ) [3] and BT.2020 color primaries. It is well understood though that these filters may be inadequate in terms of performance especially for more demanding quality environments.

In this contribution, several adaptive filtering mechanisms are presented that can result in better performance, both objectively and subjectively, compared to the filters currently used by the JCT-VC HDR/WCG activity. The implementation of these filters is also combinable with other methods such as the luma adjustment method and its refinements.

# Filtering optimization using Parallel Filters

Instead of utilizing a single filter for filtering an image, multiple filters could be used in parallel and the one determined as best, given a particular criterion, could be selected for filtering an area during down-conversion.  An area could easily be the entire image, a region in an image, or simple a column or row, or part thereof. In our implementation we have used the column or row optimization method.

More specifically, for each row or column that we need to filter and down-convert, multiple filters are considered for this process. Assuming N filter candidates, each filter is applied on each row or column independently, resulting in N possible filtered outcomes. The result of these outcomes is then compared to select the final filtered output. This comparison is performed by considering a reference upsampling filter that is applied on this samples to reconstruct the signal in its original resolution. Then this signal is compared to the original signal, using a distortion metric D, while the filtered outcome with the lowest distortion is declared the winner. This process is performed for all columns (first) and rows.

In our current implementation, 5 filters are considered for this optimization process, including a 15, a 9, an 11, and two 3 tap downsampling filters. The Lanczos4 filter is always used for the upconversion process. The Sum of Square Errors (SSE) for the current component being filtered is considered for the distortion evaluation, however, that could be extended to consider impact from other color components, given their relationships.

# Edge adaptive/Colour Variation Adaptive Filtering Methods

The parallel filtering method, although quite straightforward, requires considerable processing given the evaluation of multiple filters in parallel, as well as the consideration of an upscaling step and a distortion computation process. Instead a simpler approach can be used, without the need of such computationally expensive operations, using simple analysis on the data to be filtered.

In particular, we have observed that artifacts could potentially appear in the signal if the filtered samples contained very different values. That can obviously occur more frequently when using longer filters, and around or near edges. Therefore, we have decided to adaptively adjust the length of the filter used when filtering a particular sample so as to minimize, as much as possible, the occurrence of such scenarios. This is done by performing a neighborhood analysis and, using predefined classification conditions, selecting the most appropriate filter for filtering this sample.

HDRTools currently supports three different schemes for neighborhood analysis. In the first scheme, starting from the longest filter and for each filter that is eligible for selection, an analysis of the local neighborhood mean is performed for each sample to be filtered. The local neighborhood is defined based on the length of the filter. If the distance of any sample within this neighborhood from the local neighborhood mean exceeds a certain tolerance value, then this filter is disallowed for this sample, and the next filter is evaluated. At the last stage, and if all other earlier filters were disallowed, a 3 tap filter is considered instead.

In the second and third methods, instead of computing the local neighborhood mean, the minimum and maximum values in the neighborhood are computed instead and their distance evaluated for the filtering selection process. The two methods though differ in implementation with the first performing analysis on the image data for every possible filter and performing filtering only when the appropriate conditions are satisfied, while the second one performs an analysis only once using the longest available filter. This analysis permits the second method to immediately select and use the appropriate length filter without having to process the same data multiple times.

It should be noted that these schemes could be extended to account for the characteristics of all color components that correspond to a pixel, as well as consider diagonal and/or 2D filtering mechanisms if desired. These extensions may be included in future extensions of the software. Obviously, such filtering mechanisms can be applied for both down-sampling and up-sampling, not only for chroma conversion but also for entire image down-scaling and up-scaling.

# Results

We have implemented all of these techniques in the upcoming version of HDRTools. Several sequences that have been used for the original CfE process were evaluated in terms of conversion speed as well as a variety of distortion metrics. It should be noted that for simplicity of our measurements, no cropping of the sequences ShowGirl, Magic Hour, and Warm Night was performed (given the intent of this work, we believe that any impact would be of no consequence). If measurements with the cropping window are though desired these could be provided. In these initial results, the same upsampling filters, as used for the Anchor generation process were used. An implementation that would also support adaptive upsampling filters would also be soon provided.

Table 1. Basic conversion results (CfE Anchors)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 57.069 | 64.509 | 53.071 | 56.155 | 58.733 | 64.376 | 53.392 | 56.778 | 54.855 | 48.421 | 55.05 | 126.909 |
| Tibul | 53.761 | 62.882 | 52.225 | 54.453 | 55.158 | 60.007 | 52.677 | 55.007 | 53.982 | 45.614 | 49.462 | 151.186 |
| Market3 | 49.253 | 58.645 | 45.978 | 48.918 | 53.475 | 62.051 | 46.238 | 50.163 | 49.751 | 36.767 | 46.848 | 253.833 |
| AutoWelding | 45.363 | 53.435 | 40.763 | 44.064 | 48.196 | 58.62 | 40.967 | 44.909 | 42.573 | 39.686 | 52.333 | 270.203 |
| BikeSparklers | 43.389 | 50.782 | 35.385 | 39.346 | 45.922 | 58.474 | 35.76 | 40.092 | 38.285 | 34.069 | 48.337 | 302.799 |
| ShowGirl | 49.756 | 57.221 | 44.669 | 48.08 | 52.04 | 62.225 | 44.834 | 48.767 | 47.653 | 39.357 | 50.957 | 207.457 |
| MagicHour | 52.004 | 57.396 | 39.785 | 44.228 | 51.87 | 63.761 | 39.951 | 44.434 | 43.119 | 37.419 | 51.504 | 227.685 |
| WarmNight | 50.437 | 57.737 | 39.985 | 44.314 | 52.762 | 62.637 | 40.293 | 44.8 | 43.672 | 38.101 | 51.494 | 231.019 |
| BalloonFestival | 50.1 | 57.822 | 45.015 | 48.443 | 52.784 | 62.062 | 45.614 | 49.541 | 51.911 | 40.623 | 46.98 | 152.587 |
| Balloon | 54.751 | 63.213 | 51.018 | 54.075 | 58.598 | 67.757 | 51.337 | 55.278 | 54.619 | 37.874 | 47.7 | 126.16 |
| Average | 50.588 | 58.364 | 44.789 | 48.208 | 52.954 | 62.197 | 45.106 | 48.977 | 48.042 | 39.793 | 50.067 | 204.984 |

Table 2. Luma Micrograding (CE1 anchors) results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 60.86 | 59.463 | 51.976 | 55.574 | 63.184 | 70.214 | 52.23 | 56.601 | 55.014 | 49.155 | 58.381 | 755.192 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.259 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 | 1096.152 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 | 1518.395 |
| AutoWelding | 46.691 | 50.456 | 40.518 | 43.996 | 50.23 | 69.826 | 40.718 | 45.006 | 42.774 | 39.808 | 55.679 | 1525.742 |
| BikeSparklers | 43.974 | 49.365 | 35.232 | 39.255 | 46.915 | 69.757 | 35.609 | 40.055 | 38.301 | 34.084 | 51.927 | 1720.768 |
| ShowGirl | 50.359 | 55.131 | 44.751 | 48.145 | 53.316 | 66.861 | 44.918 | 49.051 | 48.011 | 39.411 | 53.01 | 1242.295 |
| MagicHour | 52.876 | 55.355 | 39.842 | 44.285 | 53.222 | 70.642 | 40.017 | 44.58 | 43.289 | 37.446 | 53.887 | 1280.69 |
| WarmNight | 51.901 | 55.315 | 39.756 | 44.155 | 54.553 | 70.771 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 | 1223.717 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.594 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 | 939.313 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 | 690.876 |
| Average | 52.201 | 55.411 | 44.260 | 47.930 | 55.200 | 69.715 | 44.549 | 48.902 | 48.088 | 39.995 | 52.417 | 1199.314 |

Table 3. LUT (10M elements) + Bounded Luma Micrograding results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 60.859 | 59.463 | 51.976 | 55.574 | 63.184 | 70.213 | 52.23 | 56.601 | 55.013 | 49.155 | 58.381 | 219.778 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.258 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 | 272.738 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 | 530.32 |
| AutoWelding | 46.678 | 50.428 | 40.521 | 43.99 | 50.213 | 69.772 | 40.718 | 45 | 42.767 | 39.808 | 55.675 | 540.27 |
| BikeSparklers | 43.969 | 49.355 | 35.231 | 39.252 | 46.91 | 69.746 | 35.608 | 40.052 | 38.299 | 34.085 | 51.925 | 649.365 |
| ShowGirl | 50.357 | 55.143 | 44.746 | 48.141 | 53.304 | 66.833 | 44.912 | 49.044 | 48.002 | 39.411 | 53.01 | 412.404 |
| MagicHour | 52.875 | 55.355 | 39.842 | 44.285 | 53.222 | 70.641 | 40.017 | 44.58 | 43.289 | 37.446 | 53.887 | 443.494 |
| WarmNight | 51.9 | 55.316 | 39.756 | 44.155 | 54.552 | 70.767 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 | 433.632 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.593 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 | 290.242 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 | 232.538 |
| Average | 52.199 | 55.408 | 44.260 | 47.928 | 55.196 | 69.704 | 44.548 | 48.901 | 48.086 | 39.995 | 52.417 | 402.478 |

Table 4. LUT + Bounded Luma Micrograding + Adaptive Downsampler (Multipass)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 61.701 | 60.302 | 52.627 | 56.268 | 63.965 | 70.211 | 52.877 | 57.248 | 55.513 | 49.495 | 58.38 | 471.295 |
| Tibul | 59.852 | 56.237 | 50.223 | 53.654 | 62.095 | 70.256 | 50.509 | 54.936 | 54.027 | 46.865 | 53.069 | 568.878 |
| Market3 | 50.467 | 56.017 | 46.156 | 49.242 | 56.061 | 69.613 | 46.412 | 50.717 | 49.833 | 36.862 | 47.892 | 1106.063 |
| AutoWelding | 47.054 | 50.751 | 40.863 | 44.336 | 50.586 | 69.789 | 41.061 | 45.345 | 42.685 | 39.797 | 55.676 | 1116.77 |
| BikeSparklers | 44.329 | 49.689 | 35.565 | 39.589 | 47.26 | 69.746 | 35.946 | 40.39 | 38.208 | 34.058 | 51.926 | 1311.007 |
| ShowGirl | 50.732 | 55.443 | 45.078 | 48.478 | 53.672 | 67.115 | 45.245 | 49.381 | 47.939 | 39.467 | 53.01 | 866.645 |
| MagicHour | 53.346 | 55.745 | 40.181 | 44.631 | 53.597 | 70.65 | 40.356 | 44.92 | 43.251 | 37.452 | 53.887 | 889.814 |
| WarmNight | 52.327 | 55.741 | 40.099 | 44.505 | 54.96 | 70.791 | 40.399 | 45.016 | 43.542 | 38.162 | 54.147 | 893.151 |
| BalloonFestival | 51.172 | 56.656 | 44.955 | 48.565 | 54.266 | 69.601 | 45.526 | 49.738 | 51.5 | 40.64 | 48.243 | 585.999 |
| Balloon | 56.034 | 62.389 | 51.202 | 54.496 | 60.372 | 69.609 | 51.507 | 55.688 | 55.024 | 38.402 | 47.936 | 505.602 |
| Average | 52.701 | 55.897 | 44.695 | 48.376 | 55.683 | 69.738 | 44.984 | 49.338 | 48.152 | 40.120 | 52.417 | 831.522 |

Table 5. LUT + Bounded Luma Micrograding + Adaptive Downsampler (Neighborhood)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 61.606 | 60.233 | 52.552 | 56.193 | 63.881 | 70.211 | 52.8 | 57.171 | 55.413 | 49.467 | 58.38 | 278.484 |
| Tibul | 59.835 | 56.217 | 50.199 | 53.631 | 62.079 | 70.256 | 50.486 | 54.914 | 54.022 | 46.868 | 53.069 | 345.027 |
| Market3 | 50.415 | 55.962 | 46.105 | 49.191 | 56.018 | 69.613 | 46.361 | 50.668 | 49.812 | 36.856 | 47.892 | 697.627 |
| AutoWelding | 47.015 | 50.708 | 40.826 | 44.299 | 50.548 | 69.786 | 41.025 | 45.308 | 42.653 | 39.781 | 55.676 | 686.111 |
| BikeSparklers | 44.301 | 49.651 | 35.53 | 39.555 | 47.232 | 69.746 | 35.915 | 40.36 | 38.189 | 34.05 | 51.926 | 827.036 |
| ShowGirl | 50.706 | 55.416 | 45.053 | 48.453 | 53.646 | 67.12 | 45.22 | 49.356 | 47.921 | 39.457 | 53.01 | 531.365 |
| MagicHour | 53.327 | 55.724 | 40.16 | 44.61 | 53.578 | 70.649 | 40.335 | 44.899 | 43.232 | 37.442 | 53.887 | 564.14 |
| WarmNight | 52.308 | 55.718 | 40.084 | 44.489 | 54.942 | 70.79 | 40.384 | 45.001 | 43.53 | 38.154 | 54.147 | 565.65 |
| BalloonFestival | 51.096 | 56.56 | 44.834 | 48.453 | 54.173 | 69.599 | 45.401 | 49.617 | 51.506 | 40.637 | 48.242 | 361.369 |
| Balloon | 55.989 | 62.349 | 51.162 | 54.455 | 60.338 | 69.609 | 51.466 | 55.649 | 55.014 | 38.392 | 47.936 | 299.995 |
| Average | 52.660 | 55.854 | 44.651 | 48.333 | 55.644 | 69.738 | 44.939 | 49.294 | 48.129 | 40.110 | 52.417 | 515.680 |

# Conclusion

Adaptive filtering mechanisms for chroma format conversion are presented in this document. It is suggested that these mechanisms could be further studied in terms of their subjective and objective performance compared to the fixed filters currently used for the JCTVC HDR/WCG activity.

# References

- J. Strom, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson's response to CfE for HDR and WCG”, MPEG document M36184, Warsaw, PL, Jun. 2015

- J. Strom, J. Sole, Y. He, “Report on HDR CE1”, JCTVC document JCTVC-W0021, Vancouver, CA, Jan. 2016

- SMPTE ST 2084:2014, “High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays”, 2014

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

This assumes a chroma location type 2, as dictated by the BT.2020 specification.

Extending the implementation to support only a partial column or row, with or without overlapping, could be added if there is strong interest in such methods. However, higher priority was placed on the edge adaptive/colour variation methods instead.

The current implementation only supports these filters using floating point precision filtering. We actually see no reason, apart from complexity reasons, why that option should not always be enabled, even for the basic filters used currently for anchor generation, when performing such conversions.