| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0052 |
| --- | --- |

| Title: | Enhanced Luma Adjustment Methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Yeping Su, David Singer Apple, Inc. 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple, Inc. |  |  |

_____________________________

# Abstract

Considerable improvements are introduced to the Luma Adjustment method, which is currently used for the creation of the Anchor Video material in the JCT-VC HDR/WCG video standardization development process. These improvements result in considerable speedups in the conversion process, with little if any impact in quality. Alternative implementations, that try to emphasize other video quality aspects and not only luminance, are also presented.

# Introduction

In contribution M36184 [1] it was demonstrated that the conversion quality from a 4:4:4 RGB floating point representation to a 10 bit YCbCr Non-constant luminance, 4:2:0 representation could be substantially improved if color conversion and downsampling/upsampling are jointly considered. More specifically, instead of computing Y’ (luma) and the chroma (Cb and Cr) components independently, an optimization process was used where Y’ was computed iteratively, while accounting and essentially correcting for the distortion introduced by the downsampling, quantization, and upsampling processes. The optimization process basically involved finding the Y’ value that, given the reconstructed Cb and Cr components, would result in the least possible distortion to the Luminance (Y) component.

This process is currently being used for the HDR anchor generation process described in [7] given its ability of creating better baseline video signals for compression, however it has been criticized for its slow processing speed. In this contribution, improvements upon this method that result in considerable speed improvements or provide alternative optimization targets are presented. It is suggested that the group considers these alternatives for any future anchor generation process or other development since they can considerably improve processing speed with essentially no impact in processing quality. The alternative models could also provide insights and help developers in further optimizing their processing methods if other targets than luminance are considered more important for optimization.

# Luma Adjustment Method

To better understand how the luma adjustment process works, we first refer to the Non-Constant Luminance (NCL) conversion process, i.e. from R’G’B’ to YCbCr. This can be expressed using a 3x3 matrix conversion of the form:

(1)

while the Cb and Cr components could also be expressed in relationship to Y’ in the following manner:

with  							(2)

with 								(3)

The various constants used in these equations also have the following relationships:

(4)

(5)

(6)

(7)

(8)

(9)

In [1], the conversion process for Cb and Cr still utilizes the matrix form, i.e. Cb and Cr are first computed as:

(10)

(11)

followed, however, by quantization and downscaling (QD) and subsequently upscaling and inverse quantization (IQU) (these steps could be performed in any order) as:

(12)

(13)

On the other hand, given the above formulations of luma and chroma, one can then derive the following formulations:

(14)

(15)

and

(16)

Equivalently, if we assume that = = =Y’, we can then compute that the reconstructed R’, G’, and B’ components would be of the following form:

(17)

(18)

(19)

while their respective linear domain values would then be equal to:

(20)

(21)

(22)

Where iTF is the inverse of the transfer function that was applied on the R, G, and B data to generate the luma information. Luminance (Y), unlike luma (Y’), is computed given the linear domain values using a formulation of the form:

(23)

Given the values for Brec, Rrec, and Grec we can then compute Yrec as:

(24)

The intent of the luma adjustment method is to try and locate the value of Y’ that essentially results in the smallest possible distortion for Yrec compared essentially to its original value Y. Unfortunately, due to the non-linear characteristics of the transfer function, solving for Y’ given Y and the values of Crfactor, Gfactor, and Cbfactor is not a straightforward process. However, root-finding numerical methods, such as the bisection method, can be used instead as presented in [1]. The performance, and more specifically the convergence speed and accuracy of these methods though is considerably impacted by the selection of the initial interval as well as the computations performed during the search.

In [1], as well as in its refinements and extensions [2][3][4], the initial interval is computed as assuming a direct relationship between the values of the luminance and luma signals. More specifically, given the earlier formulations for reconstructed R’, G’, and B’ and the transformed luminance signal TF(Y), bounds for R’, G’, and B’ are computed as follows:

(25)

(26)

(27)

Then the minimum and maximum bounds for Y’ can be computed as:

(28)

(29)

where Q is the quantization process used to convert the data from floating precision to fixed precision.

Unfortunately, the above bounds, although could provide some improvement versus searching the entire Y space (especially for higher bit-depths), still resulted in considerable computation spent on the optimization process, especially compared to all other conversion steps used for the creation of the anchor video input signal. Reducing such complexity is highly desirable.

# Improved Bound Computation

We can observe that equations 14-16 can already provide with an initial estimate for Y’, by essentially accounting for the distortion introduced in the chroma components by the down-sampling, up-sampling, and quantization processes, as well as the characteristics of the original R’, G’, and B’ components. These estimates were actually used in the closed loop optimization process that was described in [5], resulting also with an improved signal generation versus the naïve YCbCr conversion method.  These values could be utilized in improving the luma adjustment bounds as well.

In particular, we have found that the optimal value of Y’ is actually within these three values, allowing us to further limit the bounds defined above as follows:

(30)

(31)

This change in boundary values can result in speed improvements of over 50%, with almost no change in performance over the original computation. Very minor differences can sometimes though occur since the relationship between Y’ and Y is unfortunately not always monotonic, therefore impacting the behavior of the bisection method. These though are considered negligible.

# Use of LUTs for the Transfer Function Computation

The original implementation of [1] considered a look up table (LUT) when applying the transfer function and its inverse during the bisection method. This was done to speed up the entire computation method, given especially the complexity of the ST.2084 [8] transfer function. Unfortunately, that implementation was also quite limited since, among other things, it could only support a limited number of color spaces and transfer function.

Given the speed advantages of LUTs a new implementation is introduced in HDRTools that avoids the limitations of the original implementation. The selected transfer function, which can be different than ST.2084, is considered to populate the elements of the LUT during the initialization phase of the process. Then, during the bisection search, the LUT, in combination with bilinear interpolation, instead of the direct computation is used. The size of the LUT, which currently is set to 10,000,000 elements, is fully controllable through the code allowing one to implement this method using different memory/accuracy trade-offs. The use of LUTs can provide us with an overall software speed improvement of ~2x, in addition to the bound improvements introduced in the previous section.

It should be noted that the use of LUTs could also be considered for use in all other stages where transfer functions are used, potentially resulting in further speed improvements to the HDRTools software. However, this is likely considered as of lower priority and such scheme has not yet been implemented. We though intend to do so in the future.

# Luma Adjustment Method Alternatives

The primary goal of the luma adjustment method was to select a luma signal that minimizes the distortion to the reconstructed luminance signal. However, that minimization also tends to result in higher distortion allowed in other components, such as color. It could be argued that although maintaining luminance is important, maintaining color or information in another domain may be also as important. Therefore, we have also introduced several additional alternatives to the original Luma Adjustment method, where, instead of optimizing the distortion in luminance, distortion in other domains or distortion of not only luminance but also of other components is considered.

In particular, a very straightforward extension of the luma adjustment method involves optimizing not for luminance but for R, G, and B component distortion, or respectively R’, G’, and B’. An advantage of this method is that such consideration could be done without having to perform the rather expensive inverse transfer function computations required for the original method. Computation can be performed using equations 17, 18, and 19, and basically computing distortion as:

(32)

where wR, wG, and wB are weighting factors that can control the importance of a particular component versus the other components, and D is a distortion function, such as absolute difference, square error, etc. The value of Y’ that results in the least distortion as formulated above can be selected as the final Y’ for the signal. For this search and given its dependency on three values, a full search method, using the  same boundary values as described in Section 3, is used. The minimum in this scenario could also be computed by averaging the results from equations 14, 15, and 16. The logic behind this averaging is that the minimum could be found at the location where the derivative of the above equation is 0. This method is also inspired by the work performed in contribution JCTVC-W0107 [9], although with the difference that this is computed for Y’ and not Y. Thus, unlike in [9], no computation of the transfer function or its inverse needs to be performed. Assuming that we have selected the square error as the distortion method the derivation of this formulation is as follows:

(33)

𝐷=*+**			=

then the derivative of D(Y’) will be equal to:

while the minimum can then be found when D’=0, in which case:

(34)

and when

(35)

In an alternative, instead of optimizing quality only for Y, we consider impact also on the X and Z components in the XYZ domain. In particular, the optimization target would be instead of Y, the following overall distortion value.

The 3 defined weights allow control of the importance of each component separately. It should be obvious that if wY=1 and wG=wB=0, the method would be the exact equivalent of the original luma adjustment method. Distortion computation in other color domains, or even in addition to the XYZ distortion, such as LMS, HSV, IPT etc, could also be considered if desired.

# Results

We have implemented all of these techniques in the upcoming version of HDRTools. Several sequences that have been used for the original CfE process were evaluated in terms of conversion speed as well as a variety of distortion metrics. It should be noted that for simplicity of our measurements, no cropping of the sequences ShowGirl, Magic Hour, and Warm Night was performed (given the intent of this work, we believe that any impact would be of no consequence). If measurements with the cropping window are though desired these could be provided. Furthermore, the same downsampling and upsampling filters, as used for the Anchor generation process were used.

Table 1. Basic conversion results (CfE Anchors)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 57.069 | 64.509 | 53.071 | 56.155 | 58.733 | 64.376 | 53.392 | 56.778 | 54.855 | 48.421 | 55.05 | 126.909 |
| Tibul | 53.761 | 62.882 | 52.225 | 54.453 | 55.158 | 60.007 | 52.677 | 55.007 | 53.982 | 45.614 | 49.462 | 151.186 |
| Market3 | 49.253 | 58.645 | 45.978 | 48.918 | 53.475 | 62.051 | 46.238 | 50.163 | 49.751 | 36.767 | 46.848 | 253.833 |
| AutoWelding | 45.363 | 53.435 | 40.763 | 44.064 | 48.196 | 58.62 | 40.967 | 44.909 | 42.573 | 39.686 | 52.333 | 270.203 |
| BikeSparklers | 43.389 | 50.782 | 35.385 | 39.346 | 45.922 | 58.474 | 35.76 | 40.092 | 38.285 | 34.069 | 48.337 | 302.799 |
| ShowGirl | 49.756 | 57.221 | 44.669 | 48.08 | 52.04 | 62.225 | 44.834 | 48.767 | 47.653 | 39.357 | 50.957 | 207.457 |
| MagicHour | 52.004 | 57.396 | 39.785 | 44.228 | 51.87 | 63.761 | 39.951 | 44.434 | 43.119 | 37.419 | 51.504 | 227.685 |
| WarmNight | 50.437 | 57.737 | 39.985 | 44.314 | 52.762 | 62.637 | 40.293 | 44.8 | 43.672 | 38.101 | 51.494 | 231.019 |
| BalloonFestival | 50.1 | 57.822 | 45.015 | 48.443 | 52.784 | 62.062 | 45.614 | 49.541 | 51.911 | 40.623 | 46.98 | 152.587 |
| Balloon | 54.751 | 63.213 | 51.018 | 54.075 | 58.598 | 67.757 | 51.337 | 55.278 | 54.619 | 37.874 | 47.7 | 126.16 |
| Average | 50.588 | 58.364 | 44.789 | 48.208 | 52.954 | 62.197 | 45.106 | 48.977 | 48.042 | 39.793 | 50.067 | 204.984 |

Table 2. Luma Micrograding (CE1 anchors) results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 60.86 | 59.463 | 51.976 | 55.574 | 63.184 | 70.214 | 52.23 | 56.601 | 55.014 | 49.155 | 58.381 | 755.192 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.259 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 | 1096.152 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 | 1518.395 |
| AutoWelding | 46.691 | 50.456 | 40.518 | 43.996 | 50.23 | 69.826 | 40.718 | 45.006 | 42.774 | 39.808 | 55.679 | 1525.742 |
| BikeSparklers | 43.974 | 49.365 | 35.232 | 39.255 | 46.915 | 69.757 | 35.609 | 40.055 | 38.301 | 34.084 | 51.927 | 1720.768 |
| ShowGirl | 50.359 | 55.131 | 44.751 | 48.145 | 53.316 | 66.861 | 44.918 | 49.051 | 48.011 | 39.411 | 53.01 | 1242.295 |
| MagicHour | 52.876 | 55.355 | 39.842 | 44.285 | 53.222 | 70.642 | 40.017 | 44.58 | 43.289 | 37.446 | 53.887 | 1280.69 |
| WarmNight | 51.901 | 55.315 | 39.756 | 44.155 | 54.553 | 70.771 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 | 1223.717 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.594 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 | 939.313 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 | 690.876 |
| Average | 52.201 | 55.411 | 44.260 | 47.930 | 55.200 | 69.715 | 44.549 | 48.902 | 48.088 | 39.995 | 52.417 | 1199.314 |

Table 3. Bounded Luma Micrograding results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 60.86 | 59.463 | 51.976 | 55.574 | 63.184 | 70.214 | 52.23 | 56.601 | 55.014 | 49.155 | 58.381 | 481.001 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.258 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 | 573.112 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 | 1037.71 |
| AutoWelding | 46.686 | 50.456 | 40.518 | 43.994 | 50.226 | 69.821 | 40.717 | 45.005 | 42.77 | 39.807 | 55.678 | 1235.938 |
| BikeSparklers | 43.973 | 49.365 | 35.232 | 39.255 | 46.915 | 69.757 | 35.609 | 40.054 | 38.301 | 34.084 | 51.927 | 1446.714 |
| ShowGirl | 50.359 | 55.131 | 44.746 | 48.141 | 53.308 | 66.839 | 44.913 | 49.045 | 48.003 | 39.411 | 53.01 | 856.151 |
| MagicHour | 52.875 | 55.355 | 39.842 | 44.285 | 53.222 | 70.641 | 40.017 | 44.58 | 43.289 | 37.446 | 53.887 | 861.812 |
| WarmNight | 51.9 | 55.315 | 39.756 | 44.155 | 54.552 | 70.767 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 | 852.188 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.592 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 | 553.964 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 | 475.582 |
| Average | 52.200 | 55.411 | 44.260 | 47.929 | 55.198 | 69.711 | 44.548 | 48.902 | 48.087 | 39.995 | 52.417 | 837.417 |

Table 4. LUT (10M elements) + Bounded Luma Micrograding results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 60.859 | 59.463 | 51.976 | 55.574 | 63.184 | 70.213 | 52.23 | 56.601 | 55.013 | 49.155 | 58.381 | 219.778 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.258 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 | 272.738 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 | 530.32 |
| AutoWelding | 46.678 | 50.428 | 40.521 | 43.99 | 50.213 | 69.772 | 40.718 | 45 | 42.767 | 39.808 | 55.675 | 540.27 |
| BikeSparklers | 43.969 | 49.355 | 35.231 | 39.252 | 46.91 | 69.746 | 35.608 | 40.052 | 38.299 | 34.085 | 51.925 | 649.365 |
| ShowGirl | 50.357 | 55.143 | 44.746 | 48.141 | 53.304 | 66.833 | 44.912 | 49.044 | 48.002 | 39.411 | 53.01 | 412.404 |
| MagicHour | 52.875 | 55.355 | 39.842 | 44.285 | 53.222 | 70.641 | 40.017 | 44.58 | 43.289 | 37.446 | 53.887 | 443.494 |
| WarmNight | 51.9 | 55.316 | 39.756 | 44.155 | 54.552 | 70.767 | 40.056 | 44.671 | 43.585 | 38.145 | 54.147 | 433.632 |
| BalloonFestival | 50.649 | 56.034 | 44.503 | 48.091 | 53.726 | 69.593 | 45.062 | 49.266 | 51.725 | 40.679 | 48.243 | 290.242 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 | 232.538 |
| Average | 52.199 | 55.408 | 44.260 | 47.928 | 55.196 | 69.704 | 44.548 | 48.901 | 48.086 | 39.995 | 52.417 | 402.478 |

Table 5. LUT (1048576 elements) + Bounded Luma Micrograding results

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 59.819 | 58.464 | 51.694 | 55.092 | 61.584 | 65.129 | 51.931 | 56.028 | 54.406 | 49.146 | 58.239 | 214.683 |
| Tibul | 59.308 | 55.571 | 49.707 | 53.108 | 61.577 | 70.257 | 49.999 | 54.433 | 53.726 | 46.557 | 53.072 | 260.947 |
| Market3 | 50.034 | 55.626 | 45.704 | 48.8 | 55.601 | 69.612 | 45.961 | 50.267 | 49.886 | 36.788 | 47.892 | 464.773 |
| AutoWelding | 46.401 | 49.922 | 40.361 | 43.746 | 49.797 | 67.597 | 40.546 | 44.728 | 42.468 | 39.798 | 55.477 | 501.088 |
| BikeSparklers | 43.864 | 49.185 | 35.174 | 39.168 | 46.774 | 69.486 | 35.548 | 39.962 | 38.21 | 34.083 | 51.865 | 580.004 |
| ShowGirl | 50.275 | 55.138 | 44.704 | 48.095 | 53.148 | 65.972 | 44.868 | 48.973 | 47.916 | 39.409 | 52.976 | 374.384 |
| MagicHour | 52.793 | 55.119 | 39.84 | 44.273 | 53.163 | 66.936 | 40.015 | 44.57 | 43.258 | 37.447 | 53.867 | 395.106 |
| WarmNight | 51.811 | 54.786 | 39.746 | 44.127 | 54.371 | 64.481 | 40.045 | 44.643 | 43.501 | 38.146 | 54.104 | 395.82 |
| BalloonFestival | 50.648 | 56.033 | 44.503 | 48.09 | 53.725 | 69.567 | 45.062 | 49.266 | 51.724 | 40.679 | 48.243 | 260.628 |
| Balloon | 55.357 | 61.793 | 50.614 | 53.886 | 59.671 | 69.609 | 50.918 | 55.094 | 54.571 | 37.879 | 47.936 | 218.188 |
| Average | 52.031 | 55.164 | 44.205 | 47.839 | 54.941 | 67.865 | 44.489 | 48.796 | 47.967 | 39.993 | 52.367 | 366.562 |

Table 6. RGB based Luma Micrograding results (Search)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 56.498 | 60.043 | 55.457 | 56.927 | 58.143 | 61.347 | 55.904 | 57.924 | 56.274 | 48.282 | 53.652 | 168.129 |
| Tibul | 52.888 | 62.077 | 53.706 | 54.734 | 54.185 | 58.114 | 54.227 | 55.159 | 54.449 | 45.35 | 48.552 | 202.630 |
| Market3 | 48.263 | 54.827 | 48.04 | 49.468 | 52.58 | 56.976 | 48.373 | 51.333 | 51.057 | 36.744 | 43.807 | 342.381 |
| AutoWelding | 45.103 | 47.261 | 43.49 | 45.016 | 48.858 | 50.821 | 43.806 | 46.772 | 44.541 | 39.655 | 47.671 | 376.151 |
| BikeSparklers | 41.821 | 42.767 | 38.291 | 40.466 | 45.836 | 45.368 | 38.847 | 42.055 | 40.321 | 33.985 | 41.273 | 437.909 |
| ShowGirl | 49.991 | 50.649 | 47.463 | 49.118 | 54.226 | 54.647 | 47.727 | 50.894 | 49.817 | 39.321 | 46.669 | 282.867 |
| MagicHour | 48.563 | 47.381 | 42.899 | 45.55 | 53.313 | 50.261 | 43.189 | 46.831 | 45.476 | 37.374 | 44.097 | 310.524 |
| WarmNight | 47.24 | 48.224 | 42.901 | 45.466 | 50.304 | 50.009 | 43.378 | 46.622 | 45.47 | 37.918 | 44.166 | 310.210 |
| BalloonFestival | 49.892 | 51.35 | 47.859 | 49.463 | 53.036 | 53.57 | 48.66 | 51.158 | 53.412 | 40.433 | 43.798 | 202.405 |
| Balloon | 53.786 | 59.152 | 53.211 | 54.697 | 57.51 | 60.877 | 53.621 | 56.36 | 55.977 | 37.85 | 45.276 | 169.008 |
| Average | 49.405 | 52.373 | 47.332 | 49.091 | 52.799 | 54.199 | 47.773 | 50.511 | 49.679 | 39.691 | 45.896 | 280.221 |

Table 7. RGB based Luma Micrograding results (Average)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 56.498 | 60.043 | 55.457 | 56.927 | 58.143 | 61.347 | 55.904 | 57.924 | 56.274 | 48.282 | 53.652 | 157.375 |
| Tibul | 52.888 | 62.077 | 53.706 | 54.734 | 54.185 | 58.114 | 54.227 | 55.159 | 54.449 | 45.35 | 48.552 | 188.301 |
| Market3 | 48.263 | 54.827 | 48.04 | 49.468 | 52.58 | 56.976 | 48.373 | 51.333 | 51.057 | 36.744 | 43.807 | 313.674 |
| AutoWelding | 45.103 | 47.261 | 43.49 | 45.016 | 48.858 | 50.821 | 43.806 | 46.772 | 44.541 | 39.655 | 47.671 | 334.282 |
| BikeSparklers | 41.821 | 42.767 | 38.291 | 40.466 | 45.836 | 45.368 | 38.847 | 42.055 | 40.321 | 33.985 | 41.273 | 377.124 |
| ShowGirl | 49.991 | 50.649 | 47.463 | 49.118 | 54.226 | 54.647 | 47.727 | 50.894 | 49.817 | 39.321 | 46.669 | 257.813 |
| MagicHour | 48.563 | 47.381 | 42.899 | 45.55 | 53.313 | 50.261 | 43.189 | 46.831 | 45.476 | 37.374 | 44.097 | 283.278 |
| WarmNight | 47.24 | 48.224 | 42.901 | 45.466 | 50.304 | 50.009 | 43.378 | 46.622 | 45.47 | 37.918 | 44.166 | 283.805 |
| BalloonFestival | 49.892 | 51.35 | 47.859 | 49.463 | 53.036 | 53.57 | 48.66 | 51.158 | 53.412 | 40.433 | 43.798 | 188.698 |
| Balloon | 53.786 | 59.152 | 53.211 | 54.697 | 57.51 | 60.877 | 53.621 | 56.36 | 55.977 | 37.85 | 45.276 | 156.989 |
| Average | 49.405 | 52.373 | 47.332 | 49.091 | 52.799 | 54.199 | 47.773 | 50.511 | 49.679 | 39.691 | 45.896 | 254.134 |

Table 8. XYZ (3/16/1) based Luma Micrograding results (No LUTs)

|  | tPSNR (dB) |  |  |  |  |  |  |  |  | DE2000 (dB) |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | R | G | B | RGB | X | Y | Z | XYZ | oXYZ | DE0100 | L0100 | (sec) |
| FireEater | 61.068 | 59.12 | 52.318 | 55.804 | 63.561 | 69.532 | 52.584 | 56.94 | 55.388 | 49.169 | 57.983 | 822.947 |
| Tibul | 59.097 | 55.661 | 49.895 | 53.244 | 61.334 | 69.851 | 50.19 | 54.589 | 53.902 | 46.525 | 52.917 | 1054.196 |
| Market3 | 50.144 | 54.993 | 45.946 | 48.942 | 56.191 | 67.518 | 46.209 | 50.536 | 50.258 | 36.788 | 47.322 | 2082.321 |
| AutoWelding | 47.124 | 49.328 | 40.953 | 44.29 | 51.401 | 62.732 | 41.169 | 45.501 | 43.324 | 39.822 | 53.224 | 3379.322 |
| BikeSparklers | 44.401 | 47.719 | 35.651 | 39.587 | 48.24 | 59.101 | 36.051 | 40.533 | 38.848 | 34.08 | 47.677 | 4379.975 |
| ShowGirl | 50.866 | 53.541 | 45.294 | 48.491 | 54.661 | 63.412 | 45.477 | 49.668 | 48.678 | 39.418 | 51.129 | 2027.342 |
| MagicHour | 52.994 | 52.877 | 40.413 | 44.72 | 55.555 | 62.386 | 40.608 | 45.214 | 43.946 | 37.452 | 50.041 | 2291.607 |
| WarmNight | 51.999 | 53.653 | 40.222 | 44.531 | 55.946 | 63.55 | 40.544 | 45.17 | 44.101 | 38.127 | 50.837 | 2336.113 |
| BalloonFestival | 51.361 | 54.687 | 44.976 | 48.486 | 55.157 | 65.259 | 45.561 | 49.838 | 52.307 | 40.654 | 47.274 | 1020.933 |
| Balloon | 55.638 | 60.882 | 50.895 | 54.094 | 60.413 | 68.583 | 51.207 | 55.416 | 55.034 | 37.877 | 47.642 | 770.683 |
| Average | 52.469 | 54.246 | 44.656 | 48.219 | 56.246 | 65.192 | 44.960 | 49.341 | 48.579 | 39.991 | 50.605 | 2016.544 |

# Conclusion

This document presented several enhancements that were introduced in HDRTools and relate to the luma adjustment method. The introduced methods can provide considerable speedups to the software with essentially no detrimental impact in quality. Alternative methods that account for optimization for different or multiple color spaces are also introduced.

# References

- J. Strom, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson's response to CfE for HDR and WCG”, MPEG document M36184, Warsaw, PL, Jun. 2015

- J. Strom, M. Pettersson, J. Samuelsson, K. Andersson, “Additional Results for m36184 for Color Correction”, MPEG document M36462, Warsaw, PL, Jun. 2015

- J. Strom, J. Samuelsson, K. Andersson, “Report for CE1.b (Color Format Conversion)”, MPEG document M37272, Geneva, SW, Oct. 2015

- J. Strom, J. Samuelsson, K. Andersson, M. Pettersson, “Luma Adjustment for Hybrid Log Gamma (HLG)	”, MPEG document M37182, Geneva, SW, Oct. 2015

- A.M. Tourapis, Y. Su, D. Singer, J. Sole, D. Rusanovskyy, S. Lee, D. Rusanovskyy, A. Ramasubramonian, M. Karczewicz, C. Fogg, A. Duenas, F. Bossen, “On 4:4:4 to 4:2:0 conversion - Performance of downsampling and upsampling filters, and MinMax and Closed Loop Filtering”, MPEG document M36271, Warsaw, PL, Jun. 2015

- A.M. Tourapis, Y. Su, D. Singer, “HDRTools: A software package for Video Processing applications”, Q6/16 document C886, Geneva, SW, Oct. 2015

- J. Strom, J. Sole, Y. He, “Report on HDR CE1”, JCTVC document JCTVC-W0021, Vancouver, CA, Jan. 2016

- SMPTE ST 2084:2014, “High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays”, 2014

- A. Norkin, “Closed form HDR 4:2:0 chroma subsampling”, JCTVC document JCTVC-W0107, San Diego, CA, USA, Feb. 2016

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

This can be seen as making use of the Least Mean Squares (LMS) algorithm