| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0053 |
| --- | --- |

| Title: | HDRTools: Extensions and Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Yeping Su, David Singer Apple, Inc. 1 Infinite Loop Cupertino, CA 95014 USA Chad Fogg MovieLabs Inc. 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple Inc., Motion Picture Laboratories Inc. |  |  |

_____________________________

# Abstract

This document provides a short report about the various enhancements and extensions that were introduced in the HDRTools package.

# HDRTools Enhancements

The following enhancements were made to the HDRTools package since its last release in [1]:

- Six (6) new variants of the luma adjustment method, including improved boundary and look-up table considerations. Improvements result in considerable speed improvements as well as in the ability to evaluate different optimization target variants.

- Three (3) new adaptive downsampling filters based on parallel optimization or edge/sample variation analysis.

- Initial implementation for NLMeans video denoising.

- Addition of a modified Wiener filter (Wiener2DDark) that accounts for brightness.

- Support of additional transfer functions (i.e. Hybrid PQ).

- Support for Y4M video files (input only currently; output will be added later).

- RAW and AVI format support for three (3) 4:4:4 10 bit video formats, i.e. V410, R210, and R10K. V210 (a 4:2:2 10 bit format) and R210 formats are currently also supported on Blackmagic cards.

- Improved conversion support from YCbCr raw signals to other formats, including formats that use different color primaries, or require additional format conversion steps. For example, conversion from a YCbCr 4:2:0 BT.709 signal to a YCbCr 4:2:0 or 4:2:2 BT.2020 signal is now fully supported in a single pass.

- Additional filter support for 4:4:4 to 4:2:2 conversion.

# References

- A.M. Tourapis, Y. Su, D. Singer, “HDRTools: A software package for Video Processing applications”, Q6/16 document C886, Geneva, SW, Oct. 2015

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).