| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0054 |
| --- | --- |

| Title: | AHG on HDR and WCG: Average Luma Controlled Adaptive dQP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Andrew Segall Jie Zhao Jacob Strom Kenneth Andersson Martin Pettersson |  | asegall@sharplabs.com jzhao@sharplabs.com jacob.strom@ericsson.com kenneth.r.andersson@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Sharp Labs, Ericsson |  |  |

_____________________________

# Abstract

This document describes the average luma-controlled adaptive dQP method that is part the anchor generating process since anchor v3.0, as outlined in JCTVC-W0021/m37605. The main idea behind the method has previously been described partly in m37439, but this document provides implementation details that have previously only been communicated over email and/or as part of the software packages associated with anchors v3.0, v3.1 and v3.2.

# Introduction

VCEG contribution COM16-C1030-E highlights the fact that compressing standard dynamic range (SDR, 0-100 cd/m2) data using SMPTE-ST 2084 together with BT.2020 will produce a different bit distribution compared to compressing the same data using BT.1886 and BT.709. In particular, the ST 2084/BT.2020 combination will allocate relatively more bits to the darker areas than to the brighter areas as compared to the BT.1886/BT.709 combination. Since the BT.1886/BT.709 combination has been used for SDR compression for a long time, it can be assumed that its bit allocation is reasonably good, and that a deviation from this balance would harm perceptual quality. It was also proposed in COM16-C1030-E that the bit allocation of ST2084/BT.2020 could be made more similar to that of BT.1886/BT.709 by measuring the average luma value in a CTU pixel block and adjusting QP appropriately.

This led to work being carried out in CE1 to investigate how perceptual quality could be improved for high dynamic range (HDR).

# Implementation

For every 64x64 CTU, the average luma value, Laverage, is calculated. An integer version intL of this is then calculated using intL = floor(Laverage + 0.5). The QP to use for a particular 64x64 CTU block is then obtained by adding the picture QP with the dQP obtained by using the look-up table shown in Table 1. The source code can be found in this contribution as a patch to HM 16.7 marked with the SHARP_LUMA_DELTA_QP defines. This luma adaptive dQP method is enabled by the following parameter in config file.

LumaDeltaQP                   : 1           # Change luma delta QP based on average luma

| luma intL range | dQP |
| --- | --- |
| intL < 301 | 3 |
| 301 ≤ intL < 367 | 2 |
| 367 ≤ intL < 434 | 1 |
| 434 ≤ intL < 501 | 0 |
| 501 ≤ intL < 567 | -1 |
| 567 ≤ intL < 634 | -2 |
| 634 <= intL < 701 | -3 |
| 701 <= intL < 767 | -4 |
| 767 <= intL < 834 | -5 |
| intL >= 834 | -6 |

Table 1. Look-up table of the dQP value from the average of the luma value. As an example, an average luma value of 503 in a 64x64 block would result in a dQP value of -1.

The data can also be drawn as a diagram, which is shown in Figure 1.

Figure 1. Change in QP value as a function of the average luma value in the 64x64 pixel block.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).