| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0055 |
| --- | --- |

| Title: | HDR CE5: Report of Experiment 5.3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Wei Dai Madhu Krishnan 3097 Cortona Dr., Melbourne, FL 32940 | Tel: Email: | 321-355-7376 {pankajtva,daisy,madhupk83 @ gmail.com} |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This proposal presents two approaches to coding HDR video, by effectively modifying certain components of the anchor HDR video coding processing chain developed in CE1 (essentially an HDR10 compliant system). The following components are modified. The intermediate colour representation is changed from Y’CbCr, to Y’FbFr, a proposed colour space. The resampling filters used for 4:4:4->4:2:0->4:4:4 conversion in the anchor are replaced by a different proposed set of fixed resampling filters. In addition to these changes, the two variants of the proposal, herein called FastVDO_a, and FastVDO_b, arise from the use of two different transfer functions: (a) ST.2084, and (b) a Philips-developed transfer function. It is asserted that both systems perform well, with subjective visual quality equal to or superior to the output of either CE1 (anchor) or CE2 (reshaper). Moreover, by testing with two different transfer functions, it is asserted that the tools proposed meet the criteria of modularity and adaptability, in addition to providing performance value. The developed system constitutes a pre- and post-processing system, which acts outside of an application of the core HEVC Main10 video coding profile. Representative objective results for these systems include: (a) FV_a, Overall results for DE100, MD100, PSNRL100 were +16.5%, +48.8%, +5.8% ; (b) FV_b, Overall results for DE100, MD100, PSNRL100 were +6.1%, -28.3%, +28.6%, respectively. It is asserted (and for which there is appears to be fairly broad agreement) that such objective metrics are not very predictive of visual quality in HDR coding studies. It is furthermore asserted that the proposed coding scheme in fact provides equal or superior visual quality to the output of both CE1 and CE2.

# Introduction

It was the intention of CE5 to investigate the potential coding efficiency available by varying the internal colour representation, as well as the resampling filters, in the processing chain for HDR video coding; see JVT-W0025. It was required to use the ST-2084 transfer function in the processing chain, but a secondary transfer function, developed by Philips (MPEG m36266), was available as an optional second test candidate. FastVDO elected to test both transfer functions. The approach based on employing the Philips transfer function is similar to the method previously reported by FastVDO in the MPEG document m37065, while the approach employing the ST-2084 transfer function is a straightforward modification of the anchor CE1 coding processing chain. The core colour transforms and resampling filters used in both approaches are identical.

# Detailed Description of Methods (a) and (b)

## 2.1. FastVDO_a

A generic HDR coding chain is presented in the figure below, where we abbreviate transfer function as TF, and where an additional a conversion to 10-bit data is suppressed for clarity (the original HDR data is typically 12-bits per sample; that 10-bit conversion step can occur in a number of places in the coding chain). For FastVDO_a, these first three boxes use the ST-2084 TF, a FastVDO-developed Y’FbFr colour transform, and a FastVDO-developed set of resampling filters. Those components will be developed below. For our convenience, we often drop the Y’ notation and use YFbFr. In fact, we may even use Y’ to refer to actual luminance, but in a Standard Dynamic Range context, rather than an HDR one. (Apologies in advance for this appropriation of an existing notation; but we feel that otherwise we will have an explosion of notation on hand.)

TF

ColorTr

444->420

Encoder

InvTF

InvColorTr

420->444

Decoder

RGB_hdr

RGB_hdr

## FastVDO_b

This method uses an approach we call Color Transform Adaptive Method (CTAM).

    - Tone Mapping

The figure below describes the top level tone mapping process for CTAM.

RGB -> YFbFr

Tone Mapping

(m36266)

Divide

(Y)	(Y’)

The linear light HDR RGB signal is converted to HDR YFbFr. The luminance channel (Y) of the converted signal is used by the tone mapping process described in m36266 to generate linear domain SDR luminance (Y’). The multiplication factor calculated from Y and Y’ is used in turn to generate SDR RGB.

The above mentioned process eliminates the need for tone mapping weights (m36266) to be transmitted as metadata and the luminance channel generation is consistent with the color transform to be applied subsequently.

    - SDR RGB 4:4:4 to SDR YFbFr 4:2:0 (Direct Method)

YFbFr 4:2:0

RGB 4:4:4 -> YFbFr 4:4:4

Chroma Downsampling

Color transformation is applied to the SDR RGB 4:4:4 signal from the previous step. Then chroma downsampling is performed to produce a SDR YFbFr 4:2:0 signal. Details on the color transformation and chroma downsampling filters are provided in the next two sections.

# Color Transformations

FastVDO has designed a number of lossless integer color transforms with excellent coding gain, as presented in the figure below. In this proposal, we make use of the floating point implementation of YFbFr4 color transform (herein named YFbFr for short). It is closely matched to the YCbCr color space of BT.709; for example, the calculation of the luminance as a combination of RGB values agrees up to two decimal places. This results, among other things, in a viewable SDR video from the bitstream as a byproduct, without further processing. Of course, our full decoding process returns the signal to RGB, from which any desired representation can be computed for viewing. Nevertheless, the new color transform is preferred for coding and potential integer processing benefits.

The floating point representation for YFbFr:

// RGB BT. 2020 to YFbFr

[

{  0.2109375,   0.7187500,   0.0703125},

{ -0.3750000,   0.5000000,  -0.1250000},

{  0.5000000,   0.000000,   -0.5000000}

]

// YFbFr to RGB BT.2020

[

{  1.000000000,  -1.437500000,   0.500000000 },

{  1.000000000,   0.562500000,   0.000000000 },

{  1.000000000,  -1.437500000,  -1.500000000 }

]

# Chroma Resampling

FastVDO has designed advanced resampling filters (down and up), which are applicable for both spatial and chroma resampling. These filters are presented in JCTVC-L0175, for example. FastVDO has developed a theoretical basis from which filters of any length can be derived.

  - Chroma Downsampling from 4:4:4 to 4:2:2

Horizontal downsampling filter coefficients are list in the following table. Scale used is 32.0

Table 1. Horizontal Downsampling Filter Coefficients

| Phase k | Coefs c1[k] 4:4:4  4:2:2 (p=0) |
| --- | --- |
| -5 | 1.0 |
| -4 | 0.0 |
| -3 | -3.0 |
| -2 | 0.0 |
| -1 | 10.0 |
| 0 | 16.0 |
| 1 | 10.0 |
| 2 | 0.0 |
| 3 | -3.0 |
| 4 | 0.0 |
| 5 | 1.0 |

  - Chroma Downsampling from 4:2:2 to 4:2:0

The vertical downsampling filter is a phase shifted version of the horizontal downsampling filter. Its coefficients are listed in the following table. An example scaling factor used is 512.

Table 2. Vertical DownSampling Filter Coefficients

| Phase k | Coefs c2[k] 4:2:2  4:2:0 (p=0.5) |
| --- | --- |
| -5 | 9.0 |
| -4 | 13.0 |
| -3 | -27.0 |
| -2 | -42.0 |
| -1 | 75.0 |
| 0 | 228.0 |
| 1 | 228.0 |
| 2 | 75.0 |
| 3 | -42.0 |
| 4 | -27.0 |
| 5 | 13.0 |
| 6 | 9.0 |

  - Chroma Upsampling from 4:2:0 to 4:2:2

Two vertical upsampling filters are used to restore samples at odd and even index respectively. For both filters, an example scaling factor used is 256.0. Coefficients  for even samples filter are:

Table 3. Chroma Even Sample UpSampling filter Coefficients

| Phase k | Coefs d1[k] 4:2:0  4:2:2 (p=0) |
| --- | --- |
| -3 | 9.0 |
| -2 | -27.0 |
| -1 | 75.0 |
| 0 | 228.0 |
| 1 | -42.0 |
| 2 | 13.0 |

Coefficients  for odd samples filter are:

Table 4. Chroma Odd Sample UpSampling Filter Coefficients

| Phase k | Coefs d2[k] 4:2:0  4:2:2 (p=0) |
| --- | --- |
| -2 | 13.0 |
| -1 | -42.0 |
| 0 | 228.0 |
| 1 | 75.0 |
| 2 | -27.0 |
| 3 | 9.0 |

  - Chroma Upsampling from 4:2:2 to 4:4:4

Two horizontal upsampling filters are used to restore samples at odd and even index respectively. For both filters, an example scaling used is 256.0. Coefficients for even samples filter are:

Table 5. Chroma Even Sample UpSampling Filter Coefficients

| Phase k | Coefs c1[k] 4:2:2  4:4:4 (p=0) |
| --- | --- |
| 0 | 16.0 |

Coefficients for odd samples filter are:

Table 6. Chroma Odd Sample Upsampling Filter Coefficients

| Phase k | Coefs c1[k] 4:2:2  4:4:4 (p=0) |
| --- | --- |
| -2 | 1.0 |
| -1 | -3.0 |
| 0 | 10.0 |
| 1 | 10.0 |
| 2 | -3.0 |
| 3 | 1.0 |

# Objective Results

The objective results obtained by FastVDO_a and FastVDO_b are enclosed in the spreadsheets, FV_a, and FV_b (for convenience, these tests are also referred by us as FV_PQ and FV_PH). The anchor for these tests is the Anchor v3.2 generated in CE1. The full source code for these methods has been uploaded to the MPEG Repository, under \Explorations\HDR\CE5.

Table 1. Objective Results for FastVDO_a (FV_PQ).

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 9.3% | 5.5% | -14.2% | 1.7% | -2.8% | 13.5% | 85.7% | 5.1% |
|  | Market3Clip4000r2 | 7.4% | 5.4% | -5.1% | 2.0% | 0.2% | -3.0% | -83.3% | 5.2% |
|  | SunRise | 7.5% | 4.6% | -7.3% | 1.0% | -2.3% | 7.3% | 44.7% | 5.2% |
| class B | BikeSparklers cut 1 | 11.2% | 10.0% | -2.3% | 6.2% | 6.4% | 7.2% | -28.8% | 9.5% |
|  | BikeSparklers cut 2 | 10.8% | 9.4% | -5.1% | 5.4% | 5.1% | 7.8% | -43.1% | 9.0% |
|  | GarageExit | 9.8% | 5.6% | -4.9% | 3.0% | 1.7% | 10.8% | 92.6% | 5.4% |
| class C | ShowGirl2Teaser | 6.6% | 3.8% | -0.2% | 3.6% | 1.0% | 11.6% | -8.7% | 3.5% |
| class D | StEM_MagicHour cut 1 | 7.0% | 3.4% | -4.7% | 0.5% | -0.2% | 0.3% | 37.2% | 3.4% |
|  | StEM_MagicHour cut 2 | 5.5% | 3.0% | -5.7% | -0.3% | -0.7% | -4.2% | 28.6% | 3.0% |
|  | StEM_MagicHour cut 3 | 7.2% | 3.3% | -2.4% | 1.6% | 1.8% | 11.7% | 17.2% | 3.4% |
|  | StEM_WarmNight cut 1 | 6.6% | 2.9% | -6.0% | 0.2% | -0.4% | 9.9% | 63.2% | 2.9% |
|  | StEM_WarmNight cut 2 | 7.2% | 4.3% | -7.8% | -0.3% | -0.9% | 1.1% | 61.6% | 4.0% |
| class G | BalloonFestival | 13.7% | 7.0% | 4.9% | 8.2% | 6.7% | 25.7% | 49.8% | 7.0% |
| class H | EBU_04_Hurdles | 18.1% | 7.3% | -3.9% | 5.1% | 4.0% | 89.5% | 174.8% | 7.0% |
|  | EBU_06_Start | 25.1% | 14.0% | 2.7% | 13.1% | 11.9% | 58.2% | 240.7% | 14.1% |
|  | Overall | 10.2% | 6.0% | -4.1% | 3.4% | 2.1% | 16.5% | 48.8% | 5.8% |

Table 2. Objective results for FastVDO_b (FV_PH).

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 107.2% | 79.6% | 200.9% | 119.0% | 162.6% | 161.3% | -27.4% | 40.5% |
|  | Market3Clip4000r2 | 88.5% | 86.2% | 59.9% | 76.9% | 49.6% | -26.7% | 0.0% | 25.2% |
|  | SunRise | 139.7% | 145.4% | 107.3% | 128.9% | 65.0% | -55.3% | -33.0% | 28.6% |
| class B | BikeSparklers cut 1 | 49.0% | 50.7% | 35.0% | 44.7% | 46.9% | 18.2% | -26.7% | 33.5% |
|  | BikeSparklers cut 2 | 40.9% | 43.6% | 28.4% | 37.8% | 39.3% | 9.6% | -29.2% | 31.1% |
|  | GarageExit | 34.6% | 35.7% | 8.9% | 25.0% | 24.9% | -7.1% | -77.3% | 30.6% |
| class C | ShowGirl2Teaser | 46.3% | 42.9% | 61.5% | 50.9% | 36.9% | -2.1% | 0.0% | 9.0% |
| class D | StEM_MagicHour cut 1 | 13.3% | 10.7% | 14.4% | 13.1% | 11.8% | 0.5% | -83.7% | 4.9% |
|  | StEM_MagicHour cut 2 | 15.0% | 11.5% | 12.4% | 12.8% | 12.5% | 2.6% | -37.5% | 4.0% |
|  | StEM_MagicHour cut 3 | 17.8% | 11.6% | 18.0% | 16.3% | 17.9% | 22.5% | -25.8% | 2.6% |
|  | StEM_WarmNight cut 1 | 18.5% | 15.3% | 12.6% | 15.2% | 15.2% | 12.5% | -27.0% | 4.1% |
|  | StEM_WarmNight cut 2 | 15.1% | 11.5% | 13.2% | 13.3% | 15.9% | -0.7% | 0.0% | 1.7% |
| class G | BalloonFestival | 35.9% | 39.5% | 27.0% | 33.2% | 103.9% | 15.3% | 0.0% | 170.0% |
| class H | EBU_04_Hurdles | 32.8% | 34.5% | 6.5% | 21.4% | 2.8% | -47.1% | -56.8% | 12.1% |
|  | EBU_06_Start | 77.6% | 74.1% | 49.1% | 65.7% | 42.6% | -12.4% | 0.0% | 31.8% |
|  | Overall | 48.8% | 46.2% | 43.7% | 44.9% | 43.2% | 6.1% | -28.3% | 28.6% |

# Patent rights declaration(s)

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).