| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0056 |
| --- | --- |

| Title: | CE1-related: LUT-based luma sample adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V. Kolesnikov Building A, The Park Estate 5 Talavera Road Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia |  |  |

_____________________________

# Abstract

The ‘Common test conditions for HDR/WCG video coding experiments’ defines an HDR anchor (at version 3.2) that includes a method known as ‘luma sample adjustment’. This method compensates for a shift in luminance that occurs e.g. when downsampling from 4:4:4 to 4:2:0 using non-constant luminance and YCbCr. The method performs an iterative search to determine each luma sample values, which is asserted to be overly complex for real time implementations. This contribution shows a method for performing ‘luma sample adjustment’ that replaces an iterative search with a LUT-based approximation whereby a second-order model is applied to predict the adjusted luma sample, with the coefficients for the model obtained from the LUT. tPSNR values when no luma sample adjustment is applied are 52.15dB, 62.61dB and 43.31dB in X, Y and Z channels, respectively. The corresponding values for the v3.2 anchor are 53.79dB, 69.77dB and 43.08dB. With the LUT-based method, the values are 53.26dB, 64.44dB and 43.02dB.

# Introduction

When generating Y’CbCr data from R’G’B’ using non-constant luminance and downsampling to 4:2:0, luminance after conversion back to RGB 4:4:4 is distorted compared to the input RGB luminance. This problem is described in [2].

A luma sample adjustment process, described by Ericsson in [3], was proposed to avoid such distortions. Figure 1 shows how this process operates in the context of the encoder-side processing, as used in the MPEG HDR/WCG CfE.

Figure 1. Luma sample adjustment process.

Within the ‘luma adjustment’ block above the following steps are performed:

- Youtput is initialised from Y’initial.

- Compute R’G’B’ from Y’outputCbCr.

- Convert R’G’B’ to RGB.

- Compute Ylinear from RGB.

- Compare Ylinear to Ytarget and adjust Youtput according to a binary search.

- While Youtput differs from Ytarget repeat steps 2-5.

- Compute and output Y’ from Youtput.

The iterative nature of the search presents difficulties for real-time implementations, e.g. for the hardware case. In the worst case, the number of iterations is equal to the luma bit depth. Then, sufficient provisioning must be made for the worst case. Initialisation of Ymin and Ymax variables to a range narrower than that afforded by the PQ transfer function can provide a reduction in the iteration count, but the worst case is unaffected.

# Method description

The iterative method is replaced with a LUT-based method. A LUT containing multiple coefficient sets is used. A coefficient set contains coefficients needed to map Ytarget, Cb and Cr to Y’ using a second-order model, as follows:

Y’ = m0Y’target + m1Y’target2 + m2Cb + m3Cb2 + m4Cr + m5Cr2 + m6. (Eqn 1)

Where Y’target is the PQ encoding of Ytarget, quantised to 10 bits, and Cb and Cr are expressed in the native bit depth of the system, i.e. 10 bits.

First, an octant location LUT (yadjust_search_table) is used to locate a containing octant within a pre-defined octant hierarchy, using the most significant bits (MSBs) of the input Y’target, Cb and Cr values. After this a coefficient set LUT (yadjust_coeff_table) is used to obtain a coefficient set, i.e. seven predetermined coefficients (as implied by the use of a second order model) associated with a given octant.

The octant hierarchy and associated coefficient sets are determined offline using a recursive search function. At a given octant, a coefficient set is produced by fitting to a set of samples generated from the iterative implementation of luma sample adjustment. Then, an ‘error’ measure, being the largest discrepancy between the resulting model and the iterative implementation, is derived by sampling the octant. If the error exceeds a threshold, the octant is split and the resulting octants are search similarly. This search is applied recursively until a minimum octant size is reached. The presence of a minimum octant size constraint limits the complexity of the logic required to identify a given octant given the Ytarget’CbCr input.

Table 1 shows a summary of the coefficient sets (maximum error, average error, and coefficient count) for several different offline searches that were performed.

Table 1. Coefficient set summary.

| Coefficient set count | Average error | Maximum actual error1 | Specified maximum error | Minimum octant size | Maximum luminance | Transfer function |
| --- | --- | --- | --- | --- | --- | --- |
| 2963 | 0.160 | 0.997 | 1 | 2x2x2 | 10K | PQ |

Notes:

1. When the maximum actual error exceeds the specified maximum error, further decomposition of the octant hierarchy was prevented due to the minimum octant size constraint.

# Results

Table 2 and Table 3 show the results from HDRMetrics for the HDR anchor [1] without compression, with luma sample adjustment enabled (default) and disabled, respectively.

Table 2. HDRMetrics results for v3.2 anchor without compression.

|  |  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 63.18 | 70.21 | 52.23 | 56.60 | 55.01 | 49.16 | 24.73 | 58.38 |
|  | Market3Clip4000r2 | 55.60 | 69.61 | 45.96 | 50.27 | 49.89 | 36.79 | 21.87 | 47.89 |
|  | SunRise | 59.67 | 69.61 | 50.92 | 55.09 | 54.57 | 37.88 | 24.22 | 47.94 |
| class B | BikeSparklers cut 1 | 47.54 | 69.78 | 37.04 | 41.44 | 39.83 | 34.19 | 21.82 | 51.48 |
|  | BikeSparklers cut 2 | 48.61 | 69.76 | 38.42 | 42.79 | 41.63 | 34.52 | 21.77 | 51.08 |
|  | GarageExit | 56.37 | 69.73 | 47.09 | 51.35 | 50.05 | 38.10 | 22.85 | 49.78 |
| class C | ShowGirl2Teaser | 53.44 | 69.24 | 44.97 | 49.12 | 47.85 | 39.29 | 22.50 | 52.90 |
| class D | StEM_MagicHour cut 1 | 54.05 | 69.83 | 39.98 | 44.58 | 42.05 | 36.57 | 24.96 | 52.16 |
|  | StEM_MagicHour cut 2 | 51.52 | 69.89 | 38.72 | 43.27 | 40.81 | 36.00 | 24.71 | 52.68 |
|  | StEM_MagicHour cut 3 | 50.92 | 69.94 | 37.51 | 42.08 | 39.66 | 36.07 | 24.28 | 52.94 |
|  | StEM_WarmNight cut 1 | 52.58 | 69.94 | 38.46 | 43.07 | 40.65 | 36.67 | 25.11 | 52.94 |
|  | StEM_WarmNight cut 2 | 54.20 | 70.18 | 39.14 | 43.78 | 41.72 | 37.06 | 23.35 | 52.81 |
| class G | BalloonFestival | 53.73 | 69.59 | 45.06 | 49.27 | 51.73 | 40.68 | 21.15 | 48.24 |
| class H | EBU_04_Start | 54.15 | 69.59 | 46.67 | 50.71 | 49.18 | 36.87 | 22.23 | 47.34 |
|  | EBU_06_Hurdles | 51.23 | 69.60 | 44.07 | 48.07 | 47.75 | 36.12 | 21.28 | 47.71 |
| Average |  | 53.79 | 69.77 | 43.08 | 47.43 | 46.16 | 37.73 | 23.12 | 51.08 |

Table 3. HDRMetrics results for v3.2 anchor with no compression and luma sample adjustment disabled.

|  |  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 58.73 | 64.38 | 53.39 | 56.78 | 54.86 | 48.42 | 24.69 | 55.05 |
|  | Market3Clip4000r2 | 53.48 | 62.05 | 46.24 | 50.16 | 49.75 | 36.77 | 21.88 | 46.85 |
|  | SunRise | 58.60 | 67.76 | 51.34 | 55.28 | 54.62 | 37.87 | 24.21 | 47.70 |
| class B | BikeSparklers cut 1 | 46.39 | 58.98 | 37.27 | 41.52 | 39.81 | 34.17 | 21.82 | 48.20 |
|  | BikeSparklers cut 2 | 47.57 | 59.92 | 38.65 | 42.87 | 41.60 | 34.51 | 21.77 | 48.46 |
|  | GarageExit | 55.21 | 65.61 | 47.08 | 51.17 | 49.89 | 38.08 | 22.85 | 48.77 |
| class C | ShowGirl2Teaser | 52.11 | 62.98 | 44.86 | 48.81 | 47.49 | 39.23 | 22.50 | 50.84 |
| class D | StEM_MagicHour cut 1 | 52.66 | 63.91 | 40.06 | 44.59 | 42.01 | 36.55 | 24.95 | 50.33 |
|  | StEM_MagicHour cut 2 | 50.22 | 62.70 | 38.62 | 43.08 | 40.59 | 35.97 | 24.70 | 50.27 |
|  | StEM_MagicHour cut 3 | 49.48 | 61.22 | 37.39 | 41.88 | 39.46 | 36.03 | 24.28 | 50.10 |
|  | StEM_WarmNight cut 1 | 51.10 | 61.83 | 38.65 | 43.16 | 40.73 | 36.64 | 25.11 | 50.59 |
|  | StEM_WarmNight cut 2 | 52.05 | 61.07 | 39.45 | 43.96 | 41.81 | 37.00 | 23.35 | 49.76 |
| class G | BalloonFestival | 52.78 | 62.06 | 45.61 | 49.54 | 51.91 | 40.62 | 21.15 | 46.98 |
| class H | EBU_04_Start | 52.12 | 62.75 | 46.85 | 50.41 | 48.82 | 36.84 | 22.23 | 45.36 |
|  | EBU_06_Hurdles | 49.73 | 61.87 | 44.19 | 47.83 | 47.52 | 36.10 | 21.27 | 46.11 |
| Average |  | 52.15 | 62.61 | 43.31 | 47.40 | 46.06 | 37.65 | 23.12 | 49.02 |

Table 4. Results with LUT programmed for 10K case, PQ

|  |  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 59.01 | 60.58 | 51.69 | 55.27 | 52.47 | 48.75 | 24.72 | 55.55 |
|  | Market3Clip4000r2 | 55.26 | 64.65 | 45.92 | 50.16 | 49.50 | 36.78 | 21.88 | 45.62 |
|  | SunRise | 58.59 | 63.71 | 50.76 | 54.69 | 53.55 | 37.86 | 24.22 | 45.30 |
| class B | BikeSparklers cut 1 | 47.48 | 64.43 | 37.04 | 41.42 | 39.77 | 34.18 | 21.82 | 49.09 |
|  | BikeSparklers cut 2 | 48.53 | 64.33 | 38.41 | 42.77 | 41.54 | 34.52 | 21.77 | 48.68 |
|  | GarageExit | 55.98 | 64.81 | 47.04 | 51.22 | 49.70 | 38.08 | 22.85 | 47.77 |
| class C | ShowGirl2Teaser | 53.08 | 63.68 | 44.90 | 48.97 | 47.55 | 39.27 | 22.50 | 51.08 |
| class D | StEM_MagicHour cut 1 | 53.86 | 65.61 | 39.97 | 44.56 | 42.00 | 36.57 | 24.95 | 50.61 |
|  | StEM_MagicHour cut 2 | 51.37 | 64.57 | 38.72 | 43.25 | 40.76 | 36.00 | 24.71 | 50.84 |
|  | StEM_MagicHour cut 3 | 50.77 | 64.37 | 37.50 | 42.07 | 39.62 | 36.07 | 24.28 | 51.11 |
|  | StEM_WarmNight cut 1 | 52.33 | 64.40 | 38.46 | 43.04 | 40.60 | 36.67 | 25.11 | 51.35 |
|  | StEM_WarmNight cut 2 | 53.92 | 65.08 | 39.14 | 43.76 | 41.66 | 37.05 | 23.35 | 51.25 |
| class G | BalloonFestival | 53.61 | 66.36 | 45.05 | 49.23 | 51.38 | 40.64 | 21.15 | 46.99 |
| class H | EBU_04_Start | 54.07 | 66.36 | 46.64 | 50.65 | 49.00 | 36.86 | 22.23 | 45.94 |
|  | EBU_06_Hurdles | 51.08 | 63.63 | 44.04 | 47.98 | 47.28 | 36.10 | 21.28 | 44.58 |
| Average |  | 53.26 | 64.44 | 43.02 | 47.27 | 45.76 | 37.69 | 23.12 | 49.05 |

Table 5. BD-Rate result summary (LUT programmed for 10K case, PQ). See Excel spreadsheet for detailed BD-rate results

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 0.7% | 2.1% | -1.0% | 0.7% | -0.3% | -1.1% | -1.9% | 0.6% | -1.3% | -0.1% | 0.2% |
|  | Market3Clip4000r2 | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.4% | -74.3% | 0.5% | 0.0% | 0.0% | 1.3% |
|  | SunRise | 0.5% | 0.5% | -0.1% | 0.3% | 0.3% | 0.8% | -12.2% | 1.3% | 0.4% | 0.7% | 0.1% |
| class B | BikeSparklers cut 1 | 0.0% | 0.0% | 0.3% | 0.1% | 0.2% | 0.4% | -8.1% | 0.1% | 0.1% | 0.3% | 1.1% |
|  | BikeSparklers cut 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 1.5% | 0.2% | 0.0% | -0.1% | 0.3% |
|  | GarageExit | -0.1% | 0.2% | 0.3% | 0.1% | 0.3% | 0.3% | 7.4% | 0.5% | 0.2% | 0.3% | -0.5% |
| class C | ShowGirl2Teaser | 8.0% | 8.7% | 8.6% | 8.2% | 5.5% | 11.6% | 32.5% | 10.9% | 41.2% | 15.0% | 16.1% |
| class D | StEM_MagicHour cut 1 | 0.1% | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | -4.9% | 0.1% | 0.2% | -1.0% | 1.1% |
|  | StEM_MagicHour cut 2 | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% | 0.3% | -3.9% | 0.1% | 0.2% | 0.1% | 0.3% |
|  | StEM_MagicHour cut 3 | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% | 0.3% | 1.1% | 0.1% | 0.6% | -0.1% | 0.5% |
|  | StEM_WarmNight cut 1 | 0.2% | 0.1% | 0.6% | 0.3% | 0.2% | 0.0% | 5.1% | 0.3% | 0.5% | 0.2% | 0.5% |
|  | StEM_WarmNight cut 2 | 0.1% | 0.2% | 0.3% | 0.2% | 0.0% | 0.0% | -11.6% | 0.1% | 0.2% | 0.1% | -0.7% |
| class G | BalloonFestival | 0.1% | 0.1% | 0.0% | 0.0% | 0.4% | 0.5% | 195.6% | 0.6% | 0.1% | 0.1% | 0.5% |
| class H | EBU_04_Hurdles | -0.1% | 0.1% | 0.2% | 0.1% | 0.2% | -0.4% | 1.0% | 0.2% | 0.0% | 0.0% | -0.3% |
|  | EBU_06_Start | 0.2% | 0.3% | 0.1% | 0.2% | 1.1% | -0.3% | 7.9% | 3.4% | 0.1% | -0.2% | 0.0% |
|  | Overall | 0.7% | 0.8% | 0.6% | 0.7% | 0.5% | 0.9% | 9.0% | 1.3% | 2.8% | 1.0% | 1.4% |

# References

- E. François, P. Yin, J. Sole, “Common Test Conditions for HDR/WCG Video Coding Experiments”, Moving Pictures Experts Group, w15793, Geneva, October, 2015.

- J. Strom, “Investigation of HDR Color Subsampling”, Moving Pictures Experts Group, m35841, Geneva, February, 2015.

- J. Strom, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson’s response to CfE for HDR and WCG”, Moving Pictures Experts Group, m36184, Warsaw, June, 2015.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).