| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0058 |
| --- | --- |

| Title: | Video usability information signaling for SDR backward compatibility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun Mook Oh Jong-Yeul Suh 19, Yangjae-daero 11gil, Seocho-gu, Seoul, 137-893, Korea | Email: | hyunmook.oh@lge.com Jongyeul.suh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the developing of high dynamic range (HDR) extension of HEVC, two different output points are defined in standard dynamic range (SDR) backward compatible mode. Although the output SDR and HDR videos have different video features in terms of transfer function and/or color gamut, the video characteristic of reconstructed HDR output is not sufficiently described in current syntax. This contribution proposes to define additional VUI to describe the video characteristics of one output point while the other output point use current VUI.

# Problem Statement

In developing of high dynamic range (HDR) extension of HEVC, a new model based on HDR reconstruction, called re-shaper, is adopted to improve coding efficiency and/or to support legacy devices. When it is used in standard dynamic range (SDR) backward compatible mode, the reconstructed video at different output points have different video characteristics to support SDR and HDR devices. In Figure 1, the different output video characteristics in different output process points are described. In this example, a receiver that is conformant to point A obtains SDR video sequence whose transfer function is BT.709 and whose color space is BT.709. At point B, HDR video sequence is reconstructed where the output characteristics are described by ST 2084 transfer function and BT.2020 color space.

Figure 1. Example of input and output video characteristics for SDR mode

For the first video output at point A, the current VUI signaling might be used in considering of legacy devices. For the second video output at point B, transfer function of the reconstructed HDR video can be signaled based on the extended SPS structure in w15792. However, the current extended structure does not consider other changes, such as color gamut change.

In this proposal, we propose to extend the syntax structure in w15792 to support video characteristics such as color gamut as well as transfer characteristics. In specific, we propose to define and to signal additional VUI for point B video output. By using this additional VUI, the decoders will have correct information regarding the HDR reconstruction output that best fits the creative intent of the content authors. Moreover, this framework could be extended to be applicable to the multiple conformance point decoder.

# Proposal

To define additional video information for point B in Figure 1, we propose to extend the syntax structure of sps_hdrwcg_extension( ) in w15792 where transfer_characteristics for reconstructed HDR video is only defined. In addition to that, we propose to signal color gamut information as well as other useful information in VUI to fully describe output HDR video.

## 2.1 Sequence parameter set HDR and WCG extension syntax table.

| sps_hdrwcg_extension( ) { | Descriptor |
| --- | --- |
| hdrwcg_extension_vui_parameters_present_flag | u(1) |
| if( hdrwcg_extension_vui_parameters_present_flag ) |  |
| hdrwcg_extension_vui_parameters() |  |
| reshaper_range_id | u(2) |
| } |  |

## 2.2 Sequence parameter set HDR and WCG extension semantics

hdrwcg_extension_vui_parameters_present_flag equal to 1 specifies that an additional syntax structure vui_parameters() is present in the SPS. hdrwcg_extension_vui_parameters_present_flag equal to 0 specifies that the additional syntax structure hdrwcg_extension_vui_parameters() is not present in the SPS. When not present, the value of hdrwcg_extension_vui_parameters_present_flag is inferred to be equal to 0.

Syntax and semantics of hdrwcg_extension_vui_parameters() are defined same as the syntax and semantics of vui_parameters( ) defined in Annex E, respectively.

Semantics of reshaper_range_id follows the description in w15792.

## 2.3 General sequence parameter set RBSP syntax (Informative)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_luma_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_luma_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| st_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| sps_range_extension_flag | u(1) |
| sps_multilayer_extension_flag | u(1) |
| sps_3d_extension_flag | u(1) |
| sps_scc_extension_flag | u(1) |
| sps_hdrwcg_extension_flag | u(1) |
| sps_extension_3bits | u(3) |
| } |  |
| if( sps_range_extension_flag ) |  |
| sps_range_extension( ) |  |
| if( sps_multilayer_extension_flag ) |  |
| sps_multilayer_extension( ) /* specified in Annex F */ |  |
| if( sps_3d_extension_flag ) |  |
| sps_3d_extension( ) /* specified in I*/ |  |
| if( sps_scc_extension_flag ) |  |
| sps_scc_extension( ) |  |
| if( sps_hdrwcg_extension_flag) |  |
| sps_hdrwcg_extension( ) |  |
| if( sps_extension_3bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## 2.4 Video Usability Information table (Informative)

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = EXTENDED_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| frame_field_info_present_flag | u(1) |
| default_display_window_flag | u(1) |
| if( default_display_window_flag ) { |  |
| def_disp_win_left_offset | ue(v) |
| def_disp_win_right_offset | ue(v) |
| def_disp_win_top_offset | ue(v) |
| def_disp_win_bottom_offset | ue(v) |
| } |  |
| vui_timing_info_present_flag | u(1) |
| if( vui_timing_info_present_flag ) { |  |
| vui_num_units_in_tick | u(32) |
| vui_time_scale | u(32) |
| vui_poc_proportional_to_timing_flag | u(1) |
| if( vui_poc_proportional_to_timing_flag ) |  |
| vui_num_ticks_poc_diff_one_minus1 | ue(v) |
| vui_hrd_parameters_present_flag | u(1) |
| if( vui_hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| restricted_ref_pic_lists_flag | u(1) |
| min_spatial_segmentation_idc | ue(v) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_min_cu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).