| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0060 |
| --- | --- |

| Title: | Re-shaper syntax extension for HDR CE6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s): | Hyun Mook Oh Jong-Yeul Suh 19, Yangjae-daero 11gil, Seocho-gu, Seoul, 137-893, Korea | Email: | hyunmook.oh@lge.com Jongyeul.suh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the extension of exploratory test model (ETM) framework which is comprised of decoder and HDR post-processing, the HDR post-processing block could be considered as video format conversion process when ETM is used for SDR backward compatible mode. To support the process of different input and output picture, we proposed to extend the HDR reshaping syntax in ETM syntax structure.

# Problem Statement

In exploratory test model (ETM) for high dynamic range (HDR) extension of HEVC, a new model based on HDR reconstruction, called re-shaper, is adopted to improve coding efficiency and/or to support legacy devices. In the current model, the input and output signal format is constrained to YCbCr 4:2:0 constant luminance signal which is compatible to the current HEVC decoder output. As an extension of ETM framework, several study items, such as different input output signal format, SDR backward compatibility, gamut mapping, etc, are considered in HDR CE6.

Figure 1. ETM framework extended for HDR CE6

In Figure 1, ETM framework extended for HDR CE6 is described where the decoder output is processed in HDR post-processing block instead of re-shaper in current ETM. When we consider the HDR video sequence with ST 2084 transfer function and BT.2020 color gamut, the HDR post-processing output in output point B should be the same HDR characteristics while the output format could be different, such as RGB, YCbCr, or other color representation. When standard dynamic range-backward compatible (SDR-BC) is assumed to be targeted to the current SDR receivers, point A output should be a format with YCbCr 4:2:0 with BT.2020 OETF and BT.709 (or BT.2020) color space.

In Figure 2, the input and output signal formats of the example above are described in the perspective of HDR post-processing block. When we focus on the signal processing aspect, this block performs video format conversion with or without the signal enhancement.

Figure 2. Input and output signal characteristics for SDR-BC case

In this proposal, we propose to extent the syntax structure in w15792 to support video format conversion in HDR post-processing. As the syntax structure in w15792 is proposed for ETM with constraint on YCbCr output, the extension contains input and output signal format description. In addition, we propose to extent the re-shaper function to have flexibility on each channel.

# Proposal

In this proposal, we assume that the current structure of re-shaper is used for future extension of HDR CE 6. Based on the syntax structure in w15792, we proposed to extend hdr_reshaping_pps_table which have the detailed information of the re-shaper function and signal information of source and target video. The proposed syntax and semantics are provided in the following subsections.

## 2.1 proposed extension of HDR reshaping PPS table

| hdr_reshaping_pps_table( ) { | Descriptor |
| --- | --- |
| reshape_present_flag | ue(v) |
| if( reshape_present_flag ) { |  |
| reshape_chroma_crosschannel_flag | u(1) |
| reshape_input_luma_bit_depth_minus8 | ue(v) |
| reshape_input_chroma_bit_depth_minus8 | ue(v) |
| reshape_output_luma_bit_depth_minus8 | ue(v) |
| reshape_output_chroma_bit_depth_minus8 | ue(v) |
| reshape_default_flag | u(1) |
| if( !reshape_default_flag ) { |  |
| reshape_scale_int_bit_depth | u(4) |
| reshape_offset_int_bit_depth | u(4) |
| reshape_scale_frac_bit_depth | u(4) |
| reshape_offset_frac_bit_depth | u(4) |
| reshape_negative_scales_present_flag | u(1) |
| } |  |
| reshape_input_transfer_characteristics | ue(8) |
| reshape_input_color_primaries | ue(8) |
| reshape_input_color_signal_representation | ue(8) |
| reshape_output_transfer_ characteristics | ue(8) |
| reshape_output_color_primaries | ue(8) |
| reshape_output_color_signal_representation | ue(8) |
| reshape_function_extension_flag | u(1) |
| if(!reshape_function_extension_flag) { |  |
| reshape_num_comps_minus2 | ue(v) |
| for( c = 0; c < reshape_num_comps_minus2 + 2; c++ ) { |  |
| reshape_num_ranges_minus1[ c ] | ue(v) |
| reshape_equal_ranges_flag[ c ] | u(1) |
| reshape_global_offset_val[ c ] | u(v) |
| if( !reshape_equal_ranges_flag[ c ] ) |  |
| for ( i = 0; i < reshape_num_ranges_minus1[ c ] + 1; i++ ) |  |
| reshape_range_val[ c ][ i ] | u(v) |
| if ( c = = 0 ) { |  |
| coeff_log2_offset_minus2 | ue(v) |
| reshape_continuity_flag | u(1) |
| for( i = 0; i < reshape_num_ranges_minus1[ c ] + 2; i++ ) { |  |
| poly_coef_int[ i ][ 0 ] | se(v) |
| poly_coef_frac[ i ][ 0 ] | u(v) |
| } |  |
| if( reshape_continuity_flag = = 1 ) { |  |
| poly_coef_int[ 0 ][ 1 ] | se(v) |
| poly_coef_frac[ 0 ][ 1 ] | u(v) |
| } |  |
| } else { |  |
| for( i = 0; i < reshape_num_ranges_minus1[ c ] + 1; i++ ) { |  |
| reshape_scale_int_val[ c – 1 ][ i ] | u(v) |
| reshape_scale_fract_val[ c – 1 ][ i ] | u(v) |
| } |  |
| if ( reshape_chroma_crosschannel_flag ) { |  |
| reshape_offset1[ c – 1 ] | u(16) |
| reshape_offset2[ c – 1 ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |
| else { |  |
| reshape_interim_color_space_type_flag | u(1) |
| if(reshape_interim_color_space_type_flag) { |  |
| reshape_interim_color_space_type | ue(8) |
| } |  |
| reshape_num_comps_minus2 | ue(v) |
| for( c = 0; c < reshape_num_comps_minus2 + 2; c++ ) { |  |
| reshape_num_ranges_minus1[ c ] | ue(v) |
| reshape_equal_ranges_flag[ c ] | u(1) |
| reshape_global_offset_val[ c ] | u(v) |
| if( !reshape_equal_ranges_flag[ c ]) { |  |
| for ( i = 0; i < reshape_num_ranges_minus1[ c ] + 1; i++ ) { |  |
| reshape_range_val[ c ][ i ] | u(v) |
| reshape_curve_type[ c ][ i ] | ue(8) |
| for(j=0; j<NUM_OF_COEFF; j++) { |  |
| coef_int[ c ][ i ] [ j ] | se(v) |
| coef_frac[ c ][ i ] [ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## 2.2 Sequence parameter set HDR and WCG extension semantics

reshape_input_transfer_characteristics represent the transfer characteristic of input picture. It has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element, except that reshape_input_transfer_ characteristics specifies the colour space of the decoder output picture, rather than the colour space used for the CLVS.

reshape_input_color_primaries represent the color primary of input picture. It has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element, except that reshape_input_color_primaries specifies the colour space of the decoder output picture, rather than the colour space used for the CLVS.

reshape_input_color_signal_representation represent the color representaion of input picture.

| reshape_input_color_signal_representation | description |
| --- | --- |
| 0 | For future use by ITU-T ¦ ISO/IEC |
| 1 | RGB |
| 2 | YCbCr 4:4:4 |
| 3 | YCbCr 4:2:2 |
| 4 | YCbCr 4:2:0 |
| 5 | YCbCr 4:4:4 CL (BT.2020 only) |
| 6 | YCbCr 4:2:2 CL (BT.2020 only) |
| 7 | YCbCr 4:2:0 CL (BT.2020 only) |
| 8-255 | For future use by ITU-T ¦ ISO/IEC |

reshape_output_transfer_ characteristics represent the transfer characteristic of output picture. It has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element, except that reshape_output_transfer_ characteristics specifies the colour space of the HDR post-processing output picture, rather than the colour space used for the CLVS.

reshape_output_color_primaries represent the color primary of input picture. It has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element. except that reshape_output_color_primaries specifies the colour space of the HDR output picture, rather than the colour space used for the CLVS.

reshape_output_color_signal_representation represent the color representaion of output picture.

| reshape_output_color_signal_representation | description |
| --- | --- |
| 0 | For future use by ITU-T ¦ ISO/IEC |
| 1 | RGB |
| 2 | YCbCr 4:4:4 |
| 3 | YCbCr 4:2:2 |
| 4 | YCbCr 4:2:0 |
| 5 | YCbCr 4:4:4 CL (BT.2020 only) |
| 6 | YCbCr 4:2:2 CL (BT.2020 only) |
| 7 | YCbCr 4:2:0 CL (BT.2020 only) |
| 8-255 | For future use by ITU-T ¦ ISO/IEC |

reshape_function_extension_flag .equals 1 represent that the re-shaper function is extened.

reshape_interim_color_space_type_flag equals 1 represents that a specific colour space that is used to apply the re-shape coefficients is defined. If 0, it is assumed that default YCbCr color space is used for re-shape function.

reshape_interim_color_space_type represents the in what color space the re-shaper or post-processing function should be performed and in what color space the coefficients are defined. If not present, the default is YCbCr.

| reshape_interim_color_space_type | description |
| --- | --- |
| 0 | For future use by ITU-T ¦ ISO/IEC |
| 1 | YCbCr |
| 2 | RGB |
| 3 | Yuv |
| 4-255 | For future use by ITU-T ¦ ISO/IEC |

reshape_curve_type represents the type of curve used to describe re-shaper function in given channel and given range.

| reshape_curve_type | description |
| --- | --- |
| 0 | For future use by ITU-T ¦ ISO/IEC |
| 1 | linear |
| 2 | Second order polynomial |
| 3-255 | For future use by ITU-T ¦ ISO/IEC |

NUM_OF_COEFF is defined by reshape_curve_type. For example, when reshape_curve_type equals 1, the number of the coefficients is two for gain and offset of the linear function.

reshape_num_comps_minus2, reshape_num_ranges_minus1, reshape_equal_ranges_flag, reshape_global_offset_val, reshape_range_val, coef_int, coef_frac follows the semantics described in w15792.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).