| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0061 |
| --- | --- |

| Title: | CE7: Results Core Experiment 7.1 test a and b |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Andrew Cotton Manish Pindoria Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Tel: Email: | 0044 03030409640 matteo.naccari@bbc.co.uk andrew.cotton@bbc.co.uk manish.pindoria@bbc.co.uk |
| Source: | BBC |  |  |

_____________________________

# Abstract

This document summarises the results associated with Core Experiment 7 (CE7) which aims to investigate on the generation of Hybrid Log-Gamma (HLG) signals with the WG11 test material. Two groups of experiments are performed: Experiment 7.1 where HLG signals are generated according to what described in m36249 and Experiment 7.2 where HLG signals are generated using different values for the system gamma parameter. This document reports on the results and experimental conditions used to generate the data for Experiment 7.1.

# Introduction

During the 113th MPEG meeting in Geneva, Core Experiment 7 (CE7, see Document N15800 for further details) was established to investigate the whole processing associated with the Hybrid Log-Gamma (HLG) technology. HLG describes a scene-referred system for transmission of High Dynamic Range (HDR) content in broadcasting. Given that the test material used by the HDR/XYZ ad-hoc group of MPEG is display-referred, some additional processing is required before the Opto-Electro Transfer Function (OETF) of HLG can be applied. Documents m36249 and m36976 describe this processing which is the topic of investigation of CE7. Two groups of experiments are designed in this CE: Experiment 7.1 and 7.2. Experiment 7.1 aims to generated HLG-coded content using the WG11 test material and according to what described in m36249 and m36976. Experiment 7.1 consists of two sub-experiments, 7.1a and 7.1b where the former removes the so-called system gamma by operating on the luminance component while the latter removes the system gamma on the three components separately. Experiment 7.2 instead, generates HLG-coded content using different values for the system gamma parameter. This document provides the results associated with Experiment 7.1 a and b. In the following, Section 2 will describe the experimental setup as well as the configuration of the HEVC reference Model (HM) to produce the bitstreams. Section 3 instead reports on the results obtained for this CE.

# Configuration files and parameters

Both Experiment 7.1a and 7.1b involve two main conversions:

- From Open EXR to YCbCr 4:2:0

- From YCbCr 4:2:0 to Open EXR

These conversions are performed using a modified version of the HDRTools software where for Experiment 7.1a the software is the same as proposed in m36976 (and available at http://wg11.sc29.org under Exploration/HDR/CEs_Software_113/CE7) while for Experiment 7.1b the software package has been modified by BBC and circulated on the CE7 email reflector. The following two subsections describe the software parameters and configuration files to be used to perform the aforementioned conversions for Experiments 7.1a and 7.1b. The last subsection describes the parameters used in the compression of the obtained YCbCr 4:2:0 files.

## Experiment 7.1a

To perform the conversion from Open EXR to YCbCr 4:2:0 the configuration file HDRConvertHLG2020.cfg provided with the software package should be used. This configuration file is basically the same as HDRConvertEXR2020ToYCbCr420.cfg which ships with the HDRTools as provided by the ad-hoc group. The only parameters which change value are highlighted in Table 1. To perform the conversion from YCbCr 4:2:0 to Open EXR in the configuration file HDRConvertYCbCr420ToEXR2020HLG.cfg provided with the software package should be used. This configuration file is basically the same as HDRConvertYCbCr420ToEXR2020.cfg which ships with the HDRTools as provided by the ad-hoc group. Again, the only parameters which change value are highlighted in Table 2. For sequences belonging to Class B, C and D, the YCbCr 4:2:0 file is obtained by first converting the PQ-coded content to Open EXR (using the configuration files HDRConvertBT2020TiffToEXR2020.cfg and HDRConvertP3D65TiffToEXR2020.cfg which ship with the HDRTools software) and then generating the associated YCbCr 4:2:0 file as described above.

Table 1: Value for parameters associated with conversion from Open EXR to YCbCr 4:2:0.

| Parameter name | Value |
| --- | --- |
| OutputTransferFunction | 11 |
| SourceSystemGamma | 1.0 |
| SourceDisplayAdjustment | 0 |
| OutputSystemGamma | 1.6 |
| OutputDisplayAdjustment | 1 |

Table 2: Value for parameters associated with conversion from YCbCr to Open EXR.

| Parameter name | Value |
| --- | --- |
| SourceTransferFunction | 11 |
| SourceSystemGamma | 1.6 |
| SourceDisplayAdjustment | 1 |
| OutputSystemGamma | 1.0 |
| OutputDisplayAdjustment | 0 |

## Experiment 7.1b

To perform the conversion from Open EXR to YCbCr 4:2:0 the HDRTools provided for this experiment should be used with configuration file HDRConvertEXR2020ToYCbCr420.cfg and parameters highlighted in Table 3. To perform the conversion from YCbCr 4:2:0 to Open EXR the HDRTools provided for this experiment should be used with configuration file HDRConvertYCbCr420ToEXR2020HLG.cfg and parameters highlighted in Table 4. It should be noted that the main difference between Table 1 and Table 3 (respectively Table 2 and Table 4) is in the value for the OutputDisplayAdjustment parameter (respectively, SourceDisplayAdjustment) which is 2 rather than 1 as in Experiment 7.1a (respectively Experiment 7.1b). Again, to obtain the YCbCr 4:2:0 file for sequences belonging to Class B, C and D the same processing as described in Experiment 7.1 a and b has been applied with the parameters changed accordingly.

Table 3: Value for parameters associated with conversion from Open EXR to YCbCr 4:2:0.

| Parameter name | Value |
| --- | --- |
| OutputTransferFunction | 11 |
| SourceSystemGamma | 1.0 |
| SourceDisplayAdjustment | 0 |
| OutputSystemGamma | 1.6 |
| OutputDisplayAdjustment | 2 |

Table 4: Value for parameters associated with conversion from YCbCr 4:2:0 to Open EXR.

| Parameter name | Value |
| --- | --- |
| SourceTransferFunction | 11 |
| SourceSystemGamma | 1.6 |
| SourceDisplayAdjustment | 2 |
| OutputSystemGamma | 1.0 |
| OutputDisplayAdjustment | 0 |

## Configuration files and parameters

For both Experiments 7.1a and 7.1b the HM software Version 16.7 (commit hash 1bda724) has been used without any additional modification. In particular the macro RExt__HIGH_BIT_DEPTH_SUPPORT is set to zero. Results when this macro is set to 1 will be also provided in Section 3. As suggested in W15793, the random access coding configuration has been used. Moreover, all bistreams signal the VUI syntax element with values listed in Table 5. In particular, the transfer_characteristics syntax element has been set to 14 which corresponds to the OETF specified by the ITU Recommendation BT.2020. Given the backward compatibility of the HLG system, legacy receivers would still be able to display the bistreams as SDR material. HDR receivers can then display the decoded content provided that the hdr_compatibility_info SEI message is inserted in the bitstream (see JCTVC-U0033 and JCTVC-V1005 for more details).

Table 5: Value for VUI syntax elements transmitted in the bistreams for both Experiment 7.1a and b.

| Parameter name | Value |
| --- | --- |
| video_signal_type_present_flag | 1 |
| video_full_range_flag | 0 |
| colour_description_present_flag | 1 |
| colour_primaries | 9 |
| TransferCharacteristics | 14 |
| transfer_characteristics | 9 |
| ChromaLocInfoPresent | 1 |
| chroma_sample_loc_type_top_field | 2 |
| chroma_sample_loc_type_bottom_field | 2 |

# Configuration files and parameters

As mentioned in N15800, the YCbCr 4:2:0 files coded with the HLG OETF have been compressed in VBR mode and the Quantisation Parameter (QP) has been set to obtain a bitrate that matches the one associated with the anchors described in W15793. The selected QP values are listed in Table 6where the values for the ShowGirl sequence are missing because this video has been dropped in CE7 (see N15800 for more details).

Table 6: QP values selected for the test material.

| Class | Sequence Name | QP1 | QP2 | QP3 | QP4 |
| --- | --- | --- | --- | --- | --- |
| A | FireEaterClip4000r1 | 21 | 24 | 27 | 30 |
|  | Market3Clip4000r2 | 23 | 30 | 33 | 35 |
|  | Sunrise | 20 | 24 | 26 | 31 |
| B | BikeSparklersCut1 | 25 | 27 | 33 | 36 |
|  | BikeSparklersCut2 | 25 | 27 | 33 | 36 |
|  | GarageExit | 24 | 27 | 32 | 36 |
| C | ShowGirl | n.a. | n.a. | n.a. | n.a. |
| D | StEM_MagicHourCut1 | 21 | 25 | 29 | 33 |
|  | StEM_MagicHourCut2 | 21 | 25 | 29 | 33 |
|  | StEM_MagicHourCut3 | 22 | 25 | 29 | 33 |
|  | StEM_WarmNightCut1 | 21 | 25 | 29 | 33 |
|  | StEM_WarmNightCut2 | 21 | 25 | 29 | 33 |
| G | BalloonFestival | 25 | 27 | 31 | 33 |
| H | EBU_04_Hurdles | 23 | 28 | 32 | 36 |
|  | EBU_06_Starting | 24 | 27 | 32 | 36 |

The MD5 check sums for all the test material and both experiments are contained in the spreadsheets attached to this document. The bitrates as well as all required metric values for all sequences in both experiments are contained in the attached spreadsheets to this document. For each experiment, two sets of spreadsheets are distributed as they are associated with the encoding when the RExt__HIGH_BIT_DEPTH_SUPPORT macro is set to zero and one respectively. For the MD5 check sums, the value for the input YCbCr 4:2:0 files does not change since only the coding process is modified. Conversely, the value for bitrates and metrics changes but the difference, especially in terms BD-rate on the PSNR, is limited as shown in Table 7 and Table 8.

Table 7: BD-rates for Experiment 7.1a when coding is performed with the RExt__HIGH_BIT_DEPTH_SUPPORT macro equal to 0 (anchor) and 1.

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -1.1% | -0.7% | -1.2% | -1.0% | -3.3% | -2.1% | 2.7% | -0.3% | -0.3% | -1.9% | -1.7% |
|  | Market3Clip4000r2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -1.0% | -52.2% | 0.1% | 0.0% | -0.2% | -0.6% |
|  | SunRise | -0.2% | 0.0% | -0.4% | -0.2% | -0.9% | -9.4% | 0.0% | -0.1% | 0.0% | -1.2% | -2.3% |
| class B | BikeSparklers cut 1 | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | -8.0% | 0.1% | 0.0% | -0.1% | -0.2% |
|  | BikeSparklers cut 2 | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 31.9% | 0.1% | 0.1% | -0.3% | -0.3% |
|  | GarageExit | 0.1% | 0.1% | 0.1% | 0.1% | -0.1% | -0.6% | 3.6% | -0.1% | 0.1% | -0.1% | -0.3% |
| class C | ShowGirl2Teaser | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| class D | StEM_MagicHour cut 1 | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.1% | 78.5% | 0.2% | 0.1% | -0.6% | -0.3% |
|  | StEM_MagicHour cut 2 | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.3% | 0.8% | 0.0% | -0.1% | -0.3% | -1.2% |
|  | StEM_MagicHour cut 3 | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.3% | 1.6% | 0.0% | -0.1% | -0.2% | -0.3% |
|  | StEM_WarmNight cut 1 | -0.1% | 0.0% | 0.1% | 0.0% | -0.2% | -0.2% | 2.1% | 0.0% | -0.1% | 0.0% | -0.6% |
|  | StEM_WarmNight cut 2 | -0.1% | 0.0% | -0.1% | -0.1% | -1.1% | -0.1% | -4.5% | 0.1% | -0.1% | -0.7% | -1.2% |
| class G | BalloonFestival | -0.1% | -0.1% | -0.1% | -0.1% | -1.5% | -2.8% | 3.0% | -1.8% | -0.1% | -0.3% | -0.6% |
| class H | EBU_04_Hurdles | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.6% | -2.8% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | EBU_06_Starting | -0.1% | 0.0% | -0.2% | -0.1% | -0.7% | -4.6% | 3.3% | -1.0% | 0.0% | -0.5% | -0.6% |
|  | Overall | -0.1% | 0.0% | -0.1% | -0.1% | -0.5% | -1.4% | 4.0% | -0.2% | 0.0% | -0.4% | -0.7% |

Table 8: BD-rates for Experiment 7.1b when coding is performed with the RExt__HIGH_BIT_DEPTH_SUPPORT macro equal to 0 (anchor) and 1.

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -1.9% | -0.8% | -4.3% | -2.4% | -6.4% | -4.9% | 0.1% | -0.4% | -0.1% | -4.4% | -3.8% |
|  | Market3Clip4000r2 | -0.1% | 0.0% | -0.1% | -0.1% | -0.4% | -1.7% | 4.1% | 0.0% | 0.0% | -0.4% | -0.2% |
|  | SunRise | -0.5% | 0.0% | -1.2% | -0.6% | -2.3% | -14.3% | 0.1% | -0.1% | 0.0% | -2.8% | -4.2% |
| class B | BikeSparklers cut 1 | -0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 1.6% | -18.3% | -0.1% | -0.1% | 0.5% | -0.1% |
|  | BikeSparklers cut 2 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.6% | -3.8% | -0.1% | -0.1% | 0.0% | -0.2% |
|  | GarageExit | -0.1% | -0.1% | -0.4% | -0.2% | -0.4% | -1.3% | 0.4% | -0.3% | -0.1% | -0.5% | -0.8% |
| class C | ShowGirl2Teaser | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| class D | StEM_MagicHour cut 1 | -0.3% | 0.1% | 0.2% | 0.1% | 0.0% | -1.0% | -8.1% | 0.1% | 0.1% | -0.3% | -2.2% |
|  | StEM_MagicHour cut 2 | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.6% | 0.3% | 0.0% | 0.0% | -0.4% | -1.7% |
|  | StEM_MagicHour cut 3 | -0.2% | -0.1% | -0.2% | -0.2% | -0.6% | -0.4% | 5.5% | 0.0% | -0.1% | -0.7% | -2.2% |
|  | StEM_WarmNight cut 1 | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% | -0.8% | 1.1% | -0.1% | -0.1% | -0.2% | -2.0% |
|  | StEM_WarmNight cut 2 | -0.3% | 0.0% | 0.0% | 0.0% | -1.2% | -0.6% | -6.4% | 0.1% | 0.0% | -0.7% | -1.8% |
| class G | BalloonFestival | 0.0% | 0.0% | -0.3% | -0.1% | -1.3% | -1.9% | 1.0% | -1.7% | 0.0% | -0.4% | -0.3% |
| class H | EBU_04_Hurdles | -0.2% | -0.1% | -0.1% | -0.1% | -0.3% | -0.5% | 4.7% | -0.2% | -0.1% | -0.1% | -0.2% |
|  | EBU_06_Starting | -0.4% | -0.1% | -0.3% | -0.3% | -1.4% | -9.0% | 8.6% | -1.1% | -0.1% | -0.5% | -0.8% |
|  | Overall | -0.3% | -0.1% | -0.4% | -0.3% | -1.0% | -2.4% | -0.7% | -0.3% | -0.1% | -0.7% | -1.4% |

# Visual inspection of the content

The content produced for Experiment 7.1a and b has been visually inspected over a SIM2 monitor and a consumer TV set. The main conclusion for this viewing is that the content associated with test b, i.e. the removal of the system gamma on a per component basis, produces content which is more washed out than when the gamma is remove on the luminance. This conclusion was also acknowledged during the viewing performed at the interim meeting in Vancouver. For SDR backward compatibility, document W0035 points out a potential issue with a sequence outside the test set where a hue saturation is observed. This colour issue is still under investigation. Finally, no particular problems with coding artefacts visibility were noted, i.e. the same amount of artefacts are comparable with the ones associated with the anchors.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).