| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0063 |
| --- | --- |

| Title: | HDR CE6: Core Experiments 4.3 and 4.6a: description of the Philips system in 4:2:0 and with automatic reshaper parameter derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wiebe de Haan (Philips) Robert Brondijk (Philips) Rocco Goris (Philips) Rene van der Vleuten (Philips) | Tel: Email: | +31 6 51684796 (W. de Haan) wiebe.de.haan@philips.com robert.brondijk@philips.com rocco.goris@philips.com |
| Source: | Philips |  |  |

_____________________________

# Abstract

This document describes the operation of the Philips CE system. This system is an adapted version of the earlier Philips system as proposed for the CfE (M36266). It now uses automatic reshaper parameter derivation, internal 4:2:0 processing in gamma domain instead of 4:4:4 processing in linear light domain, and desaturation.

Two variants of the system are described in this contribution (W0063) depending on the transfer of the reshaper curve. In version a) the curve shape is transferred using a limited number of parameters (CE6 4.3). In version b) the curve shape transferred using a piece wise linear approximation of the curve itself (CE6 4.6a).

It is shown that the newly adapted Philips system performs similar to the original Philips CfE system, and it is shown that both version a) and version b) of the newly adapted Philips system also perform similar quality-wise, but that version a) is more efficient from a transmission bandwidth point of view.

# Introduction

Philips proposes a parameter based single layer HDR+SDR solution transmitting either an HDR or an SDR signal with parameters that enable the reconstruction of both the HDR and SDR signals. This has been described and presented in more details in M35067 at MPEG 110 in Strasbourg.

The Philips submission for the CE is based on transmitting SDR (YCbCr 1886) with parameters in order to derive the original HDR signal at the decoder. This is shown below in Figure 1.

Automatic

Parameter

Derivation

(Autometadata)

Figure 1: Philips parameter based single layer HDR solution

The HDR Video Master enters the processing chain in the left top corner of the figure. Grading (Dynamic range conversion) results in an SDR Video Master. This grading is done automatically by an Automatic Parameter Derivation (APD) algorithm. After SDR Video Encoding the Conversion parameters from the grading process are added as (SEI) messages to the coded video bitstream. This results in an SDR Video Stream + Metadata for transmission.

A standard SDR video decoder decodes the coded video bitstream to reproduce SDR Video. A new HDR decoder will discover the metadata embedded in the content and be capable of performing the inverse dynamic range conversion of the SDR Video back to the original HDR Video. A display may then perform its own special operations to adapt the recreated HDR to its own display levels.

# Reshaping process

The Philips reshaping process is similar to what was described earlier in MPEG M36266 and is included in this section for completeness.

The processing chain of the previous Philips CfE system can be represented on a high level as shown in Figure 2. The figure shows the Reshaper before conversion to the gamma and 4:2:0 domain for the pre-processing path. For the post-processing path the operations are inverse.

SDR

transmission

Reshaper

Gamma

to 4:2:0

Desat.

Linear Light RGB

Gamma YUV

Reshaper

Gamma

to 4:4:4

Sat.

Metadata

Figure 2: High level overview of Philips CfE system

In the current Philips CE system the gamma and 4:2:0 conversion is done before the Reshaper, allowing the Reshaper processing to be in 4:2:0 as used in compressed video transmission. Because the gamma function is a pure power function, the old CfE system and new CE system are mathematically equivalent. The new Philips CE system is as shown in Figure 3.

SDR

transmission

Gamma

to 4:2:0

Reshaper

Desat.

Linear Light RGB

Gamma YUV

Gamma

to 4:4:4

Reshaper

Sat.

Metadata

Figure 3: High level overview of Philips CE system

The Reshaper is as shown in Figure 4 and will be elaborated on in the following section. It receives an YCbCr input.

Figure 4: Philips 4:2:0 Reshaper at encoder side

## Reshaping top-level process

In the reshaper first an RGB signal is derived from YCbCr to be able to perform the Max(R,G,B,Y) process and calculate the Tone mapping input:

where (α,β,y,δ) represent the tone mapping weights. Tone mapping weights determine the relative weight of the R, G, B and Y components in tone mapping. Defaults for the weights are (1,1,1,0.3), but also other values can be used by transferring them via the metadata.

## Reshaping detailed block diagram

Figure 5 shows the detailed block diagram of the TM curve block in the reshaper. It may consist of 5 processing blocks of which 4 will be explained in this document. In this MPEG CE, default values are used for the block adjustment curve that correspond to no correction.

Figure 5 Detailed HDR to SDR Tone Mapping block diagram

### Perceptual uniform signal

First step of the TM curve block is converting the linear light MaxRGBY input signal to a perceptual uniform signal. The inverse Philips EOTF as presented in m34274 (Proposed electro-optical transfer function (EOTF) for high dynamic range (HDR) video delivery).

Figure 6 Perceptual uniform signal

The following formula represents the inverse Philips EOTF.

Where γ = 2.4 and ρ is determined based on the Mastering Display Maximum Luminance [cd/m2] (Lsource).

### Level offset

The first processing block which is applied on the perceptual signal is black and /or white level offset. This block provides the possibility to apply an offset in the low and high luminance area in case there are no or a limited amount of pixels in those areas.

Figure 7 Black / White Level offset

### Local Slope Adjustment

The tone mapping curve is applied in perceptual uniform domain and is a piece-wise constructed transfer curve out of three parts, see Figure 8.
The bottom section is linear of which the steepness is determined by the Shadow Gain Control. (y = SGC * x)
The upper section is also linear of which the steepness is determined by the Highlight Gain Control. (y = HGC * x + (1-HGC))
The mid-section is a parabola connecting the two linear section in a smooth cross-over. The width of the cross-over is determined by the Mid-Tones Adjustment.
This transfer curve can be implemented as a 1-D LUT.

Figure 8 Local Slope Adjustment

### Perceptual to linear SDR

Finally the perceptual signal is converted back to the linear domain by means of the Philips EOTF as presented in m34274.

Figure 9 Perceptual to linear SDR

The formula of the Philips EOTF is shown below:

`

Where γ = 2.4 and ρ is determined based on the Target Display Maximum Luminance (e.g. 100 [cd/m2]).

## SDR to HDR Reshaping process

In the Philips CE submission the HDR signal is reshaped to an SDR signal which is, together with the metadata describing the reshaping process, provided to the HEVC codec. The decoded SDR signal is converted back to the original HDR signal by means of the inverse Reshaping process as used for the conversion from HDR to SDR, see Figure 10.

Figure 10: Reshaper at decoder side

The contents of the TM curve block of this SDR to HDR Reshaping process is as shown in Figure 11. So all Tone Mapping curve blocks in the SDR to HDR process are inverse with respect to the HDR to SDR process as shown in Figure 5.

Black

White correction

Tone Mapping Curve

To linear signal

Local Slope Adjustment

Mastering Display Max Lum

Adjustment Function

Target Display Max Lum

100 cd/m^2

Custom Curve

Black / White

level offset

To perceptual uniform signal

Figure 11 Detailed SDR to HDR Reshaping block diagram

# Automatic parameter derivation (Autometadata)

Auto metadata generation consists of two steps:

- Automatic parameter generation from an HDR image

- Temporal filtering

These steps are described in the following subsections.

## Automatic parameter generation from an HDR image

The tone mapping parameters will automatically be calculated given an HDR image. It may also be used during a live HDR transmission, e.g. sports, for automatically generating a tone mapping to SDR, and eliminating manual grading.

The tone mapping curve is variable, depending at least on the black and peak-white levels and on the average luminance of the HDR image.

First the black stretch and white stretch are calculated, derived only from black and white level measurements (the 0.01% percentile of Y and the 99.999% percentile of MAX(R,G,B).

The average lightness of the HDR image is easily calculated from the HDR average luminance. Lightness_HDR is the average luminance of the HDR converted to the perceptually uniform domain. The perceptually uniform lightness is defined by an OETF_HDR() function that is based on Barten.

The shadow gain bg in our tone mapping is a function of the average lightness of the HDR:

nom_gain = Vmax_out / Vmax_in

bg = nom_gain * (2.0 – Lightness_HDR / Lightness_HDR_max)

In this formula, nom_gain is a constant for the nominal shadow gain that maps e.g. 4000 nits HDR to 100 nits SDR with a straight line in the PU domain. This constant gain draws the darkest tone mapping curve. Lightness_HDR_max is a constant that represents the average lightness of one (very) bright HDR image for which we want to achieve this lowest shadow gain, typically 400 to 700 nit for a “true HDR image”. For example, for 700 nit Lightness_HDR_max = 0.7.

The shadow gain needs to be independent of the gain that is applied by the black- and white-stretch; therefore it is divided by the desired overall gain by the black-white-stretch gain:

BW_gain = (Vmax_in – 0.0) / (wh – bl)

bg = nom_gain * (2.0 – Lightness_HDR / Lightness_HDR_max) / BW_gain

The resulting shadow gain must be clipped between nom_gain (darkest) and 1.0 (equal nits):

bg = min(nom_gain * max(1.0, (2.0–Lightness_HDR/Lightness_HDR_max)/BW_gain), 1.0)

This is a clipped linear function, it can also be implemented as a piecewise-linear interpolator:

This means that bg reaches the minimum value for e.g. Lightness_HDR = 0.7, which corresponds to average luminance = 700 nits. For scenes with an average luminance of 700 nits or more we will apply our darkest tone mapping. This is necessary for protecting white areas during video compression of the SDR.

The other 2 tone mapping parameters dg (highlights differential gain) and xp (mid-tones width) can be calculated from bg or can be kept constant. For, the current experiment it is chosen to keep dg and xp constant, except for low bg when xp will be reduced.

From these the external metadata values are calculated as:

BlackLevel    =       bl / Vmax_in

WhiteLevel    = 1.0 – wh / Vmax_in

ShadowGain    = 4.0 * (bg – 0.50)

MidToneWidth  = 2.0 * xp / Vmax_in

HighlightGain = 4.0 * dg / nom_gain

## Temporal filtering of tone mapping parameters

Parameters automatically generated in real-time from a live video stream must be temporally low-pass filtered to reduce unrest and to avoid unnecessary responses to short-term changes.

The generic code for a temporal filter is, with Y = raw, YF = filtered parameter, in floating point:

for i in frames:

e     = Y[i] – YF[i-1]

d     = linterp(E_LUT, D_LUT, e)

YF[i] = d + YF[i-1]

param = int(Quantize(YF[i]))

The speed of the filter is determined by the 2 LUTs E_LUT and D_LUT for a piecewise-linear interpolator. A deliberate asymmetry in the response speed is introduced though the LUTs:

→ slow

quick ←

In this case the filter responds quickly to a decreasing shadow gain (increasing lightness), and more slowly to an increasing shadow gain (decreasing lightness).

The same filter can be applied to the calculated black level, responding quickly to a decreasing black level, and vice versa. For the calculated white level we will want the opposite: responding quickly to an increasing white level, and vice versa.

The 3 dynamic parameters BlackLevel (bl), WhiteLevel (wh), ShadowGain (bg) are filtered separately.

The other parameters HighlightGain (dg) and MidToneWidth (xp) are not filtered separately, but they are derived from the filtered parameter ShadowGain (bg) or set constant.

As an example of the parameters originating from the temporal filtering see Figure 13, which plots the parameters for ShowGirl over the frames. As an example of the curve changes as result from metadata changes see Figure 13, which shows the different curves for frame 100 and frame 300.

Figure 12: Automatic metadata for ShowGirl

Figure 13: Curves for reshaping ShowGirl frame 100 and 300

# Results

Initially coding was done with HM-16.7. Results of CE6.4.3 were crosschecked by Technicolor. Results of CE6.4.6a were cross-checked by FastVDO.

Next, coding was done again using “anchor HM” as used for the anchors in CE1. Results were cross-checked by FastVDO.

Visual inspection of the results with HM-16.7 showed that both CE6 4.3 and 4.6a in general perform on par with the anchor. In a number of cases the Philips SDR backward compatible solution shows better detail. The following screenshots from Market3Clip4000r2 at rate point 3 show that the Philips system has a better reconstruction of the road and license plate compared to the anchor. There is also more detail visible in the bright white dress. Notice that these results were observed in the case where the bitrate of CE6 4.3 was almost 10% lower compared to the anchor.

Figure 14 Anchor (CE1) Market3Clip4000r2 rate 3 (1725.042kbps)

Figure 15 CE6 4.3 Market3Clip4000r2 rate 3 (1584.631kbps)

A second example of a better preservation of detail of the Philips solution compared to the anchor is the bright sky around the sun in SunRise

Figure 16 Anchor (CE1) SunRise rate 4 (318.629kbps)

Figure 17 CE6 4.3 SunRise rate 4 (304.337kbps)

Coding results with the “anchor HM” were clearly better than with HM-16.7 and better than the CE1 anchors in some areas (more detailed textures, wires). Some examples are shown in a PowerPoint presentation submitted with this document.

# Version a) vs version b)

Two variants of the system are described depending on the transfer of the reshaper curve. In version a) the curve shape is transferred using a limited number of parameters (CE6 4.3). In version b) the curve shape transferred using a piece wise approximation of the curve itself. In the previous sections version a) of the system was described. In this section we (briefly) target version b) of the system.

In version b) a piece wise linear approximation of the curve is transferred instead of more generic parameters describing the curve (CE6 4.6a). In the CE submitted Philips approximated the parabolic part of the curve using 30 pair 16 bit points. This has been done according to the principle as described in M37285. Including some formatting exchanging this curve data took 142 bytes of data per frame.

The experiments of version a) and version b) of the system show that version b) and version a) are equal from a subjective point of view and which is confirmed by the metrics data. The experiments therefore prove that both methods can be used.

The use of parameters, however, give less additional data (71 bytes dynamic data with all options used; quantization and coding of the parameters will bring that further down below 50 bytes) and the option to control far more parameters, e.g. black and white stretch, saturation, …. Also together with the piece wise linear curve these additional data can be transmitted at the expensive of a higher bitrate. Version a) therefore is the preferred option for the Philips system.

# Patent rights declaration(s)

Philips may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).