| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0064 |
| --- | --- |

| Title: | HDR CE6-related : Cross Check of CE6.4.6.b |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Brondijk Philips Innovation Services HTC 37 6.022 5656 AE EIndhoven | Tel: Email: | +31 646945350 Robert.Brondijk@philips.com |
| Source: | Philips |  |  |

_____________________________

# Abstract

This document describes the result of the cross check of CE6.4.6.b, described in document JCTVC-W0059.  It reports a perfect match with the md5 sums and objective metrics. Further it reports that the, for the sequences viewed, the result was similar to the anchors.

# Introduction

Core Experiment JCTVC-W0059 presents results of the CE6.4.6b, in which the technology based on m37285 that demonstrates an HDR  backwards compatible single layer system. Essential difference with the Test model, is that the metadata is conveyed via an SEI message.  The software delivered is architected such the reshaping is integrated in the HM decoder. The effect is that a system can now be demonstrated in which all of the “new” functionality is in the de HM decoder, and the post processing is either as if the HM decoder emitted an HDR10 stream, with EOTF the ST.2084, or as a standard SDR stream, with EOTF the ITU 1886.

This document gives further a brief assessments  of the results compared to the HDR CE1 v3.2 anchors.

# Objective Results

After tailoring the validation system to support a HM decoder with two different modes, the simulations showed a perfect match for both the md5 sums as the objective metrics.

Important is that the QPs are just slightly higher than the anchor, enabling similar quantization results. Important is also the improvement in the color metric compared to the anchor, in the subjective results, this improvement seems to offset the diminished results in the Y. These kind of trade-offs clearly demonstrate once again, that the objective metrics are difficult to relate to subjective quality.

# Subjective Results

Due to timing constraints, not all sequences have been  viewed yet.

The ones we did (all at rate 3)

- FireEater : Slightly more blocking than Anchor in red smoke. Almost on par with Anchor.

- Market: Slighty more chroma artifacts in for example pavement. Rest on par with Anchor.

- Warm Night cut2: WP 29.0: On par with Anchor

- EBU hurdles: Slighty more chroma artifacts. Somewhat more detail in track. Rest on par.

- EBU starting: Red line in floor less stable compared to anchor. Sometimes disappears.

- Balloon: On par with Anchor

- SunRise: Slightly better than the Anchor