| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0066 |
| --- | --- |

| Title: | CE1-related: Optimization of HEVC Main 10 coding in HDR videos Based on Disorderly Concealment Effect |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Jung, S. Yu, Q. Lin Xidian University 2 South Taibai Road Xian 710071, China M. Li, P. Wu No.6 Huashendadao Road, ZTE R&D Center, Nanjing 210012, China | Tel: Email: | +86-29-88202654 zhengzk@xidian.edu.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

# Abstract

Proposed is an optimization of coding HDR videos based on disorderly concealment effect using HEVC encoder. Perceptual block merging is implemented on HEVC Main 10 Profile-based HDR video coding. Firstly, luminance adaptation (LA) for quantized HDR video is calculated based on the Barten contrast sensitivity function (CSF) model. Then a free-energy based just-noticeable-difference (FEJND) map is derived to represent the disorderly concealment effect for each picture. Finally, the perceptual Lagrange multiplier is derived using the average FEJND values of coding unit (CU) and picture. Experimental results demonstrate that the proposed method achieves average bit-rate reductions of 5.23% in tPSNR XYZ and 8.02% in PSNR_DE over Anchor, while producing block partitioning merging results correlated with human visual perception.

# Introduction

Since high dynamic range (HDR) videos represent a wide range of intensity levels of real world scenes, they provide ultimate fidelity to users. A major challenge for HDR videos is an efficient compression, which requires a huge amount of storage space and transmission bandwidth. Several methods have been proposed to deal with this challenge in two ways. First, HDR image formats, such as OpenEXR [1], Radiance [2], and LogLuv [3], have been proposed to describe floating-point colorimetric HDR values. Second, traditional video coding standards have been extended to the HDR video compression. [4]-[6] aim at exploiting the high profile of H.264/AVC codec [7], which support the bit-depth more than 8 bits up to 14 bits. For HDR video compression, [8] and [9] divide an HDR image into low dynamic range (LDR) image and residual data, which represent differences between HDR and LDR pixel values to support backward-compatibility. Among the existing video coding standards, the next generation of video compression technology, known as High Efficient Video Coding (HEVC) [10], is the most advanced and efficient compression standard. While recent studies on HDR video coding have concentrated on the extension of traditional video coding standards, little work has been made to optimize the compression framework without any HEVC specification changes.

(a)

(b)

Fig. 1 Block diagram of the proposed method (a) Whole framework of the proposed HDR video compression. (b) Perceptual block merging.

In this contribution, we propose optimization of HEVC Main 10 coding in HDR videos using perceptual block merging. We perform perceptual block merging for quadtree-based partitioning in HEVC based on the disorderly concealment effect of the human visual system (HVS). To represent the disorderly concealment effect, we adopt the FEJND model [11]. However, LA in the FEJND model is optimized for 8bit LDR videos [11], which has a limit to be directly used for HDR ones. Thus, we obtain new LA based on Barten CSF model [12] to generate the FEJND maps for HDR videos. First, we compute the FEJND map in each HDR video frame based on the new LA. Then, we calculate average FEJND values of each coding unit (CU) and the frame from the FEJND map to obtain the perceptual Lagrange multiplier for rate-distortion optimization (RDO). Finally, we perform perceptual block merging by minimizing rate-distortion (RD) costs based on the perceptual Lagrange multiplier. Experimental results demonstrate that the proposed method produces the block merging results more correlated with human visual perception and successfully improves the perceptual quality of the reconstructed HDR videos.

# Proposed Method

In our previous contribution [13], we have proposed a perceptual block merging method for quadtree-based partitioning in HEVC based on the disorderly concealment effect. The key of this method is to represent the disorderly concealment effect by the FEJND model and use it for perceptual block merging in HEVC. This method conceals more distortions in disorderly regions while improving the visual quality of the reconstructed videos. Therefore, we propose an HDR video coding framework with HEVC Main 10 Profile based on perceptual block merging as illustrated in Fig. 1. As shown in Fig. 1, input HDR videos in 4:4:4 RGB color space are pre-processed by coding transfer function (TF), color space conversion from RGB to Y’CbCr, quantization and chroma down-sampling to obtain the standard format (10-bit, 4:2:0, Y’CbCr) of HEVC Main 10 Profile [14]. Next, we perform perceptual block merging for quadtree-based partitioning in HEVC as follows. We first compute the FEJND map from the 10-bit frame, and then calculate average FEJND values of each CU and the frame from the FEJND map to obtain the perceptual Lagrange multiplier for RDO. Based on the perceptual Lagrange multiplier, we perform perceptual block merging by minimizing RD costs. In the decoder part, we reconstruct output HDR videos by chroma up-sampling, inverse quantization, color space conversion (Y’CbCr to RGB), and inverse coding TF [14]. In [13], we have used LA in [11] for perceptual block merging to get FEJND maps. However, LA in [11] is optimized for 8bit LDR videos; and if the LA-based FEJND map is directly used for HDR videos, it causes distortions due to the inconsistent dynamic and bit-depth between LDR and HDR videos. In this contritution, we propose new LA based on Barten CSF model [12] to generate the FEJND maps for HDR videos. Based on the Anchor of HDR/WCG CfE [14], the input HDR videos after pre-processing are compressed by HEVC Main 10 Profile. Thus, luma in the 10-bit quantized HDR videos, i.e. perceptually quantized luminance, is within the range of [64, 940] [15]. The values of 64 and 940 correspond to the lowest and highest luminance, respectively [16][17]. It is well known that the HVS sensitivity highly depends on the adaptation and background luminance. The sensitivity, known as LA, is often measured in psychophysical experiments as the just-noticeable-difference (JND) threshold that is distinguished on the uniform background luminance. LA has been obtained by a threshold versus intensity (t.v.i.) function or CSF. There are several LAs to represent the relationship between luminance and JND [18][19]. However, they cannot be directly used for HDR videos because the quantized HDR values represent luma rather than luminance. The perceptual TF for HDR videos describes the relationship between luminance and luma [20]. Thus, if we combine the LA and the perceptual TF, we can obtain the relationship between luma and JND. This is the main idea of the proposed LA in this contribution. Compared with other TFs, Perceptual Quantizer (PQ)-TF has better perceptual uniformity in the considered luminance range with modest bit-depth [21]. In PQ-TF, Barten’s CSF [12] has been used to obtain the relationship between luminance and luma. The 12-bit PQ-TF can cover the range from 10-6 cd/m2 to 104 cd/m2 with the interval of 0.9 JNDs. By considering this property, we obtain LA for 10-bit quantized HDR videos based on the Barten CSF model as follows:

(1)

with

(2)

(3)

(4)

Its inverse PQ-TF is obtained as follows:

(5)

where m1, m2, c1, c2, and c3 are parameters for inverse PQ-FT; min() denotes the minimum value; and max() denotes the maximum value. The proposed LA model is a two-step mapping. The first step of Eq. (4) is to map luma to luminance using the inverse PQ-TF [16][17]. The second step of Eq. (1) is to map luminance to JND based Barten CSF model [12]. After the two-step mapping, the proposed LA model maps luma to JND. The constant coefficient in Eq. (1) reflects different numbers of luma between 10-bit quantized HDR videos [15] and 12-bit PQ-TF [16]. The advantage of the two-step mapping is that the new LA for HDR videos is not limited to only 10-bit. That is, the new LA is still effective for the latest version of HEVC Range Extension (HEVC-RExt) which specifies profiles up to 16-bit [22]. We only need to change the luma range supported by HEVC-RExt instead of the range by the HEVC Main 10 Profile, i.e. [64, 940]. From Eq. (1), it can be observed that the new LA model of LA_10 has 877 values which correspond to the range of [64,940], which is suitable for 10-bit quantized HDR videos. Fig. 2 shows the new LA model LA_10. We use an autoregressive (AR) model to predict the orderly regions of an image [11]. The prediction residual is regarded as the disorderly regions of an image. To compute JND in the orderly part, we use spatial masking (SM) and the proposed LA model as follows:

(6)

(7)

(8)

Fig. 2 Barten CSF model-based LA for 10-bit quantized HDR videos.

where B(x,y) is the background luminance of a pixel (x, y); G(x,y) is the maximum edge height of its neighborhood [23]; min() denotes the minimum value; and JNDp(x,y) is JND of the orderly part at (x, y). Moreover, JND in the disorderly part, JNDc(x,y), is computed as follows:

(9)

where  is an adjustable parameter; and C is the disorderly map. Therefore, the FEJND is computed as follows:

(10)

The quadtree structure allows partitioning of a video frame into blocks of variable sizes, and thus keeps the tradeoff between the distortion D and the bit-rate R. It is an optimization problem which is formulated by the optimization of a Lagrangian function as follows:

(11)

where J is the RD cost; and λ is the Lagrange multiplier that controls the tradeoff between distortion and bit-rate. In the IGM theory [24], HVS tends to ignore details in the disorderly regions, which only extracts its primary structure. Since JND refers to the minimum visibility threshold of HVS, more distortion can be tolerated in the disorderly regions, and thus disorderly regions have higher FEJND values than orderly ones. Therefore, if one pixel has a relatively greater FEJND value, its corresponding original information is highly uncertain, and of less importance to human eyes. Thus, it can be classified as a candidate pixel within disorderly regions. FEJND of each CU is defined as follows:

(12)

where CUi means the i-th CU in the frame, and Mi×Ni is the resolution of the i-th CU. Different from [13], we compute FEJND of each CU instead of largest CU (LCU). This is because LCU can split into smaller CUs in HEVC [25], and smaller CU corresponds to regions with details. Therefore, block merging based on LCU [13] often causes distortions in details. Furthermore, FEJND of the whole video frame is defined as the average of all pixels’ FEJND in a frame as follows:

(13)

(a) 		                                        (b)

(c) 		                                       (d)

Fig. 3. Original image and its FEJND maps. (a) Original video frame in Market3. (b) Luminance channel. FEJND maps computed by (c) 8-bit LDR LA [13], (d) 10-bit HDR LA.

where M×N is the resolution of the frame. Thus, if one CU has an FEJND value lager than FEJNDframe, it is classified as a disorderly CU; otherwise, it is an orderly CU. Fig. 3 shows the FEJND map computed by using the 8-bit LDR LA and the proposed new LA, where light regions in Figs. 3(c) and (d) represent disorderly regions. By using the FEJND map we calculate the weight for the Lagrange multiplier as follows:

η=a+b⋅11+exp−c⋅FEJNDCUi−FEJNDframeFEJNDframe2             (14)

where three coefficients a, b and c are set to 0.7, 0.6, and 4 empirically. A smaller visual weight η (smaller than 1) indicates that the CU is orderly and more sensitive to distortions. Such a CU might be perceived at a higher visual sensitivity, which cannot tolerate higher distortions. In contrast, a larger visual weight η (larger than 1) describes a disorderly CU and is less sensitive to the distortion so that it can tolerate more distortions. When η equals 1, it is not required to adjust the Lagrange multiplier. Then, we get the perceptual Lagrange multiplier for each CU as follows:

(15)

where QP is the frame-level quantization parameter in a group of pictures (GOP) structure. Therefore, the final RD cost function in the merging process is expressed as follows:

(16)

TABLE I

Average Bit-Rate Reductions of [13] and the Proposed Method Compared with Anchor

| Video Sequences | QP=22 |  | QP=27 |  | QP=32 |  | QP=37 |  | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [13] | Proposed | [13] | Proposed | [13] | Proposed | [13] | Proposed | [13] | Proposed |
| Market3 | -3.89% | -16.66% | -4.73% | -17.46% | -4.71% | -18.81% | -5.40% | -20.65% | -4.68% | -18.40% |
| Balloon | -3.53% | -10.82% | -2.52% | -12.13% | -2.46% | -12.12% | -1.81% | -11.19% | -2.58% | -11.57% |
| Tibul2 | -0.73% | -4.85% | -0.77% | -4.88% | -0.27% | -0.66% | -0.25% | 0.02% | -0.51% | -2.59% |
| Average | -2.72% | -10.78% | -2.67% | -11.49% | -2.48% | -10.53% | -2.49% | -10.61% | -2.59% | -10.85% |

(a)                                                     (b)

(c)                                                     (d)

(e)                                                     (f)

Fig. 4 The R-D curve performance. (a) Bit-tPSNR XYZ curve in Market. (b) Bit-PSNR_DE curve in Market. (c) Bit-tPSNR XYZ curve in Ballon. (d) Bit-PSNR_DE curve in Balloon. (e) Bit-tPSNR XYZ curve in Tibul. (f) Bit-PSNR_DE curve in Tibul.

# Experimental Results

To verify the accuracy and efficiency of the proposed method, we perform the experiments in the HEVC test model (HM16.2) Main 10 Profile low delay configuration, which uses a hierarchical P picture structure with a GOP size of 4 frames and only the first frame is intra frame. For the tests, we use a PC with Intel (R) Core (TM) i3 CPU (2.60GHZ) and 4.00GB RAM running a Windows 7 environment and Microsoft Visual Studio 2010. We apply the proposed method to the encoding of three HDR video test sequences (Market3, Balloon, and Tibul2) with the resolution of 1920×1080 (OpenEXR format) [14]. In (5), we set m1, m2, c1, c2, and c3 to 0.1593, 78.8438, 0.8359, 18.8516, and 18.6875, respectively. Four quantization parameter (QP) points {22, 27, 32, 37} are used to encode each sequence. We measure the bit-rate reductions with respect to the anchor [14]. Moreover, we use Bjøntegaard Delta (BD) rate in [26] to calculate the average bit-rate reduction at the same tPSNR (X, Y, Z, XYZ) and PSNR_DE [14].

TABLE II

BD Rates of [13] and the Proposed Method Compared with Anchor

| Video Sequences | tPSNR X |  | tPSNR Y |  | tPSNR Z |  | tPSNR XYZ |  | PSNR_DE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [13] | Proposed | [13] | Proposed | [13] | Proposed | [13] | Proposed | [13] | Proposed |
| Market3 | -1.49% | -5.27% | -1.19% | -4.30% | -1.41% | -6.81% | -1.37% | -6.15% | -0.67% | -12.42% |
| Balloon | -0.76% | -6.61% | -0.79% | -6.71% | -1.48% | -7.81% | -1.06% | -7.12% | -2.01% | -9.43% |
| Tibul2 | 0.91% | -1.80% | 0.05% | -2.65% | -0.74% | -3.50% | 0.29% | -2.42% | 0.54% | -2.20% |
| Average | -0.45% | -4.56% | -0.64% | -4.55% | -1.21% | -6.04% | -0.71% | -5.23% | -0.71% | -8.02% |

Fig. 5 Visual quality comparison in Balloon. (a) The reconstructed HDR frame by Anchor. (b), (c) Enlarged regions of (a). (d) The reconstructed HDR frame by [13]. (e), (f) Enlarged regions of (d). (g) The reconstructed HDR frame by the proposed method. (h), (i) Enlarged regions of (g).

A. Quantitative Measurements

TABLE I shows the bit-rate reductions by [13] and the proposed method. From TABLE I, it can be observed that the proposed method has better ability to save bit-rate than [13]. The only exception is when QP=37 in Tibul2; in this case, our method has a little bit-rate increment. This is because many regions have similar background luminance in Tibul2 and QP=37 is a large quantization step size. Fig. 4 shows the R-D curves for the three test sequences by Anchor and [13] including the proposed method. From the results, it can be observed that the proposed method has better bit-PSNR XYZ and bit-PSNR_DE performance than Anchor and [13]. That is, the proposed method produces better HDR reconstruction results in perceptual quality than Anchor and [13] under the same bits. TABLE II shows BD rates of [13] and the proposed method compared with Anchor. The metric tPSNR [14] measures intensity distortions between the reconstructed videos and their original ones. The larger the tPSNR is, the better the reconstruction video is. The metric PSNR_DE [14] measures color distortions between the reconstructed videos and their original ones. The larger the PSNR_DE is, the better color preservation the reconstruction video has. From TABLE II, it can be observed that the proposed method achieves more bit-rate reductions than [13] and Anchor under the same perceptual quality (both tPSNR and PSNR_DE). This is because the proposed method utilizes the LA model based on Barten’s CSF which is suitable for HDR videos.

B. Visual Comparisons

Moreover, due to the perceptual bit allocation in different regions, the subjective perceptual quality by the proposed method is better than that by the anchor and [13] at the same low bit-rate level as shown in Fig. 5. Specifically, the proposed method preserves more details and structural information in the orderly regions of the reconstructed frame. In the regions of the red rectangles in Balloon, edges are more obvious than those in the anchor and [13]. Therefore, we can safely conclude that the proposed method successfully improves the perceptual quality of the reconstructed HDR videos using the disorderly concealment effect, and thus can be applied to quadtree-based partitioning of HEVC.

# Conclusions

We have proposed perceptual block merging for HEVC Main 10 Profile-based HDR video coding using Barten CSF model. We have obtained new LA for quantized HDR videos based on Barten CSF model, and generated the FEJND map in each frame. Based on the FEJND map, we have obtained the perceptual Lagrange multiplier in each CU for RDO, and then performed perceptual block merging by minimizing RD costs. Experimental results demonstrate that the proposed method achieves average bit-rate reductions of 5.23% in tPSNR XYZ and 8.02% in PSNR_DE compared with Anchor. Moreover, experimental results show that the proposed method preserves more details and structural information in the orderly regions than them. It is recommended to consider the proposed method in HDR Anchor generation.

# References

[1] F. Kains, R. Bogart, and D. Hess, “OpenEXR image file format,” Proc. ACM SIGGRAPH Sketches & Applications, 2003.

[2] G. W. Larson and R. Shakespeare, Rendering with radiance: The art and science of lighting visualization, Morgan Kaufmann Publishers, 1998.

[3] G.W. Larson, “LogLuv encoding for full-gamut high-dynamic range images,” J. Graph. Tools, vol. 3, pp.15–31, 1998.

[4] A. Motra and H. Thoma, “An adaptive logluv transform for high dynamic range video compression,” Proc. IEEE ICIP, pp. 2061–2064, 2010.

[5] J. U. Garbas and H. Tahoma, “Temporally coherent luminance-to-luma mapping for high dynamic range video coding with H.264/ AVC,” Proc. IEEE ICASSP, pp. 829–832, 2011.

[6] Y. Zhang, E. Reinhard, and D. Bull, “Perception-based high dynamic range video compression with optimal bit depth transformation,” Proc. IEEE ICIP, pp. 1321–1324, 2011.

[7] T. Wiegand, G. J. Sullivan, G. Bjontegaard, and A. Luthra, “Overview of the H.264/AVC video coding standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 13, no. 7, pp. 560–576, 2003.

[8] A. Segall, “Scalable coding of high dynamic range video,” Proc. IEEE ICIP, vol. 1, pp. 1–4, 2007.

[9] R. Mantiuk, A. Efremov, K. Myszkowski, and H. -P. Seidel, “Backward compatible high dynamic range MPEG video compression,” ACM Transactions on Graphics, vol. 25, no. 3, pp. 713–723, 2006.

[10] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, “High Efficiency Video Coding (HEVC) Text Specification Draft 9,” ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC), JCTVC-K1003, Oct. 2012.

[11] J. Wu, G. Shi, W. Lin, A. Liu, and F. Qi, “Just noticeable difference estimation for images with free-energy principle,” IEEE Trans. Multimedia, vol. 15, no. 7, pp. 1705-1710, 2013.

[12] P. G. J. Barten, “Formula for the contrast sensitivity of the human eye,” Proc. SPIE-IS&T, vol. 5294, pp. 231-238, 2004.

[13] C. Jung, Y. Chen, Q. Lin, M. Li, and P. Wu, "Optimization of HEVC Main 10 Coding in HDR Videos Using Perceptual Block Merging," ISO/IEC JTC1/SC29/WG11 MPEG2015/M37073, 2015

[14] A. Luthra, E. Francois, and W. Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding,” ISO/IEC JTC1/SC29/WG11 MPEG2015/N15083, Geneva, Switzerland, February 2015.

[15] ITU-R BT.709-6, “Parameter values for the HDTV standards for production and international programme exchange,” https://www.itu.int/dms_pubrec/itu-r/rec/bt/R-REC-BT.709-6-201506-I!!PDF-E.pdf, 2015.

[16] S. Miller, M. Nezamabadi, and S. Daly, “Perceptual signal coding for more efficient usage of bit codes,” SMPTE Motion Imaging Journal, vol. 122, no. 4, pp. 52-59, 2013.

[17] Society of Motion Picture and Television Engineers, FCD ST 2084, Electro-Optical Transfer Function for High Dynamic Range Reference Display, 2014.

[18] R. Mantiuk, G. Krawczyk, K. Myszkowski, and H. Seidel, “Perception-motivated high dynamic range video encoding,” ACM Trans. Graph., vol. 23 no. 3, pp. 733-741, 2004.

[19] R. Mantiuk, K. Myszkowski, and H. Seidel, “Lossy Compression of high dynamic range images and video,” Proceedings of SPIE, vol. 6057, pp. 311-320, 2006.

[20] A. M. Tourapis, Y. Su, D. Singer, C. Fogg, R. Van der Vleuten, and E. Francois, “Report on the XYZ/HDR Exploratory Experiment 1 (EE1): Electro-Optical Transfer Functions for XYZ/HDR Delivery,” ISO/IEC JTC1/SC29/WG11 MPEG2014/M34165, Sapporo, Japan, July 2014.

[21] R. Boitard and R. Mantiuk, “Evaluation of Color Encodings for High Dynamic Range Pixels,” Proc. SPIE 9394, Human Vision and Electronic Imaging, pp. 93941K-93941K, 2015.

[22] D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, J. Sole, G. Sullivan, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions Text Specification Draft 7,” ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC), JCTVC-Q1005, Apr. 2014.

[23] X. Yang, W. Lin, Z. Lu, E. Ong, and S. Yao, “Just noticeable distortion model and its applications in video coding,” Signal Processing, vol. 20, no. 7, pp. 662-680, 2005.

[24] K. J. Friston, J. Daunizeau, and S. J. Kiebel, “Reinforcement learning or active inference?,” Public Library of Science One, vol. 4, pp. e6421–, 2009.

[25] I.-K. Kim, J. Min, T. Lee, W.-J. Han, and J. Park, “Block partitioning structure in the HEVC standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1697–1706, 2012.

[26] G. Bjøntegaard, “Calculation of average PSNR difference between RD curves,” Proc. VCEG Meeting ITU-T Q.6/SG16, VCEG-M33, Austin, TX, Apr. 2010.

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).