| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0068 |
| --- | --- |

| Title: | CE2-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Jung, Q. Fu, G. Yang Xidian University 2 South Taibai Road Xian 710071, China M. Li, P. Wu No. 6 Huashendadao Road, ZTE R&D Center, Nanjing, 210012, China | Tel: Email: | +86-29-88202654 zhengzk@xidian.edu.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

# Abstract

The perceptual quantization (PQ) for HDR video coding has perceptual uniformity in the luminance range with the modest bit-depth. However, the dynamic range of HDR videos is not fully utilized in PQ, especially PQ for chrominance channels. Thus, there exist the wasted dynamic ranges in PQ which cause detail loss and color distortions. In this contribution, adaptive quantization-based HDR video coding is proposed. Adaptive mapping of sample values with bit depths of 16 bit and 10 bit is established to perform conversion based on cumulative distribution function (CDF), i.e. adaptive quantization, instead of linear mapping. First, CDF is derived from the histogram of each picture. Then, adaptive quantization is performed based on CDF. The metadata for adaptive conversion are coded for performing the inverse conversion at destination. Experimental results demonstrate that the proposed method achieves average 1.7 dB gains in tPSNR XYZ and 2 dB gains in PSNR_DE over Anchor. Subjective comparisons show that the proposed method preserves image details while reducing color distortion.

# Introduction

The human visual system (HVS) can perceive the dynamic range of 10,000,000,000:1 [1]-[4], which far exceeds the standard dynamic range in images and videos that most of existing displays support. Thus, high dynamic range (HDR) imaging aims at presenting the similar range to the human eyes. HDR videos remarkably enhance the visual experience compared to low dynamic range (LDR) ones. However, they need a huge amount of the storage space and transmission bandwidth because of the high bit-depth for represent the dynamic range [5][6]. High Efficient Video Coding (HEVC) is the most recent video coding standard proposed by Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG. HEVC reduces nearly 50% bit rate compared with the last video coding standard of H.264/AVC [7]. To effectively compress HDR videos, HEVC is extended to HDR video coding. In 2015, MPEG has released an anchor framework of compressing HDR and Wide Color Gamut (WCG) video based on HEVC Main 10 Profile. In recent years, researchers have proposed several solutions to HDR video coding. Mantiuk et al. [8][9] proposed a HDR video coding scheme in the hybrid luminance-frequency space. They considered the characteristic of HVS related to the spatial frequency sensitivity and calculated the difference between neighbor pixels, which saved low frequency information and edge information in DCT block and edge block separately.

Fig. 1 Histograms of Y, U, V channels in an HDR video frame. (a) Y channel. (b) U channel. (c) V channel.

Zhang et al. [10] considered the luminance masking effect of HVS and proposed the intensity difference quantization (IDQ) profile which quantized the coding residual with different quantization steps related to the average luminance of one block. Motra et al. [11] proposed an adaptive LogLuv transform method for HDR video coding. In this method, they transformed the RGB color space into the integer LogLuv color space, and minimized the quantization error in the mapping stage. However, we observe that the dynamic range of HDR videos are sometimes not well-used in HDR video coding, especially for chrominance channels, as shown in Fig. 1. That is, wasted dynamic ranges exist in HDR video coding, which causes detail loss and color distortion when we quantize them linearly without considering contents. Therefore, we propose adaptive quantization-based HDR video coding with HEVC Main 10 Profile. The proposed method is based on the Anchor of HDR/WCG CfE, i.e. PQ [1]. We adopt adaptive quantization, i.e. adaptive mapping for 16bit to 10bit conversion based on CDF, instead of linear mapping.

# Proposed Method

Based on the Anchor of HDR/WCG CfE [1], the input HDR videos after pre-processing are compressed by HEVC Main 10 Profile. In the Anchor, luma and color values of the input video are represented by integers and the bit depth is set to 10 bits. That is, the range of available values, i.e. available code words, is restricted to 210=1024. However, the histogram of HDR video frames generally occupies a part of the whole dynamic range, especially in chrominance channels. Thus, we utilize the histogram distributions of HDR video frames to make full use of the dynamic range in HDR video coding based on contents. We perform adaptive quantization, i.e. adaptive mapping for 16bit to 10bit conversion based on CDF, instead of linear mapping, which remarkably improves colors and details of the decoded video frames. Based on the Anchor [1], we propose adaptive quantization based HDR video coding with HEVC Main 10 Profile as illustrated in Fig. 2. In the encoding part (Fig. 2(a)), the input HDR video sequences are successively pre-processed by coding transform function (TF), color space conversion, and float-to-16bit conversion. We replace the original float-to-10bit quantization in Anchor [1] with the float-to-16bit conversion to achieve good quantization performance. After color subsampling, we perform adaptive quantization for 16bit-to-10bit conversion based on contents to make full use of the dynamic range in HDR video frames. At the same time, we generate metadata for inverse mapping, i.e. 10bit-to-16bit conversion, in the decoding part. After that, the transformed 10bit video data is encoded by HEVC encoder and then outputs video bit stream. In the decoding part (Fig. 2(b)), the inverse processes for decoding are performed. First, the input video bit stream is decoded by HEVC decoder. Then, we perform inverse adaptive quantization to convert 10bit data into 16bit data, where metadata-based inverse linear interpolation is applied to obtain the inverse mapping curve. Then, the 16bit data is transformed to float one after color upsampling of 4:2:0 to 4:4:4. Finally, we reconstruct HDR video frames after color space conversion and inverse coding TF.

(a)

(b)

Fig. 2 Block diagram of the proposed adaptive quantization-based HDR video coding with HEVC Main 10 Profile. (a) Encoding. (b) Decoding.

In the proposed method, adaptive quantization is a key process for HDR video coding with HEVC Main 10 Profile, which significantly affects the quality of reconstructed HDR videos. We utilize CDF of video frames as content information for adaptive quantization. CDF at a value x is obtained as follows:

CDF(x)=k=0KPDF(x)                                (1)

PDF(x)=NxN                                   (2)

where PDF() is the probability density function (PDF) of an image, Nx is the number of pixels that have the x value and N is the total number of pixels in the image. Based on the statistical information of CDF(x), we provide an adaptive mapping function for 16bit-to-10bit data conversion, i.e. adaptive quantization, instead of using linear mapping. Inspired by [12], we formulate the adaptive mapping function as follows:

y=(210−1)⋅x216−1a⋅b−CDF(x)                         (3)

where a and b are two parameters to adjust the mapping curve, x denotes the pixel value of 16bit data, and y represents its corresponding 10bit mapped value of x. To reduce the computational complexity and minimize the volume of metadata, we perform partitioning for the dynamic range. In this contribution, the whole 16bit dynamic range of input frame data is equally divided into 64 sections, which generates 65 points, i.e. 0, 1023, 2047, …, and 65535. We calculate the corresponding 10bit mapped values at the 65 points by (3). Based on the 65 mapped values, we perform linear interpolation to obtain the 16bit-to-10bit mapping curve in the whole 16bit dynamic range, i.e. from 0 to 65535 as follows:

y=y1+y2−y1x2−x1⋅x−x1                               (4)

where x represents the pixel value of 16bit image; x1 and x2 are two adjacent dividing points of each section in 16bit dynamic range (x1<x<x2); y1 and y2 are the corresponding 10bit mapped values of x1 and x2. Fig. 3 illustrates an adaptive mapping function for 16bit-to-10bit data conversion. The blue points denote 10bit mapped values from 65 points in 16bit dynamic range. The blue line obtained by linear interpolation in (4) is the mapping curve which is divided into 64 sections from the whole dynamic range. As shown in Fig. 3, a narrow dynamic range [x1, x2] in 16bit data, where most pixels are concentrated, is mapped into a broader dynamic range [y1, y2] in 10bit data. Then, we perform adaptive quantization by mapping the 16bit data into 10bit data according to the mapping curve.

Fig. 3 Adaptive mapping function for 16bit-to-10bit data conversion.

The set of 65 mapped values is metadata, which is used for inverse mapping in the decoding part, i.e. inverse adaptive quantization. In decoding part, we perform the inverse adaptive quantization to convert 10bit data into 16bit data by linear interpolation as follows:

x=x1+x2−x1y2−y1⋅y−y1                            (5)

where x represents the pixel value of 16bit data; x1 and x2 are two adjacent points of each section in 16bit dynamic range (x1<x<x2); y1 and y2 are values in metadata, which represent the corresponding 10bit mapped values of x1 and x2. After the interpolation, we get a mapping curve for inverse adaptive quantization. We convert 10bit data into 16bit data using this mapping curve, and transform it into float data to reconstruct HDR video frames.

# Experimental Results

In the experiments, we evaluate the proposed method on three HDR video sequences using HEVC HM16.2 and HDRTools 0.9 software. We perform experiments using a PC with Intel (R) Core (TM) i3 CPU (3.60GHZ) and 4.00GB RAM running a Windows 7 environment and Microsoft Visual Studio 2013. In our experiments, we set a=1.25, b=1.1 for Y channel, and a=1.5, b=1.1 for U and V channels in (3) because U and V channels generally have a more wasted dynamic range than Y one. To verify the superiority of the proposed method, we perform objective evaluations on the reconstructed HDR videos in terms of tPSNR, mPSNR, and PSNR_DE. The evaluation metrics are recommended to evaluate the performance of HDR video coding in HDR/WCG CfE [1]. tPSNR measures distortions between the original and reconstructed images after PQ/Philips TF, which avoids bias by a particular TF [13]. mPSNR measures the average PSNR values of virtual LDR contents which are obtained from HDR ones by tone mapping [14]. PSNR_DE measures color distortions between the original and reconstructed images in Lab color space [15]. Fig. 4 shows HDR reconstruction results by Anchor [1] and the proposed method with their original HDR video frames. As shown in Fig. 4(a), the reconstructed frame in Tibul produces visible blocking artifacts by Anchor because of using the linear quantization [1]. However, the proposed method effectively removes the blocking artifacts and reconstructs natural-looking results. Furthermore, the line in Balloon is lost in the reconstructed frame by Anchor [1], while it is successfully preserved after decoding by the proposal method (Fig. 4(b)). Also, in Market, the Anchor [1] introduces color distortions in the woman's arm in Market, but the proposed method reconstructs much closer color to the original frame (Fig. 4(c)). This is because the proposed method effectively utilizes the dynamic range of the original frame by adaptive quantization.

(a)                                      (b)                                     (c)

Fig. 4 Visual comparisons of HDR reconstruction results between Anchor [1] and the proposed method. (a) Original video frame. (b) Anchor [1]. (c) Proposed method. Top to bottom: Tibul, Balloon, and Market.

TABLE I

Quantitative comparisons between anchor and the proposed method for HDR video coding

| Sequences | QP | Method | tPSNR-XYZ | mPSNR | PSNR_DE | tPSNR-Yu'v' | tPSNR-Y | tPSNR-u' | tPSNR-v' |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tibul | 37 | Anchor | 34.562 | 29.658 | 34.973 | 35.937 | 34.284 | 34.247 | 50.203 |
|  |  | Proposed | 37.302 | 32.012 | 36.780 | 39.196 | 36.958 | 38.325 | 52.180 |
|  | 32 | Anchor | 36.226 | 31.247 | 35.779 | 36.931 | 36.057 | 34.586 | 50.801 |
|  |  | Proposed | 39.248 | 33.833 | 37.711 | 40.698 | 38.956 | 39.235 | 53.735 |
|  | 27 | Anchor | 38.355 | 33.193 | 36.791 | 38.354 | 38.317 | 35.508 | 51.835 |
|  |  | Proposed | 41.622 | 35.978 | 38.871 | 42.364 | 41.456 | 40.149 | 55.589 |
|  | 22 | Anchor | 40.884 | 35.471 | 37.997 | 40.064 | 41.053 | 36.774 | 53.120 |
|  |  | Proposed | 44.343 | 38.407 | 40.181 | 43.980 | 44.426 | 40.956 | 57.594 |
| Market | 37 | Anchor | 31.309 | 29.112 | 30.472 | 34.437 | 31.868 | 35.728 | 37.918 |
|  |  | Proposed | 32.232 | 29.884 | 32.959 | 36.157 | 32.442 | 39.830 | 42.798 |
|  | 32 | Anchor | 33.770 | 31.413 | 31.134 | 36.448 | 34.563 | 36.885 | 39.042 |
|  |  | Proposed | 34.864 | 32.342 | 33.740 | 38.487 | 35.184 | 41.008 | 43.747 |
|  | 27 | Anchor | 36.152 | 33.645 | 31.914 | 38.358 | 37.138 | 37.947 | 40.340 |
|  |  | Proposed | 37.640 | 35.002 | 34.537 | 40.881 | 38.103 | 42.382 | 44.947 |
|  | 22 | Anchor | 39.417 | 36.724 | 32.967 | 40.896 | 40.855 | 39.856 | 42.343 |
|  |  | Proposed | 40.842 | 37.860 | 35.320 | 43.435 | 41.566 | 43.749 | 46.215 |
| Balloon | 37 | Anchor | 32.784 | 30.737 | 32.311 | 35.339 | 34.221 | 34.689 | 38.026 |
|  |  | Proposed | 33.428 | 30.967 | 35.253 | 37.202 | 34.091 | 39.369 | 41.852 |
|  | 32 | Anchor | 35.233 | 32.967 | 33.455 | 37.333 | 37.125 | 36.099 | 39.441 |
|  |  | Proposed | 35.871 | 33.155 | 36.188 | 39.312 | 36.749 | 40.656 | 42.720 |
|  | 27 | Anchor | 37.848 | 35.333 | 34.595 | 39.527 | 40.234 | 37.934 | 41.071 |
|  |  | Proposed | 38.622 | 35.503 | 37.156 | 41.671 | 39.777 | 42.281 | 44.070 |
|  | 22 | Anchor | 40.517 | 37.723 | 35.865 | 41.826 | 43.335 | 40.077 | 42.852 |
|  |  | Proposed | 41.518 | 37.905 | 38.120 | 44.035 | 43.099 | 43.912 | 45.457 |

To quantitatively analyze the performance of the proposed method, we evaluate the performance of Anchor [1] and the proposed method in terms of tPSNR, mPSNR, and PSNR_DE. TABLE I lists the average values of tPSNR, mPSNR, and PSNR_DE on three HDR video sequences (46 frames for each sequence) with QP=22, 27, 32, and 37. When we evaluate tPSNR for U and V channel, we adopt  tPSNR-u’ and tPSNR-v’ because the original Y, U, V channels have been converted into the transform function domain Y, u’, v’ by PQ-TF. It can be observed that the proposed method achieves more than 2dB gains in PSNR_DE over Anchor [1]. The results indicate that the proposed method produces lower color distortions than Anchor. Above all, in the PSNR results of chrominance channels (u’ and v’), the proposed method performs much better, i.e. more than 3dB gain, in each channel.

Fig. 5 The R-D curve performance. (a) Bit-PSNR Y curve in Market. (b) Bit-tPSNR U curve in Market. (c) Bit-PSNR V curve in Market. (d) Bit-tPSNR_DE curve in Market.

We achieve about 1.7dB gains in tPSNR-XYZ, 2.6dB gains in tPSNR-Yu’v’, and 1.3dB gains in mPSNR by the proposed method, which demonstrates that the proposed method achieves good reconstruction results with low distortions in intensity and color.

Fig. 5 shows the R-D curve performance in Market in terms of Bit-PSNR Y, Bit-PSNR U, Bit-PSNR V, and PSNR-DE. As shown in the figure, although there is a tPSNR improvement in Y channel by the proposed method, the performance improvement is a little and the coding efficiency of the proposed method is worse than Anchor. However, U and V channels are much different from Y channel. In U and V channels, the tPSNR performance even at QP=37 in the proposed method is slightly better than that at QP=22 in Anchor. That is, the proposed method is very effective in improving the perceptual quality of the chrominance channels, which lead to enhancing both colors and details in HDR reconstruction results. Also, tPSNR_DE verifies them. This is because the proposed method adopts adaptive quantization, adaptive mapping for 16bit to 10bit conversion based on CDF, instead of linear mapping in Anchor. Consequently, we can safely conclude that the proposed method successfully preserves image details while remarkably reducing color distortions, and outperforms Anchor for HDR video coding.

# Conclusions

In this contribution, we have proposed adaptive quantization-based HDR video coding with HEVC Main 10 Profile. We have performed adaptive quantization for 16bit to 10bit conversion based on CDF of the original HDR frame to effectively use the dynamic range of HDR video frames. Experimental results demonstrate that the proposed method outperforms Anchor for HDR video coding in detail preservation and color reproduction, and achieves a significant improvement in performance in terms of tPSNR, mPSNR, and PSNR_DE. Furthermore, the proposed method successfully removes blocking artifacts caused by narrow dynamic range of HDR videos at high compression ratio in Anchor. The proposed method mainly improves the chrominance, and can be combined with adaptive PQ (another contribution: M37613) which focuses on improving the luminance. Also, it is recommended to set-up a sub experiment in CE2 on content adaptive HDR video coding.

# References

[1] A. Luthra, E. Francois and W. Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding,” ISO/IEC JTC1/SC29/WG11 MPEG2015/N15083, Geneva, Switzerland, February 2015.

[2] J. Ferwerda, “Elements of early vision for computer graphics,” IEEE Computer Graphics and Applications, vol. 21, no. 5, pp. 22-33, 2001.

[3] Y. Dong, P. Nasiopoulos, and M. T. Pourazad, “HDR video compression using high efficiency video coding (HEVC),” Proc. International Conference on Mobile Ubiquitous Computing, Systems, Services and Technologies, pp. 205-2009, 2012.

[4] J. J. McCann, “Art, science, and appearance in HDR,” Journal of the Society for Information Display, vol. 15, no. 9, pp. 709-719, 2007.

[5] P. Debevec, E. Reinhard, G. Ward, and S. Pattanaik, “High dynamic range imaging,” ACM SIGGRAPH 2004 Course Notes, 2004.

[6] Y. Zhang, D. Agrafiotis, and D. R. Bull, “High Dynamic Range image & video compression a review,” IEEE International Conference on Digital Signal Processing (DSP), pp. 1-7, 2013.

[7] G. J. Sullivan, J. R. Ohm, W. J. Han, and T. Wiegand, “Overview of the high efficiency video coding (HEVC) standard,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 22, no. 12, pp. 1649-1668, 2012.

[8] R. Mantiuk, G. Krawczyk, K. Myszkowski, and H.-P. Seidel, “High dynamic range image and video compression-fidelity matching human visual performance,” Proc. IEEE International Conference on Image Processing (ICIP), pp. 9-12, 2007.

[9] R. Mantiuk, G. Krawczyk, K. Myszkowski, and H.-P. Seidel, “Perception-motivated high dynamic range video encoding,” ACM Transactions on Graphics, vol. 23, no. 3, pp. 733-741, 2004.

[10] Y. Zhang, M. Naccari, D. Agrafiotis, M. Mrak, and D. Bull, “High dynamic range video compression by intensity dependent spatial quantization in HEVC,” Proc. IEEE Picture Coding Symposium, pp. 353-356, 2013.

[11] A. Motra and H. Thoma, “An adaptive LogLuv transform for high dynamic range video compression,” Proc. IEEE International Conference on Image Processing (ICIP), pp. 2061-2064, 2010.

[12] S. C. Huang, F. C. Cheng, and Y. S. Chiu, “Efficient contrast enhancement using adaptive gamma correction with weighting distribution,” IEEE Transactions on Image Processing, vol. 22, no. 3, pp. 1032-1041, 2013.

[13] A. Banitalebi-Dehkordi, M. Azimi, and M. T. Pourazad, “Saliency-Aided HDR Quality Metrics," ISO/IEC JTC1/SC29/WG11/MPEG2014 /m37317, Geneva, Switzerland, 2015.

[14] J. Strom and J. Samuelsson, “Investigation of mPSNR as an error measure for HDR video,” ISO/IEC JTC1/SC29/WG11/MPEG2014 /M34148, Japan, July.2014.

[15] Bruce Justin Lindbloom, Delta E (CIE2000), [Online] http://www.brucelindbloom.com/index.html?Eqn_DeltaE_CIE2000.html, 2009.

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).