| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0069 |
| --- | --- |

| Title: | CE5: Results of the Core Experiment CE5.3.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Brondijk Jeroen Stessen Wiebe de Haan |  | robert.brondijk@philips.com Jeroen.stessen@philips.com Wiebe.de.haan@philips.com |
| Source: | Philips |  |  |

_____________________________

# Abstract

This document describes the core experiment CE5.3.1. In this core experiment, the quality of Y”u”v” is investigated when used with the ST.2084 instead of the Philips Transfer Function. The purpose is to compare the results of this experiment to other pixel representations identified in CE5.

# Introduction

The Y”u”v” pixel format was first described as an MPEG contribution in m36461 and later in m37063, when it was integrated in HDRTools. In this core experiment, yet a more efficient version of the code has been created for HDRTools. In this most recent version, the bit-depth of u”v” as the transfer function can be tailored in by a user of HDRTools.

Figure 1 Y"u"v" Processing

Figure 1 shows the implementation of Y”u”v” as realized in HDRTools. In this implementation, the entire content is subsampled to a quarter resolution in RGB linear light. This is then transformed from RGB to u”v” via u’v’ and quantized to either 9 or 10 bits.

# Comparing Color Spaces

The purpose of CE5 is to compare color spaces. One of the questions is on basis of what criteria the colors spaces are to be compared. The most important basis for comparison is the subjective results. However, for the color spaces whose subject results are similar, additional traits must be used for comparison.   Below is a proposed set of differentiating traits of a color space.

Table 1 Differentiating Aspects of a Color Space

|  | Y”u”v” | Y”CbCr | Y”FbFr | IPT-PQ |
| --- | --- | --- | --- | --- |
| Color space | Contains all visible colors | Restricted to a BT.2020 gamut | Restricted to a BT.2020 gamut | Contains all visible colors |
| Luminance in Color | Uncoupled: except for low luminance to avoid coding of dark noise | Coupled: non-constant luminance | Coupled: non-constant luminance | Y is luminance, but the chroma correlates with the luminance |
| Constant-luminance issues due to 4:2:0 | No problem, almost 4:4:4 quality | Major problem near the edge of BT.2020 gamut | Major problem near the edge of BT.2020 gamut | Minor problem due to larger color gamut |
| Chromaticity for gamut mapping | Very easy to make (u’,v’) for gamut mapping | Not easy, must go through linear-light | Not easy, must go through linear-light | Not easy, must go through linear-light |
| Representation | Representation in maximum needed for professional viewing: 10 bits per component, independent of the luminance. Only 9 is needed in the current CE case, where video is limited and the bit-rates are low. | Depends on the luminance range: all three equal 10 bits, later 12 as luminance level increases | Depends on the luminance range: all three equal 10 bits, later 12 as luminance level increases | Depends on the luminance range: color may now be 9 bits, benefits from “re-shaping” = a form of dynamic tone mapping |
| Transmitter complexity | Medium, high bit-widths are needed to support linear light operations, but the operations themselves are of low complexity | Low, traditional processing with OETF replaced by ST.2084 PQ | Low, just a variation of Y”CbCr | Medium, just re-programming of existing matrices, but dynamic re-shaping is needed |
| Receiver complexity | Can be low, no linear-light needed, mostly lookup tables | Can be low, unless color space conversion is needed | Can be low, unless color space conversion is needed | High, always re-shaping and color space conversion needed |

# Objective Results

The table below shows the results of the objective metrics. For the Y”u”v’ system, 10bits standard range was used for Y” and 9bits standard range was used for u”v”.

# Observations

When the QP, that is needed to match the bit-rate of the anchors, is much higher than QP that was used for the anchors, Y”u”v” showed more coding artifacts than the anchors. This causes the subjective quality of Y”u”u” to become less than the anchors.  The coding artifacts in Y”u”v” can be reduced by reducing the needed QP and, to match the bit-rate, simultaneously decreasing the bit-width of either the u”v” or Y” or both.  Reducing the bit-widths of u”v” to 9bits will reduce the uncompressed quality, but increases the quality of compressed, because the QP can be decreased. For some content, even the bit-width of Y” can be reduced to decrease the QP and thereby increasing the quality, and thereby matching the visual quality of the anchors.

However, reducing the number of bits used for Y to 9 does not seem to be possible in the current version of the HM.  The only way to reduce the number of bits for Y is to reduce the number of “active” bits used in the 10 bit word. For uv, the HM does support 9bits, but the yuv created by the encoder is often corrupt. The decoder however seems to be able to create valid yuv files.

It is a well known trade off in encoding: one can reduce the compression factor by pre-filtering, but pre-filtering reduces the contrast. The trade off is coding artifacts versus contrast.

HM Encoding tools:

- QP

- QPoffset

- (new) Control when the QPoffset is stopped, so as to not over quantize the chrominance

For YCbCr, the coding tools mentioned above seem to be adequate. This may have everything to do with the amplitude correlation between Cb,Cr and Y, this is because there is luminance information in Cb and Cr.  Y” u”v”, on the other hand, are already totally uncorrelated. That means that they can be compressed separately of each other.

What does work for Y”u”v”, is to use the following, more primitive tools, to reduce the bit-rate:

Pre-Processing Tools:

- Bit-width of Y : 10 or 9 bits

- Bit-range of Y : full or standard

- Bit-width of uv : 10,9 or 8 bits

However, due to issues in the HM 16.7, setting the Y bit-depth to 9 is not supported. Also, when setting the color bit-widths to 9, the yuv created by the encoder is often corrupted, while that created by the decoder is mostly ok.

By using only 9 of the 10 bits the HEVC encoder can be tricked.  Depending on the sequence the resulting picture became “on-par” with the anchor. For example the sequences SunRise, EBU Starting and EBU Hurdles are showing the same quality as the anchors. We expect therefore that 9 bits Y will show an improvement, but we could not test this in the prescribed way and for all sequences.

# Conclusion

The experiment have shown that Y”u”v” pixel representation can match the quality of the anchors with less bits. How much less, is dependent on the bit-rate and content. Some preliminary simulations are showing that for some content, like the EBU, the Y” can even go down to 9 bits; however because the HEVC encoder does not support 9bits Y, it had to be simulated by using only the 9 LSB bits of a 10 bit word.

In general the Y”u”v” representation is offering the same quality for less bits.

References:

- C. POYNTON, J. STESSEN, AND R. NIJLAND (2014), “Deploying Wide Colour Gamut and High Dynamic Range in HD and UHD” (IBC Conference 2014).

- C. POYNTON, J. STESSEN, AND R. NIJLAND , SMPTE Motion Imaging Journal April 2015,“Deploying Wide Color Gamut and High Dynamic Range in HD and UHD”