| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0071 |
| --- | --- |

| Title: | CE2-related: Adaptive PQ: Adaptive Perceptual Quantizer for HDR video Coding with HEVC Main 10 Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Jung, S. Yu, P. Ke Xidian University 2 South Taibai Road Xian 710071, China M. Li, P. Wu No.6 Huashendadao Road, ZTE R&D Center, Nanjing 210012, China | Tel: Email: | +86-29-88202654 zhengzk@xidian.edu.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

# Abstract

Proposed is an adaptive transform function based perceptual quantizer (PQ) for HDR videos, called adaptive PQ. Adaptive PQ is to solve the problem that PQ is not adaptive to HDR content because of its using a fixed mapping function from luminance to luma. By introducing a ratio factor derived from the luminance information of HDR content, adaptive PQ maps luminance to luma adaptively according to the content. Experimental results demonstrate that adaptive PQ achieves average bit-rate reductions of 3.51% in tPSNR XYZ and 5.51% in PSNR_DE over PQ.

# Introduction

With the rapid advances in the HDR technology, efficient storage and transmission of such data are crucial for HDR value chain. Several methods have been proposed to handle this challenge. First, backward compatible approaches [1][2] have been proposed to compress HDR videos. In the backward compatible approaches, HDR videos are divided into two layers: base layer of low dynamic range (LDR) data and enhancement layer of residual data. LDR data is obtained by tone mapping on the HDR videos, while residual data is used to reconstruct the HDR videos. Second, lossless HDR image formats such as Radiance [3], OpenEXR [4], and LogLuv [5] have been proposed to store floating-point colorimetric HDR data [6][7]. Although lossless formats accurately record luminance in the scene, floating-point data needs huge memory to store. Moreover, compared with integer, floating-point data is inefficient to encode. In general, the input data is integer in digital image and video processing. To widely use HDR contents, floating-point data of HDR pixels should be transformed into integer data. Aiming at solving the problem, several perceptual transfer functions (TFs) for HDR contents have been proposed to convert floating-point luminance to integer data [8]-[13]. In [9], the integer data is luma which means the perceptually encoded and quantized luminance.

(a)

(b)

Fig. 1 Whole framework of HDR video coding based on adaptive PQ. (a) Encoding. (b) Decoding

Mantiuk et al. [8] first proposed perceptual TF (PTF) based on the threshold versus intensity (t.v.i.) model. To offer a more general model and improve the compression efficiency, Mantiuk et al. [9] presented several properties for the color space. For instance, a unit distance in the color space correlates with the just-noticeable-difference (JND), and a half-unit distance in the color space is below 1 JND). Moreover, they proposed several PTFs based on t.v.i. and more complex contrast sensitivity function (CSF) models. Based on Barten CSF model [14], Perceptual Quantizer (PQ) TF was presented in [10][11]. [12] and [13] provided various comparisons and evaluations between several PTFs. They evaluated them in terms of the perceptual uniformity and the minimum bit-depth [15]. The perceptual uniformity was obtained if a small perturbation to a component value was equally perceptible across the range of luminance. The minimum bit-depth reflected where quantization distortions remain invisible for any luminance. Compared with other PTFs, PQ had the best perceptual uniformity in the considered luminance range. Moreover, the bit-depth of PQ was the modest, and the 12-bit PQ was recommended in [10], while the results showed that the bit-depth of PQ was less than 12 in [13]. The difference in the bit-depth was mainly caused by the different testing environments in [10] and [13]. Thus, PQ has been selected as Anchor for HDR/WCG video coding [16]. In the 12-bit PQ [10], the encoded luminance range is fixed from 10-6 cd/m2 to 104 cd/m2. However, through various experiments, we have discovered that most HDR videos do not cover the full luminance range that PQ supports. Therefore, the inconsistent dynamic range between HDR videos and PQ causes inefficiency of HDR video coding based on HEVC Main 10 Profile. In [17], Motra and Thoma proposed adaptive PTF based on LogLuv transform [5]. They utilized the complete quantization interval of the encoder, and thus efficiently compressed HDR videos. Inspired by this method, we propose adaptive PTF based on PQ, called adaptive PQ, for HDR video coding with HEVC Main 10 Profile. In adaptive PQ, PTF is adjusted according to the HDR content. Different from PQ which uses a fixed mapping function from luminance to luma, the proposed PTF adaptively map luminance to luma according to contents. Compared with PQ, adaptive PQ has two main advantages as follows:

1) The proposed PTF adaptively maps luminance to luma according to HDR contents. Since most HDR videos do not cover the full luminance range that PQ supports, adaptive PQ utilizes wider quantization interval than PQ. Thus, quantization distortions by adaptive PQ are smaller than those by PQ.

2) Bit-depth of adaptive PQ is the same as that of PQ, but adaptive PQ has better perceptual uniformity in the luminance range than PQ.

# Proposed Method

Fig. 1 illustrates the whole framework of HDR video coding based on adaptive PQ. In the encoding part, the input of the HDR video coding system is an HDR video sequence in the linear domain with 4:4:4 RGB color space with the floating-point accuracy. First, we transform the input video sequence from RGB to YCbCr color space, and extract the maximum and minimum luminance of each frame, Ymax and Ymin. From Ymax and Ymin, we calculate the adjustment factor ratio. Then, we adjust adaptive PQ based on ratio, and perform adaptive coding TF on HDR video frames. Since ratio is different for each frame, it is sequentially updated in each frame. To use ratio as the metadata for HDR video coding, we add ratio in the bit stream, more specifically in the Video Usability Information (VUI) syntax element [18]. Next, the video sequence is processed by color space conversion from R’G’B’ to Y’CbCr, quantization and chroma down-sampling to get the standard input format, i.e. 10-bit, 4:2:0, non-constant luminance, and Y’CbCr, for HEVC Main 10 coding [16]. The processed sequence is encoded and decoded by HEVC Main 10 Profile.

In the decoding part, the decoded sequence is processed by chroma up-sampling, inverse quantization (10bit-to-16bit), color space conversion (Y’CbCr to R’G’B’) [16]. Then, we adjust inverse PQ based on the metadata ratio [18], and perform inverse adaptive PQ for HDR video reconstruction. In inverse PQ [10], the relationship between normalized luma and luminance is given. Let N denote the normalized luma, L denotes the normalized luminance. We would like to find out the relationship between normalized luminance and JND. Since JND is related to luminance rather than luma, we consider inverse PQ transfer function f as follows:

(1)

where m1, m2, c1, c2, and c3 are set as 0.1593, 78.8438, 0.8359, 18.8516, and 18.6875, respectively. Because N represents the normalized luma,  only when N=0. Thus, we simplify (1) for  N>0 as follows:

(2)

Let Ni and Ni+1 denote adjacent normalized luma; Li and Li+1 denote their corresponding normalized luminance. We have , and the normalized quantization step  is a positive constant. Also, we have and . By using the 1st-order Taylor series expansion, we obtain as follows:

f(Ni+1)≈f(Ni+ΔN2)+ΔN2⋅f'(Ni+ΔN2)             (3)

f(Ni)≈f(Ni+ΔN2)−ΔN2⋅f'(Ni+ΔN2)              (4)

where f’ means the derivative of f. In PQ [10], Barten CSF model [14] shows the relationship between luminance and JND [10], and thus we get the JND, mt, from luminance by its inverse representation, i.e. mt=1/CSF, as follows:

mt=f(Ni+1)−f(Ni)f(Ni+1)+f(Ni)≈g1⋅g2(Ni)⋅g3                   (5)

where

(6)

(7)

(8)

where a value g1 is related to normalized quantization step N because N is determined only by bit-depth [19] (the bit-depth is 10 in HEVC Main 10 Profile); therefore, g1 is a constant; g2(Ni) is highly dependent on Ni, according to the derivation in [10], which is parallel to the JND, i.e. g2(Ni) is equal to JND multiplied by a constant of less than 1. In (3), a value g3 is only related to the constant parameters [10]. For PQ [10], if the covered luminance range is from 10-6 cd/m2 to 102 cd/m2, then the precision of each luma (i.e. the half-unit distance in the color space) is 0.46 JNDs; if the covered luminance range is from 10-6 cd/m2 to 103 cd/m2, then the precision of each luma is 0.68 JNDs; if the covered luminance range is from 10-6 cd/m2 to 104 cd/m2, then the precision of each luma is 0.9 JNDs. Thus, the precision is closely related to the encoded luminance range. Based on this observation, we obtain a new parameter ratio which is determined by the luminance range of the HDR content as follows:

ratio=0.46+0.22⋅[log10(Ymax−Ymin)−2]0.9                      (9)

where Ymax and Ymin are the maximum and minimum luminance of the HDR content, respectively, log10() is the logarithmic function with base of 10. From Eqs. (5)-(8), it can be observed that all parameters of m2, c1, c2 and c3 contribute to g2(Ni), which is parallel to the JND. Thus, if they are adjusted by ratio, the perceptual uniformity of PQ is destructed [10][13]. However, since m1 only contributes to g3, if we use ratio to adjust it, the perceptual uniformity of PQ still remains. Moreover, it is adaptive to HDR content. Therefore, we use ratio to adjust m1, and obtain adaptive PQ as follows:

(10)

Besides, the corresponding inverse adaptive PQ is obtained as follows:

(11)

where

(12)

# Experimental Results

To verify the accuracy and efficiency of the proposed method, we perform experiments in the HEVC test model (HM16.2) Main 10 Profile low delay configuration, which uses a hierarchical P picture structure with a Group of Pictures (GOP) size of 4 frames and only the first frame is Intra frame. For the tests, we use a PC with Intel (R) Core (TM) i3 CPU (2.60GHZ) and 4.00GB RAM running a Windows 7 environment and Microsoft Visual Studio 2010. We apply adaptive PQ to the encoding of four HDR video test sequences: Market3, Tibul2, Balloon, and FireEater2 with the resolution of 1920×1080 (openEXR format) [16]. Four quantization parameter (QP) points {22, 27, 32, 37} are used to encode each sequence. In (11), m1, m2, c1, c2 and c3 are set to 0.1593, 78.8438, 0.8359, 18.8516, and 18.6875, respectively, the same as [10]. We use Bjøntegaard Delta (BD) rate in [20] to calculate the average bit-rate reduction at the same tPSNR (X, Y, Z, XYZ) and PSNR_DE [16] as shown in TABLE I. tPSNR [16] measures the distortions between the reconstructed and the original videos; therefore, the larger the tPSNR is, the better the reconstructed content is. Different from PSNR which is computed in the spatial domain, tPSNR is computed in the transform domain.

TABLE I

BD Rates of Adaptive PQ in Comparison with Anchor

| Video Sequences | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | PSNR_DE |
| --- | --- | --- | --- | --- | --- |
| Market3 | -3.97% | -3.77% | -2.70% | -3.37% | -7.44% |
| Tibul2 | -3.38% | -2.57% | -3.19% | -3.17% | -5.04% |
| Balloon | -2.39% | -1.69% | -1.14% | -1.60% | -3.43% |
| FireEater2 | -4.18% | -5.27% | -6.14% | -5.88% | -6.12% |
| Average | -3.48% | -4.43% | -4.39% | -3.51% | -5.51% |

(a)                                          (b)

(c)                                          (d)

Fig. 2 Visual quality comparison in Market3. (a) Reconstructed HDR frame by Anchor. (b) Enlarged region of (a). (c) Reconstructed HDR frame by adaptive PQ. (d) Enlarged region of (c).

(a)                                   (b)           (c)

(d)                                   (e)           (f)

Fig. 3 Visual quality comparison in Balloon. (a) Reconstructed HDR frame by Anchor. (b) and (c) are enlarged regions of (a). (d) Reconstructed HDR frame by adaptive PQ. (e) and (f) are enlarged regions of (d).

That is, the reconstructed and the original videos are perceptually encoded by PTF in tPSNR. Therefore, tPSNR is suitable for perceptually assessing the quality of HDR videos. PSNR_DE [16] measures the color distortions between the reconstructed and the original videos. Therefore, the larger the PSNR_DE is, the better color preservation the reconstructed content has. TABLE I presents BD rates of adaptive PQ in comparison with Anchor [16]. From TABLE I, it can be observed that the proposed method achieves bit-rate reductions compared with Anchor under the same tPSNR (X, Y, Z, and XYZ). Moreover, under the same PSNR_DE, the proposed method achieves bit-rate reductions from 3.43% to 7.44% (average: 5.51%). The bit-rate reductions under the same tPSNR (X, Y, Z, XYZ) is from the fact that the proposed PTF utilizes the quantization interval more efficiently than PQ. Bit-rate reductions under the same PSNR_DE are from the fact that the proposed PTF has better perceptual uniformity in the luminance range than PQ. Since the proposed PTF can more efficiently utilize the quantization interval than PQ, quantization distortions by the proposed PTF are smaller than those by PQ, which means adaptive PQ preserves more details and structural information than Anchor. Moreover, since the proposed PTF has better perceptual uniformity in the luminance range than PQ, the proposed method achieves better color preservation than Anchor.

Figs. 2 and 3 provide visual quality comparisons between Anchor and adaptive PQ in Market3 and Balloon, respectively. As shown in Fig. 2, adaptive PQ produces clearer image details than Anchor, while preserving colors better than Anchor (see the region in the blue rectangle). As shown in Fig. 3, the similar results to Market3 are observed in Balloon (see the enlarged regions). Especially, adaptive PQ in Fig. 3(e) effectively preserves color information better than Anchor in Fig. 3(b). Also, adaptive PQ in Fig. 3(f) successfully reconstructs image details than PQ in Fig. 3(c).

# Conclusions

We have proposed adaptive PQ for HDR video coding with HEVC Main 10 Profile. Adaptive PQ is based on PQ and adaptively maps luminance to luma because its PTF is adjusted according to the HDR content. First, we have extracted the maximum and minimum luminance of the HDR content, and used them to calculate the adjustment factor for coding TF. Then, we have adjusted PQ based on the adjustment factor, called adaptive PQ. Since the adjustment factor is adaptive to HDR content, we have obtained it per frame. We have used the adjustment factor as the metadata, and saved it in the VUI syntax element. In comparison with PQ, adaptive PQ utilizes the quantization interval more efficiently than PQ, which is useful for reducing the quantization distortions. Moreover, adaptive PQ has better perceptual uniformity in the luminance range than PQ, which contributes to the color preservation. Experimental results demonstrate that adaptive PQ achieves a significant performance improvement in HDR video coding over PQ in terms of bit-rate reduction, detail preservation, and color reproduction. Adaptive PQ mainly improves the luminance, and can be combined with adaptive quantization-based method (another contribution: M37612) which focuses on enhancing the chrominance. Also, it is recommended to set-up a sub experiment in CE2 on content adaptive HDR video coding.

# References

[1] R. Mantiuk, A. Efremov, K. Myszkowski, and H. -P. Seidel, “Backward Compatible High Dynamic Range MPEG Video Compression,” ACM Trans. Graph., vol. 25, no. 3, pp. 713-723, 2006.

[2] Z. Mai, H. Mansour, R. Mantiuk, P. Nasiopoulos, R. Ward, and W. Heidrich, “Optimizing a Tone Curve for Backward-Compatible High Dynamic Range Image and Video Compression,” IEEE Trans. Image Process., vol. 20, no. 6, pp. 1558-1571, 2011.

[3] G. W. Larson and R. Shakespeare, Rendering With Radiance: The Art and Science of Lighting Visualization, Morgan Kaufmann Publishers, 1998.

[4] F. Kains, R. Bogart, and D. Hess, “OpenEXR Image File Format,” Proc. ACM SIGGRAPH Sketches & Applications, 2003.

[5] G. W. Larson, “LogLuv Encoding for Full-Gamut High-Dynamic Range Images,” J. Graph. Tools 3, pp.15-31, 1998.

[6] G. W. Larson, “High Dynamic Range Image Encodings,” [online]. Available: www.anyhere.com/gward/hdrenc/hdr_encodings.html.

[7] E. Reinhard, High Dynamic Range Imaging: Acquisition, Display and Image-Based Lighting, Morgan Kaufmann Publishers, 2006.

[8] R. Mantiuk, G. Krawczyk, K. Myszkowski, and H. -P. Seidel, “Perception-motivated High Dynamic Range Video Encoding,” ACM Trans. Graph., vol. 23, no. 3, pp. 733-741, 2004.

[9] R. Mantiuk, K. Myszkowski, and H. Seidel, “Lossy Compression of High Dynamic Range Images and Video,” Proceedings of SPIE, vol. 6057, pp. 311-320, 2006.

[10] S. Miller, M. Nezamabadi, and S. Daly, “Perceptual Signal Coding for More Efficient Usage of Bit Codes,” SMPTE Motion Imaging Journal, vol. 122 no. 4, pp. 52-59, 2013.

[11] Society of Motion Picture and Television Engineers, FCD ST 2084 (2014), Electro-Optical Transfer Function for High Dynamic Range Reference Display.

[12] A. M. Tourapis, Y. Su, D. Singer, C. Fogg, R. van der Vleuten, and E. Francois, “Report on the XYZ/HDR Exploratory Experiment 1 (EE1): Electro-Optical Transfer Function for XYZ/HDR Delivery”, ISO/IEC JTC1/SC29/WG11 MPEG2014/M34165, Sapporo, Japan, July 2014.

[13] R. Boitard and R. Mantiuk, “Evaluation of Color Encodings for High Dynamic Range Pixels,” Proc. SPIE, vol. 9394, pp. 93941K-93941K, 2015.

[14] P. G. J. Barten, “Formula for the contrast sensitivity of the human eye,” Proc. SPIE-IS&T, vol. 5294, pp. 231-238, 2004.

[15] C. Poynton and B. Funt, “Perceptual uniformity in digital image representation and display,” Color Research & Application, vol. 39, no. 1, pp. 6-15, 2014.

[16] A. Luthra, E. Francois, and W. Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding,” ISO/IEC JTC1/SC29/WG11 MPEG2015/N15083, Geneva, Switzerland, February 2015.

[17] A. Motra and H. Thoma, “An Adaptive LogLuv Transform for High Dynamic Range Video Compression,” In Proceedings of IEEE ICIP, pp. 2061-2064, 2010.

[18] G. J. Sullivan, J. -R. Ohm, W. –J. Han, and T. Wiegand, “Overview of the High Efficient Video Coding (HEVC) Standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1649-1668, 2012.

[19] ITU-R BT.709-6 (2015), “Parameter values for the HDTV standards for production and international programme exchange,” https://www.itu.int/dms_pubrec/itu-r/rec/bt/R-REC-BT.709-6-201506-I!!PDF-E.pdf.

[20] G. Bjøntegaard, “Calculation of average PSNR difference between RD curves,” Proc. 13th VCEG Meeting ITU-T Q.6/SG16, Doc. VCEG-M33.doc., Austin, TX, Apr. 2010.

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).