| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0075 |
| --- | --- |

| Title: | Palette encoder improvements for the 4:2:0 chroma format and lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Guillaume Laroche, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

At the 22nd JCT-VC meeting in October 2015, the proponents’ non-normative modifications for palette lossy coding of non-4:4:4 content was adopted. It was asserted then that the fact superfluous chroma samples are coded was not properly taken into account, resulting in suboptimal coding. It is asserted that similar considerations affect lossless encoding. In addition, it is proposed that the encoder tracks whether an entry is only associated with pixels with superfluous chroma samples. Finally, it is asserted that various palette evaluations need not be run, the removal of which is asserted to result in encoder runtime improvements. It is reported that the non-normative changes provide, for 4:2:0 content, up to 1.1%, 0.5% and 0.5% BDR gains, for runtimes of 90%, 94% and 95%, for respectively the AI, RA and LDB lossless configurations.

# Description

## Normative decoding algorithm as of Draft Text 5

The following description is extracted for the reader’s convenience from [1], although the proposal is similar in a number of ways to various other proposals from Qualcomm or InterDigital.

In the adopted implementation, a YUV420 palette-coded CU is treated as YUV444 in the way that Chroma values in position 1, 2, 3 are padded with the Chroma value in position 0, as in Figure 1. Then the usual 4:4:4 palette derivation and coding processes are applied.

Figure 1 Pad position 1, 2, 3 Chroma value with Chroma value in position 0 for YUV420 CU

During the reconstruction, Chroma values in position 1, 2 and 3 are ignored, such that a YUV420 CU is reconstructed. In one approach, when Escape happens at positions 1, 2 and 3, the Chroma values are signalled, but discarded during the reconstruction process. As a result, the current palette syntax remains the same [as for 4:4:4]. In the other approach, when Escape happens at positions 1, 2 and 3, the Chroma values are not signalled as they will not be used in reconstruction process.

Furthermore, it was noted, e.g. in Error: Reference source not found, that in addition the escape coding of a pixel may also entail signalling either Y only or Y,U,V depending to said position

## Pixel mapping

Lossless coding requires strict matching between the pixel and palette entry it is mapped to. However, it can be seen that pixels 1, 2 and 3 do not suffer from being mapped to an entry whose chroma sample values differ from their own. Given two triplets P1 and P2 (either entries or pixels), this can be summarized in the following metric:

- If P1[Y] ! = P2[Y], then D = maximum;

- Otherwise D = maximum if:

  - The pixel doesn’t have superfluous chroma; and

  - P1[Cb] ! = P2[Cb];

  - P1[Cr] ! = P2[Cr].

- Else D = L(P1[Cb], P2[Cb], P1[Cr], P2[Cr]).

It was found that, although all matching entries for the last case are equivalent, it is preferable to use the first matching one, except if chroma sample values are also equal, in which case that later entry should be preferred.

## Entry merging and forcing

When deriving the palette, knowing when palette entries are only used for pixels with superfluous chroma, they can be merged in the following manner:

- If 2 entries are in this case, their chroma values do not matter, and only one entry can be used;

- If only one of the 2 entries is in this case (i.e., not the other), but luma values are however equal, then the former entry chroma values do not matter and can be set to the ones of the later entry, resulting in only one entry being used.

This can also be used for palette prediction: when an entry is only used for pixels with superfluous chroma, then a predictor entry can be used instead, as long as luma values are equal, even if chroma values are not.

## Palette evaluation

In the lossy case, the palette can be optimized for prediction: forcing prediction of an entry can result in increased distortion for the mapped pixels, but the overall signaling is reduced. However, in lossless coding, even with the above modifications, this has almost no consequence. Simply disabling these evaluations for the lossless case results in an encoder runtime reduction with no coding efficiency loss.

# Results

As the proposed methods are meant for lossless encoding, only these results are reported here, according to the Common Test Conditions. However, results for lossy are still attached to the contributions, and show no change in BDR or runtime, as expected. Decoding runtimes are inaccurate due to higher and unaccounted-for I/O during decoding.

## Speedup, lossless 4:2:0 configuration

## All combined, lossless 4:2:0 configuration

This is the only configuration showing coding gains, as expected.

## Speedup, lossless 4:4:4 configuration

## All combined, lossless 4:4:4 configuration

# Conclusion

Various non-normative modifications to 4:2:0 lossless coding result in encoder runtime improvements and coding efficiency gains. It is therefore suggested to adopt these changes into the next SCC test model.

# References

- JCTVC-T0072, “CE1-related: Palette Coding for non-444 format content,” J. Ye, S. Liu, S. Lei (MediaTek), Geneva, CH, Feb. 2015.

- JCTVC-V0034, “Palette encoder improvements for the 4:2:0 chroma format”, C. Gisquet, G. Laroche, P. Onno (Canon), Geneva, CH, Oct. 2015.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).