| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0076 |
| --- | --- |

| Title: | Comments on alignment of SCC text with multi-view and scalable |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Guillaume Laroche, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

At the 21st JCT-VC meeting in June 2015, disabling of weighted prediction parameters syntax for Intra Block Copy was adopted, by checking whether a reference picture has the same POC as the current frame. It is asserted that the text specifications modifications have consequences outside of the Screen Content Coding extension. It is proposed to study a solution, and an example of an additional verification to alleviate those consequences is proposed.

# Weighted prediction signaling

## Text specifications

As of HEVC SCC Draft text 5 [1], the signaling of section 7.3.6.3 in pred_weight_table( ) compares the POC of reference pictures to the current picture, and if equal, implicitly disables signaling.

Table 1: excerpt of weighted prediction parameters syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| if( PicOrderCnt( RefPicList0[ i ] ) != PicOrderCnt( CurrPic ) ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| … |  |

At the last meeting, it was debated whether weighted prediction should be enabled for Intra Block Copy, as there was a potential for gains [2]. The discussion led to the decision of keeping the above syntax; meeting notes indicate concerns on the enabling negatively impacting implementations.

## Analysis of the consequences

The text sections below underlines how checking the POC can accidentally impact other extensions and potentially disables weighted prediction for reference pictures in them, thereby accidentally breaking conformity of previously conformant streams. In particular, emphasis has been put in bold on the cases which are not related to Intra Block Copy.

It is therefore recommended to fix this “breakage”.

### Scalable extension

However, this has to be contrasted with SHVC, whose section H.8.1.4, “Derivation process for inter-layer reference pictures,” states (emphasis in bold):

Output of this process is the inter-layer reference picture ilRefPic

[…]

The following ordered steps are applied to derive the inter-layer reference picture ilRefPic.

[…]

If equalPictureSizeAndOffsetFlag is equal to 1, BitDepthRefLayerY is equal to BitDepthCurrY, BitDepthRefLayerC is equal to BitDepthCurrC, SubWidthRefLayerC is equal to SubWidthCurrC, SubHeightRefLayerC is equal to SubHeightCurrC, and currColourMappingEnableFlag is equal to 0,

–	ilRefPic is set equal to rlPic.

Otherwise, the following steps apply:

–	The inter-layer reference picture ilRefPic is generated as follows:

–	The PicOrderCntVal value of ilRefPic is set equal to the PicOrderCntVal value of rlPic.

### 3D coding extension

Similarly, 3D-HEVC may be affected, as section I.7.4.7.1, “General slice segment header semantics,” states:

When in_comp_pred_flag is equal to 1, the following applies for i in the range of 0 to numCurCmpLIds − 1, inclusive:

–	It is a requirement of bitstream conformance that there is a picture in the DPB with PicOrderCntVal equal to the PicOrderCntVal of the current picture, and a nuh_layer_id value equal to ViewCompLayerId[ inCmpRefViewIdcs[ i ] ][ !DepthFlag ].

## Example of a solution

One simple fix given as example is to augment the check in section 7.3.6.3 in the following manner:

Table 2: excerpt of proposed weighted prediction parameters syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| if( pps_curr_pic_ref_enabled_flag && PicOrderCnt( RefPicList0[ i ] ) != PicOrderCnt( CurrPic ) ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| … |  |

As the pps_curr_pic_ref_enabled_flag flag is defined in section 7.4.3.3.3 as:

pps_curr_pic_ref_enabled_flag equal to 1 specifies that a picture referring to the PPS may be included in a reference picture list of a slice of the picture itself. pps_curr_pic_ref_enabled_flag equal to 0 specifies that a picture referring to the PPS is never included in a reference picture list of a slice of the picture itself. When not present, the value of pps_curr_pic_ref_enabled_flag is inferred to be equal to 0.

The complete proposed modifications are attached to the present contribution.

# Conclusion

A potential loss of compliance for scalable and 3D-HEVC extensions, introduced during the development of the SCC extension, was identified in HEVC Screen Content Coding Draft Text 5. It is recommended to evaluate the actual situation and, if an issue is actually identified, to remediate it before any draft issuance at this meeting.

# References

- JCTVC-V1005, “HEVC Screen Content Coding Draft Text 5,” R. Joshi, S. Liu, G. J. Sullivan, Y.-K. Wang, J. Xu, Y. Ye

- JCTVC-V0044, “Weighted prediction for intra block copy,” V. Seregin, R. Joshi, M. Karczewicz, (Qualcomm), Geneva, CH, Oct. 2015.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).