| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0077r1 |
| --- | --- |

| Title: | Bug fix for DPB operations when current picture is a reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC SCC specification, the current decoded picture prior to the loop filtering operations is used as a reference picture. This reference picture is also put and managed in the decoded picture buffer (DPB), together with other decoded pictures. A bug fix is proposed in this contribution for the DPB operation such that before the start of decoding current picture, one empty picture buffer is created when TwoVersionsOfCurrDecPicFlag is equal to 0; alternatively, two empty picture buffers are created when TwoVersionsOfCurrDecPicFlag is equal to 1.

# Introduction

In HEVC, the decoded picture buffer (DPB) operates in such a way that, before decoding the current picture, some of the previously decoded pictures that reside in DPB may need to be removed, if there is no enough space for storing the upcoming current decoded picture.

In the current SCC specification [1], the current decoded picture prior to loop filter operations may be put in the DPB, in addition to the current decoded picture after loop filter operations, when IBC is used (signalled via SPS flag sps_curr_pic_ref_enabled_flag). The unfiltered version of current decoded picture is put in the reference picture list (either list 0 or list 1 or both) and later used as a reference picture for decoding slice(s) of current picture [2]. However, when no loop filter (either SAO or deblocking) is present for the current picture, there will be only one version of the current decoded picture stored therefore no need to create and preserve a duplicated picture buffer in the DPB.

Therefore, the DPB update operation before decoding current picture should be different:

- When no loop filter is present or when IBC is not used for current picture, only the current decoded picture after loop filter will be stored in the DPB

- Otherwise,  the current decoded picture both prior to and after loop filter will be stored in the DPB

- When the DPB is full, only one picture will be removed from DPB before decoding current picture if sps_max_num_reorder_pics[ HighestTid ] is equal to sps_max_dec_pic_buffering_minus1[ HighestTid ]. In this case, all remaining pictures in the DPB are marked as “needed for output” and cannot be removed from the DPB. Therefore, no empty picture buffer can be created for the extra version of current picture.

# Proposed method

In the first aspect of this contribution, a solution is provided to solve the above mentioned mismatch (the need to create two empty picture buffers and the capability of removing only one picture from DPB).

It is proposed that depending on the use of loop filters and IBC, there is a need to store an extra version of current decoded picture, therefore at least two empty picture buffers need to be created in the DPB, before the start of decoding current picture; otherwise, only one empty picture buffer is needed. The two different cases are illustrated in Fig. 1. The value of the derived flag “TwoVersionsOfCurrDecPicFlag” can be used to differentiate the above two cases.

Fig. 1: DPB consideration when current picture is a reference picture

Solution: a change is made to the maximum allowed number of decoded pictures in DPB, when TwoVersionsOfCurrDecPicFlag is equal to 1. The proposed text change is shown as follows:

......

C 5.2.2 Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows:

–	The decoding process for RPS as specified in clause Error: Reference source not found is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, bit_depth_chroma_minus8, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in clause Error: Reference source not found, and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in clause Error: Reference source not found is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

- The number of pictures in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ].

- sps_max_latency_increase_plus1[ HighestTid ] is not equal to 0 and there is at least one picture in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to SpsMaxLatencyPictures[ HighestTid ].

- The number of pictures in the DPB is greater than or equal to Max(1, sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 – TwoVersionsOfCurrDecPicFlag).

In the second aspect of this contribution, some editorial improvements are provided for describing the process of putting current picture (before loop filtering) into DPB and removing current decoded picture (before loop filtering). Details are listed below:

8.1.3	Decoding process for a coded picture with nuh_layer_id equal to 0

......

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause Error: Reference source not found.

- The processes in clause Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in clause Error: Reference source not found is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	A picture storage buffer in the DPB is allocated for storage of the decoded sample values of the current picture after the invocation of the in-loop filter process as specified in clause Error: Reference source not found. When TwoVersionsOfCurrDecPicFlag is equal to 0 and pps_curr_pic_as_ref_enabled_flag is equal to 1, this picture storage buffer is marked as "used for long-term reference". When TwoVersionsOfCurrDecPicFlag is equal to 1 and sps_max_dec_pic_buffering_minus1[ HighestTid ] is greater than 0, another picture storage buffer in the DPB is allocated for storage of the decoded sample values of the current picture immediately before the invocation of the in-loop filter process as specified in clause Error: Reference source not found, and is marked as "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in clause Error: Reference source not found is invoked, which needs to be invoked only for the first slice segment of a picture.

- The processes in clauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each forms a partitioning of the picture.

- After all slices of the current picture have been decoded, the current decoded picture after the invocation of the in-loop filter process as specified in clause Error: Reference source not found, is marked as "used for short-term reference". When TwoVersionsOfCurrDecPicFlag is equal to 1 and sps_max_dec_pic_buffering_minus1[ HighestTid ] is greater than 0, the current decoded picture before the invocation of the in-loop filter process as specified in clause Error: Reference source not found, is marked as "unused for reference".

8.3.4	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

The variables CurrPicInList0Flag and CurrPicInList1Flag are both set equal to 0.

Reference pictures are addressed through reference indices as specified in clause Error: Reference source not found. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

When TwoVersionsOfCurrDecPicFlag is equal to 1and sps_max_dec_pic_buffering_minus1[ HighestTid ] is greater than 0, let the variable currPic point to the current decoded picture before the invocation of the in-loop filter process as specified in clause Error: Reference source not found; otherwise (TwoVersionsOfCurrDecPicFlag is equal to 0), let the variable currPic point to the current decoded picture after the invocation of the in-loop filter process as specified in clause Error: Reference source not found.

C.3.4	Current decoded picture marking and storage

The current decoded picture after the invocation of the in-loop filter process as specified in clause Error: Reference source not found is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one. When TwoVersionsOfCurrDecPicFlag is equal to 0 and pps_curr_pic_ref_enabled_flag is equal to 1, this picture is marked as "used for long-term reference". After all the slices of the current picture have been decoded, this picture is marked as "used for short-term reference".

When TwoVersionsOfCurrDecPicFlag is equal to 1 and sps_max_dec_pic_buffering_minus1[HighestTid] is greater than 0, the current decoded picture before the invocation of the in-loop filter process as specified in clause Error: Reference source not found is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and this picture is marked as "used for long-term reference".

NOTE – Unless more memory than required by the level limit is available for storage of decoded pictures, decoders should start storing decoded parts of the current picture into the DPB when the first slice segment is decoded and continue storing more decoded samples as the decoding process proceeds.

C.3.5	Removal of pictures from the DPB after decoding of the current picture

When TwoVersionsOfCurrDecPicFlag is equal to 1 and sps_max_dec_pic_buffering_minus1[HighestTid] is greater than 0, immediately after decoding of the current picture, at the CPB removal time of the last decoding unit of access unit n (containing the current picture), the current decoded picture before the invocation of the in-loop filter process as specified in clause Error: Reference source not found is removed from the DPB, and the DPB fullness is decremented by one.

# Conclusions

In this contribution, a bug fix is proposed to address the issue when updating the decoded pictures in the DPB. In particular, when IBC is used (current decoded picture is a reference picture), one or two empty picture buffer needs to be created before decoding the current picture by outputting and removing some pictures from the DPB, depending on the value of the picture level derived flag TwoVersionsOfCurrDecPicFlag.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- R. Joshi, S. Liu, G. J. Sullivan, Y.-K. Wang, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 5, JCTVC-V1005, Geneva, CH, Oct. 2015

- X. Xu, S. Liu, C. Liu, S. Lei, Y.-K. Wang, K. Rapaka, V. Seregin  “On storage of filtered and unfiltered current decoded pictures” Document of Joint Collaborative Team on Video Coding, JCTVC-U0181, Warsaw, PL, 19–26 Jun. 2015.