| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0079 |
| --- | --- |

| Title: | HDR CE7-related: Additional results for Experiment 7.2a |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Matteo Naccari Manish Pindoria Andrew Cotton Block D, Centre House 56 Wood Lane, W12 7SB London UK | Tel: Email: | 0044 (0)3030409640 matteo.naccari@bbc.co.uk manish.pindoria@bbc.co.uk andrew.cotton@bbc.co.uk |
| Source: | BBC |  |  |

_____________________________

# Abstract

This contribution presents additional results for Experiment 7.2a as defined in Core Experiment 7 (CE7, see N15800). In Experiment 7.2a two values of the so-called system gamma are tested: 1.2 and 1.4 as described in W0067 and associated cross-check report W0043. In this document, the system gamma is set to 1.45 which is the suggested value reported in m37535 and discussed during the ad-hoc group interim meeting in Vancouver. AVI files for the SIM2 display have been generated for SDR and HDR and will be shown in the viewing sessions held during the meeting.

# Introduction

Core Experiment 7 (CE7) is described in N15800 and investigates the generation of HDR material with the Hybrid Log-Gamma (HLG) technology as described in m36249. Two experiments are envisaged: 7.1 and 7.2 where the former uses a fixed value of 1.6 for the system gamma parameter while the latter tests different values. Each experiment has in turn two tests denoted as a and b. In test a the system gamma is removed over the luminance component as originally proposed in m36249 while test b removes the system gamma on each RGB component separately.

During the interim meeting of the HDR ad-hoc-group it has been reported in contribution m37535 that a value of 1.6 would provide too bright backward compatible images. That value of system gamma was computed from the assumption that the OETF curve for HDR would have the same slope as per the SDR regime. This contribution reports on an additional value tested for Experiment 7.2a where the system gamma is set 1.45 which is the preferred value as suggested in m37535. The next section will describe how the HDR and SDR sequences have been generated. These sequences will be displayed during one of the viewing sessions organised during the meeting.

# Generation of SDR and HDR material

The content coded with HLG and system gamma 1.45 has been visually inspected to assess the video quality of both HDR and backward compatible SDR images. Particularly, this latter aspect is important to understand the effect of different values for the system gamma. To appreciate the quality associated with backward compatible images, a professional SDR monitor would be ideal. Unfortunately given the BT.2020 container of the test material and the lack of a display supporting this colour gamut in the proponent’s laboratory, a different workflow has been used. Figure 1 depicts the workflow used to inspect the visual quality associated with the backward compatible image. More precisely the YCbCr file coded with HLG and system gamma 1.45 is pushed through two different chains: one using an HLG EOTF to produce an HDR video and the other using a BT.1886 EOTF which corresponds to what a legacy SDR receiver would apply to display the video. The sequences obtained from these two processing chains are then played on a SIM2 display one after the other. Uncompressed files were used in this test since compression artefacts do not seem to influence the perceived quality associated with backward compatible images.

The sequences will be available for expert viewing during the meeting. The overall conclusion from this inspection is the same of the one reported in W0043 where 1.2 and 1.4 were tested as system gamma values. In particular system gamma 1.45 gives a good compromise between the highlights and the backward compatible images. Lower values for the system gamma (e.g. 1.2) will result in too dark images with a significant loss of details.

Figure 1: Workflow used to visually inspect the tested material.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).