| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0080 |
| --- | --- |

| Title: | Crosscheck of HDR CE5.3.1 (JCTVC-W0069) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jacob Strom Martin Pettersson Jonatan Samuelsson | Tel: Email: | +46107136883 jacob.strom@ericsson.com martin.m.pettersson@ericsson.com jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for the CE5.3.1 proposal by Philips (W0069). All sequences could be correctly reconstructed with matching md5sums, but the metrics could not be reproduced for all sequences. Results calculated with the HDRMetrics version used for anchor 3.2 have been provided instead. A subjective viewing has also been performed.

# Introduction

This contribution reports the cross-checking results for CE5.3.1 (W0069). The proposal suggests using the Y”u”v” color space for ST.2084 in combination with 10 bits representation for luma and 9 bits representation for each of the two chroma components. The cross-check has been performed according to:

- Examination of the code to verify that the changes are as proposed

- Compilation and simulation of the code to verify that the same results are achieved

- Visual examination of the results to identify where the contribution is different from anchor version 3.2.

## Examination of code

The code (based on HDRTools) has been examined to verify that the implementation is as described.

## Compilation and simulation of code

The code was compiled on linux using gcc version 4.5.1. The code was run with the following parameters changed in the HDRTools config file compared to the CfE anchor configuration:

EXR->YUV:

FilterUsingFloats  = 1

OutputSampleRange  = 4

OutputColorSpace   = 11

OutputBitDepthCmp1 = 9

OutputBitDepthCmp2 = 9

YUV->EXR:

FilterUsingFloats  = 1

SourceSampleRange  = 1

SourceColorSpace   = 11

SourceBitDepthCmp1 = 9

SourceBitDepthCmp2 = 9

HM16.7 was used as is, to encode the bitstream, i.e. without any modifications like for the anchor v3.2 (luma adjustment, chroma QP offset and luma based QP selection). HM16.7 was compiled with the HIGHBITDEPTH flag set to 1.

It was noted that the reconstructed yuv on the encoder side did not match the decoded yuv. The decoded yuv looked ok, while the encoder reconstructed yuv looked shifted in the chroma components. It seems therefor likely that there is a bug in HM16.7 in the encoder-side reconstruction for 9 bit encoding.

# Results

## Objective results

The bitstreams, decoded yuv, and reconstructed exr files were correctly reconstructed with matching md5 sums for all sequences.

The objective metrics produced by the proponents using the HDRMetrics of CE3 could not be reproduced. Below are instead the results using the same version of HDRMetrics as was used to calculate the metrics for anchor version 3.2.

Table 1	Results of CE5.3.1 compared to anchor version 3.2. The same version of HDRMetrics as was used to produce the anchor metrics has been used.

## Informal subjective evaluation

A visual examination was made on a 47” Sim2 6000 nits display.

In general, the CE5.3.1 proposal had worse subjective quality compared to the 3.2 anchor. The following scenes are good examples of where differences between the proposal and the anchor can be seen.

BalloonFestival (All rate points)	The rope of the balloon was less sharp in the proposal and also disappeared to a larger extent than in the anchor.

BikeSparklers cut2 (BR4)	The proposal had somewhat more detail in the puddle, but also more chroma artifacts in the windows and outside the hangar.

EBU04Hurdles (All rate points)	The proposal had significantly less details on the track already at BR1. There were also some chroma artifacts at lower rate points.

EBU06Starting (All rate points)  	The cable disappeared more in the proposal than in the anchor. The sky also had less details and the red stripe in the proposal disappears already in BR3.

FireEater (BR3)	The text on the banner in the background is slightly more readable in the proposal than in the anchor.

GarageExit (BR2, BR3)	The cables at the end of the scene disappears to a larger extent in the proposal compared to the anchor.

MarketPlace (BR2-BR4)	Significant loss of details of the wall for the proposal compared to the anchor, also more chroma artifacts on the wall and the windows.

ShowGirl (All rate points)	Less details in face and body for the proposal compared to the anchor.

SunRise (BR3, BR4)	Less details in the sky and around the cables for the proposal, but more details in the trees.