| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0081 |
| --- | --- |

| Title: | Crosscheck of Luma delta QP adjustment based on video statistical information (JCTVC-W0039) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Jacob Ström Per Hermansson Färögatan 6 16480 Stockholm | Tel: Email: | +46107136883 jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports on the crosscheck for “Luma delta QP adjustment based on video statistical information” described in JCTVC-W0039 by Samsung. Simulation results indicated a match for all sequences except FireEater where results and Md5sums had been switched for the lowest rate points.

# Introduction

The proponent has proposed two methods to further control how the delta QP, which was initially proposed by Sharp in [1], is adjusted. The first method is to disable adaptive QP for pictures which has low average luma. The second method is to not use any negative delta QP for CTU blocks with high variance. This document reports results for the case when both methods are used together. The authors of this crosscheck would like to highlight that the method to use variance as a basis for the adaptive QP is outside of the framework set out for the adaptive QP method in the improved anchor. In m37501 [2] one of the restrictions for the QP selection method is “QP control based only on average luma value”, which is not the case for the second method in JCTVC-W0039 since the variance is being used.

# Objective Results

All MD5 sums match those given by the proponent except for FireEater at rate R3 and R4. It is recognized that when switching the md5sums for those two rate points they then also match.

For the objective results all numbers match the ones provided by the proponent except for FireEater. The HDR metrics for FireEater rate R3 and R4 have been switched in excel file. As a result BD-rate calculations for that sequence are also wrong. The corrected result indicates tPSNR-Y gains of -7.5% instead of -1.2% and DeltaE gains of -17.4% instead of -10.1% of as reported by the proposer.

Table 1 Results as indicated by proponent

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -5,3% | -1,2% | -6,9% | -4,2% | -11,4% | -10,1% | 21,7% | 6,4% |
|  | Market3Clip4000r2 | -2,2% | -2,0% | -1,9% | -2,0% | -0,1% | -1,3% | -95,4% | 4,3% |
|  | SunRise | -7,1% | -7,2% | -8,6% | -7,7% | -3,9% | -6,4% | -37,7% | 4,5% |
| class B | BikeSparklers cut 1 | -3,6% | -3,5% | -4,4% | -3,7% | -6,2% | -11,0% | -19,4% | -0,1% |
|  | BikeSparklers cut 2 | -2,0% | -1,9% | -2,8% | -2,1% | -4,2% | -7,2% | -15,0% | 1,1% |
|  | GarageExit | -0,6% | -0,3% | -1,7% | -1,0% | -1,2% | -5,0% | -3,6% | 1,5% |
| class C | ShowGirl2Teaser | -1,0% | 0,8% | -1,4% | -0,5% | 0,8% | -7,3% | -6,4% | 5,7% |
| class D | StEM_MagicHour cut 1 | -1,7% | 1,4% | -7,7% | -3,6% | -4,4% | -18,5% | 40,5% | 2,9% |
|  | StEM_MagicHour cut 2 | -0,4% | 1,3% | -5,6% | -2,3% | -3,3% | -15,2% | -9,5% | 2,3% |
|  | StEM_MagicHour cut 3 | -1,1% | 0,9% | -6,8% | -3,3% | -4,7% | -14,8% | 6,5% | 2,7% |
|  | StEM_WarmNight cut 1 | -2,1% | -0,2% | -5,2% | -2,8% | -3,8% | -17,3% | 3,0% | 2,0% |
|  | StEM_WarmNight cut 2 | -2,4% | 0,8% | -5,3% | -2,7% | -4,1% | -13,6% | -2,7% | 3,1% |
| class G | BalloonFestival | -0,4% | -0,2% | -1,3% | -0,8% | -0,6% | -2,3% | -19,7% | 1,3% |
| class H | EBU_04_Hurdles | -0,7% | -0,6% | -0,5% | -0,6% | -0,4% | -0,4% | 4,9% | -0,1% |
|  | EBU_06_Start | -2,1% | -1,5% | -3,0% | -2,3% | -2,2% | -4,4% | 4,1% | 0,8% |
|  | Overall | -2,2% | -0,9% | -4,2% | -2,6% | -3,3% | -9,0% | -8,6% | 2,6% |

Table 2 Results from cross-checker

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -12,3% | -7,5% | -14,0% | -11,2% | -18,2% | -17,4% | -2,2% | 1,6% |
|  | Market3Clip4000r2 | -2,2% | -2,0% | -1,9% | -2,0% | -0,1% | -1,3% | -95,4% | 4,3% |
|  | SunRise | -7,1% | -7,2% | -8,6% | -7,7% | -3,9% | -6,4% | -37,7% | 4,5% |
| class B | BikeSparklers cut 1 | -3,6% | -3,5% | -4,4% | -3,7% | -6,2% | -11,0% | -19,4% | -0,1% |
|  | BikeSparklers cut 2 | -2,0% | -1,9% | -2,8% | -2,1% | -4,2% | -7,2% | -15,0% | 1,1% |
|  | GarageExit | -0,6% | -0,3% | -1,7% | -1,0% | -1,2% | -5,0% | -3,6% | 1,5% |
| class C | ShowGirl2Teaser | -1,0% | 0,8% | -1,4% | -0,5% | 0,8% | -7,3% | -6,4% | 5,7% |
| class D | StEM_MagicHour cut 1 | -1,7% | 1,4% | -7,7% | -3,6% | -4,4% | -18,5% | 40,5% | 2,9% |
|  | StEM_MagicHour cut 2 | -0,4% | 1,3% | -5,6% | -2,3% | -3,3% | -15,2% | -9,5% | 2,3% |
|  | StEM_MagicHour cut 3 | -1,1% | 0,9% | -6,8% | -3,3% | -4,7% | -14,8% | 6,5% | 2,7% |
|  | StEM_WarmNight cut 1 | -2,1% | -0,2% | -5,2% | -2,8% | -3,8% | -17,3% | 3,0% | 2,0% |
|  | StEM_WarmNight cut 2 | -2,4% | 0,8% | -5,3% | -2,7% | -4,1% | -13,6% | -2,7% | 3,1% |
| class G | BalloonFestival | -0,4% | -0,2% | -1,3% | -0,8% | -0,6% | -2,3% | -19,7% | 1,3% |
| class H | EBU_04_Hurdles | -0,7% | -0,6% | -0,5% | -0,6% | -0,4% | -0,4% | 4,9% | -0,1% |
|  | EBU_06_Start | -2,1% | -1,5% | -3,0% | -2,3% | -2,2% | -4,4% | 4,1% | 0,8% |
|  | Overall | -2,7% | -1,3% | -4,7% | -3,1% | -3,8% | -9,5% | -10,2% | 2,3% |

# Subjective Results

Subjective viewing on SIM2 monitor indicates an overall better visual quality on the tested sequences. No noticeable quality degradations were observed when comparing with the SuperAnchor v3.2. On several sequences improvements to texture detail and sharpness were observed, most noticeably in darker areas. These improvements are mostly visible in the BikeSparklers, Market and Sunrise sequences at lower bit-rates.

# Conclusion

It is reported that the source code provided by the proponent was verified to be consistent with the results in JCTVC-W0039. Subjective viewing shows improvements to visual quality compared to SuperAnchor v3.2 on most sequences and otherwise similar quality as the anchor.

# References

[1] J. Zhao, S.-H. Kim, A. Segall, K. Misra(Sharp) “Performance investigation of high dynamic range and wide color gamut video coding techniques”, MPEG 113, m37439

[2] J. Strom, J.Sole, Y. He,“ HDR CE1: Optimization without HEVC Specification Change”, MPEG 113, m37501