| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0086r1 |
| --- | --- |

| Title: | Indication of SMPTE 2094-10 metadata in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Raymond Yeung, Sheng Qu, Peng Yin, Taoran Lu, Tao Chen, Walt Husak 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes text changes in the HEVC specification to support metadata of SMPTE 2094-10 standard: Dynamic Metadata for Color Volume Transform - Application #1. Specifically, in Annex D, a new SEI message is proposed for carriage of such metadata.

# Introduction

The ST 2094 suite of documents defines metadata for use in color volume transforms of content. The metadata are content-dependent and can vary scene-by-scene or frame-by-frame. The metadata are intended to transform High Dynamic Range and Wide Color Gamut (HDR/WCG) source content for presentation on a display having a smaller color volume than the source content’s mastering display [1].

ST 2094 Part 10 specifies the content-dependent color volume transform metadata items for Application #1 [2]. This color volume transform is based on an adaptive tone mapping curve parameterized by the source content characteristics as well as manually set adjustments. This contribution proposes a new SEI message in Annex D to carry such metadata.

# Proposed Text

## SEI payload Syntax for ST 2094-10

D.2.4x Colour volume transform information SEI message syntax

| colour_volume_transformation_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_volume_transform_id | ue(v) |
| colour_volume_transform_cancel_flag | u(1) |
| if( !colour_volume_transform_cancel_flag ) { |  |
| colour_volume_transform_persistence_flag | u(1) |
| num_metadata_sets_minus1 | ue(v) |
| for ( i = 0; i < num_metadata_sets_minus1 + 1; i++) { |  |
| application_identifier[ i ] | ue(v) |
| application_version[ i ] | ue(v) |
| proc_win_num_id[ i ] | ue(v) |
| proc_win_left_offset[ i ] | ue(v) |
| proc_win_right_offset[ i ] | ue(v) |
| proc_win_top_offset[ i ] | ue(v) |
| proc_win_bottom_offset[ i ] | ue(v) |
| for( c = 0; c < 3; c++ ) { |  |
| target_display_primaries_x[ i ][ c ] | u(16) |
| target_display_primaries_y[ i ][ c ] | u(16) |
| } |  |
| target_display_white_point_x[ i ] | u(16) |
| target_display_white_point_y[ i ] | u(16) |
| max_target_display_luminance[ i ] | u(32) |
| min_target_display_luminance[ i ] | u(32) |
| min_PQ_maxRGB[ i ] | u(32) |
| avg_PQ_maxRGB[ i ] | u(32) |
| max_PQ_maxRGB[ i ] | u(32) |
| manual_adjustment_flag | u(1) |
| if ( manual_adjustment_flag = = 1) { |  |
| min_PQ_maxRGB_offset[ i ] | i(32) |
| avg_PQ_maxRGB_offset[ i ] | i(32) |
| max_PQ_maxRGB_offset[ i ] | i(32) |
| tone_mapping_offset[ i ] | i(32) |
| tone_mapping_gain[ i ] | u(16) |
| tone_mapping_gamma[ i ] | u(16) |
| chroma_compensation_weight[ i ] | i(16) |
| saturation_gain[ i ] | i(16) |
| tone_detail_factor[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

## SEI payload semantics

### D3.4x Colour volume transform information SEI message semantics

This SEI message provides information to enable colour volume transformation of the reconstructed colour samples of the output pictures. The input to the indicated colour volume transform process is either in luminance-chrominance color spaces or directly on RGB color components. In both cases, the set of decoded sample values shall apply an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format).

The colour volume transformation model used in the colour volume transformation information SEI message is based on a parametrically defined tone mapping curve, the shape of which is defined both by the image essence characteristics (algorithmically computed from the input image essence) as well as by manually-set adjustments.

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of Society of Motion Picture and Television Engineers ST 2094-10.

NOTE x – Colour volume transform of the output pictures for the display process (which is outside the scope of this Specification) is optional and does not affect the decoding process specified in this Specification.

colour_volume_transform_id contains an identifying number that may be used to identify the purpose of the colour volume transform information. When more than one colour volume transform information SEI message is present with the same value of colour_volume_transform_id, the content of these colour volume transform information SEI messages shall be the same. When colour volume transform information SEI messages are present that have more than one value of colour_volume_transform_id, this indicates that the volume transform processes indicated by the different values of colour_volume_transform_id are alternatives that are provided for different purposes (not that a cascading of volume transform processes is to be applied). The value of colour_volume_transform_id shall be in the range of 0 to 232 − 2, inclusive.

Values of colour_volume_transform_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of colour_volume_transform_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour volume transform information SEI messages containing a value of colour_volume_transform_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE x – The colour_volume_transform_id can be used to support different colour volume transform processes that are suitable for different display scenarios. For example, different values of colour_volume_transform_id may correspond to different transform colour spaces supported by displays.

colour_volume_transform_cancel_flag equal to 1 indicates that the colour volume transform information SEI message cancels the persistence of any previous colour volume transform information SEI message in output order that applies to the current layer. colour_volume_transform_cancel_flag equal to 0 indicates that colour volume transform information follows.

colour_volume_transform_persistence_flag specifies the persistence of the colour volume transform information SEI message for the current layer.

colour_volume_transform_persistence_flag equal to 0 specifies that the colour volume transform information applies to the current picture only.

Let picA be the current picture. colour_volume_transform_persistence_flag equal to 1 specifies that the colour volume transform information persists for the current layer in output order until either of the following conditions is true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a colour volume transform information SEI message with the same value of colour_volume_transform_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

num_metadata_sets_minus1 specifies the number of metadata sets identified in the SEI message. The values of num_metadata_sets_minus1 shall be in the range of 0 to 28 − 1 inclusive.

application_identifier[ i ] identifies an application and its defining document in ST-2094 suite in the i-th metadata set. For this SEI message, application_identifier[ i ] shall be set to 1.

application_version[ i ] specifies the application version in the application defining document in ST-2094 suite in the i-th metadata set. For this SEI message, application_identifier[ i ] shall be set to 0.

proc_win_num_id[ i ] identifies the number of the processing window in the i-th metadata set. The values of proc_win_num_id[ i ] shall be in the range of 0 to 15, inclusive.

proc_win_left_offset[ i ], proc_win_right_offset[ i ], proc_win_top_offset[ i ], and proc_win_bottom_offset[ i ] specify the samples of the pictures in the i-th metadata set, in terms of a rectangular region specified in picture coordinates. When not present, the values of proc_win_left_offset[ i ], proc_win_right_offset[ i ], proc_win_top_offset[ i ], and proc_win_bottom_offset[ i ] are inferred to be equal to 0.

The processing window is specified for 4:4:4 chroma format. It contains the colour samples with horizontal picture coordinates from proc_win_left_offset[ i ] to pic_width_in_luma_samples − ( proc_win_right_offset[ i ] + 1 ) and vertical picture coordinates from proc_win_top_offset[ i ] to pic_height_in_luma_samples − ( proc_win_bottom_offset[ i ] + 1 ), inclusive.

The value of ( proc_win_left_offset[ i ] + proc_win_right_offset[ i ] ) shall be less than pic_width_in_luma_samples, and the value of ( proc_win_top_offset[ i ] + proc_win_bottom_offset[ i ] ) shall be less than pic_height_in_luma_samples.

target_display_primaries_x[ i ][ c ] and target_display_primaries_y[ i ][ c ] specify the normalized x and y chromaticity coordinates, respectively, of the c-th colour primary component in i-th metadata set in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing target displays that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Error: Reference source not found). The values of display_primaries_x[ i ][ c ]  and display_primaries_y[ i ][ c ]shall be in the range of 0 to 50 000, inclusive.

target_display_white_point_x[ i ] and target_display_white_point_y[ i ] specify the normalized x and y chromaticity coordinates, respectively, of the white point of the i-th metadata set in normalized increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). The values of target_display_white_point_x[ i ] and target_display_white_point_y[ i ] shall be in the range of 0 to 50 000.

max_target_display_luminance[ i ] and min_target_display_luminance[ i ] specify the nominal maximum and minimum display luminance, respectively, of the i-th metadata set in units of 0.0001 candelas per square metre. min_target_display_luminance[ i ] shall be less than max_target_display_luminance[ i ].

min_PQ_maxRGB[ i ], avg_PQ_maxRGB[ i ], and max_PQ_maxRGB[ i ] specify the normalized minimum, average and maximum PQ-encoded maxRGB value of samples in the processing window of the i-th metadata set in normalized increments of 0.00001. The values of min_PQ_maxRGB[ i ], avg_PQ_maxRGB[ i ], and max_PQ_maxRGB[ i ] shall be in the range of 0 to 100 000.

maxRGB specifies the maximum of color component values in R, G and B color components of a sample.

PQ-encoded maxRGB specifies the value of maxRGB encoded according to the Inverse-EOTF specified in ST-2084 (see also Annex Error: Reference source not found and Error: Reference source not found).

manual_adjustment_flag[ i ] equal to 1 indicates that min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], max_PQ_maxRGB_offset[ i ], tone_mapping_offset[ i ], tone_mapping_gain[ i ], tone_mapping_gamma[ i ], chroma_compensation_weight[ i ], saturation_gain[ i ], and tone_detail_factor[ i ] are present for the i-th metadata set. manual_adjustment_flag[ i ] equal to 0 indicates that min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], max_PQ_maxRGB_offset[ i ], tone_mapping_offset[ i ], tone_mapping_gain[ i ], tone_mapping_gamma[ i ], chroma_compensation_weight[ i ], saturation_gain[ i ], and tone_detail_factor[ i ] are not present for the i-th metadata set.

min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], and max_PQ_maxRGB_offset[ i ] specify the offset value for minimum, average and maximum PQ-encoded maxRGB, respectively, of the i-th metadata set in normalized increments of 0.00001. The value of min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], and max_PQ_maxRGB_offset[ i ] shall be in the range of −50 000 to 50 000. When not present, their values are inferred to be equal to 0.

The variable minPQmaxRGB, avgPQmaxRGB, and maxPQmaxRGB are derived as follows:

minPQmaxRGB = ( min_PQ_maxRGB[ i ] + min_PQ_maxRGB_offset[ i ]) * 0.00001	(D-xx)

avgPQmaxRGB = ( avg_PQ_maxRGB[ i ] + avg_PQ_maxRGB_offset[ i ]) * 0.00001	(D-xx)

maxPQmaxRGB = ( max_PQ_maxRGB[ i ] + max_PQ_maxRGB_offset[ i ]) * 0.00001	(D-xx)

The value of minPQmaxRGB, avgPQmaxRGB, and maxPQmaxRGB, shall obey the following constraints:

–	minPQmaxRGB shall be greater than or equal to 0.0 and less than or equal to avgPQmaxRGB;

–	avgPQmaxRGB shall be less than or equal to maxPQmaxRGB;

–	maxPQmaxRGB shall be less than or equal to 1.0.

NOTE x – The generation of the parametric tone mapping curve is outside the scope of this Specification. An example of defining a parametric tone mapping curve on adopting an analytically defined sigmoid curve using provided syntax is shown as following:

The variable D represents input value of the colour component (R, G, or B) in units of cd/m2. The variable c1, c2, and c3 are coefficients, derived in Equation D-xx.  The variable E of output value for corresponding colour component is derived as following:

E = ( c1 + c2 * D ) ÷ ( 1 + c3 * D )							(D-xx)

The variable x1, x2 and x3 denote the minimum luminance value of the input image in units of cd/m2, the average luminance value of the input image in units of cd/m2, and the maximum luminance value of the input image in units of cd/m2, respectively, and are derived as following:

x1 = EOTFPQ ( minPQmaxRGB )							(D-xx)

x2 = EOTFPQ ( avgPQmaxRGB )							(D-xx)

x3 = EOTFPQ ( maxPQmaxRGB )							(D-xx)

The variable y1 and y3 denote the minimum luminance value in units of cd/m2, and the maximum luminance value in units of cd/m2, respectively, for target display, and are derived as following:

y1 = min_target_display_luminance[ i ] * 0.0001						(D-xx)

y3 = max_target_display_luminance[ i ] * 0.0001						(D-xx)

The varible y2 denoted the derived targeted system display adaptation point in units of cd/m2 and is derived as following:

y2 = sqrt( x2 * sqrt( y3 * y1 ) )								(D-xx)

The variable α is derived as

α = x3 * y3 * ( x1 − x2 ) + x2 * y2 * ( x3 − x1 ) + x1 * y1 * ( x2 − x3 )				(D-xx)

The variable c1, c2, and c3 are derived as following:

c1 = ( x2 * x3 * (y2 − y3 ) * y1 + x1 * x3 * (y3 − y1 ) * y2 + x1 * x2 * ( y1 − y2 ) * y3 ) ÷ α		(D-xx)

c2 = ( ( x3 * y3 − x2 * y2 ) * y1 +( x1 * y1 − x3 * y3 ) * y2 + ( x2 * y2 − x1 * y1 ) * y3 ) ÷ α		(D-xx)

c3 = ( ( x3 − x2 ) * y1 +( x1 − x3 ) * y2 + ( x2 − x1 ) * y3 ) ÷ α					(D-xx)

tone_mapping_offset[ i ] specifies the value of tone mapping offset for the i-th metadata set in normalized increments of 0.00001. The value of tone_mapping_offset[ i ] shall be in the range of −50 000 to 50 000. When not present, its value is inferred to be equal to 0.

tone_mapping_gain[ i ] specifies the value of tone mapping gain for the i-th metadata set in normalized increments of 0.0001. The value of tone_mapping_gain[ i ] shall be in the range of 5 000 to 15 000. When not present, its value is inferred to be equal to 10 000.

tone_mapping_gamma[ i ] specifies the value of tone mapping gamma for the i-th metadata set in normalized increments of 0.001. The value of tone_mapping_gain[ i ] shall be in the range of 500 to 1 500. When not present, its value is inferred to be equal to 1 000.

The variable toneMappingOffset, toneMappingGain, and toneMappingGamma are derived as follows:

toneMappingOffset = tone_mapping_offset [ i ] * 0.00001				(D-xx)

toneMappingGain = tone_mapping_gain [ i ] * 0.0001				(D-xx)

toneMappingGamma = tone_mapping_gamma [ i ] * 0.001				(D-xx)

The variable x represents the normalized input value. The variable y represents the normalized output value. The tone mapping function is described as follows:

y = max( 0, ( x * toneMappingGain + toneMappingOffset ) ) toneMappingGamma		(D-xx)

chroma_compensation_weight[ i ] specifies the amount of chroma adjustment for the i-th metadata set in normalized increments of 0.0001. The value of chroma_compensation_weight[ i ] shall be in the range of −5 000 to 5 000. When not present its value is inferred to be equal to 0.

saturation_gain[ i ] specifies the amount of saturation adjustment for the i-th metadata set in normalized increments of 0.0001. The value of saturation_gain[ i ] shall be in the range of −5 000 to 5 000. When not present its value is inferred to be equal to 0.

NOTE x – The usage of chroma_compensation_weight and saturation_gain is outside the scope of this Specification. An example to use such syntax is shown as following:

The variable chromaCompensationWeight and staurationGain are derived as following:

chromaCompensationWeight = chroma_compensation_weight[ i ] * 0.0001			(D-xx)

saturationGain = saturation_gain[ i ] * 0.0001						(D-xx)

The variable F represents input value of the colour component (R, G, or B) in units of cd/m2. The variable Y represents the luminance of the input pixel in units of cd/m2. The variable G of output value for corresponding colour component is described as following:

G = F * ( ( 1 + chromaCompensateWeight) * F ÷ Y ) saturationGain				(D-xx)

tone_detail_factor[ i ] specifies the contribution of the detail management after tone mapping for the i-th metadata set in normalized increments of 0.001. The value of tone_detail_factor[ i ] shall be in the range of 0 to 1000. When not present its value is inferred to be equal to 0.

NOTE x – The usage of tone_detail_factor is outside the scope of this Specification. An example to use such syntax is shown as following:

The variable toneDetailFactor is derived as

toneDetailFactor = tone_detail_factor [ i ] * 0.001					(D-xx)

The variable p represents the normalized input value. The variable q represents the normalized input value with detail management processing. The variable y of output value is described as follows:

y = p * (1 − toneDetailFactor )+q * toneDetailFactor					(D-xx)

# Reference

- SMPTE ST 2094-1:2016: "Dynamic Metadata for Color Volume Transform – Core Components"

- SMPTE ST 2094-10:2016: "Dynamic Metadata for Color Volume Transform – Application #1"

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).