| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0097r1 |
| --- | --- |

| Title: | HDR CE2: CE2.c-Chroma QP offset study report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D.Rusanovskyy, J.Sole, A.K.Ramasubramonian, D.Sansli Bugdayci, M. Karczewicz (Qualcomm) | Tel: Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports results of CE2.c sub-test which studies harmonization of chroma QP offset proposed in and CrCb processing in HDR/WCG ETM. As a result of the study, modification of encoder side algorithm is proposed to the ETM. Proposed in this document solution employs an independent multirange ATF (reshaper) for each color component. Proposed solution provides objective gain as well as improvement in visual quality over reference ETM design as well as over the CE1 anchor.

# Introduction

The chroma QP offset technique was proposed to suppress color artifacts in HDR/WCG video coding in m37179 and it was adopted in the HDR/WCG anchor. This technique utilizes a content (native color gamut) and base QP dependent chroma QP offset technique. Independent chroma QP offset for each of Cb and Cr components is utilized. Figure 1 shows actual chroma QP for Cb and Cr components as they are derived in CE1 anchor 3.2 from base luma QP.

Alternative approach to compensate the color distortion in coded HDR video data was proposed in m37064. It was proposed to apply to each of chroma samples of HDR video data an ATF (resaper), parameters of which are derived from the native color gamut of coded HDR video signal. This approach was adopted to the Exploratory Test Model design and enabled in the mode 0 (HDR only solution). Current implementation, deployed in the ETM utilizes a single range ATF, parameters of which are derived from color primaries of native color gamut of the signal, e.g. BT.709 or P3 and target color container as described in []. Resulting parameters of ATF are shown in the Table 1.

Table 1. Parameters of ATF applied to Cb and Cr color components utilized in ETM

| Chroma components | BT709 in ct. BT2020 | P3 in ct. BT2020 |
| --- | --- | --- |
| Cb | Scale = 1.2344, Offset = 0.0 | Scale = 1.3991, Offset = 0 |
| Cr | Scale = 1.7913, Offset = 0.0 | Scale = 2.1735, Offset = 0 |

# Proposal

Considering, that chroma QP offset is being deployed at the HEVC encoder itself, is capable to take into consideration target bitrates, therefore it is capable to handle a color distortion which are function of base QP.

However, a hypothetical HDR video codec which utilizes a pre-processing and normative post-processing which is applied to CbCr samples is potentially capable to benefit from following three techniques:

- Internal dynamical range extension for video compression

  - E.g. compression at the full range, decoded output at standard range

- Impact of ATF applied to luma samples (luma reshaper)

  - ATF applied on luma may lead to bitrate budget increase for luma, thus penalizes the chroma coding

- Optimize perceptual visual quality of colors

  - Color artifacts are more noticeable in neutral colors (white/grey) and less noticeable in saturated colors

It is proposed to combine a QP-dependent chromaQp offset utilized in HEVC encoder with multirange ATF applied to CbCr samples. Parameters of resulting ATF applied to Cb and Cr components are derived as following:

- Approximate available bitrate budget

  - Extension from SR to FR

  - Extension of the Luma component

- Determine target ranges in CbCr representation for bitrate budget distribution

- Determine bitrate budget distribution between CbCr components

  - Take native color gamut into account

Examples of the target bitbadget distribution is shown in Figure 1, where a target ranges (associated with neutral colors) of Cb and Cr components are being scaled with larger scale value compare to low priority ranges

Figure 1. Resulting histogram of Luma samples after applying reference ETM ATF

Figure 2. Resulting histogram of Cb, Cr components after applying proposed multirange ATF

It should be noted, that current ETM design supports independent multirange ATF for all three components and therefore no normative changes to ETM spec are proposed.

## Simulation results 1

Proposed CbCr processing was integrated in the ETM reference scheme. And CE1 HM design was updated with ETM signaling and normative post-processing. The luma reshaper utilized in the reference ETM design is utilized in the reference configuration, with standard range. Forward ATF for CbCr channels was derived to emulate a full range utilization for video compression. Parameters of inverse ATF at the decoder side has been derived to produce output YUV signal in standard range, as specified by the CTC. No changes to the ETM syntax of semantic was introduced.

Proposed scheme was evaluated under the CTC and compared against the anchor. Simulation results against the Anchor 3.2 are shown in Table 2

# Table 2 Proposed scheme vs anchor 3.2

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | -9.7% | -4.7% | -17.4% | -10.2% | -16.7% | -17.5% | -9.4% | 4.3% |
| Market3Clip4000r2 | 19.1% | 18.3% | 13.5% | 16.7% | 10.1% | 26.4% | -96.3% | -5.2% |
| SunRise | -3.5% | -4.8% | -9.0% | -6.0% | 3.5% | 59.7% | -65.0% | 17.3% |
| BikeSparklers cut 1 | -0.5% | -0.5% | -7.2% | -2.7% | -5.5% | -17.9% | -51.4% | 6.3% |
| BikeSparklers cut 2 | 1.2% | 1.3% | -6.6% | -1.2% | -3.0% | -14.1% | -44.1% | 7.6% |
| GarageExit | 31.8% | 30.8% | 23.0% | 28.6% | 17.8% | -2.4% | -21.5% | 11.5% |
| ShowGirl2Teaser | 11.4% | 13.0% | 10.7% | 11.7% | 15.2% | -4.1% | -13.3% | 3.4% |
| StEM_MagicHour cut 1 | -1.5% | 3.2% | -10.1% | -4.3% | -5.4% | -28.6% | -38.7% | 4.7% |
| StEM_MagicHour cut 2 | 1.2% | 3.8% | -8.8% | -2.7% | -4.3% | -25.4% | -15.7% | 4.9% |
| StEM_MagicHour cut 3 | 1.9% | 4.4% | -7.3% | -2.0% | -3.6% | -20.7% | 8.9% | 6.5% |
| StEM_WarmNight cut 1 | -1.3% | 1.2% | -7.8% | -3.3% | -4.9% | -25.5% | 0.0% | 3.3% |
| StEM_WarmNight cut 2 | 3.4% | 7.4% | -9.6% | -1.5% | 0.9% | -23.1% | -32.1% | 2.0% |
| BalloonFestival | 36.5% | 45.5% | 17.9% | 31.0% | 20.4% | 13.2% | -44.8% | -0.6% |
| EBU_04_Hurdles | 27.4% | 14.4% | 16.7% | 18.7% | 8.4% | 68.6% | 83.5% | -8.5% |
| EBU_06_Start | 43.9% | 36.0% | 27.6% | 35.0% | 23.3% | 46.4% | -2.7% | -1.6% |
| Overall | 10.8% | 11.3% | 1.7% | 7.2% | 3.7% | 2.3% | -22.8% | 3.7% |

Proposed scheme was also evaluated under the CTC and compared against the reference ETM design. Simulation results against the Anchor 3.2 are shown in Table 3

# Table 3 Proposed scheme compared vs reference ETM design (Multirange ATF vs single range)

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 3.7% | -0.4% | -1.4% | 1.1% | 1.3% | 7.4% | 18.0% | 0.0% |
| Market3Clip4000r2 | 1.4% | 1.1% | -2.5% | 0.0% | -1.3% | -15.3% | -9.8% | 1.2% |
| SunRise | 1.5% | -0.8% | 0.2% | 0.3% | 0.0% | 4.4% | 7.7% | -0.6% |
| BikeSparklers cut 1 | 7.5% | 7.7% | -5.1% | 2.8% | 1.0% | -14.2% | -22.8% | 7.5% |
| BikeSparklers cut 2 | 5.5% | 5.6% | -7.7% | 1.2% | -1.3% | -15.7% | -19.7% | 5.4% |
| GarageExit | 2.7% | 2.4% | -6.6% | -0.9% | -2.8% | -12.0% | 4.3% | 2.3% |
| ShowGirl2Teaser | -1.2% | -0.5% | -7.7% | -3.3% | -14.6% | -12.1% | -11.9% | 0.5% |
| StEM_MagicHour cut 1 | 3.4% | 3.9% | -10.1% | -3.1% | -4.1% | -13.1% | -44.9% | 3.8% |
| StEM_MagicHour cut 2 | 3.3% | 3.1% | -7.5% | -1.8% | -2.9% | -9.6% | -8.9% | 3.1% |
| StEM_MagicHour cut 3 | 4.3% | 4.0% | -8.6% | -2.3% | -3.2% | -9.6% | -6.8% | 4.1% |
| StEM_WarmNight cut 1 | 2.2% | 2.4% | -7.9% | -2.0% | -3.2% | -13.1% | -6.7% | 2.4% |
| StEM_WarmNight cut 2 | 3.2% | 2.6% | -5.1% | -0.6% | -1.3% | -5.1% | 2.3% | 2.6% |
| BalloonFestival | 0.7% | -0.4% | -2.1% | -0.7% | -1.6% | -4.2% | -0.4% | -0.1% |
| EBU_04_Hurdles | 1.0% | 2.5% | -10.8% | -4.0% | -7.4% | -30.9% | -28.9% | 2.5% |
| EBU_06_Start | 2.6% | 3.3% | -4.6% | 0.1% | -2.4% | -32.7% | -29.3% | 4.0% |
| Overall | 2.8% | 2.4% | -5.8% | -0.9% | -2.9% | -11.7% | -10.5% | 2.6% |

Proposed scheme was also evaluated under the CTC and compared against the solution consisting of Luma ATF of the ETM and chroma QP offset mechanism from CE1 (DeltaQP for luma was disabled). Simulation results against the Anchor 3.2 are shown in Table 4

# Table 4 Proposed scheme compared vs solution consisting of Luma ATF of the ETM and ChromQP offset scheme of CE1

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | -0.7% | 0.4% | 0.8% | 0.1% | 0.6% | -2.4% | -6.0% | 0.7% |
| Market3Clip4000r2 | 0.5% | 0.9% | -0.9% | 0.2% | -0.6% | -17.5% | -60.1% | 1.0% |
| SunRise | 0.1% | 1.5% | -3.7% | -0.9% | -2.9% | -14.0% | -41.6% | 1.3% |
| BikeSparklers cut 1 | 2.5% | 3.6% | 0.7% | 2.3% | 1.3% | -12.4% | -22.1% | 3.4% |
| BikeSparklers cut 2 | 2.5% | 3.8% | -0.5% | 1.9% | 0.8% | -14.7% | -64.6% | 3.7% |
| GarageExit | 2.6% | 3.3% | -0.3% | 1.8% | 1.0% | -11.6% | -26.5% | 3.4% |
| ShowGirl2Teaser | 0.5% | 1.4% | -3.2% | -0.4% | -1.0% | -14.6% | -10.0% | 1.1% |
| StEM_MagicHour cut 1 | 0.0% | 2.3% | -2.7% | -0.6% | -1.2% | -14.3% | -29.9% | 2.3% |
| StEM_MagicHour cut 2 | 0.9% | 2.0% | -3.3% | -0.8% | -1.3% | -12.2% | -4.9% | 1.9% |
| StEM_MagicHour cut 3 | 1.5% | 2.3% | -2.9% | -0.4% | -0.8% | -10.2% | 1.8% | 2.4% |
| StEM_WarmNight cut 1 | -0.4% | 1.6% | -3.1% | -0.9% | -1.5% | -15.5% | -4.3% | 1.6% |
| StEM_WarmNight cut 2 | 0.0% | 1.7% | -1.3% | -0.1% | -0.2% | -9.5% | -27.3% | 1.9% |
| BalloonFestival | 0.8% | 1.9% | 0.3% | 0.9% | -0.2% | -10.1% | -32.5% | 1.9% |
| EBU_04_Hurdles | -0.1% | 1.3% | -2.6% | -0.8% | -1.8% | -18.1% | -21.3% | 1.3% |
| EBU_06_Start | 1.2% | 2.8% | -1.0% | 0.9% | -0.3% | -16.7% | -43.7% | 2.8% |
| Overall | 0.8% | 2.1% | -1.6% | 0.2% | -0.5% | -12.9% | -26.2% | 2.1% |

Simulation results shows that proposed solution provide very a improvement in detlaE metric, upto 32.7% dBR for EBU_Start with 12% of dBR on average for DeltaE metric. Introduced improvements comes at the cost of moderate penalty to tPSNRY, 2.4% on average.

## Simulation results 2

Proposed CbCr processing was integrated in the SW solution refered in JCT-W0084.

Modified HDRTools features an alternative to ETM luma ATF and outputs preprocessed YUV data at the full range. Parameters of forward ATF for CbCr components are derived from associated Luma ATF and took full range processing into consideration. The dynamical increase budget, has been weighted with native color gamut of the signal and adaptively distributed between target ranges of CbCr components. No modification to HM codec of W0084 has been introduced. Decoder outputs the YUV PQ in standard range with the help of non-normative scaling at the decoder.

At first, proposed scheme was tested with QP setting utilized in the reference codebase. Comparison of these, non-rate matched results against the Anchor 3.2 are shown in Table 5.

It is seen from the Table 5, introduced CbCr processing results in the 2.6% reduction of total bitrate on average for the same QP without noticeable degradation on visual quality.

# Table 5 Simulation results of Test2 (rate matching) vs anchor 3.2

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | BR saving |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 11.4% | 6.5% | 1.7% | 7.2% | 10.2% | 17.3% | 29.6% | 3.9% | -5.4% |
| Market3Clip4000r2 | 4.3% | 3.4% | 2.1% | 3.2% | -0.8% | 14.7% | 208.4% | -11.1% | -1.5% |
| SunRise | 6.4% | 5.5% | 5.3% | 5.7% | 6.8% | 83.7% | -11.0% | -0.4% | -2.3% |
| BikeSparklers cut 1 | -5.8% | -8.2% | -2.0% | -5.1% | -10.1% | 0.5% | -32.0% | -2.6% | -3.2% |
| BikeSparklers cut 2 | -3.0% | -5.6% | 0.5% | -2.7% | -7.8% | 7.5% | -40.5% | -2.0% | -3.3% |
| GarageExit | 8.4% | 5.5% | 14.0% | 9.5% | 7.3% | 25.3% | 37.3% | 2.5% | -2.2% |
| ShowGirl2Teaser | 14.3% | 13.0% | 12.8% | 13.4% | 26.5% | 9.5% | 18.1% | -0.9% | -1.8% |
| StEM_MagicHour cut 1 | -0.5% | -0.9% | 3.0% | 1.3% | 0.7% | 3.5% | -5.8% | -1.7% | -1.8% |
| StEM_MagicHour cut 2 | 1.5% | 0.0% | 4.4% | 2.5% | 2.6% | 8.5% | 6.9% | -0.5% | -2.4% |
| StEM_MagicHour cut 3 | 1.4% | 0.3% | 5.9% | 3.5% | 3.8% | 8.4% | 16.0% | -0.7% | -1.9% |
| StEM_WarmNight cut 1 | 10.3% | 9.8% | 2.8% | 7.1% | 5.4% | -1.6% | 2.9% | -4.8% | -1.2% |
| StEM_WarmNight cut 2 | 5.3% | 3.6% | -3.3% | 1.0% | 4.9% | 0.0% | 5.6% | -2.9% | -2.6% |
| BalloonFestival | 16.1% | 17.7% | 11.5% | 14.6% | 10.8% | 25.5% | -10.4% | -6.1% | -2.6% |
| EBU_04_Hurdles | 5.0% | -2.5% | 2.4% | 1.5% | -1.3% | 43.1% | 50.7% | -12.0% | -2.8% |
| EBU_06_Start | 14.5% | 7.3% | 7.7% | 9.5% | 7.1% | 64.8% | 25.8% | -8.2% | -4.8% |
| Overall | 6.0% | 3.7% | 4.6% | 4.8% | 4.4% | 20.7% | 20.1% | -3.2% | -2.6% |

At the second, proposed scheme was evaluated under the CTC with rate matching and compared against the anchor. Simulation results against the Anchor 3.2 are shown in Table 6. Rate matching resulted in the QP which are lower for all sequences, which lead to general quality improvement due to lower QP value.

# Table 6 Simulation results of Test2 (rate matching) vs anchor 3.2

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 11.0% | 6.3% | 1.6% | 6.9% | 9.6% | 16.4% | 33.6% | 3.8% |
| Market3Clip4000r2 | 4.4% | 3.5% | 2.4% | 3.3% | -0.6% | 16.0% | -30.2% | -11.0% |
| SunRise | 6.1% | 5.2% | 5.3% | 5.4% | 6.6% | 85.3% | -8.2% | -0.8% |
| BikeSparklers cut 1 | -6.1% | -8.6% | -1.7% | -5.2% | -10.2% | 1.1% | -51.4% | -3.0% |
| BikeSparklers cut 2 | -2.9% | -5.6% | 1.3% | -2.4% | -7.3% | 8.8% | -31.4% | -1.8% |
| GarageExit | 8.4% | 5.5% | 13.9% | 9.4% | 7.2% | 25.7% | 33.5% | 2.5% |
| ShowGirl2Teaser | 13.9% | 12.5% | 12.1% | 12.9% | 26.3% | 9.9% | 18.0% | -1.0% |
| StEM_MagicHour cut 1 | -0.4% | -0.9% | 3.2% | 1.3% | 0.9% | 3.7% | -7.3% | -1.6% |
| StEM_MagicHour cut 2 | 1.5% | 0.0% | 4.4% | 2.6% | 2.6% | 8.6% | 6.8% | -0.5% |
| StEM_MagicHour cut 3 | 1.3% | 0.2% | 5.7% | 3.4% | 3.6% | 8.4% | 20.8% | -0.8% |
| StEM_WarmNight cut 1 | 10.4% | 9.9% | 2.9% | 7.2% | 5.5% | -1.2% | 1.8% | -4.7% |
| StEM_WarmNight cut 2 | 5.5% | 3.8% | -3.1% | 1.2% | 5.1% | 0.5% | 3.9% | -2.7% |
| BalloonFestival | 16.1% | 17.7% | 11.5% | 14.6% | 10.8% | 25.5% | -6.4% | -6.1% |
| EBU_04_Hurdles | 5.3% | -2.3% | 2.8% | 1.9% | -1.0% | 44.5% | 48.6% | -11.9% |
| EBU_06_Start | 15.0% | 7.6% | 7.7% | 9.8% | 7.2% | 64.0% | 38.6% | -8.0% |
| Overall | 6.0% | 3.7% | 4.7% | 4.8% | 4.4% | 21.1% | 4.7% | -3.2% |

Rate matched results presented in Table 6, will be made available for viewing during the meeting.

Example of observed visual quality improvement can be assed from the following fragments:

Figure 1. Fragment of WarmNight_cut1, coded at RP4 with W0084 (up) and Test (Bottom)

Figure 2. Fragment of GarageExit, coded at RP4 with W0084 (up) and Test (Bottom).

Figure 3. Fragment of Hurdles, coded at RP4 with W0084 (up) and Test (Bottom).

Figure 2. Fragment of Balloons, coded at RP4 with W0084 (left) and Test (right)

# Conclusion

This document reports results of CE2.c sub-test which studies harmonization of chroma QP offset proposed in and CrCb processing in HDR/WCG ETM. As a result of the study, a modified ATF for CbCr is proposed to the ETM. Proposed in this document solution employs an independent multirange ATF (reshaper) for each color component. Proposed solution has been tested in the framework of ETM and SW package of W0084. In both framework, proposed solution resulted in bitrate reduction without noticeable quality degradation and improvement in visual quality over reference scheme.

# Acknowledgement

We would like to thank Dolby for providing a crosscheck for this proposal.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).