| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0101r1 |
| --- | --- |

| Title: | HDR CE2.a-1: Report on LCS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Joel Sole Dmytro Rusanovskyy Done Bugdayci Sansli Marta Karczewicz | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports the results of the study of CE2.a-1 on luma-based chroma scaling (LCS). The chroma sample values are scaled by values that are dependent on the co-sited luma sample values, and the scale values are computed based on a function that increases with luma values. The document reports objective evaluation on the tested configuration.

In r1 version of this document package, slides presented in the meeting are attached.

# Introduction

In proposal m37245 [1], a method to scale the chroma sample values based on the co-sited luma sample values was described (a.k.a. luma-based chroma scaling or LCS). This work investigates the scaling functions that were used in m37245 and reports the performance of the method in the latest ETM software.

The ETM has reshaper modules that are used to scale the luma and the chroma sample values of the sequences before they are encoded. The ETM has several modes; the chroma reshapers can be classified into two major categories – independent-channel scaling and cross-channel scaling. In independent-channel scaling, a piece-wise linear reshaper is applied to the chroma sample values directly. In the cross-channel scaling mode, a scale value that is dependent on the corresponding/co-sited luma values is applied to zero-offset chroma values.

One feature that has been observed in coding HDR video has been that the errors in the chroma channels are often perceived by viewers differently based on the luminance of the samples; e.g., errors in chroma channels are typically perceived more often when the luminance values of the samples are high when compared to the case when the luminance values of the sample are less. At low luminance values, the errors in chroma channels are perceived less and hence the chroma samples at the lower luminance levels may be less emphasized. The reshaping of the chroma samples – differently for different corresponding luma values – has the potential to provide a better trade-off in the quality of the encoded HDR content.

# Tested configuration

The luma reshaper used in the ETM for Mode 0 is used, and the scale values with which chroma values are scaled are obtained using the following function:

The scaled chroma value CS corresponding to the chroma sample value C, when the co-sited luma sample value is Y’ and x denotes the value of the corresponding parameter Smax, is obtained by multiplying C with f(Y’, x). The value of Smax used to determine the scaling functions to be used for the different sequences is given in Table 1. The luma values at the encoder that are forward-reshaped are used to compute the scale values of LCS.

Table 1: Value of Smax used for different sequences and different colour components

| Colour gamut of the sequences | For Cb | For Cr |
| --- | --- | --- |
| 709 | 2.3 | 3.9 |
| DCI-P3 | 1.2 | 2.8 |

# Results

Table 2 reports the objective metric results obtained for the tested configuration on the HDR test sequences. The BD-rate numbers reported are with respect to the Anchor 3.2 version.

Table 2: Summary of the objective metrics obtained  for the tested configuration of LCS

# References

[1] S. Lee, J. Sole, D. Rusanovskyy, A. Ramasubramonian, D. Bugdayci, M. Karczewicz, ``CE2-related: Report of LumaATF with luma-driven chroma scaling (LCS),” m37245, MPEG 2015, October 2015, Geneva, Switzerland.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).