| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0103 |
| --- | --- |

| Title: | HDR CE6: Test 4.1 Reshaper from m37064 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Done Bugdayci Sansli, Adarsh K. Ramasubramonian, Dmytro Rusanovskyy, Joel Sole, Marta Karczewicz | Email: | dbsansli@qca.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports the results of CE6 sub-test 4.1 that studies the Backwards Compatible configuration of the technology proposed in m37064. The technology is based on the Dynamic Range Adjustment (DRA) proposed in m37064 which is similar to mode 0 in ETM and provides a method for a guided, end-user side dynamic range conversion. Parameters of the DRA are proposed to be conveyed to the decoder through an SEI message. Objective and subjective test results are provided for an example application of reconstructing an HDR signal from compressed SDR bitstreams. It is proposed to adopt the technology described in this contribution, to provide a guided, end-user side dynamic range conversion.

# Introduction

An HDR/WCG video received by a legacy receiver may not be backwards compatible although it is compressed using HEVC main 10 profile. In order to provide backwards compatibility, a viewable picture quality is necessary for the legacy receivers. For this purpose, HDR signal may need to be reshaped at the pre-processing stage so that the decoded video is viewable to the SDR receiver. Furthermore, adjustment parameters are signaled to the HDR receiver so that the HDR signal can be recovered from the reshaped video. Similarly, an HDR signal can be converted to an SDR signal at the receiver side through the usage of adjustment parameters.

Backwards compatibility with legacy SDR receivers, can be achieved by setting transfer_characteristics syntax element in the VUI to indicate the SDR BT.709, IEC 61966-2-4, or BT.2020 OETFs (i.e. values 1, 6, 11, 14 or 15). Conversely, for HDR receivers, a new signaling enabling HDR reconstruction process is provided in DRA SEI.

A configuration of the proposed mechanism allows a backward compatible solution, which would provide recipients with a SDR BT2020 compatible bitstreams and would provide HDR-capable decoders with signaling to reconstruct HDR data. Example of such solution includes a two DRA modules, first is deployed in non-linear RGB domain and second in CbCr domain, as it is shown in Figure 1.

Figure 1. Post-processing chain of the tested configuration that employs DRA application in RGB and YCbCr domains

# Parameter generation for SDR

For SDR backwards compatibility, the parameters of DRA block are derived by using either a reference algorithm where a graded SDR is not available or from the graded SDR directly when graded SDR is available. Figure 2 illustrates the block diagram of the pre-processing when a certain reference algorithm (HLG) is applied; whereas, Figure 3 illustrates the pre-processing chain when a graded SDR signal is available. In the latter, obtained DRA parameters are utilized for the recovery of the HDR signal from the graded SDR.

Figure 2. Pre-processing block diagram of the tested configuration that employs usage of SystemGamma Removal + HLG as the reference.

Figure 3. Pre-processing block diagram when a graded SDR signal is available.

## Parameter Derivation from Reference Algorithm

In this report, we have used Hybrid Log Gamma (HLG) Transfer function combined with “System Gamma Removal applied on R,G,B samples” for DRA-1 parameter generation and HLG combined with “System Gamma Removal applied on Y samples” for the DRA-2 parameter generation. The output of DRA-1 has color distortions that are due to the application of system gamma removal on R, G, B samples and they are corrected by the DRA-2 block applied on chroma samples.

This configuration is tested with 20 subranges in DRA1 (RGB domain), with a single scale value in each partitioning same for all 3 R,G and B components and a single range in DRA2 (Cb,Cr domain). Each of the color component have independently computed temporally varying scales. DRA-1 is the, R, G, B domain reshaper that provides the mapping from PQ coded signal to the reference SDR signal. Hence the parameters of this block changes only when the required system gamma value changes. DRA-2 provides chroma correction for the color distortions caused by the application of system gamma removal on R, G, and B samples instead of Y. DRA-2 required only 2 scale values that needs to be signaled for each frame. It has been observed that different gamma values are needed for different types of content in order to achieve a good looking SDR picture. Therefore gamma values used in “System Gamma Removal” are calculated from the first frame of each test sequence and the same gamma value is used throughout the test. Calculation of the gamma values are implemented in the HDRTools software and it does not require external information. The algorithm selects to lower the value of system gamma depending on the distribution of luminance in the first frame of each content. As a result, lower system gamma values are assigned for brighter content.

Figure 4. First frame of EBU_06_Starting sequence coded with proposed method, rate point 2

Figure 5. First frame of GarageExit sequence coded with proposed method, rate point 2

Figure 6. First frame of Market sequence coded with proposed method, rate point 2

Figure 7. First frame of SunRise sequence coded with proposed method, rate point 2

Figure 8. First frame of BikeSparklers cut2 sequence coded with proposed method, rate point 2

## Parameter Derivation from Graded SDR

As shown in Figure 2, one DRA block in YCbCr domain is sufficient to replicate the graded SDR picture from PQ coded signal. Least square fitting is used to minimize the difference between the graded SDR and DRA applied to PQ coded signal in YCbCr domain.

# Signaling of Parameters through SEI message

The DRA parameters are signaled to the HDR receiver through an SEI message. The SEI message proposed here is implemented in HM16.7 and used in testing.

This SEI message provides information to enable the reconstruction of the HDR colour samples of the reconstructed HDR pictures from the reconstructed colour samples of the decoder output pictures. The HDR reconstruction process may be applied to the YCbCr or RGB components (specified in Annex Error: Reference source not found), directly produced by the decoder, or resulting from colour space conversion of the decoded pictures colour samples.

General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ... |  |
| else if( payloadType = = XXX ) |  |
| hdr_reconstruction_info( payloadSize ) |  |
| ... |  |
| } |  |

HDR reconstruction information SEI message syntax

| hdr_reconstruction_info( payloadSize ) { | Descriptor |
| --- | --- |
| hdr_recon_id | ue(v) |
| hdr_recon_cancel_flag | u(1) |
| if( !hdr_recon_cancel_flag ) { |  |
| hdr_recon_persistence_flag | u(1) |
| hdr_recon_transfer_characteristics | u(8) |
| hdr_recon_default_flag | u(1) |
| if( !hdr_recon_default_flag ) { |  |
| hdr_recon_scale_bit_depth | u(4) |
| hdr_recon_offset_bit_depth | u(4) |
| hdr_recon_scale_frac_bit_depth | u(4) |
| hdr_recon_offset_frac_bit_depth | u(4) |
| hdr_recon_num_comps_minus1 | ue(v) |
| } |  |
| for( c = 0; c <= hdr_recon_num_comps_minus1; c++ ) { |  |
| hdr_recon_num_ranges[ c ] | ue(v) |
| hdr_recon_equal_ranges_flag[ c ] | u(1) |
| hdr_recon_global_offset_val[ c ] | u(v) |
| for( i = 0; i <= hdr_recon_num_ranges[ c ]; i++ ) |  |
| hdr_recon_scale_val[ c ][ i ] | u(v) |
| if( !hdr_recon_equal_ranges[ c ] ) |  |
| for( i = 0; i <= hdr_recon_num_ranges[ c ]; i++ ) |  |
| hdr_recon_range_val [ c ][ i ] | u(v) |
| } |  |
| } |  |

Semantics of the HDR reconstruction SEI message (DRA SEI)

hdr_recon_id contains an identifying number that may be used to identify the purpose of the HDR reconstruction information. The value of hdr_recon_id shall be in the range of 0 to 232 − 2, inclusive. Values greater than 2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore all HDR reconstruction information SEI messages that contain a value of hdr_recon_ id greater than 2, and bitstreams shall not contain such values.

hdr_recon_id equal to 0 specifies that the mapping applies in the YCbCr domain, and that the mapping of each colour sample is achieved by applying a piece-wise linear based- function.. If input colour samples of the mapping process are not in YCbCr domain, colour space conversion of input colour samples should be applied first. For each colour component an individual piece-wise linear function is signalled, function for Y first, followed by functions for Cb and Cr if applicable.

NOTE 1 – When hdr_recon_id is equal to 0, the mapping process of input samples Yin, Uin, Vin into output samples Yout, Uout, Vout consists of the following equations:

Yout = map[ 0 ]( Yin )

Uout = map[ 1 ]( Uin )

Vout = map[ 2 ]( Vin )

where map[ c ]( ) is the function modeled by the syntax elements hdr_recon_scale_bit_depth, hdr_recon_offset_bit_depth, hdr_recon_scale_frac_bit_depth, hdr_recon_offset_frac_bit_depth, hdr_recon_num_comps_minus1, hdr_recon_num_ranges[ c ], hdr_recon_equal_ranges_flag[ c ], hdr_recon_global_offset_val[ c ], hdr_recon_scale_val[ c ] and hdr_recon_range_val[ c ], for c = 0, 1 and 2.

hdr_recon_id equal to 1 specifies that the mapping applies in the YCbCr domain and RGB domain, and that the mapping of each colour sample is achieved by applying a piece-wise linear based- function. The mapping process is first applied on samples of Cr and Cb first, followed by colour space conversion to RGB samples, and followed by mapping process applied on R,G and B colour samples.  A single piece-wise linear function is signalled for R,G and B colour components, followed by signalling of an individual piece-wise lienar function for Cr and Cb colour components.

NOTE 2 – When hdr_recon_id equal to 1, the mapping process of input samples Uin, Vin, Rin, Gin, Bin into output samples Uout, Vout, Rout, Gout, Bout consist of the following equations:

Uout = map[ 1 ]( Uin )

Vout = map[ 2 ]( Vin )

Rout = map[ 0 ]( Rin )

Gout = map[ 0 ]( Gin )

Bout = map[ 0 ]( Bin )

where map[ c ]( ) is the function modeled by syntax elements hdr_recon_scale_bit_depth, hdr_recon_offset_bit_depth, hdr_recon_scale_frac_bit_depth, hdr_recon_offset_frac_bit_depth, hdr_recon_num_comps_minus1, hdr_recon_num_ranges[ c ], hdr_recon_equal_ranges_flag[ c ], hdr_recon_global_offset_val[ c ], hdr_recon_scale_val[ c ] and hdr_recon_range_val[ c ], for c = 0, 1 and 2.

The mapping of sample x from component c to sample y = map[ c ]( x ) is specified as follows:

–	The variable ScaleValue[ c ][ i ], for i in the range of 0 to hdr_recon_num_ranges[ c ] – 1, inclusive, is derived as follows:

ScaleValue[ c ][ i ] = hdr_recon_scale_val[ c ][ i ](1  <<  hdr_recon_scale_frac_bit_depth ) 	(D-xx)

–	The variable RangeValue[ c ][ i ], for i in the range of 0 to hdr_recon_num_ranges[ c ] – 1, inclusive, is derived as follows:

–	If hdr_recon_equal_ranges_flag[ c ] is equal to 0, the following applies:

RangeValue[ c ][ i ] = hdr_recon_range_val[ c ][ i ]  >>  hdr_recon_offset_frac_bit_depth ) 
		( 1  <<  hdr_recon_offset_frac_bit_depth ) 	(D-xx)

–	Otherwise ( hdr_recon_equal_ranges_flag[ c ] is equal to 1 ), the following applies:

RangeValue[ c ][ i ] = InputDynamicRangeValuehdr_recon_num_ranges[ c ] 	(D-xx)

where InputDynamicRangeValue is equal to 1 when the sample range is normilized from 0 to 1.

–	The variables InputRanges[ c ][ i ] and OutputRanges[ c ][ i ], for i in the range of 0 to hdr_recon_num_ranges[ c ], inclusive, are derived as follows:

–	If i is equal to 0, the following applies:

OutputRanges[ c ][ i ] = - hdr_recon_global_offset_val[ c ] * ScaleValue[ c ][ i – 1 ]	(D-xx)

InputRanges[ c ][ i ] = 0		(D-xx)

–	Otherwise (i is not equal to 0), the following applies:

InputRanges[ c ][ i ] = InputRanges[ c ][ i − 1 ] + RangeValue[ c ][ i – 1 ] 	(D-xx)

OutputRanges[ c ][ i ] = OutputRanges[ c ][ i − 1 ] + 
		RangeValue[ c ][ i – 1 ] * ScaleValue[ c ][ i – 1 ] 	(D-xx)

–	The parameters OffsetValue[ c ][ i ], for i in the range of 0 to hdr_recon_num_ranges[ c ] – 1, inclusive, are derived as follows:

OffsetValue[ c ][ i ] = InputRanges[ c ][ i+1 ] − OutputRanges[ c ][ i+1 ] / ScaleValue[ c ][ i – 1 ] 	(D-xx)

–	The parameter y = map[ c ]( x ) is derived as follows:

–	If x is lower than or equal to OutputRanges[ c ][ 0 ], the following applies:

y = InputRanges[ c ][ 0 ] 		(D-xx)

–	Otherwise if x is larger than OutputRanges[ c ][ hdr_recon_num_ranges[ c ] ], the following applies:

y = InputRanges[ c ][ hdr_recon_num_ranges[ c ] ]	(D-xx)

–	Otherwise, the following applies:

for( i = 1; i < = hdr_recon_num_ranges[ c ]; i++ )
			if( OutputRanges[ i – 1 ] < x  &&  x < = OutputRanges[ i ] )
				y = x / ScaleValue[ c ][ i – 1 ]  + OffsetValue[ c ][ i – 1 ] 	 (D-xx)

hdr_recon_cancel_flag equal to 1 indicates that the HDR reconstruction information SEI message cancels the persistence of any previous HDR reconstruction information SEI message in output order that applies to the current layer. hdr_recon_cancel_flag equal to 0 indicates that HDR reconstruction information follows.

hdr_recon_persistence_flag specifies the persistence of the HDR reconstruction information SEI message for the current layer.

hdr_recon_persistence_flag equal to 0 specifies that the HDR reconstruction information applies to the current picture only.

Let picA be the current picture. hdr_recon_persistence_flag equal to 1 specifies that the HDR reconstruction information persists for the current layer in output order until either of the following conditions is true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a HDR reconstruction information SEI message with the same value of hdr_recon_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

hdr_recon_transfer_characteristics has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element, except that hdr_recon_transfer_characteristics specifies the colour space of the reconstructed HDR picture, rather than the colour space used for the CLVS. When not present, the value of hdr_recon_transfer_characteristics is inferred to be equal to the value of transfer_characteristics.

hdr_recon_default_flag equal to 0 specifies that the syntax elements hdr_recon_scale_bit_depth, hdr_recon_offset_bit_depth, hdr_recon_scale_frac_bit_depth, hdr_recon_offset_frac_bit_depth, and hdr_recon_num_comps_minus1, are explictly signalled. hdr_recon_default_flag equal to 1 specifies that the syntax elements hdr_recon_scale_bit_depth, hdr_recon_offset_bit_depth, hdr_recon_scale_frac_bit_depth, hdr_recon_offset_frac_bit_depth, and hdr_recon_num_comps_minus1 are not explictly signalled, and the values of these syntax elements are derived from Table X.

hdr_recon_scale_bit_depth specifies the number of bits used to signal the syntax element hdr_recon_scale_val[ c ][ i ]. The value of hdr_recon_scale_bit_depth shall be in the range of 0 to 15, inclusive. Default value is determined from Table X

hdr_recon_offset_bit_depth specifies the number of bits used to signal the syntax elements hdr_recon_global_offset_val[ c ] and hdr_recon_offset_val[ c ][ i ]. The value of hdr_recon_offset_bit_depth shall be in the range of 0 to 15, inclusive. Default value is determined from Table X

hdr_recon_scale_frac_bit_depth specifies the number of LSBs used to indicate the fractional part of the scale parameter of the i-th partition of the c-th component. The value of hdr_recon_scale_frac_bit_depth shall be in the range of 0 to 15, inclusive. The value of hdr_recon_scale_frac_bit_depth shall be less than or equal to the value of hdr_recon_scale_bit_depth. Default value is determined from Table X

hdr_recon_offset_frac_bit_depth specifies the number of LSBs used to indicate the fractional part of the offset parameter of the i-th partition of the c-th component and global offset of the c-th component. The value of hdr_recon_offset_frac_bit_depth shall be in the range of 0 to 15, inclusive. The value of hdr_recon_offset_frac_bit_depth shall be less than or equal to the value of hdr_recon_offset_bit_depth. Default value is determined from Table X.

hdr_recon_num_comps_minus1 plus 1 specifies the number of components for which the component scaling function is specified. hdr_recon_num_comps_minus1 shall be in the range of 0 to 2, inclusive. Default value is determined from Table X

hdr_recon_num_ranges[ c ] specifies the number of ranges in to which the output sample range is partitioned in to. The value of hdr_recon_num_ranges[ c ] shall be in the range of 0 to 63(32), inclusive.

hdr_recon_equal_ranges_flag[ c ] equal to 1 indicates that that output sample range is partitioned into hdr_recon_num_ranges[ c ] nearly equal partitions, and the partition widths are not explicitly signalled. hdr_recon_equal_ranges_flag[ c ] equal to 0 indicates that that output sample range may be partitioned into hdr_recon_num_ranges[ c ] partitions not all of which are of the same size, and the partitions widths are explicitly signalled.

hdr_recon_global_offset_val[ c ] is used to derive the offset value that is used to map the smallest value of the valid input data range for the c-th component. The length of hdr_recon_global_offset_val[ c ] is hdr_recon_offset_bit_depth bits.

hdr_recon_scale_val[ c ][ i ] is used to derive the offset value that is used to derive the width of the of the i-th partition of the c-th component. The length of hdr_recon_global_offset_val[ c ] is hdr_recon_offset_bit_depth bits.

hdr_recon_range_val[ c ][ i ] is used to derive the width of the of the i-th dynamic range partition of the c-th component that is used to derive the offset value for the i-th dynamic range partition of the c-th component. The length of hdr_recon_range_val[ c ] is hdr_recon_offset_bit_depth bits.

Table X. Specification of default values signalled parameters

| hdr_recon_id | 0 | 1 |
| --- | --- | --- |
| hdr_recon_scale_bit_depth | 8 | 8 |
| hdr_recon_offset_bit_depth | 8 | 8 |
| hdr_recon_scale_frac_bit_depth | 6 | 6 |
| hdr_recon_offset_frac_bit_depth | 8 | 8 |
| hdr_recon_num_comps_minus1 | 2 | 2 |

# Results

Table 1 shows the simulation results compared to v3.2 of CE 1 test.

Table 1. Objective test results compared to CE 1 anchor v3.2

# Conclusion

In this document, subjective and objective results of CE6 test 4.1 are provided.

# References

[1] D. Rusanovskyy, A. Ramasubramonian, D. Bugdayci, S. Lee, J. Sole, M. Karczewicz, “Report on CE

2.1.3 test: Single-layer HDR video coding based on m36256”, m37064, Geneva, CH, Oct. 2015.

[2] D. Rusanovskyy, S. Lee, D. Bugdayci, A. Ramasubramonian, J. Sole, M. Karczewicz (QCOM), A.M. Tourapis, Y. Su, D. Singer (Apple), C. Fogg (MovieLabs), A. Duenas, F. Bossen (NGCodec), “Single layer non-normative (category 3a) NCL and CL responses to the Call for Evidence on HDR/WCG”, m36256, Warsaw, PL, Jun. 2015

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).