| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC- W0115 |
| --- | --- |

| Title: | Software implementation of visual information fidelity (VIF) for HDRTools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Informative Document |  |  |
| Author(s) or Contact(s): | Hamid Reza Tohidypour1, M. Azimi1, Mahsa T. Pourazad2,3, and Panos Nasiopoulos1, 3 | Tel: Email: | (604) 827-4878 htohidyp@ece.ubc.ca maryama@ece.ubc.ca mahsa.pourazad@gmail.com panos.p.n@ieee.org |
| Source: | 1University of British Columbia (UBC), 2TELUS Communications Inc., 3 The Institute for Computing, Information and Cognitive Systems (ICICS) |  |  |

_____________________________

# Abstract

Visual Information Fidelity (VIF) measure is a full reference quality assessment method. In this contribution, the implementation of VIF for HDRTools is presented.

# VIF extension for HDRTools

To implement VIF [1, 2] to the HDRTools package (ver. 0.8.1) [3], the following modifications have been made:

- A new define preprocessor macro called ‘VIF_SUPPORT’ is added to the HDRTools package. To enable the VIF feature of the code, this module is set to one.

- A variable (VIFBitDepthY) is added in the configuration, which needs to be set to the bit depth of the input signals.

- Two types of steerable pyramid filters [1, 4] are added including sp1filters and sp5filters [1]. For each of them a define macro is introduced. The default version of software uses sp5filters [1].

- The new functions related to VIF computations have been implemented in Common library of HDRTools package.

- VIF value is reported as a new distortion measure per frame and average is also reported in summary.

Note that the current version supports only OpenEXR files of type BT.709 and BT. 2020.

# References

- H. R. Sheikh and A. C. Bovik “A Visual Information Fidelity measure for image quality assessment”, Available: http://live.ece.utexas.edu/research/quality/VIF.htm

- H. R. Sheikh and A. C. Bovik , "A Visual Information Fidelity Approach to Video Quality Assessment" (Invited Paper), The First International Workshop on Video Processing and Quality Metrics for Consumer Electronics, Scottsdale, AZ, January 23-25, 2005.

- A.M. Tourapis, Y. Su, D. Singer, “HDRTools: A software package for Video Processing applications”, Q6/16 document C886, Geneva, SW, Oct. 2015

- E P Simoncelli and W T Freeman, “The Steerable Pyramid: A Flexible Architecture for Multi-Scale Derivative Computation”, Second Int. Conf. on Image Processing, Washington, DC, Oct. 1995.