| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0125 |
| --- | --- |

| Title: | CE6-related: Cross check report for Test 4.1 Reshaper from m37064 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Block D, Centre House 56 Wood Lane, W12 7SB London – UK | Tel: Email: | +44 (0)3030409640 matteo.naccari@bbc.co.uk |
| Source: | BBC |  |  |

_____________________________

# Abstract

This report describes the cross check made by the BBC for Test 4.1 belonging to the Core Experiment 6. The data distributed by the proponent match with the ones recreated by the cross-checker. Moreover, source code inspection revealed a good mapping between algorithmic description and implementation.

# Introduction

Core Experiment 6 (CE6) aims to investigate post-processing techniques to reconstruct the HDR signal from the output of an HEVC Main10 decoder. Different tests have been defined for this core experiment and in Test 4.1 the reshaper proposed in Document m37064 and W0103 is assessed. This reshaper defines an Adaptive Transfer Function (ATF) to adjust the dynamic range of an HDR signal coded with a particular transfer function (e.g. ST 2084). The information needed by the ATF is encoded and compressed in an HEVC bitstream using a dedicated SEI message. The use case considered in Test 4.1 is backward compatibility for SDR legacy receivers. Accordingly, a decoded video graded with ST 2084 is dynamically adjusted to make it SDR. The parameters used by the ATF to recover the HDR component are sent via the SEI message and inferred using a reference transfer characteristics curve. The curve selected for this test is the Hybrid Log Gamma (HLG) with EOTF applied on a per component basis as specified in Experiment 7.1a of CE7 (see N15800 for more details).

This document reports on the cross check performed by the BBC to match the MD5 sums and metric values circulated by the proponent. It is reported a full match for both these data sets. The following sections will report on the processing work flow, the cross check results and source code inspection.

# Processing workflow and data matching

To produce the content coded with the proposed reshaper two software tools are used:

- HDRTools modified by the proponent to derive and/or use the parameters associated with the ATF

- HM modified by the proponent to output the ATF SEI message

At a very high level of description, the overall processing workflow for each video can be described as follows:

- Produce the nonlinear YCbCr 4:2:0 content from the test material using the HDRTools provided

- Write onto text files the parameters needed by the ATF to reconstruct the HDR signal at the receiver always using the HDRTools provided

- Compress the YCbCr 4:2:0 content using the modified HM software which reads the aforementioned text files and embeds in the bitstream the ATF SEI message(s)

- Decode the produced bitstream using the modified HM software which also writes onto text files the parameters used by the ATF to recover the HDR signal

- Produce the OpenEXR HDR content using the modified HDRTools which reads the aforementioned text files produced by the decoder

For each generated file, an MD5 sum string was provided by the proponent. More specifically the following MD5 sums have been circulated:

- YCbCr 4:2:0 file resulting from Step 1 above

- HEVC elementary stream produced at Step 3 above

- Decoded YCbCr 4:2:0 file resulting from Step 3 above

- Concatenated OpenEXR file associated to all decoded frames and produced according to Step 5 above

The cross checker has reproduced the aforementioned files and was able to match all MD5 sums. Moreover, the values for the metrics agreed in the ad-hoc group were provided by the proponent and confirmed by the cross checker. The spreadsheet attached to this report show the matching for all material.

# Source code inspection

The two software tools provided by the proponent have been inspected by the cross-checker. For the modified HM software a macro (QC_ATF_SEI_MSG) has been defined to enclose all code associated with the SEI message generation and writing. This code implements the machinery to read the text files with the parameters for ATF as well as the setting and writing of the SEI message.

For the modified HDRTools software a macro (QC_ATF_SDR_AUTO) has been defined to track all changes associated with the use of the proposed transfer function as well as the read and write of its parameters on text files. Given time constraints at the cross checker’s side, the code hasn’t been thoroughly inspected. A revision of this report may be uploaded at a later stage to provide more details on this aspect.

# Specification text inspection

Not provided at the time of cross-checking

# Points for discussion

Some observations to be discussed/addressed during the meeting are as follows.

- Is not clear why the proponent decided to use the HLG transfer function with removal of the system gamma on the RGB components. This solution is not what the proponent of HLG advocates and the data shown during the interim meeting in Vancouver confirmed that the images produced look washed out

- It’d good to have a demo of the proposal during the meeting to appraise the quality for the HDR signal. Given time constraints at the cross checker’s side, a visual inspection couldn’t be performed

- It’d good to understand how the SDR backward compatible image can be shown. Given that there are no TV sets implementing the proposal, a possible solution would be to push the YCbCr files through a legacy chain (e.g. BT.1886, gamma 2.4) and assume 100 nits peak brightness of the screen. The produced EXR files can be cat together in an AVI file using the Sim2Convert tool and then viewed on a SIM2 display

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).