| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0128 |
| --- | --- |

| Title: | HDR CE2: Cross-check of JCTVC-W0084 (combines solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D.Rusanovskyy | Tel: Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This report describes the cross check made by the Qualcomm Inc. for the SW of the JCTVC-W0084 (combines solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3). Simulation results distributed by the proponent for two test configurations (new luma reshaper for ETM mode 0 and its combination with encoder optimization) has been successfully cross-checked.

Visual quality improvement over the reference ETM mode 0 design is observed for some of the test sequences. Especially noticeable quality improvement by a new reshaper design is observed in a form of is black-hole artifacts removal, which are very critical for luma reshaper currently being utilized in the reference ETM. For some of the test sequences mixed results are observed either in a form of bitrate redistribution between different partitions of the dynamical range or different pictures within prediction structure. In comparison against the CE1 anchor 3.2, it was observed that W0084 seems to provide better visual quality, especially at the low bitrates rate points.

Further discussion of the proposed technology, current implementation and visual quality assessment during this meeting is suggested.

# Introduction

The proposal W0084 targets the ETM mode 0 design and provide a solution, which address several CE2 subtests jointly: CE2.a-2 on luma forward reshaping improvement, CE2.c on chromaQPOffset, CE2.d on DeltaQP adjustment for luma, and CE2.e-3 on automatic selection of ETM parameters for mode 0.

# Distributed SW package

Proponents distributed the SW package consisting of modified ETM components: modified HDRTools with new a luma reshaping derivation and modified HM codec consisting of new encoder side optimization, as well as a minor change to the inverse reshaper at the decoder side.

Modified HDRTools package features an alternative to ETM luma reshaper derivation algorithms (incorporating signal range extension) and modified automatic reshaper update algorithm.

Modified HM codec features encoder optimization methods are based on three tools from CE1 test: deblocking filter parameter selection, chromaQPOffset and DeltaQP adjustment for luma. These 2 encoder-only methods are being adjusted by parameters of luma and chroma reshapers produced at the preprocessing stage. No changes to the current ETM syntax are proposed in the W0084. In addition to above, further discussion is encouraged on currently proposed implementation of full/standard range conversion at the inverse reshaper.

# Simulation results

Proponents of W0084 proposed two SW configurations and shared simulation results for those two configuration. The first, Test 1, studied luma reshaper with no changes to the HM encoder, and the second, Test 2, studied combination of luma reshaper with proposed changes to HM encoder.

Both test configuration has been successfully cross-checked with simulation results produced by QCOM matching those, reported by Dolby. Cross-checked simulation results for Test 1 and Test 2 are provided in Table 1 and 2 accordingly.

Table 1. Objective simulation results for Test 1 (Luma reshaper) being compared against the CE1 anchor 3.2

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | -10.2% | -3.2% | -11.9% | -8.7% | -13.7% | -17.6% | -17.7% | 3.5% |
| Market3Clip4000r2 | -4.4% | -4.7% | 1.5% | -2.4% | 2.1% | 42.3% | -82.6% | -3.2% |
| SunRise | -6.3% | -6.6% | -2.0% | -4.8% | 7.3% | 60.8% | -41.6% | 10.9% |
| BikeSparklers cut 1 | -8.3% | -9.7% | 2.1% | -5.0% | -8.8% | 8.4% | -35.0% | -4.3% |
| BikeSparklers cut 2 | -4.7% | -5.9% | 7.3% | -1.4% | -4.2% | 18.3% | -25.3% | -2.4% |
| GarageExit | 2.9% | 0.5% | 24.7% | 10.1% | 10.8% | 34.5% | 18.1% | 0.3% |
| ShowGirl2Teaser | 3.8% | 5.6% | 7.7% | 5.7% | 11.4% | 3.4% | -1.5% | 2.8% |
| StEM_MagicHour cut 1 | -3.8% | -2.8% | 13.2% | 4.6% | 5.0% | 8.2% | 51.0% | -2.1% |
| StEM_MagicHour cut 2 | -0.8% | 0.6% | 6.5% | 3.0% | 2.8% | -0.4% | -3.3% | 0.9% |
| StEM_MagicHour cut 3 | -1.3% | -0.2% | 9.3% | 4.3% | 4.3% | 4.0% | 34.3% | 0.5% |
| StEM_WarmNight cut 1 | 9.6% | 9.5% | 11.3% | 10.2% | 9.3% | 8.7% | 19.1% | -5.4% |
| StEM_WarmNight cut 2 | -1.2% | 0.9% | 0.5% | 0.1% | 3.3% | -6.8% | -11.8% | -3.0% |
| BalloonFestival | 7.0% | 10.0% | 9.6% | 9.0% | 14.0% | 36.7% | -23.5% | -5.5% |
| EBU_04_Hurdles | 2.0% | -5.7% | 15.5% | 5.4% | 8.4% | 90.9% | 113.5% | -11.2% |
| EBU_06_Start | -0.9% | -6.6% | 7.2% | 0.3% | 3.6% | 77.5% | 46.1% | -12.8% |
| Overall | -1.1% | -1.2% | 6.8% | 2.0% | 3.7% | 24.6% | 2.7% | -2.1% |

Table 2. Objective simulation results for Test 2 (Luma reshaper and joint pre- and encoder optimization) being compared against the CE1 anchor 3.2

|  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 1.9% | 7.1% | -6.8% | 1.0% | 0.0% | -6.3% | -17.7% | 3.4% |
| Market3Clip4000r2 | 3.4% | 3.6% | 1.6% | 2.8% | -1.0% | 12.8% | -80.3% | -10.8% |
| SunRise | 4.7% | 6.6% | 3.1% | 4.7% | 4.9% | 34.4% | -40.4% | 0.4% |
| BikeSparklers cut 1 | -5.5% | -6.3% | -2.9% | -4.7% | -9.6% | -7.9% | -27.2% | -0.6% |
| BikeSparklers cut 2 | -3.1% | -3.7% | -1.4% | -2.6% | -7.4% | -2.9% | -45.3% | 0.0% |
| GarageExit | 7.0% | 6.4% | 11.1% | 8.3% | 5.6% | 11.7% | 0.3% | 3.3% |
| ShowGirl2Teaser | 10.7% | 12.8% | 11.9% | 11.7% | 16.1% | -4.9% | -2.8% | -0.3% |
| StEM_MagicHour cut 1 | -2.9% | 0.0% | 1.0% | -0.1% | -1.2% | -10.2% | 0.6% | -0.9% |
| StEM_MagicHour cut 2 | -0.9% | 0.8% | 2.3% | 1.2% | 0.7% | -7.6% | -5.0% | 0.2% |
| StEM_MagicHour cut 3 | -0.5% | 1.1% | 3.6% | 2.1% | 1.9% | -5.5% | 23.2% | 0.0% |
| StEM_WarmNight cut 1 | 8.7% | 10.5% | 1.4% | 6.3% | 4.1% | -11.9% | 1.3% | -4.2% |
| StEM_WarmNight cut 2 | 2.1% | 4.4% | -4.6% | -0.2% | 2.9% | -12.8% | -29.3% | -2.0% |
| BalloonFestival | 14.9% | 19.4% | 9.1% | 13.7% | 9.2% | 19.0% | -21.9% | -5.1% |
| EBU_04_Hurdles | 2.9% | -1.4% | 1.4% | 0.9% | -2.6% | 28.5% | 64.9% | -11.0% |
| EBU_06_Start | 13.6% | 10.1% | 6.9% | 9.9% | 6.2% | 50.3% | -17.0% | -5.7% |
| Overall | 3.8% | 4.8% | 2.5% | 3.7% | 2.0% | 5.8% | -13.1% | -2.2% |

Visual assessment was conducted for both test configuration. For the Test 1 visual quality improvement observed for several test sequences due to modified luma reshaper and usage of a full range instead of standard range. Introduced change to luma component representation resulted in bit allocation shift from chroma to luma and resulted in more severe chroma artifacts, large then those observed in ETM. It should be noted, that luma reshaper proposed in W084 resolves a critical for the reference ETM problem of “black hole artifacts”. Proposed reshaper does not feature those artifacts.

For the Test 2, in direct comparison with ETM mode 0, visual quality improvement was observed for several test sequences. For a group of test sequences, mixed improvement is observed, where improvement in some ranges is compensated by degradation in another range. It is understood that this results originate from bitrate redistribution between different partitions of the dynamical range. For a group of test sequences, temporally varying quality was observed. It was observed by a cross-checker for several sequences, that Test 2 results in HDR picture being coded with intra prediction typically features higher quality than picture produced by the ETM design. At the same time, an HDR picture being coded with inter-prediction down the cascaded prediction feature worse quality than respective ETM coded picture.

In direct comparison of W0084 configurations against the CE1 anchor v3.2, visual quality improvement is observed.

# Conclusion

This report describes the cross check made by the Qualcomm Inc. for the SW of the JCTVC-W0084 (combines solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3). Simulation results distributed by the proponent for two test configurations (new luma reshaper and its combination with encoder optimization) has been successfully cross-checked.