| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0129 |
| --- | --- |

| Title: | SCC Level Limits Based on Chroma Format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Seung-Hwan Kim 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | 360-817-8486 sdeshpande@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

Current calculation in HEVC SCC Draft 5 for MaxDpbSize does not account for different chroma subsampling. This proposal describes extending the current design to support using higher MaxDpbSize at lower chroma format values. In this document MaxDpbSize calculation which accounts for memory use due to chroma components is proposed for Screen-Extended Main 10 4:4:4 profile.

# Introduction

The current HEVCScreen Content Coding draft 5 [1] defines the calculation of MaxDpbSize in Annex A. This calculation calculates different value for MaxDpbSize based on PicSizeInSamplesY to take advantage of available memory capacity. However the MaxDpbSize is not calculated based on the chroma format.

In this document MaxDpbSize calculation which accounts for memory use due to chroma components is proposed for Screen-Extended Main 10 4:4:4 profile.

# Proposal

The existing process for computing MaxDpbSize is extended by introducing a scaling factor (SFactor) which accounts for chroma subsampling for Screen-Extended Main 10 4:4:4 profile.

The proposed text changes are highlighted below compared to JTVC-V1005-v1 [1].

A.4.2 Profile-specific level limits for the video profiles

NOTE – Video profiles refer to those profiles that are not still picture profiles. The still picture profiles include the Main Still Picture profile, the Main 4:4:4 Still Picture profile, and the Main 4:4:4 16 Still Picture profile.

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

The variable HbrFactor is defined as follows:

- If the bitstream or sub-layer representation is indicated to conform to the Main profile or the Main 10 profile, HbrFactor is set equal to 1.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4, High Throughput 4:4:4 10, High Throughput 4:4:4 14, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, or Screen-Extended High Throughput 4:4:4 14  profile, HbrFactor is set equal to 6.

- Otherwise, if the bitstream or sub-layer representation is indicated to conform to the High Throughput 4:4:4 16 Intra profile, HbrFactor is set equal to 24 − ( 12 * general_lower_bit_rate_constraint_flag ) or 24 − ( 12 * sub_layer_lower_bit_rate_constraint_flag[ i ] ).

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag or 2 − sub_layer_lower_bit_rate_constraint_flag[ i ].

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

The variable SFactor is defined as follows:

- If the bitstream or sub-layer representation is indicated to conform to the Screen-Extended Main 10 4:4:4 profile, and if chroma_format_idc is equal to 1, SFactor is set equal to (1 2)

- Otherwise, SFactor is set equal to 1

When the specified level is not level 8.5, the value of sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf/SFactor, 16 )
else if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf/SFactor, 16 )	(A-1)
else if( PicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf/SFactor ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf/SFactor

where MaxLumaPs is specified in Table A.6, and maxDpbPicBuf is equal to 6 for all profiles where the value of sps_curr_pic_ref_enabled_flag is required to be equal to 0 and 7 for all profiles where the value of sps_curr_pic_ref_enabled_flag is not required to be equal to 0.

# Results of proposed extension of MaxDpbSize

Results for MaxDpbSize from our proposed formula for Screen-Extended Main 10 4:4:4 profile are shown below for when chroma format is 4:2:0.

Table 1- MaxDpbSize for various Scale Factors when sps_curr_pic_ref_enabled_flag=0

| PicSizeInSamplesY | SFactor=1 (HEVC SCC Draft 5) | SFactor=12 |
| --- | --- | --- |
| PicSizeInSamplesY ≤ (MaxLumaPS>>2) | 16 | 16 |
| (MaxLumaPS>>2) < PicSizeInSamplesY ≤ (MaxLumaPS>>1) | 12 | 16 |
| (MaxLumaPS>>1) < PicSizeInSamplesY ≤ ((3*MaxLumaPS)>>2) | 8 | 16 |
| ((3*MaxLumaPS)>>2) < PicSizeInSamplesY ≤ 1.0 * MaxLumaPS | 6 | 12 |

Table 2- MaxDpbSize for various Scale Factors when sps_curr_pic_ref_enabled_flag!=0

| PicSizeInSamplesY | SFactor=1 (HEVC SCC Draft 5) | SFactor=12 |
| --- | --- | --- |
| PicSizeInSamplesY ≤ (MaxLumaPS>>2) | 16 | 16 |
| (MaxLumaPS>>2) < PicSizeInSamplesY ≤ (MaxLumaPS>>1) | 12 | 16 |
| (MaxLumaPS>>1) < PicSizeInSamplesY ≤ ((3*MaxLumaPS)>>2) | 8 | 16 |
| ((3*MaxLumaPS)>>2) < PicSizeInSamplesY ≤ 1.0 * MaxLumaPS | 7 | 14 |

# Conclusion

Modification is proposed for MaxDpbSize calculation for Screen-Extended Main 10 4:4:4 profile to account for chroma format. We propose to adopt the proposed approach in HEVC SCC Draft.

# References

[1] R. Joshi, S. Liu, G. J. Sullivan, G. Tech, J. Xu and Y. Ye, “High efficiency video coding (HEVC) Screen Content Coding: Draft 5,” JCTVC-V1005-v1, Geneva, January 2016.

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).