| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0133 |
| --- | --- |

| Title: | Indication of SMPTE 2094-20 metadata in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wiebe de Haan Rutger Nijland Leon van de Kerkhof Robert Brondijk | Tel: Email: | +31 40 27 96248 wiebe.de.haan@philips.com rutger.nijland@philips.com leon.van.de.kerkhof@philips.com Robert.brondijk@philips.com |
| Source: | Philips |  |  |

_____________________________

# Abstract

This contribution proposes text changes in the HEVC specification to support metadata of SMPTE 2094-20 standard: Dynamic Metadata for Color Volume Transform - Application #2. Specifically, in Annex D, a new SEI message is proposed for carriage of such metadata.

# Introduction

The ST 2094 suite of documents defines metadata for use in color volume transforms of content. The metadata are content-dependent and can vary scene-by-scene or frame-by-frame. The metadata are intended to transform High Dynamic Range and Wide Color Gamut (HDR/WCG) source content for presentation on a display having a smaller color volume than the source content’s mastering display [1].

ST 2094 Part 20 specifies the content-dependent color volume transform metadata items for Application #2 [2]. This color volume transform uses content-dependent dynamic metadata to reproduce the artistic intent of High Dynamic Range and Wide Color Gamut content as accurately as possible on displays having a smaller color volume than that of the display with which that content was mastered. This contribution proposes a new SEI message in Annex D to carry such metadata.

# Proposed Text

## SEI payload Syntax for ST 2094-20

D.2.4x Colour volume transform information SEI message syntax

| Color_Volume_Transform_processing_data( metadataSize) { | Descriptor |
| --- | --- |
| application_identifier | u(4) |
| application_version | u(3) |
| broadcast_flag | u(1) |
| if( broadcast_flag == 1 ) { |  |
| local_slope_adjustments( 0 ) |  |
| } |  |
| else { |  |
| scaled_video_flag | u(1) |
| targeted_system_display_maximum_luminance | u(32) |
| targeted_system_display_minimum_luminance | u(32) |
| nr_of_windows | u(4) |
| for( w=0; w < nr_of_windows; w++ ) { |  |
| if( w !=0 ) { |  |
| window_x_low[ w ] | u(12) |
| window_y_low[ w ] | u(12) |
| window_x_high[ w ] | u(12) |
| window_y_high[ w ] | u(12) |
| luminance_lower_bound[ w ] | u(12) |
| luminance_upper_bound[ w ] | u(12) |
| u_prime_chromaticity_disk_center[ w ] | u(10) |
| v_prime_chromaticity_disk_center[ w ] | u(10) |
| chromaticity_disk_radius[ w ] | u(10) |
| luminance_range_selector[ w ] | u(1) |
| chromaticity_area_selector[ w ] | u(1) |
| } |  |
| tone_mapping_input_signalweights_R[ w ] | u(8) |
| tone_mapping_input_signalweights_G[ w ] | u(8) |
| tone_mapping_input_signalweights_B[ w ] | u(8) |
| tone_mapping_input_signalweights_Y[ w ] | u(8) |
| local_slope_adjustments( w ) |  |
| } |  |
| } |  |
| } |  |
| local_slope_adjustments( w ) { |  |
| tone_mapping_input_signal_black_level_offset[ w ] | u(8) |
| tone_mapping_input_signal_white_level_offset[ w ] | u(8) |
| shadow_gain_control[ w ] | u(8) |
| highlight_gain_control[ w ] | u(8) |
| mid_tone_width_adjustment_factor[ w ] | u(8) |
| nr_of_ tone_mapping_fine_tuning_curve_points[w] | u(4) |
| for( j = 0; j < nr_of_tone_mapping_fine_tuning_curve_points[ w ]; j++ ) { |  |
| tone_mapping_fine_tuning_x[ w ][ j ] | u(8) |
| tone_mapping_fine_tuning_y[ w ][ j ] | u(8) |
| } |  |
| nr_of_saturation_gain_function_curve_points[ w ] | u(4) |
| for( j = 0; j < nr_of_saturation_gain_function_curve_points[ w ]; j++ ) { |  |
| saturation_gain_function_x[ w ][ j ] | u(8) |
| saturation_gain_function_y[ w ][ j ] | u(8) |
| } |  |
| } |  |

## SEI payload semantics

### D3.4x Colour volume transform information SEI message semantics

This SEI message provides information to enable colour volume transformation of the reconstructed colour samples of the output pictures. The input to the indicated colour volume transform process is either in luminance-chrominance color spaces or directly on RGB color components. In both cases, the set of decoded sample values shall apply an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format).

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of Society of Motion Picture and Television Engineers ST 2094-20.

application_identifier identifies an application and its defining document in ST-2094 suite. For this SEI message, application_identifier shall be set to 2.

application_version specifies the application version in the application defining document in ST-2094 suite. For this SEI message, application_identifier shall be set to 0.

broadcast_flag indicates whether a restricted or the full metadata set is used.

targeted_system_display_maximum_luminance specifies the nominal maximum display luminance, in units of 0.01 candelas per square metre.

targeted_system_display_minimum_luminance specifies the nominal minimum display luminance, in units of 0.0001 candelas per square metre. targeted_system_display_minimum_luminance shall be less than targeted_system_display_maximum_luminance.

scaled_video_flag indicates whether the video is scaled (=1) or not (=0).

nr_of_windows indicates the number of windows (for the whole picture and the pixel selectors) following. The first window definition shall be for the entire picture. The value of nr_of_windows shall be in the range of 1 to 3, inclusive, i.e. a maximum of 2 pixel selectors can be present.

window_x_low[ w ], window_y_low[ w ] indicates the upper left corner of the local processing window rectangle in pixels for window w. The value of window_x_low[ w ] and window_y_low[ w ] shall be in the range of 0 to 4095, inclusive.

window_x_high[ w ], window_y_high[ w ] indicates the right lower corner of the local processing window rectangle in pixels for window w. The value of window_x_high[ w ] and window_y_high[ w ] shall be in the range of 0 to 4095, inclusive. The value of window_x_high[ w ] shall be larger than the value of window_x_low[ w ]. The value of window_y_high[ w ] shall be larger than the value of window_y_low[ w ].

luminance_lower_bound[ w ] indicates the lower threshold for Luminance for window w. The value of luminance_lower_bound[ w ] shall be in the range of 0 to 4095, inclusive.
Y_low_value = 10(luminance_lower_bound ÷ 1000)-1.

luminance_upper_bound[ w ] indicates the upper threshold for Luminance for window w. The value of luminance_upper_bound[ w ] shall be in the range of 0 to 4095, inclusive. The value of luminance_upper_bound[ w ] shall be larger than the value of luminance_lower_bound[ w].
Y_high_value = 10(luminance_upper_bound ÷ 1000)-1.

u_prime_chromaticity_disk_center[ w ], v_prime_chromaticity_disk_center[ w ] indicate the center values for the chromaticity coordinates in a CIE 1976 uniform color space for window w. The value of u_prime_chromaticity_disk_center[ w ] and v_prime_chromaticity_disk_center[ w ] shall be in the range of 0 to 1023, inclusive.
u_prime_center, v_prime_center = u_prime_chromaticity_disk_center, v_prime_chromaticity_disk_center ÷ 1632.

chromaticity_disk_radius[ w ] indicates the threshold for the distance between the threshold center values (u_prime_center, v_prime_center) and the actual pixel data for window w. The value of chromaticity_disk_radius[ w ] shall be in the range of 0 to 1023, inclusive.

This distance to actual pixel data (u_prime, v_prime) is measured as 
Sqrt( (u_prime-u_prime_center)2 + (v_prime-v_prime_center)2 ).

The threshold center values and the threshold distance in fact define a disk in the uv-space. 
radius_uv_prime = chromaticity_disk_radius ÷ 1632.

luminance_range_selector[ w ] indicates whether the active area is within or outside the specified threshold range for luminance values for window w. A value equal to 0 means outside. A value equal to 1 means inside.

chromaticity_area_selector[ w ] indicates whether the active area is within or outside the specified threshold range for chromaticity values for window w, i.e. u_prime_value, v_prime_value, and radius_uv_prime. A value equal to 0 means outside. A value equal to 1 means inside.

tone_mapping_input_signal_black_level_offset[ w ] indicates black offset for window w. The value of tone_mapping_input_signal_black_level_offset[ w ] shall be in the range of 0 to 255, inclusive.
blo = tone_mapping_input_signal_black_level_offset ÷ 2040.

tone_mapping_input_signal_white_level_offset[ w ] indicates white offset for window w. The value of tone_mapping_input_signal_white_level_offset[ w ] shall be in the range of 0 to 255, inclusive.
wlo = tone_mapping_input_signal_white_level_offset ÷ 510.

shadow_gain_control[ w ] indicates the adjustment to the shadow (darker) region of the tone mapping for window w. The value of shadow_gain_control[ w ] shall be in the range of 0 to 255, inclusive.
shadow_gain = shadow_gain_control ÷ 127.5.

highlight_gain_control[ w ] indicates the adjustment to the highlight (brighter) region of the tone mapping for window w. The value of highlight_gain_control[ w ] shall be in the range of 0 to 255, inclusive.
highlight_gain = highlight_gain_control ÷ 127.5.

mid_tone_width_adjustment_factor[ w ] indicates the adjustment to the mid-tone region of the tone mapping for window w. The value of mid_tone_width_adjustment_factor[ w ] shall be in the range of 0 to 255, inclusive.
midtone_adjustment = mid_tone_width_adjustment_factor ÷ 127.5.

tone_mapping_input_signalweights_R[ w ], tone_mapping_input_signalweights_G[ w ], tone_mapping_input_signalweights_B[ w ], tone_mapping_input_signalweights_Y[ w ] indicate the relative weights of the R, G, B, and Y components in the tone mapping for window w. The value of tone_mapping_input_signalweights_R[ w ], tone_mapping_input_signalweights_G[ w ], tone_mapping_input_signalweights_B[ w ] and tone_mapping_input_signalweights_Y[ w ] shall be in the range of 0 to 255, inclusive.
tone_mapping_weight_x = tone_mapping_input_signalweights_x ÷ 255.

nr_of_tone_mapping_fine_tuning_curve_points[ w ] indicates the number of curve points between the fixed lowest and highest point for window w. 
The value of nr_of_ tone_mapping_fine_tuning_curve_points shall be ≤ 8.

nr_of_saturation_gain_function_curve_points[ w ] indicates the number of curve for window w. The value of nr_of_saturation_gain_function_curve_points shall be ≤ 6.

tone_mapping_fine_tuning_x[ w ][ j ], tone_mapping_fine_tuning_y[ w ][ j ] represents the j-th point for the luminance Custom Curve (CC) for window w. The value of tone_mapping_fine_tuning_x and tone_mapping_fine_tuning_y shall be in the range of 0 to 255, inclusive.
CC_lum_x,y = tone_mapping_fine_tuning_x,y ÷ 255. 
Custom Curve points with tone_mapping_fine_tuning_x equal to 0 and 255 shall not be included in the metadata as the associated tone_mapping_fine_tuning_y values in the receiver shall be fixed to 0 and 255 respectively.

saturation_gain_function_x[ w ][ j ], saturation_gain_function_y[ w ][ j ] represents the j-th point for the saturation Custom Curve (CC) for window w. The value of saturation_gain_function_x[ w ][ j ] and saturation_gain_function_y[ w ][ j ] shall be in the range of 0 to 255, inclusive.
The value for calculation CC_sat_x = 2-15*(saturation_gain_function_x  255). 
The value for calculation CC_sat_y = 2-15*( saturation_gain_function_y  255).

Note: the actual color volume transform method is outside the scope of this specification. A transform method using this SEI message is described in Annex B of reference [2].

# Reference

- SMPTE ST 2094-1:2016: "Dynamic Metadata for Color Volume Transform – Core Components"

- SMPTE ST 2094-20:2016: "Dynamic Metadata for Color Volume Transform – Application #2"

# Patent rights declaration(s)

Philips may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).