| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0014 |
| --- | --- |

| Title: | Ad hoc group report: HDR/WCG technology for backward compatibility and display adaptivity (AHG14) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | E. Francois, W. Husak, D. Rusanovskyy (co-chairs) | Tel: Email: |  |
| Source: | AHG14 |  |  |

_____________________________

# Abstract

This document provides a report of the AhG14 activity on HDR/WCG technology for backward compatibility and display adaptivity conducted between the 23rd and 24th JCTVC meetings. This document reports the mandates, summary of AhG activities and email discussions, list of AhG-related input contributions to 24th JCTV meeting and recommendations.

# Introduction

During 23rd JCTVC meeting there were multiple input contributions on HDR video coding. The contributions targeted several aspects of HDR video coding, including new tools for the improvement of HEVC compression efficiency for HDR/WCG video signals, providing backward compatibility as well advanced encoding configuration of HEVC main10 video codec specific for HDR/WCG signals.

As output of the study conducted in MPEG and JCTVC, it was concluded that no technology provided   greater than 20% compression efficiency improvements that would justify the creation of a new HDR specific HEVC profile. JCTVC initiated a work on producing a technical report on conversion and coding practices for HDR/WCG video coding using HEVC main10 profile.

JCTVC also identified several standard technologies that can be utilized for backward compatible HDR services, among which are HLG in combination with alternative transfer characteristics SEI message, the colour remapping information SEI message, and SHVC.

AhG14 was formed to continue study on HDR/WCG technologies for backward compatibility and display adaptivity, including but not limiting to study of single-layer solutions with existing and potentially new SEI messages, dual-layer coding with SHVC and study on relevant test conditions and experiments.

# Mandates

- Study the technical characteristics of single-layer coding with HEVC using existing SEI and VUI indicators for backward compatibility and display adaptivity.

- Study the technical characteristics of two-layer coding with SHVC for backward compatibility and  display adaptivity.

- Identify and study the technical characteristics of proposed approaches to backward compatibility and display adaptivity with potential additional new SEI messages.

- Study and propose test conditions for associated experiments.

# Activity

A kick-off message was sent on March 9, 2016, listing the mandates of ad hoc group 14 and suggesting an emails discussion on these issues to be submitted to JCT-VC reflector, jct-vc@lists.rwth-aachen.de.

There were around 20 emails submitted to the reflector. The topic related to the mandate 2 of the AhG14 received the most of expert’s attention.

There are 12 input contributions to this meeting which address problems of all four mandates of the AhG14.

## Mandate 1: Study the technical characteristics of single-layer coding with HEVC using existing SEI and VUI indicators for backward compatibility and display adaptivity.

There was no discussion over the reflector on issues related to the mandate 1. There are 2 contributions to this meeting which may be categorized as related to the mandate 1 of the AhG14.

Contribution [1] addresses the JCTVC conclusion from the 23rd meeting about possibility to utilize the Colour Remapping Information SEI message for providing a single-layer HDR coding with backward compatibility. Proposal [1] presents simulation results of utilizing CRI signaling and CRI post-processing to enable 2 types of backward compatibility: SDR display compatibility and SDR bitstream compatibility.  The contribution also proposes a draft text to document “Conversion and Coding Practices for HDR/WCG Video” on utilizing the CRI for enabling backward compatible HDR video.  The contribution [2] provides the cross-check of these results.

## Mandate 2: Study the technical characteristics of two-layer coding with SHVC for backward compatibility and display adaptivity.

During the discussion on mandate 2 this was pointed out, that SHVC verification test conducted for the previous meeting included a test on color gamut scalability. In this configuration, SDR/BT.709 video signal was coded at the base layer and HDR/BT.2020 video was coded as enhanced layer. Another useful scenario of SDR to HDR scalability was identified, namely when SDR/BT.2020 signal is coded at the base layer and HDR/BT.2020 is coded at the enhanced layer.  It was suggested to supplement the SHVC verification test with additional results addressing this use case.

Between 23rd and 24th meetings, expert collected additional test material, updated the encoder configuration and generated a draft supplemental SHVC verification test plan. The following is the timeline of the conducted SHVC-related activity:

- Test material in HDR/BT.2020 and SDR/BT.2020 collected by 12th of April 2016

- Updated HDRTools configurations and md5 sums distributed on 15th of April 2016

- Updated SHVC software and suggested tested rate points distributed on 25th of April 2016

- Test set is updated to address some recommendations of experts on 9th of May, 2016

- Encoded bitstreams made available to the test labs by May 11, 2016

- Subjective evaluation preparation and test conducted on: May 11-22, 2016

Cross-check of the bitstreams used in the tests is provided in [7].Detailed plan of the draft supplemental SHVC verification test is available in [3] and discussion on the test plan is suggested in [4]. Details on utilized SHVC configuration is available in [5] and subjective evaluation results conducted by GBTech Laboratory and Waitek Laboratory available in [6].

## Mandate 3: Identify and study the technical characteristics of proposed approaches to backward compatibility and display adaptivity with potential additional new SEI messages.

There was no discussion over the reflector on issues related to the mandate 3. There are 4 contributions to this meeting which are addressing upcoming standardization of SMPTE ST2094 group of specifications and may be categorized as related to the mandate 3 of the AhG14.

Contributions [8], [9], and [10] proposes to introduce to the HEVC new SEI messages to support metadata of SMPTE 2094-10, 20, 40 specifications. Individual SEI for each specification is proposed. Contribution [11] proposes regional nesting SEI to address the carriage means for region descriptors specified in SMPTE 2094-1.

## Mandate 4: Study and propose test conditions for associated experiments.

There was no discussion over the reflector on issues related to this mandate. There are 2 input contributions to this meeting which may be categorized as related to the mandate 4 of the

Contributions [12] and [13] propose two approaches for enabling the backward compatibility HDR video.

# Input contributions

There 12 input contributions related to AhG14 were submitted to the 24th JCTVC meeting.

- D. Rusanovskyy, D. Sansli, A. Ramasubramonian, J. Sole, M. Karczewicz, Usage of CRI for guided mapping (dynamic range adaptation) , JCTVC document X0060

- F. Hiron, E. Francois (Technicolor), AHG13: cross-check of JCTVC-X0060 on usage of CRI for DRA, JCTVC document X0076.

- A.K. Ramasubramonian, J. Sole, D. Rusanovskyy, D.B. Sansli, “SHM encoder improvements for HDR verification testing”, JCTVC-X0035, Geneva, CH, May-June, 2016

- A.K. Ramasubramonian, J. Sole, Y.-K. Wang, V. Baroncini, “Draft supplemental SHVC Verification test plan,” JCTVC-X0046, Geneva, CH, May–June 2016.

- W. Husak, F. Pu, T. Lu, P. Yin, T. Chen, AHG14: On Test Plan of SHVC Main 10 for HDR/WCG Backward Compatibility, JCTVC document X0055

- V. Baroncini (MPEG Test Chair), “Draft supplement to the SHVC verification test”, Geneva, CH, 26 May– 1 June 2016

- Y. Ye, Y. He, Cross check of JCTVC-X0046: Draft supplemental SHVC verification test plan, JCTVC document X0058,

- R. Yeung, S. Qu, P. Yin, T. Lu, T. Chen, W. Husak (Dolby), "Indication of SMPTE 2094-10 metadata in HEVC", JCTVC document X0042

- W. de Haan, R. Brondijk, L. van de Kerkhof, R. Goris, R. Nijland, Indication of SMPTE ST 2094-20 metadata in HEVC, JCTVC document X0075

- L. Tao, Y-T Kim, S. Park, B. Min, S. Rhyu, H. Najaf-Zadeh, M. Budagav (Samsung), Indication of SMPTE 2094-40 metadata in HEVC, JCTVC document X0061

- A. K. Ramasubramonian, J. Sole, Y.-K. Wang, D. Rusanovskyy, D. Bugdayci Sansli, M. Kacrzewicz (Qualcomm), P. Andrivon, E. Francois (Technicolor), W. de Haan, R. Brondijk  (Phillips), Regional nesting SEI message, JCTVC document X0062.

- P. Topiwala, W. Dai, M. Krishnan, Improvements to HDR10 with Backward Compatibility Options, JCTVC document X0044-r1

- C. Jung, Q. Lin, S. Yu, Backward Compatible Opto-Electrical Transfer Function for HDR Video Coding Based on Rational Quantization, JCTV document X0064

# Recommendations

The ad hoc group recommends to review the input contributions falling under the AhG14 mandates.