| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0033 |
| --- | --- |

| Title: | Results of the HDR/WCG Verification test using HEVC Main 10 Profile in Stockholm and Rome |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | V. Baroncini (MPEG Test Chair) K. Andersson, V. Kulyk Färögatan 6 164 80 Stockholm Sweden | Email: | baroncini@gmx.com kenneth.r.andersson@ericsson.com |
| Source: | MPEG Test Chair, Ericsson AB |  |  |

_____________________________

# Abstract

This document reports the testing procedure and results from the HDR verification testing with naïve viewers held in Stockholm during April 11 to April 15 and in Rome during May 11 to May 22. The results shows that HEVC Main 10 Profile can be used to compress 1080p HDR content in BT.2020 format with close to visually lossless quality compared to HDR source signal at bitrates below 6500 kbps for HDR sequences including two sport sequences. It was also shown significant improvements of using state-of-the-art technology when pre-processing and encoding HDR content using HEVC Main 10 Profile compared to pre-processing and encoding without considering that the content was HDR. BD-rate(MOS) numbers are presented based on the subjective scores in the test, showing an average bitrate reduction of 27% over all sequences of both test sites.

# Introduction

This document reports results for the HDR (High Dynamic Range) verification test for HEVC Main 10 Profile using both state-of-the-art technology for HDR pre-processing and coding, and without such considerations according to test plan in JCTVC-W1018 [1]. In this document the term “using state-of-the-art technology” is used to represent what is sometimes called “Anchor 3.2” and which represent technology described in more detail in JCTVC-W1017 [2]. The term “without using stat-of-the-art” is used to represent what is sometimes called “Anchor 1.0” and which represents conventional processing generally described in [5]. Ericsson has generated the processed test sequences and they have been cross-checked by Qualcomm and Apple.

A formal subjective test with 26 naïve viewers was conducted in Stockholm during April 11 to April 15 in Ericsson media test room and in Rome during May 11 to May 22 in Waitek laboratory testing area.

The test in Stockholm was done using DCR (Degradation Category Rating) Variant II (A B A* B* Vote) comparing the degradation of six 1080p HDR test sequences processed with and without state-of-the-art technology versus uncompressed source in 4:4:4. The test in Rome was done testing the same video clips by means of the DCR Variant I (A B Vote) test method.

The graphs illustrating the test results at Stockholm and Rome can be found at paragraphs 3.2 and 4.2 respectively.

# Processing chain

The 1080p HDR source sequences is originally in TIF or EXR. HDRTools is used to produce reference for viewing on SIM2 and pre-processed data for video coding and then to produce post-processed data for viewing on SIM2 after video decoding. One version  of HDRTools can be found here [4].

## Reference (Ref)

The source is first pre-processed to RGB values and then to a logLuv avi file (S01 to S06) intended for viewing on SIM2.

## Processed test sequences (P01 / P02)

The source is first pre-processed to RGB values, then converted using the inverse of the ST. 2084 EOTF to R’G’B’, then converted to CbCr, then quantized to 10 bits, then Cb and Cr are down-sampled to produce Y’CbCr in 4:2:0. The following processing is different for P01 and P02. P01 converts R’G’B’ to Y’ and then quantize to 10 bits while P02 use luma adjustment to derive Y’ in 10 bits.  P01 is encoded by using unmodified HM-16.7 while P02 is encoded by using the modified version of HEVC encoder (modified version of HM-16.7) which distribute bits from luma to chroma and from darker areas to brighter areas. For both P01 and P02, HEVC decoding is applied to produce reconstructed Y’CbCr 4:2:0. Then for both P01 and P02, Cb and Cr are upsampling to full resolution, then inverse quantization of Y’CbCr is performed, then conversion to R’G’B’ then transformation using ST. 2084 to RGB and then to logLuv avi file P01S0X / P02S0X intended for viewing on SIM2. Details on HEVC encoder and HDRTools settings can be found in JCTVC-W1018 [1].

Table 1: 1080p source sequences processed according to P01 (without state-of-the-art technology) and P02 (state-of-the-art technology) at four bitrates

| Label | Sequence | Frame rate (Hz) | Rate 1 (kbps) |  | Rate 2 (kbps) |  | Rate 3 (kbps) |  | Rate 4 (kbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | P01 | P02 | P01 | P02 | P01 | P02 | P01 | P02 |
| S01 | Market3 | 50 | 5371 | 5332 | 2676 | 2659 | 1684 | 1676 | 1290 | 1284 |
| S02 | Showgirl | 25 | 3358 | 3342 | 1686 | 1680 | 997 | 995 | 599 | 595 |
| S03 | EBU_06_Starting | 50 | 2679 | 2675 | 1590 | 1587 | 794 | 793 | 499 | 499 |
| S04 | EBU_04_Hurdles | 50 | 6454 | 6453 | 2994 | 2983 | 1895 | 1882 | 1093 | 1088 |
| S05 | Garage | 24 | 2693 | 2682 | 1398 | 1386 | 744 | 738 | 456 | 453 |
| S06 | BalloonFestival | 24 | 4175 | 4166 | 2577 | 2568 | 1587 | 1582 | 1192 | 1189 |

# Test in Stockholm

## Test setup

The test setup follows the test plan in JCTVC-W1018 [1].

The monitor in the test was a 47” SIM2 with a peak luminance of 6000 cd/m2 (model HDR47ES6MB) that was fed with AVI in LogLuv format via DVI cable.

The source sequence was presented first and then a processed sequence was shown, this was repeated two times, then the viewers had 5 seconds to vote.

The test subjects were asked to express their opinion according to DCR protocol, i.e. to answer the question “ if and in the case how much the processed sample is different from the reference”. The impairment scale is shown below. The viewers were also instructed that some sequences contain slow motion video and that the playout could be uneven but that it was not part of the assessment.

Before the test the viewers were trained on five test sequences not used in the test (different compressions of SunRise) to make them familiar with the type of impairments that will be present during the test. The viewers were also checked for both visual acuity and color blindness. Only viewers having visual acuity 20/30 or better and passed color blindness test performed the test.

Table 2: 11 grade DCR impairment scale

| Score | Impairment in the test sequence compared to the reference |  | Voting scale |
| --- | --- | --- | --- |
| 10 | Imperceptible | No differences |  |
| 9 | Slightly perceptible | Hardly visible in one area |  |
| 8 |  | Hardly visible in many areas |  |
| 7 | Perceptible | Easily visible in one area |  |
| 6 |  | Easily visible in many areas |  |
| 5 | Clearly perceptible | Evident differences in one area |  |
| 4 |  | Evident differences in many areas |  |
| 3 | Annoying | Severely affect video in one area |  |
| 2 |  | Severely affect video in many areas |  |
| 1 | Severely annoying | Heavily corrupted in one area |  |
| 0 |  | Heavily corrupted everywhere |  |

The test was divided into two test sessions each about 23 minutes long where the beginning of each session included three stabilization test sequences with same content as in the test. Voting was discarded for those. A 10 minute break was held between the two test sessions. Different randomizations were used for every pair of the viewers i.e. unique for the pair of users. The viewers were seated in front of the SIM2 at a distance of 2.5H.

The scores were collected in an electronic way (see Figure 1) by means of a small touch screen, where the 11 grades degradation scale was reproduced reporting (see table 2) both numerical and adjective labels. Subjects were instructed to press the proper button when the message “Vote N” was appearing on the main HDR screen.

The post screening of the viewers was performed using Pearson correlation index and 2 viewers were removed due to bad correlation with other subjects (less correlation than 0.75 as stated in JCTVC-W1018). Additional post screening of the viewers was performed according to procedure described in Recommendation ITU-T P.910 [3]. No viewers eligible for rejection were detected. Thus the results are based on 24 viewers.

Figure 1: Viewing room at Stockholm

## Results

Figure 2: MOS with 95% confidence interval for Market (left) and ShowGirl (right).

Figure 3: MOS with 95% confidence interval for EBU_06 Starting (left) and EBU_04 Hurdles (right).

Figure 4: MOS with 95% confidence interval for GarageExit (left) and Balloon (right).

Figure 5: MOS with 95% confidence interval of P01 (without state-of-the-art technology ) and P02 (state-of-the-art technology) at highest bitrate, and also for reference (uncompressed source).

| Label | Sequence | BD-rate(%) |
| --- | --- | --- |
| S01 | Market3 | -32,9% |
| S02 | Showgirl | -49,3% |
| S03 | EBU_06_Starting | -4,3% |
| S04 | EBU_04_Hurdles | -15,4% |
| S05 | Garage | -13,1% |
| S06 | BalloonFestival | -28,3% |
|  | Average | -23,9% |

Table 3: BD-rate for P01 (without state-of-the-art technology) versus P02 (state-of-the-art technology). A negative number correspond to less bitrate for P02 than P01 at equal MOS.

# Test in Rome

The test in Rome was conducted in the new Waitek testing area, dedicated to the video demos and visual testing activities.

## Test setup

The test was done using a prototype SIM2 HDR display for which the manufacturer declared a peak luminance close to 9.000 nits in the “HDR-mode”, over the 40% of the whole screen.

A screen luminance uniformity measurement was done by means of an X-Rite luminance meter (model i1Display Pro), getting a mean luminance value around 6.400 nits, with a variation around to 2% across the 9 quadrants of the screen (up-left, up-center, up-right, center-left, center-center, center-right, down-left, down-center, down-right). The measurement was done with the display in the “non HDR” mode.

All the test signals (included the caption messages: A B A* B* and VOTE N) were sent to the monitor through the LogLuv DVI input. The video signals were all in AVI LogLuv file format, while the caption messages were bitmap fixed images, whose presence on the screen was set by means of a script of the MUP (Multimedia Universal Player) HDR player.

The Basic Test Cell of the DCR Variant I test method [3] foresees the presentation on the screen of the original (uncompressed) sequence (always as first) and of the coded sequence (always as second); a 5 seconds voting time was left to the viewers to rate the 11 grades degradation score.

Figure 6: DCR Variant I Basic Test Cell

The test subjects were asked to express their impairment scores answering to the question “ if and in the case how much the coded video clip was different from the original”. The same 11 grades numerical impairment scale used during the Stockholm experiment was adopted, but without adjective labels.

The viewers were asked to rate their scores using a paper scoring sheet (see example in Figure 7).

Figure 7: Example of scoring sheet

At the very beginning of the test the viewers were trained by means of a tutorial phase during which a “Training Session” was run. The Training Session was made of 6 BTCs randomly extracted form the 48 BTC to test, including the widest range of impairments. The training session allowed the viewers to get used with the scoring procedure (they have to write a number in the box labelled with the same number of the message “Votte N” on the screen) and to get a chance to see the feature of the video clips. The Test Manager helped the viewers in exploring the artefacts they saw during the Training Session and in answering any question raised by them.

All the viewers were university students aging between 20 and 30 years, and screened for visual acuity and color blindness. A post screening of the viewers was made applying of the Pearson regression function among each viewer scores and the MOS values. Subjects with a regression index lower than 0,75 were possibly discarded; non of the 24 viewers were discarded.

The 48 coded video clips were evaluated creating 4 test sessions of 18 BTCs each. Each session included:

# stabilization phase: 3 BTCs extracted among the 12 to evaluate, taking care to select the highest, the lowest and a middle quality cases, were shown to the viewers at the beginning of each test session; (the scores collected during the “stabilization phase” were discarded)

# test phase: 12 BTCs with coded clips to be evaluated;

# check phase: 3 Original vs. Original cases; useful also to verify the level of attention of the viewers; the check phase was randomly mixed up with the test phase.

The 24 viewers participating to the test were divided in groups twelve groups of two persons each; when a group was running a test the other groups were getting rest; in this way he fatigue of the viewers were reduced to the minimum. For each group of viewers the presentation order of a session was different, to minimize the possibility contextual effects in the judgments.

## Results

Figure 8: MOS with 95% confidence interval for Market (left) and ShowGirl (right).

Figure 9: MOS with 95% confidence interval for EBU_06 Starting (left) and EBU_04 Hurdles (right).

Figure 10: MOS with 95% confidence interval for GarageExit (left) and Balloon (right).

Figure 11: MOS with 95% confidence interval of P01 (without state-of-the-art technology ) and P02 (state-of-the-art technology) at highest bitrate, and also for reference (uncompressed source).

| Label | Sequence | BD-rate(%) |
| --- | --- | --- |
| S01 | Market3 | -46,5% |
| S02 | Showgirl | -40,7% |
| S03 | EBU_06_Starting | -25,2% |
| S04 | EBU_04_Hurdles | -19,9% |
| S05 | Garage | -4,4% |
| S06 | BalloonFestival | -46,2% |
|  | Average | -30,5% |

Table 4: BD-rate for P01 (without state-of-the-art technology) versus P02 (state-of-the-art technology). A negative number correspond to less bitrate for P02 than P01 at equal MOS

# Conclusions

The result of the test with naïve viewers shows that HEVC Main 10 Profile resulted in a small or no significant difference in quality compared to the uncompressed source at a bitrate of less than 6500 kbps for the six 1080p HDR sequences including two sport sequences (encoded at 100 Hz but played out at 50 Hz). The results also show that state-of-the-art technology gives significant improvements on several rate points for many sequences used in the test compared to without state-of-the-art technology. On average the bitrate gain for state-of-the-art technology at a fixed MOS is more than 20%.

# References

[1] R. Sjöberg, V. Baroncini, A. K. Ramasubramonian , “Verification test plan for HDR/WCG coding using HEVC Main 10 Profile”, JCTVC-W1018, San Diego, USA, Feb. 2016

[2] J. Samuelsson, “Conversion and Coding Practices for HDR/WCG Video, Draft 1”, JCTVC-W1017, San Diego, USA, Feb. 2016

[3] ITU-T Recommendation P.910 “Subjective video quality assessment methods for multimedia applications” 04/2008

[4] HDRTools software package for processing of HDR video content available at https://gitlab.com/standards/HDRTools/

[5] Ajay Luthra, Edouard François, Walt Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding”, n15083, 110th MPEG meeting, Geneva