| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0038 |
| --- | --- |

| Title: | AHG 3 Recommended settings for HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, P. Wennersten, J.Samuelsson, J. Ström, P. Hermansson, M. Pettersson Färögatan 6 16480 Stockholm | Tel: Email: | +46107141538 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

It is proposed to update the common test conditions for HM to fix the misalignment between QP and lambda and also to increase the GOP size from 8 to 16 for cases when random access configuration functionality is not affected. To solve the misalignment between QP and lambda some additional configuration parameters are introduced for HM namely, a QP offset for intra pictures, and parameters for a linear model to adjust QP pending on the magnitude of the QP for respective hierarchical level. This is an alternative implementation of the proposed fix in JCTVC-W0062 which produces similar QPs for respective picture. The contribution reports luma BD rate improvements for common conditions of 1.9%/2%/2% for RA/LD/LP for the QP fix and 7.8% for QP fix with GOP 16. To verify that the GOP extension not reduce the performance for difficult to encode sequences, 8 additional sequences were tested. An average improvement in luma BD rate of 4.5% is reported for this additional test set. Results for HDR content is also presented.

Proposed changes to the common test conditions in JCTVC-L1100 and reference software (HM-16.9) are also provided in the contribution.

# Introduction

In JCTVC-W0062 [1] it was proposed to fix the mismatch between lambda and QP for HM. The fix was based on one line of code adjusting the QP so it better matches the used lambda. The approach was adopted at the San Diego meeting and was thus integrated into HM-16.9 but turned off by default. The reason it was turned off mainly due to some experts during software integration time were uncomfortable with changing an initially set QP. We therefore propose an alternative implementation that produces near identical QPs but allows the user to specify the behaviour of QP and let that determine the lambda. To this end it is proposed an intra picture QP offset and a linear QP offset model that together with the QPoffset parameter already available in HM adjusts the given QP. The linear QP offset model defines an adjustment of the given QP for respective GOP layer based on the sum of the given QP and QPoffset. The model gives a larger increase of QP for higher QPs than for lower QPs.

A second aspect which also was proposed in JCTVC-W0062 is to extend the GOP size to 16 pictures instead of 8 pictures. The reason for doing so is that coding efficiency can be improved. The reason this was not adopted at the San Diego meeting was expressed fear of quality fluctuations for HDR and that the random access functionality would be affected for 24Hz content. In this contribution it is proposed to use GOP size 16 when it is possible to avoid affecting the random access functionality. HDR coding’s will be available for review at the meeting.

# Alternative QP fix to align with lambda

To use a QP that corresponds to the lambda change for intra slices derived from the GOP size as in HM it is proposed to add an intraQPOffset parameter that defines the QP for intra slices to be equal to the sum of the given QP and intraQPOffset.

Config example and code:

IntraQPOffset                 : -2  // GOP size 8

if(eSliceType==I_SLICE)

{

dQP += m_pcCfg->getIntraQPOffset();

}

To use a QP that that corresponds to the scaling of lambda with QPfactor pending on the hierarchical level in the GOP as in HM, it is proposed to change the QPoffset and set the QPfactor to 1.

To use a QP that corresponds to the scaling of lambda according to the QP for a picture as in HM it is proposed to adjust a given QP by a linear QP offset model for each hierarchial level.

The lambda is then specified from the derived QP by:

lambda = QPfactor*0.57*2 (QP-12)/3 		(Eq. 1)

For random access the config change looks like:

IntraQPOffset                 : -2 // GOP size 8

#        Type POC QPoffset CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 temporal_id #ref_pics_active #ref_pics reference pictures     predict deltaRPS #ref_idcs reference idcs  QPOffsetModelOffset GradQPOffsetModel

Frame1:  B    8   0        0          0          1.0   0            0              0           2                3         -8 -12 -16             0 0.0 0.0

Frame2:  B    4   3        0          0          1.0   0            0              1           2                3         -4  -8   4             1       4        4          1 1 0 1   -6.25 0.25

Frame3:  B    2   4        0          0          1.0   0            0              2           2                4         -2  -6   2 6           1       2        4         1 1 1 1   -6.25 0.25

Frame4:  B    1   8        0          0          1.0   0            0              3           2                4         -1   1   3 7           1       1        5         1 0 1 1 1 -7.0 0.245

Frame5:  B    3   8        0          0          1.0   0            0              3           2                4         -1  -3   1 5           1      -2        5         1 1 1 1 0 -7.0 0.245

Frame6:  B    6   4        0          0          1.0   0            0              2           2                3         -2  -6   2             1      -3        5          0 1 1 1 0  -6.25 0.25

Frame7:  B    5   8        0          0          1.0   0            0              3           2                4         -1  -5   1 3           1       1        4         1 1 1 1     -7.0 0.245

Frame8:  B    7   8        0          0          1.0   0            0              3           2                4         -1  -3  -7 1           1      -2        5         1 1 1 1 0  -7.0 0.245

The change looks like this in the software:

Double dQPoffset2 = Clip3(0.0,3.0,floor((dQP+m_pcCfg->getGOPEntry(iGOPid).m_QPOffset)*m_pcCfg->getGOPEntry(iGOPid).m_GradQPOffsetModel+m_pcCfg->getGOPEntry(iGOPid).m_QPOffsetModelOffset+0.5));

if(m_pcCfg->getGOPEntry(iGOPid).m_GradQPOffsetModel==0.0 && m_pcCfg>getGOPEntry(iGOPid).m_QPOffsetModelOffset==0.0)

{

dQP += m_pcCfg->getGOPEntry(iGOPid).m_QPOffset;

}

Else

{

dQP += m_pcCfg->getGOPEntry(iGOPid).m_QPOffset+dQPoffset2;

}

# Extension of hierarchy to GOP size 16

For random access we also propose to extend the GOP size to 16 using the following configuration file.

GOPSize                       : 16           # GOP Size (number of B slice = GOPSize-1)

IntraQPOffset                 : -3

#        Type POC QPoffset CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 temporal_id #ref_pics_active #ref_pics reference pictures     predict deltaRPS #ref_idcs reference idcs QPmodelOffset QPmodelGradient

Frame1:  B   16   1        0          0          1.0      0            0              0           2                3         -16 -24 -32            0  0.0 0.0

Frame2:  B    8   1        0          0          1.0      0            0              1           2                3         -8  -16   8            1       8        4         1 1 0 1     -4.8848 0.2061

Frame3:  B    4   4        0          0          1.0      0            0              2           2                4         -4  -12   4  12        1       4        4         1 1 1 1     -5.7476 0.2286

Frame4:  B    2   5        0          0          1.0      0            0              3           2                5         -2  -10   2   6  14    1       2        5         1 1 1 1 1   -5.90 0.2333

Frame5:  B    1   6        0          0          1.0      0            0              4           2                5         -1    1  3    7  15    1       1        6         1 0 1 1 1 1 -7.1444 0.3

Frame6:  B    3   6        0          0          1.0      0            0              4           2                5         -1   -3   1   5  13    1      -2        6         1 1 1 1 1 0 -7.1444 0.3

Frame7:  B    6   5        0          0          1.0      0            0              3           2                4         -2   -6   2  10        1      -3        6         0 1 1 1 1 0 -5.90 0.2333

Frame8:  B    5   6        0          0          1.0      0            0              4           2                5         -1   -5   1   3  11    1       1        5         1 1 1 1 1   -7.1444 0.3

Frame9:  B    7   6        0          0          1.0      0            0              4           2                5         -1   -3  -7   1   9    1      -2        6         1 1 1 1 1 0 -7.1444 0.3

Frame10: B   12   4        0          0          1.0      0            0              2           2                3         -4  -12   4            1      -5        6         0 0 1 1 1 0 -5.7476 0.2286

Frame11: B   10   5        0          0          1.0      0            0              3           2                4         -2  -10   2   6        1       2        4         1 1 1 1     -5.90 0.2333

Frame12: B    9   6        0          0          1.0      0            0              4           2                5         -1   -9   1   3   7    1       1        5         1 1 1 1 1   -7.1444 0.3

Frame13: B   11   6        0          0          1.0      0            0              4           2                5         -1   -3 -11   1   5    1      -2        6         1 1 1 1 1 0 -7.1444 0.3

Frame14: B   14   5        0          0          1.0      0            0              3           2                4         -2   -6 -14   2        1      -3        6         0 1 1 1 1 0 -5.90 0.2333

Frame15: B   13   6        0          0          1.0      0            0              4           2                5         -1   -5 -13   1   3    1       1        5         1 1 1 1 1   -7.1444 0.3

Frame16: B   15   6        0          0          1.0      0            0              4           2                5         -1   -3 -7  -15   1    1      -2        6         1 1 1 1 1 0 -7.1444 0.3

Since HM requires that the intra period is a multiple of the GOP length we propose to only use GOP 16 for content that allows that. However it should be noted that better gains can be achieved if instead the intra period is adjusted. For example if your content is 24 Hz you could use an intra period of 32 instead of 24 which then allows you to use GOP 16 instead of GOP 8.

# Results

## Objective performance for common conditions

Table 1: Alternative QP fix to align with lambda:

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) | -2,3% | -6,5% | -7,4% | -2,2% | -7,1% | -7,9% |
| Class B | -1,6% | -7,9% | -9,2% | -1,7% | -8,5% | -9,6% |
| Class C | -2,1% | -8,6% | -9,0% | -2,1% | -9,0% | -9,4% |
| Class D | -1,8% | -7,9% | -8,0% | -1,7% | -8,6% | -8,7% |
| Class E |  |  |  |  |  |  |
| Overall | -1,9% | -7,5% | -8,0% | -1,9% | -7,9% | -8,4% |
|  | -1,9% | -7,1% | -7,6% | -1,9% | -7,5% | -7,9% |
| Class F | -1,8% | -6,0% | -5,9% | -1,8% | -5,9% | -5,8% |
| Enc Time[%] | 96% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) |  |  |  |  |  |  |
| Class B | -1,7% | -5,5% | -5,7% | -1,7% | -5,2% | -5,6% |
| Class C | -2,0% | -6,0% | -6,6% | -2,0% | -6,1% | -6,4% |
| Class D | -1,6% | -5,4% | -5,9% | -1,5% | -5,5% | -5,7% |
| Class E | -2,8% | -5,3% | -3,8% | -2,7% | -6,3% | -5,5% |
| Overall | -2,0% | -5,6% | -5,5% | -1,9% | -5,5% | -5,9% |
|  | -2,0% | -5,3% | -5,2% | -1,9% | -5,1% | -5,5% |
| Class F | -2,1% | -5,6% | -4,8% | -2,0% | -4,5% | -6,2% |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 109% |  |  | 106% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) |  |  |  |  |  |  |
| Class B | -1,8% | -5,4% | -5,8% | -1,8% | -5,5% | -5,8% |
| Class C | -2,0% | -6,1% | -6,5% | -1,9% | -5,9% | -6,5% |
| Class D | -1,6% | -5,5% | -6,1% | -1,6% | -5,5% | -5,4% |
| Class E | -3,2% | -6,2% | -4,3% | -3,2% | -6,8% | -5,7% |
| Overall | -2,0% | -5,6% | -5,5% | -2,0% | -5,7% | -5,8% |
|  | -2,0% | -5,3% | -5,1% | -2,1% | -5,3% | -5,4% |
| Class F | -1,9% | -4,9% | -4,5% | -2,0% | -5,0% | -5,6% |
| Enc Time[%] | 99% |  |  | 107% |  |  |
| Dec Time[%] | 105% |  |  | 119% |  |  |

Table 2: Alternative QP fix to align with lambda with GOP 16:

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) | -7,8% | -9,1% | -11,4% | -7,8% | -10,1% | -12,3% |
| Class B | -7,0% | -12,9% | -16,5% | -7,2% | -13,9% | -17,5% |
| Class C | -8,4% | -12,6% | -12,8% | -8,3% | -13,3% | -13,5% |
| Class D | -7,5% | -14,2% | -14,3% | -7,5% | -14,9% | -15,0% |
| Class E |  |  |  |  |  |  |
| Overall | -7,8% | -12,1% | -13,2% | -7,8% | -12,9% | -14,0% |
|  | -7,8% | -11,8% | -13,0% | -7,8% | -12,6% | -13,8% |
| Class F | -8,5% | -9,9% | -9,5% | -8,5% | -10,5% | -10,0% |
| Enc Time[%] | 87% |  |  | 93% |  |  |
| Dec Time[%] | 80% |  |  | 90% |  |  |

## Objective performance for additional sequences

Table 3: Alternative QP fix to align with lambda and GOP 16

|  | Random Access Main HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | -4,1% | -12,6% | -11,5% |
| Class A2 | -5,0% | -9,5% | -9,0% |
| Overall | -4,5% | -11,0% | -10,2% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 4: Alternative QP fix to align with lambda and GOP 16 versus anchor 3.2 [3]. Yellow marked sequence also using GOP equal to 16. The other sequences have intra period of 24 so for those only alternative QP fix were used.

|  |  | X | Y | Z | DE100 |
| --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | -4,4% | -3,5% | -5,2% | -5,3% |
|  | Market3Clip4000r2 | -12,6% | -12,0% | -15,5% | -33,3% |
|  | SunRise | -3,6% | -2,9% | -5,3% | -22,4% |
| class B | BikeSparklers cut 1 | -4,5% | -4,3% | -5,2% | -12,2% |
|  | BikeSparklers cut 2 | -3,5% | -3,2% | -4,1% | -11,5% |
|  | GarageExit | -3,3% | -2,8% | -5,0% | -9,3% |
| class C | ShowGirl2Teaser | -1,6% | -1,1% | -2,4% | -9,0% |
| class D | StEM_MagicHour cut 1 | -3,5% | -2,3% | -5,2% | -10,8% |
|  | StEM_MagicHour cut 2 | -2,0% | -1,5% | -4,2% | -7,5% |
|  | StEM_MagicHour cut 3 | -1,6% | -0,9% | -3,6% | -6,6% |
|  | StEM_WarmNight cut 1 | -4,5% | -3,8% | -5,1% | -10,3% |
|  | StEM_WarmNight cut 2 | -4,1% | -3,0% | -4,8% | -9,2% |
| class G | BalloonFestival | -1,6% | -1,0% | -3,3% | -11,1% |
| class H | EBU_04_Hurdles | -10,2% | -10,3% | -12,9% | -24,8% |
|  | EBU_06_Start | -12,8% | -10,4% | -15,8% | -29,0% |
|  | Overall | -4,9% | -4,2% | -6,5% | -14,1% |

## Subjective performance

A brief visual inspection on Market (HDR) and DayLightRoad (4K SDR) has indicated improved visual quality from using the QP fix with GOP16.

# Conclusions

It has been shown that objective and subjective performance of HEVC coding using HM can be improved by fixing the QP to match the used lambda and by increasing the B-frame hierarchy to 16 for random access. The contribution proposes that both aspects are included in the reference software for HEVC and common test conditions.

# Reference

[1] K. Andersson, P. Wennersten, R. Sjöberg, J. Samuelsson, J. Ström, P. Hermansson, M. Pettersson , “Non-normative HM encoder improvements”, JCTVC-W0062, San Diego, USA, 19–26 February 2016

[2] F. Bossen, “Common Test Conditions and Software Reference Configurations for HM”, JCTVC-L1100, Geneva, Switzerland, January 2013

[3] E. Francois, J. Sole, P. Yin, J. Ström, “Updated Common Test Conditions for HDR/WCG Video Coding Experiments”, JCTVC-W1020, San Diego, USA, Feb. 2016

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).