| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0040 |
| --- | --- |

| Title: | Content colour gamut SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun Mook Oh Jang Won Choi Jong-Yeul Suh. 19, Yangjae-daero 11gil, Seocho-gu, Seoul, 137-893, Korea | Email: | hyunmook.oh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In this proposal, an SEI message is proposed to signal content colour gamut which describes actual colour distribution of a video content. This information is mostly useful when the content colour is partially distributed in the possible colour area within the container colour gamut. Given the content colour gamut information, the performance of colour gamut mapping could be improved in terms of colour saturation.

# Introduction

## Background

In variety of industry fields, colour characteristics of cameras, displays, or video contents are represented by colour gamut, which is the area of colour distribution defined by red, green, and blue colour primaries. To have a consistent colour understanding and colour representation in different devices, standard colour gamut, such as ITU-R BT.709 and ITU-R BT.2020, have been developed for HDTV and UHDTV broadcasting services. In addition, industry defined colour gamut, such as DCI-P3, sRGB, or AdobeRGB, are also widely used in various applications such as film industry and computer graphics, etc.

As ITU-R BT.2020 colorimetry is adopted as a baseline standard for ultra-high definition (UHD) and high dynamic range (HDR) video services, it is anticipated that the more colourfulness would be delivered to viewers. However, due to the limitations on the current capturing and displaying technologies, the whole range of BT.2020 colour area is not fully used in imaging and displaying devices.

Take into account the gap between ideal and reality, display devices adopt gamut mapping process which aims to fit the content colour gamut into the display colour gamut. As the process manipulates the colour distribution of a video content into a smaller colour gamut, algorithms often suffer from de-saturated colour video output. It is clear that the colour distortion becomes more severe as the gap between the source and the target colour gamut is larger.

As an approach to alleviate this issue, we focus on the additional gamut information which provides actual distribution of colours in a video content. Since the actual colour distribution is bounded in the given container colour gamut, the additional information reduces the gap between the source and target colour gamut, which results in improved colour mapping output.

## Definition of content colour gamut

In this proposal, additional gamut information called ‘content colour gamut’, which is the area representing actual distribution of colours present in a video content is proposed to use. The term ‘content’ helps us to distinguish it with ‘container colour gamut’, which represents the area of possible colours presented by code value. Colour gamut comprised by colour primaries defined in BT.2020 is a representative container colour gamut in UHDTV video format.

Conceptually, the mastering display colour gamut provides a boundary of content colour distribution since the area of colour distribution of a video content cannot exceed the mastering display colour gamut. However, they are not identical since the whole area of colours defined by mastering display is not always used to describe a video content. Therefore, the area of content colour gamut is defined within the area of the others as follows,

content colour gamut ⊂ mastering display colour gamut,

content colour gamut ⊂ container colour gamut.

As an example for different colour gamut concept, consider the live broadcasting environment: captured by BT.709 colour gamut camera, mastered on DCI-P3 colour gamut display and then distributed over BT.2020 container. For visual description of the concept, those colour gamut are depicted in Figure 1, where blue dots represents actual distribution of colour samples which are sampled from a frame of Market3 MPEG test video sequence. As the content and container colour gamut of the sequence is known for BT.709 and BT.2020, respectively, the only assumption for this example is that the content is mastered in DCI-P3 mastering monitor.

Figure 1. The concept of the different colour gamut (left, ‘Market3’ BT709 test sequence, right, actual distribution of colour samples and boundary of different colour gamut)

For HEVC signaling, the syntax value colour_primaries in VUI could be set to 9 to inform that the video signal is formatted with BT.2020 container. Also, if necessary, DCI-P3 colour gamut of mastering environment could be signaled by using the display_primaries_x and display_primaries_y for red, green and blue colour primaries in mastering display colour volume SEI message. The only missing part is content colour gamut where it is mostly precise and direct information in describing colour distribution of a video content.

## Example usage of content colour gamut

In the application of colour gamut mapping from large to small colour gamut, it is clear that the closer the two colour gamut boundaries are located the less colour distortion is resulted. In the following, the benefit of using content colour gamut is described with gamut mapping examples with and without content colour gamut information.

Consider a case where a video distributed in container colour gamut BT.2020 and displayed in a device with BT.709 panel. If the colours are actually distributed within BT.709 area and the content colour gamut information is known, one of the best images that could be presented in the display device is to clip out the empty out-of gamut colours and show the original colours as it is intended. However, when there is no information about the content colour gamut, some display device performs colour gamut mapping only rely on the container colour gamut information. As shown in Figure 2, the colour gamut mapping without using content colour gamut information results in de-saturated colour compare to the colour gamut clipping output using content colour gamut information.

Figure 2. Comparison of display process using and without using content colour gamut. (left, colour gamut clipping result when content and display colour gamut are matched, right, gamut mapping result using container colour gamut only)

In advanced colour gamut mapping algorithm, such as ISO/IEC JTC1/SC29/WG11 MPEG2015/m37368, also adopt the content colour gamut. As the algorithm aims to reconstruct wide colour gamut video content from colour gamut mapped source, target colour gamut is signaled to describe content colour gamut after the gamut mapping processing while the container colour gamut is signaled separately.

# Proposal

We propose an SEI message that can convey a representation of the content colour gamut, which represents actual area of colour distribution of a video content. This information is distinguished with other colour primaries in VUI which describes the output video format or in mastering display colour volume SEI message which described the mastering environment. It is useful in colour gamut mapping process that manages the colour gamut of a video content to fit into the colour gamut of the target application (e.g. display).

In the proposed SEI message, content colour gamut is signaled by standard colour gamut type or by using manually selected colour primaries. For precise description of the content colour gamut, it is allowed to use more than three colour primaries. The syntax and semantics proposed for this SEI message are provided in the following subsections.

## Content colour gamut SEI message syntax

| content_colour_gamut_info( payloadSize ) { | Descriptor |
| --- | --- |
| content_colour_gamut_type | u(8) |
| if(content_colour_gamut_type == 255) { |  |
| number_of_primaries_minus3 | u(4) |
| for( c = 0; c < number_of_primaries_minus3; c++ ) { |  |
| content_colour_primaries_x[ c ] | u(16) |
| content_colour_primaries_y[ c ] | u(16) |
| } |  |
| } |  |
| } |  |

## Content colour gamut SEI message semantics

This SEI message identifies the colour gamut of a video content, which represents the area of actual colours presented in a video content. The content colour gamut is defined by standard colour gamut or manually defined by colour primaries. It is highly recommended that the maximum boundary of content colour gamut is defined within container colour gamut signaled by colour_primaries in VUI as well as mastering display primaries described in Mastering display colour volume SEI message.

content_colour_gamut_type specify the type of the colour gamut representation. It follows the definition of colour_primaries in E.3.1 VUI parameters semantics and Table E.3 Colour primaries.

When the content_colour_gamut_type syntax element is not present, the value of colour_primaries is inferred to be equal to the value of colour_primaries syntax element presented in VUI. When the value is 255, the content colour gamut is manually described by using content_colour_primaries_x, and content_colour_primaries_y.

number_of_primaries_minus3 specify total number of colour primaries those are used to describe the content colour gamut boundary. The total number of colour primaries is defined by number_of_primaries_minus3 + 3 where the range of number_of_primaries_minus3 value is 0 to 15.

content_colour_primaries_x[ c ] and content_colour_primaries_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the video content in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing video content that use 3 colour primaries with green, blue, and red colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary and c equal to 2 should correspond to the red colour primary (see also Annex E and Table E.3). The values of content_colour_primaries_x[ c ] and content_colour_primaries_y[ c ] shall be in the range of 0 to 50 000, inclusive.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).