| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0042 |
| --- | --- |

| Title: | Indication of SMPTE 2094-10 metadata in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Raymond Yeung, Sheng Qu, Peng Yin, Taoran Lu, Tao Chen, Walt Husak 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

The ST 2094 suite of documents define metadata for use in color volume transforms of content. The metadata are content-dependent and can vary scene-by-scene or image-by-image. The metadata are intended to transform High Dynamic Range and Wide Color Gamut (HDR/WCG) image essence for presentation on a display having a smaller color volume than that of the mastering display used for mastering the image essence. Multiple applications provide particular color volume transforms. Among them, SMPTE 2094-10 standard: Dynamic Metadata for Color Volume Transform - Application #1 has completed the document development process and is in the publication queue pending SMPTE audit and therefore it is very stable. This contribution proposes text changes in the HEVC specification to support SMPTE-2094-10 dynamic metadata. Specifically, in Annex D, a new SEI message is proposed for the carriage of such metadata.

# Overview of ST 2094

The recent advancement in technologies and displays enable the content producers to produce content in high dynamic range (HDR) and wide color gamut (WCG) with a greater artistic palette than ever before. But at the same time, the range of capabilities of the endpoint displays varies significant. How to ensure HDR/WCG content to be presented consistently over a wide range of professional and consumer displays is a very challenging topic. The SMPTE ST 2094 provides a solution through the use of content-dependent, dynamic color volume transform metadata rather than using only static display color volume metadata.

The ST 2094 suite of documents defines metadata for use in color volume transforms of content. The metadata are content-dependent and can vary scene-by-scene or frame-by-frame. The metadata are intended to transform HDR/WCG source content for presentation on a display having a smaller color volume than the source content’s mastering display [1]. Multiple applications provide particular color volume transforms.

The layout of ST 2094 document is as follows:

- ST 2094-1: Core Components. It specifies a “core” set of common metadata items and provide a framework for specialized applications specified in the documents SMPTE ST 2094-10 (Application #1), 2094-20 (Application #2), 2094-30 (Application #3) and 2094-40 (Application #4). ST 2094-1 has completed the document development process and is very stable.

- ST 2094-2: Syntax and Carrier. It specifies KLV representations of color volume transform metadata sets defined in SMPTE ST 2094-10, 2094-20, 2094-30 and 2094-40, and aggregates of such metadata sets. Furthermore, it specifies frame- and clip-based mappings of KLV-encoded metadata sets into the MXF constrained generic container, as defined in SMPTE ST 379-2. Therefore the scope of this document does not cover that of SEI messages.

- ST 2094 applications: #1, #2, #3, and #4. Each application specifies a specialized model of the generic color volume transform described in Annex A of the core components document ST 2094-1.

A metadata set in ST 2094 incorporates a time interval, a window, metadata describing the targeted system display and parameters controlling the color volume transform [1]. A receiver of the metadata can use the targeted system display metadata to select the metadata sets that are most applicable to the actual output device. A receiver of the metadata can use the targeted system display metadata to adjust the output of the color volume transform from the specified output device to the actual output device. Two different approaches have been identified as transform methods in ST 2094 applications: parametric mapping and reference based numerical derivation. Application #1, #2 and #4 belong to the parametric model, and #3 belongs to the numerical data-fitting model.

This contribution is focused on ST 2094 Part 10. It specifies the content-dependent color volume transform metadata items for Application #1 [2]. This color volume transform is based on an adaptive tone mapping curve parameterized by the source content characteristics as well as manually set adjustments. This contribution proposes a new SEI message in Annex D to carry such metadata.

# HDR/WCG Display Mapping related SEI messages in HEVC spec

There are multiple SEI messages in the HEVC specification [3] which relate to HDR/WCG display mapping. Some key ones are:

- Tone mapping information SEI message

- Mastering display colour volume SEI message

- Colour remapping information SEI message

- Content light level information SEI message

- Ambient viewing environment SEI message

Tone mapping information SEI message only provides the information for luminance mapping, not color volume mapping. The gamut (chromaticity) mapping is not considered at all. Such luminance mapping requires the mapping to be applied to either luma component or each RGB component.

Mastering display colour volume SEI message describes static metadata in ST. 2086. It specifies the colour volume (the colour primaries, white point, and luminance range) of the mastering display that was used for viewing while authoring the video content. It does not provide information on colour transforms that would be appropriate to preserve creative intent on displays with colour volumes different from that of the described mastering display.

Colour remapping information SEI message carries colour volume transform metadata for ST. 2094-30. It performs color volume mapping using numerically-derived dynamic metadata for the Pre Tone Mapper (1D LUT), Creative Matrix (3x3 scalar), and Post Tone Mapper (1D LUT). Therefore it can specify the remapping of the content to specific targeted color volumes in corresponding color coding workspaces. However, it does not carry metadata describing the targeted system display and it does not carry metadata describing the color volume transform color coding workspace. Therefore the display needs some other means to decide which CRI SEI message to be used for that particular display as well as a particular color coding workspace for the content to be transformed.

Content light level information SEI message specifies the Maximum Content Light Level, which is the largest individual pixel light value of any picture in the CLVS, and Maximum Frame Average Light Level, which is the largest average pixel light value of any picture in the CLVS. The information may be used to improve the mapping for target display. But it does not directly provide information on colour volume transforms that would be appropriate to preserve creative intent on target displays.

Ambient viewing environment SEI message identifies the characteristics of the nominal ambient viewing environment for the display of the associated video content. It may assist the receiving system in improving mapping for target display which has similar or different viewing environments from those when mastering the video content. But it does not directly provide information on colour volume transforms that would be appropriate to preserve creative intent on target displays.

The major difference of ST 2094 metadata from the above SEI messages (except CRI) is that it specifies content-dependent color volume transform parameters. The metadata also ties the color volume transform parameters with the target display, where none of the above SEI messages does.  Furthermore, when metadata for an actual output display is not available, a receiver of the metadata can use the received targeted system display metadata to select the metadata sets that are most applicable to the actual output device or adjust the output of the color volume transform from one or more intended output devices in the metadata to the actual output device.

# ST 2094-10 Scope and Metadata

ST 2094-10 is Application #1 color volume transform in the framework of the generalized Color Volume Transform Model described in Annex A of ST 2094-1. Unlike CRI (ST 2094-30) which uses a numerical data-fitting model that finds an optimal mapping between two master files, ST 2094-10 uses a parameterized color transform similar to what a colorist could use in a color grading session. This color volume transform is based on a parametrically-defined tone mapping curve, the shape of which is defined both by the image essence characteristics (algorithmically computed from the input image essence) as well as by manually-set adjustments [2]. It mainly contains three processing blocks: Tone Mapping, Gamut Adjustment and Detail Management, as described in Annex A of ST 2094-10. A mathematical description of an example color volume transform in RGB output-referred linear light output is provided in Annex B of ST 2094-10. ST 2094-10 metadata set shall contain all the common metadata in ST 2094-1 core, which relates to ApplicationIdentifier, ApplicationVersion, TimeInterval, ProcessingWindow, and TargetedSystemDisplay. For ColorVolumeTransform, the metadata shall contain ImageCharacteristicsLayer, and optional ManualAdjustmentLayer. ImageCharacteristicsLayer shall contain exactly MinimumPqencodedMaxrgb, AveragePqencodedMaxrgb and MaximumPqencodedMaxrgb. ManualAdjustmentLayer may optionally contain, MinimumPqencodedMaxrgbOffset, AveragePqencodedMaxrgbOffset, MaximumPqencodedMaxrgbOffset, ToneMappingOffset, ToneMappingGain, ToneMappingGamma, ChromaCompensationWeight, SaturationGain and ToneDetailFactor.

The tone mapping curve adopts an analytically defined sigmoidal curve specified by three control parameters: minimum luminance, mid-point / adaptation point and maximum luminance. The initial values of the three control parameters for the source are algorithmically computed from the input pixel values, and are carried as metadata items MinimumPqencodedMaxrgb, AveragePqencodedMaxrgb and MaximumPqencodedMaxrgb. The three values can be adjusted manually by offsetting the minimum, average and maximum control points by using the parameters specified in MinimumPqencodedMaxrgbOffset, AveragePqencodedMaxrgbOffset and MaximumPqencodedMaxrgbOffset, respectively.  The tone mapping curve maps the input image essence minimum, average and maximum luminance values to the TargetedSystemDisplayMinimumLuminance, derived adaptation point and TargetedSystemDisplayMaximumLuminance. For the target system, to provide further manual control for the mapping curve, ST 2094-10 also respects the common color correction technique of offsetting the minimum targeted system display output level, applying a gain factor for the entire luminance range and applying a gamma function affecting the mid-tones. These parameters are specified by ToneMappingOffset, ToneMappingGain and ToneMappingGamma. The slope of the tone mapping curve affects image saturation. A SaturationGain control parameter provides a saturation boost or diminution, and a ChromaCompensationWeight control parameter provides an overall chrominance compensation weighting factor for the tone mapping. These parameters are intended for manual adjustments. In addition, ST 2094-10 provides a single manually set parameter defined by ToneDetailFactor to control the contribution from image detail management.

# Proposed Text for ST 2094-10

## SEI payload Syntax for ST 2094-10

D.2.4x Colour volume transform information SEI message syntax

| colour_volume_transformation_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_volume_transform_id | ue(v) |
| colour_volume_transform_cancel_flag | u(1) |
| if( !colour_volume_transform_cancel_flag ) { |  |
| colour_volume_transform_persistence_flag | u(1) |
| application_identifier | ue(v) |
| application_version | ue(v) |
| num_proc_wins_minus1 | ue(v) |
| for ( i = 0; i < num_proc_wins_minus1 + 1; i++) { |  |
| proc_win_num_id[ i ] | ue(v) |
| proc_win_left_offset[ i ] | ue(v) |
| proc_win_right_offset[ i ] | ue(v) |
| proc_win_top_offset[ i ] | ue(v) |
| proc_win_bottom_offset[ i ] | ue(v) |
| min_PQ_maxRGB[ i ] | u(32) |
| avg_PQ_maxRGB[ i ] | u(32) |
| max_PQ_maxRGB[ i ] | u(32) |
| } |  |
| num_target_display_minus1 |  |
| for ( i = 0; i < num_target_display_minus1 + 1; i++) { |  |
| target_display_id[ i ] | ue(v) |
| for( c = 0; c < 3; c++ ) { |  |
| target_display_primaries_x[ i ][ c ] | u(16) |
| target_display_primaries_y[ i ][ c ] | u(16) |
| } |  |
| target_display_white_point_x[ i ] | u(16) |
| target_display_white_point_y[ i ] | u(16) |
| max_target_display_luminance[ i ] | u(32) |
| min_target_display_luminance[ i ] | u(32) |
| } |  |
| num_metadata_sets_minus1 | ue(v) |
| for ( i = 0; i < num_metadata_sets_minus1 + 1; i++) { |  |
| meta_proc_win_num_id[ i ] | ue(v) |
| meta_target_display_id[ i ] | ue(v) |
| manual_adjustment_flag[ i ] | u(1) |
| if ( manual_adjustment_flag[ i ] = = 1) { |  |
| min_PQ_maxRGB_offset[ i ] | i(32) |
| avg_PQ_maxRGB_offset[ i ] | i(32) |
| max_PQ_maxRGB_offset[ i ] | i(32) |
| tone_mapping_offset[ i ] | i(32) |
| tone_mapping_gain[ i ] | u(16) |
| tone_mapping_gamma[ i ] | u(16) |
| chroma_compensation_weight[ i ] | i(16) |
| saturation_gain[ i ] | i(16) |
| tone_detail_factor[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

## SEI payload semantics

### D3.4x Colour volume transform information SEI message semantics

This SEI message provides information to enable colour volume transform of the reconstructed colour samples of the output pictures. The input to the indicated colour volume transform process is either in luminance-chrominance color spaces or directly on RGB color components. In both cases, the set of decoded sample values shall apply an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format).

The colour volume transformation model used in the colour volume transform information SEI message is based on a parametrically defined tone mapping curve, the shape of which is defined both by the image essence characteristics (algorithmically computed from the input image essence) as well as by manually-set adjustments.

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of Society of Motion Picture and Television Engineers ST 2094-10.

NOTE x – Colour volume transform of the output pictures for the display process (which is outside the scope of this Specification) is optional and does not affect the decoding process specified in this Specification.

colour_volume_transform_id contains an identifying number that may be used to identify the purpose of the colour volume transform information. When more than one colour volume transform information SEI message is present with the same value of colour_volume_transform_id, the content of these colour volume transform information SEI messages shall be the same. When colour volume transform information SEI messages are present that have more than one value of colour_volume_transform_id, this indicates that the volume transform processes indicated by the different values of colour_volume_transform_id are alternatives that are provided for different purposes (not that a cascading of volume transform processes is to be applied). The value of colour_volume_transform_id shall be in the range of 0 to 232 − 2, inclusive.

Values of colour_volume_transform_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of colour_volume_transform_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour volume transform information SEI messages containing a value of colour_volume_transform_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE x – The colour_volume_transform_id can be used to support different colour volume transform processes that are suitable for different display scenarios. For example, different values of colour_volume_transform_id may correspond to different transform colour spaces supported by displays.

colour_volume_transform_cancel_flag equal to 1 indicates that the colour volume transform information SEI message cancels the persistence of any previous colour volume transform information SEI message in output order that applies to the current layer. colour_volume_transform_cancel_flag equal to 0 indicates that colour volume transform information follows.

colour_volume_transform_persistence_flag specifies the persistence of the colour volume transform information SEI message for the current layer.

colour_volume_transform_persistence_flag equal to 0 specifies that the colour volume transform information applies to the current picture only.

Let picA be the current picture. colour_volume_transform_persistence_flag equal to 1 specifies that the colour volume transform information persists for the current layer in output order until either of the following conditions is true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a colour volume transform information SEI message with the same value of colour_volume_transform_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

application_identifier identifies an application in its defining document in ST-2094 suite. For this SEI message, application_identifier shall be set to 1.

application_version specifies the application version in the application defining document in ST-2094 suite. For this SEI message, application_identifier shall be set to 0.

num_proc_wins_minus1 specifies the number of processing windows in the SEI message. The values of num_proc_wins_minus1 shall be in the range of 0 to 15 inclusive.

proc_win_num_id[ i ] identifies the number of the i-th processing window. The values of proc_win_num_id[ i ] shall be in the range of 0 to 15, inclusive.

proc_win_left_offset[ i ], proc_win_right_offset[ i ], proc_win_top_offset[ i ], and proc_win_bottom_offset[ i ] specify the samples of the pictures in the processing window with the number proc_win_num_id[ i ], in terms of a rectangular region specified in picture coordinates. When not present, the values of proc_win_left_offset[ i ], proc_win_right_offset[ i ], proc_win_top_offset[ i ], and proc_win_bottom_offset[ i ] are inferred to be equal to 0.

The processing window is specified for 4:4:4 chroma format. It contains the colour samples with horizontal picture coordinates from proc_win_left_offset[ i ] to pic_width_in_luma_samples − ( proc_win_right_offset[ i ] + 1 ) and vertical picture coordinates from proc_win_top_offset[ i ] to pic_height_in_luma_samples − ( proc_win_bottom_offset[ i ] + 1 ), inclusive.

The value of ( proc_win_left_offset[ i ] + proc_win_right_offset[ i ] ) shall be less than pic_width_in_luma_samples, and the value of ( proc_win_top_offset[ i ] + proc_win_bottom_offset[ i ] ) shall be less than pic_height_in_luma_samples.

min_PQ_maxRGB[ i ], avg_PQ_maxRGB[ i ], and max_PQ_maxRGB[ i ] specify the normalized minimum, average and maximum PQ-encoded maxRGB value of samples in the processing window with the number proc_win_num_id[ i ] in normalized increments of 0.00001. The values of min_PQ_maxRGB[ i ], avg_PQ_maxRGB[ i ], and max_PQ_maxRGB[ i ] shall be in the range of 0 to 100 000.

maxRGB specifies the maximum of color component values in R, G and B color components of a sample.

PQ-encoded maxRGB specifies the value of maxRGB encoded according to the Inverse-EOTF specified in ST-2084 (see also Annex Error: Reference source not found and Error: Reference source not found).

NOTE x –x  The reduced pixel set, which is the half resolution version of the selected pixels, is used for calculating the minimum, average and maximum PQ-encoded maxRGB value. The reduced pixel set is sampled from the selected pixels throughout all image essence associated with a metadata set. The reduced pixel set is calculated as follows:

Starting at the upper-left corner of the processing window, the processing window shall be partitioned into non-overlapping sample areas, each comprising 2x2 pixels. A component-by-component average shall be calculated for the pixels in each sample area. If a sample area falls partly outside the processing window, only the (1 or 2) pixels that are in the intersection of the sample area and the processing window shall be used in calculating the average. The reduced pixel set shall be the set of computed averages.

num_target_display_minus1 specifies the number of target displays in the SEI message. The values of num_target_display_minus1 shall be in the range of 0 to 15 inclusive.

target_display_id[ i ] specifies the identifier of the i-th target display. The values of target_display_id[ i ] shall be in the range of 0 to 15, inclusive.

target_display_primaries_x[ i ][ c ] and target_display_primaries_y[ i ][ c ] specify the normalized x and y chromaticity coordinates, respectively, of the c-th colour primary component for the i-th target display with the identifier target_display_id[ i ], in increments of 0.0001, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing target displays that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Error: Reference source not found). The values of target_display_primaries_x[ i ][ c ]  and target_display_primaries_y[ i ][ c ]shall be in the range of 0 to 10 000, inclusive.

target_display_white_point_x[ i ] and target_display_white_point_y[ i ] specify the normalized x and y chromaticity coordinates, respectively, of the white point for the i-th target display with the identifier target_display_id[ i ], in normalized increments of 0.0001, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). The values of target_display_white_point_x[ i ] and target_display_white_point_y[ i ] shall be in the range of 0 to 10 000.

max_target_display_luminance[ i ] specifies the maximum display luminance, respectively, for the i-th target display with the identifier target_display_id[ i ], in units of 0.01 candelas per square metre. The values of max_target_display_luminance[ i ] shall be in the range of 100 to 1 000 000.

min_target_display_luminance[ i ] specifies the minimum display luminance, respectively, for the i-th target display with the identifier target_display_id[ i ], in units of 0.0001 candelas per square metre. The values of min_target_display_luminance[ i ] shall be in the range of 0 to 1 000 000. The value of min_target_display_luminance[ i ] * 0.0001 shall be less than the value of max_target_display_luminance[ i ] * 0.01.

num_metadata_sets_minus1 specifies the number of metadata sets identified in the SEI message. The values of num_metadata_sets_minus1 shall be in the range of 0 to 28 − 1 inclusive.

meta_proc_win_num_id[ i ] is the proc_win_num_id for the i-th metadata set.

meta_target_display_id[ i ] is the target_display_id for the i-th metadata set.

manual_adjustment_flag[ i ] equal to 1 indicates that min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], max_PQ_maxRGB_offset[ i ], tone_mapping_offset[ i ], tone_mapping_gain[ i ], tone_mapping_gamma[ i ], chroma_compensation_weight[ i ], saturation_gain[ i ], and tone_detail_factor[ i ] are present for the i-th metadata set. manual_adjustment_flag equal to 0 indicates that min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], max_PQ_maxRGB_offset[ i ], tone_mapping_offset[ i ], tone_mapping_gain[ i ], tone_mapping_gamma[ i ], chroma_compensation_weight[ i ], saturation_gain[ i ], and tone_detail_factor[ i ] are not present for the i-th metadata set.

min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], and max_PQ_maxRGB_offset[ i ] specify the offset value for minimum, average and maximum PQ-encoded maxRGB, respectively, of the i-th metadata set in normalized increments of 0.00001. The value of min_PQ_maxRGB_offset[ i ], avg_PQ_maxRGB_offset[ i ], and max_PQ_maxRGB_offset[ i ] shall be in the range of −50 000 to 50 000. When not present, their values are inferred to be equal to 0.

The variable minPQmaxRGB, avgPQmaxRGB, and maxPQmaxRGB are derived as follows:

minPQmaxRGB = ( min_PQ_maxRGB[ i ] + min_PQ_maxRGB_offset[ i ]) * 0.00001	(D-xx)

avgPQmaxRGB = ( avg_PQ_maxRGB[ i ] + avg_PQ_maxRGB_offset[ i ]) * 0.00001	(D-xx)

maxPQmaxRGB = ( max_PQ_maxRGB[ i ] + max_PQ_maxRGB_offset[ i ]) * 0.00001	(D-xx)

The value of minPQmaxRGB, avgPQmaxRGB, and maxPQmaxRGB, shall obey the following constraints:

–	minPQmaxRGB shall be greater than or equal to 0.0 and less than avgPQmaxRGB;

–	avgPQmaxRGB shall be less than maxPQmaxRGB;

–	maxPQmaxRGB shall be less than or equal to 1.0.

NOTE x – The generation of the parametric tone mapping curve is outside the scope of this Specification. An example of defining a parametric tone mapping curve on adopting an analytically defined sigmoid curve using provided syntax is shown as follows:

The variable D represents input value of the colour component (R, G, or B) in units of cd/m2. The variable c1, c2, and c3 are coefficients, derived in Equation D-xx.  The variable E of output value for corresponding colour component is derived as follows:

E = ( c1 + c2 * D ) ÷ ( 1 + c3 * D )							(D-xx)

The variable x1, x2 and x3 denote the minimum luminance value of the input image in units of cd/m2, the average luminance value of the input image in units of cd/m2, and the maximum luminance value of the input image in units of cd/m2, respectively, and are derived as follows:

x1 = EOTFPQ ( minPQmaxRGB )							(D-xx)

x2 = EOTFPQ ( avgPQmaxRGB )							(D-xx)

x3 = EOTFPQ ( maxPQmaxRGB )							(D-xx)

The variable y1 and y3 denote the minimum luminance value in units of cd/m2, and the maximum luminance value in units of cd/m2, respectively, for target display, and are derived as follows:

y1 = min_target_display_luminance[ i ] * 0.0001						(D-xx)

y3 = max_target_display_luminance[ i ] * 0.0001						(D-xx)

The varible y2 denoted the derived targeted system display adaptation point in units of cd/m2 and is derived as follows:

y2 = sqrt( x2 * sqrt( y3 * y1 ) )								(D-xx)

The variable α is derived as

α = x3 * y3 * ( x1 − x2 ) + x2 * y2 * ( x3 − x1 ) + x1 * y1 * ( x2 − x3 )				(D-xx)

The variable c1, c2, and c3 are derived as follows:

c1 = ( x2 * x3 * (y2 − y3 ) * y1 + x1 * x3 * (y3 − y1 ) * y2 + x1 * x2 * ( y1 − y2 ) * y3 ) ÷ α		(D-xx)

c2 = ( ( x3 * y3 − x2 * y2 ) * y1 +( x1 * y1 − x3 * y3 ) * y2 + ( x2 * y2 − x1 * y1 ) * y3 ) ÷ α		(D-xx)

c3 = ( ( x3 − x2 ) * y1 +( x1 − x3 ) * y2 + ( x2 − x1 ) * y3 ) ÷ α					(D-xx)

tone_mapping_offset[ i ] specifies the value of tone mapping offset for the i-th metadata set in normalized increments of 0.00001. The value of tone_mapping_offset[ i ] shall be in the range of −50 000 to 50 000. When not present, its value is inferred to be equal to 0.

tone_mapping_gain[ i ] specifies the value of tone mapping gain for the i-th metadata set in normalized increments of 0.0001. The value of tone_mapping_gain[ i ] shall be in the range of 5 000 to 15 000. When not present, its value is inferred to be equal to 10 000.

tone_mapping_gamma[ i ] specifies the value of tone mapping gamma for the i-th metadata set in normalized increments of 0.001. The value of tone_mapping_gain[ i ] shall be in the range of 500 to 1 500. When not present, its value is inferred to be equal to 1 000.

The variable toneMappingOffset, toneMappingGain, and toneMappingGamma are derived as follows:

toneMappingOffset = tone_mapping_offset [ i ] * 0.00001				(D-xx)

toneMappingGain = tone_mapping_gain [ i ] * 0.0001				(D-xx)

toneMappingGamma = tone_mapping_gamma [ i ] * 0.001				(D-xx)

The variable x represents the normalized input value. The variable y represents the normalized output value. The tone mapping function is described as follows:

y = ( min( max( 0, ( x * toneMappingGain + toneMappingOffset ) ), 1 ) toneMappingGamma		(D-xx)

chroma_compensation_weight[ i ] specifies the amount of chroma adjustment for the i-th metadata set in normalized increments of 0.0001. The value of chroma_compensation_weight[ i ] shall be in the range of −5 000 to 5 000. When not present its value is inferred to be equal to 0.

saturation_gain[ i ] specifies the amount of saturation adjustment for the i-th metadata set in normalized increments of 0.0001. The value of saturation_gain[ i ] shall be in the range of −5 000 to 5 000. When not present its value is inferred to be equal to 0.

NOTE x – The usage of chroma_compensation_weight and saturation_gain is outside the scope of this Specification. An example to use such syntax is shown as follows:

The variable chromaCompensationWeight and staurationGain are derived as follows:

chromaCompensationWeight = chroma_compensation_weight[ i ] * 0.0001			(D-xx)

saturationGain = saturation_gain[ i ] * 0.0001						(D-xx)

The variable F represents input value of the colour component (R, G, or B) in units of cd/m2. The variable Y represents the luminance of the input pixel in units of cd/m2. The variable G of output value for corresponding colour component is described as follows:

G = F * ( ( 1 + chromaCompensateWeight) * F ÷ Y ) saturationGain				(D-xx)

tone_detail_factor[ i ] specifies the contribution of the detail management after tone mapping for the i-th metadata set in normalized increments of 0.001. The value of tone_detail_factor[ i ] shall be in the range of 0 to 1000. When not present its value is inferred to be equal to 0.

NOTE x – The usage of tone_detail_factor is outside the scope of this Specification. An example to use such syntax is shown as follows:

The variable toneDetailFactor is derived as

toneDetailFactor = tone_detail_factor [ i ] * 0.001					(D-xx)

The variable p represents the normalized input value. The variable q represents the normalized input value with detail management processing. The variable y of output value is described as follows:

y = p * (1 − toneDetailFactor ) + q * toneDetailFactor					(D-xx)

# Reference

- SMPTE ST 2094-1:2016: "Dynamic Metadata for Color Volume Transform – Core Components"

- SMPTE ST 2094-10:2016: "Dynamic Metadata for Color Volume Transform – Application #1"

- JCTVC-W1005, “HEVC Screen Content Coding Draft Text 6”, 24 March 2016.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).