| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0043 |
| --- | --- |

| Title: | AHG13: on Luma Adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Tao Chen, Walt Husak 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

Luma adjustment algorithm is used in the pre-encoding step for the conversion of source data into HDR10 format (NCL Y'CbCr PQ 4:2:0 10bit) in Anchor 3.2 for HDR/WCG video coding and is also listed as one of the major tools in the JCTVC output document of conversion and coding practices for HDR/WCG Video. It tries to fix chroma sub-sampling distortion due to the nature of non-constant luminance encoding of Y'CbCr. It is a closed loop conversion system, in the sense that both chroma downsampling and chroma upsampling are performed as part of the luma adjustment process, assuming certain filters are used. This contribution shares our understanding on the problem luma adjustment algorithm tries to solve and its effectiveness. The contribution includes test results for luma adjustment. It is reported that in terms of its impact on compression, if luma adjustment is disabled, the BDrate loss is only 0.5% for tPSNY, 0.5% for DE100, 0.3% for PSNRL100 and subjective quality remain indistinguishable from HDR10 Anchors. Due to the nature of being a closed-loop conversion system, luma adjustment is subject to the potential problem of the chroma upsampling filter used during pre-processing being different from the chroma upsampling filter used in post-processing. For this potential mismatch problem, it is reported that unmatched pre- and post-processing upsampling filters may introduce new color artifacts, compared to not using luma adjustment. It is therefore suggested that more careful study is required to justify recommending luma adjustment to be used in real applications. Further, it is asserted that the ICTCP color representation achieves better performance without the additional complexity for chroma subsampling or the potential problem of mismatched upsampling filters in pre- and post-processing..

# Background

In the early development of MPEG HDR/WCG AHG, it was observed that when using BT. 709 container for some HDR content, like Market, FireEater, etc, the conversion process with chroma resampling, which can simply be described as source RGB->NCL Y'CbCr 4:2:0->reconstructed RGB, can have some visible artifact even without compression [1]. The observation was that the artifact mostly appears in the color close to BT. 709 gamut boundary [1]. Later, after the AHG decided to follow the industry practice and use BT. 2020 container, the chroma resampling artifact disappeared for those HDR content.

In [2][4], luma adjustment algorithm was proposed to solve the artifact for those clips in BT. 709 container. Though the artifact disappears in BT. 2020 container, the Anchor generation process still employed the luma adjustment algorithm. There were two asserted benefits of using this algorithm:

1) When only pre-processing is applied (without compression), the algorithm can significantly improve a few objective metrics under the HDR CTC [9]. For example average tPSNR-Y may be increased from 62.61 dB to 69.77 dB, though it was also acknowledged that subjectively it was very hard to observe any improvement since the average tPSNR-Y is above 60dB;

2) It is expected that in the future, when we have content which can really cover BT. 2020 gamut, hypothetically we might get such kind of artifact again.

It was recently reported that the luma adjustment algorithm has the following limitations [3]:

- Substantial complexity increase: due to computations of chroma downsampling and upsampling, and of matrix conversion and transfer function in multiple iterations;

- A closed-loop system: assuming upsampling filter is known at pre-processing step;

- The close-loop ends before compression: after compression, the benefit is diminished.

In the past several meetings, several proposals [12][13][14] were presented, focusing mainly on solve problem 1) above, which is to reduce the complexity of luma adjustment algorithm. This contribution, instead, focuses on problems 2) and 3). In addition, we also try to clarify what problem the algorithm tries to solve and why the artifact is most visible in container gamut boundary. We hope the contribution can help JCT-VC gain further understanding of the luma adjustment algorithm.

# Problem statement

Figure 1 shows a simplified diagram on how luma adjustment algorithm works. The basic idea is to modify Y' so the luminance value is closer to the source (or target) luminance. In order to compute the luminance value, chroma pixels needs to go through the downsampling and upsampling processes and then go through the conversion from Y'CbCr 444 to luminance Y. Multiple iterations might be needed in order to search for Y'. For more details of the algorithm, readers are referred to [4].

Figure 1 Luma Adjustment diagram [4]

The algorithm essentially tries to solve the problem due to non-constant luminance (NCL) representation of Y'CbCr. NCL Y'CbCr does not fully de-correlate luminance from color. Therefore, when performing chroma downsampling, the distortions introduced in chroma can propagate to luminance where they become much more noticeable. Figure 2 shows the luminance difference of NCL Luma Y'-PQ against PQ luminance Y. The difference tends to become larger when color is more saturated. This also explains why chroma subsampling artifacts were mostly visible in colors close to gamut boundary where the color is more saturated.

Figure 2 Luminance correlation with Luma Y' of Y'CbCr PQ [7][8]

A clearer demonstration of the problem can be shown using a test pattern, as in Figure 3. The pattern is created by two very similar colors with ∆E2000 of 0.3 as in [3]. The pattern was downsampled to 10bits 4:2:0 in Y'CbCr and upsampled to 4:4:4 using default chroma subsampling filters defined in CTC [9]. Due to poor correlation of Y' and PQ Lumainance (Y), errors introduced in chroma during sub-sampling spread to the luminance and becomes very visible with ∆E2000 of 13.5. Luma adjustment fixes the problem with ∆E2000 of 0.25.

RGB1 = (1000, 0.005, 100), Y1 = 268.6 nits

RGB2 = (1000, 4, 100), Y2 = 271.3 nits

Figure 3 Test pattern to illustrate color sampling distortion in Y'CbCr and luma adjustment results

The explanation can be further justified when using a color representation where luma channel is much closer to luminance, for exampling, the ICTCP PQ representation [6]. Figure 4 shows the correlation of I against PQ luminance Y. When applying the same chroma resampling process as for YCbCr (without Luma Adjustment) using ICTCP PQ representation for the above test pattern, we will not observe any visible color distortions any more (with a ∆E2000=0.12). Figure 5 shows the ICTCP PQ results using the same pattern as in Figure 3.

Figure 4 Luminance correlation with Luma I of ICTCP PQ [7][8]

Figure 5 Test pattern to illustrate ICTCP PQ subsampling results

# Impact on compression performance

Table 1 and  show the conversion results with (ClosedLoopConversion=5) and without (ClosedLoopConversion=0) luma adjustment algorithm using HDR/WCG content in BT. 2020 container. All other settings are exactly the same following HDR/WCG CTC [9]. The objective metrics shows that tPSNR-Y has gained about 7dB. However, since the tPSNR-Y value is above 60dB, subjectively no differences could be observed.  shows the compression results (BD rate) using the same HM encoding setting as Anchor 3.2. The Anchor 3.2 has enabled luma adjustment algorithm. Though luma adjustment can improve tPSNR-Y by about 7 dB on average, after compression, the BDrate penalty for not using luma adjustment is only 0.5% for tPSNY, 0.5% for DE100, 0.3% for PSNRL100, as shown in . We also checked the subjective performance on HDR display for all bitrates and could not observe any visible subjective quality difference between Anchor v3.2 and Anchor v3.2 without luma adjustment.

The results show that luma adjustment in Anchor 3.2 cannot bring any benefit after compression under the HDR/WCG CTC.

Table 1 Conversion results of Y'CbCr with luma adjustment under HDR/WCG CTC

Table 2 Conversion results of Y'CbCr without luma adjustment under HDR/WCG CTC

Table 3 Compression results of Anchor 3.2 without luma adjustment against Anchor 3.2 (with luma adjustment) under HDR/WCG CTC

# Mismatch of chroma upsampling filter in pre-/post- processing

In Anchor 3.2, a Lanczos-2 chroma upsampling filter [-1 9 9 -1]/16 is used by default in the closed-loop luma adjustment process. The proponent claimed to have checked a bilinear filter in preprocessing and could not see any mismatch issue. In real applications, various upsampling filters might be used in pre- and post-processing, and what upsampling filters will be used in post-processing is to a pre-processing step such as luma adjustment.  We conducted experiments with different upsampling filters in HDRTools (like other type of Lanczos filters and the TM filter) to study the potential impact on video quality when mis-matched chroma upsampling filters are used. It is observed that when different upsampling filters are used in pre-processing and post-processing, the luma adjustment algorithm may introduce more visible color distortions than not using luma adjustment.

The chroma subsampling experiments are conducted using a set of real HDR/WCG content and subjective quality issues are observed when mismatched upsampling filters are used. Due to copyright restriction of the content, we instead use the same pattern in Figure 3 as an example to help illustrate the problem.  shows the result which uses default Lanczos-2 upsampling filter in post-processing, but TM upsampling filter ([+21.0, -52.0, +159.0, +159.0, -52.0, +21.0]/256) in pre-processing. Compared with not using luma adjustment, the luma adjustment process creates new rippling artifacts in some parts of the test pattern. The corresponding DE2000 metrics are computed and shown in Figure 7, where the larger DE distortions correspond to the pixels where larger luminance derivation is introduced by luma adjustment when upsampling filters are not matched in pre- and post-processing.

The preliminary study suggests that more careful study needs to be conducted to fully understand the potential impact of mismatched chroma upsampling filters in pre-/post- processing.

Figure 6 YCbCr Luma Adjustment with different upsampling filters in pre-/post processing, comparing with original YCbCr and ICtCp

Figure 7 DE2000 error metric of the examples in Figure 3 and

# Conclusion

This contribution tries to clarify the problem that the luma adjustment algorithm tries to solve, which is to fix chroma sub-sampling distortion due to the nature of non-constant luminance encoding of Y'CbCr. Two preliminary studies have been carried out on the luma adjustment algorithm. The first study shows that despite the large gain in tPSNR, the luma adjustment algorithm does not show any subjective benefits when no compression is applied. Further, when compression is applied, there is hardly any benefit left both subjectively and objectively. The second study shows that if there is a mismatch of upsampling filters in pre- and post- processing, applying luma adjustment may introduce new chroma sub-sampling artifact.

This contribution also discussed alternative ways of addressing the problem that the luma adjustment method tries to solve by using alternative iso-luminance color difference encodings the ICTCP PQ color space. It is shown that the ICTCP PQ color space could achieve better quality without the additional complexity associated with luma adjustment or the potential problem of mismatched upsampling filters in pre- and post-processing.

It is suggested that more careful study is required to justify JCT-VC’s recommendation of using luma adjustment in real applications.

# Reference

- E. François, P. Lopez, Y. Olivier, (Technicolor) “About using a BT.2020 container for BT.709 content”, doc. m35255, 110th MPEG meeting Sapporo 2014

- J. Strom, “Investigation of HDR Color Subsampling”, doc. m35841, 110th MPEG meeting, Geneva, Feb. 2015

- A. Tourapis, “Best practices for HDR Encoding”, USNB tutorial, San Francisco, USA, April, 2016

- “Conversion and Coding Practices for HDR/WCG Video, Draft 1”, JCTVC-W1007v5, May, 2016

- “Common Test Conditions for HDR/WCG video coding experiments”, JCTVC-W1020, March 2016.

- Rec. ITU-R BT.[HDR-TV], Image parameter values for high dynamic range television for use in production and international programme exchange.

- J. Pytlarz, “Overview of IPT-PQ Color Space”, JCTVC-W0050, March 2016.

- Report ITU-R BT. 2390-0, High dynamic range television for production and international programme exchange, 2016.

- “Common Test Conditions for HDR/WCG video coding experiments”, JCTVC-W1020, March 2016.

- J. Pytlarz, R., T. Lu, F. Pu, P. Yin, T. Chen and W. Husak, “Overview of IPT-PQ Color Space”, MPEG contribution, doc. m37266, Oct. 2015.

- HDRTools: https://gitlab.com/standards/HDRTools/

- C. Rosewarne and V. Kolesnikov, “CE1-related: LUT-based luma sample adjustment”, JCTVC-W0056, March 2016.

- A. M. Tourapis, Y. Su, D. Singer, “Enhanced Luma Adjustment Methods”, JCTVC-W0052, March 2016.

- Andrey Norkin, “Closed form HDR 4:2:0 chroma subsampling”, JCTVC-W0052, March 2016.

# Acknowledge

We would like to thank InterDigital for helping to crosscheck this contribution.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).