| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, Switzerland, 26th May–1st June 2016 | Document: JCTVC-X0044-r2 |
| --- | --- |

| Title: | Improvements to HDR10 with Backward Compatibility Options |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Wei Dai Madhu Krishnan 3097 Cortona Dr., Melbourne, FL 32940 | Tel: Email: | 321-355-7376 {pankajtva, daisy, madhupk83 @ gmail.com} |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This proposal presents a backward compatible solution to coding HDR video, by effectively modifying the pre and post processing components of the anchor HDR video coding processing chain (essentially an HDR10 compliant system). Moreover, potential coding efficiency gains from using advanced resampling filters for 4:4:4->4:2:0->4:4:4 conversion and alternate intermediate color representation (Y’FbFr) are also studied. Representative objective results for the system (compared with HDR10 anchor v3.2) include: (a) Overall results for RGB-PSNR, DE100, MD100, PSNRL100 were -43.7%, -3.3%, -33.7%, 5.6%. The proposed coding scheme provides equal or superior visual quality to the HDR10 anchor.

# Introduction

As part of the investigations done to leverage existing video standards for efficiently encoding HDR/WCG videos, FastVDO developed several effective approaches for HDR video coding. All our methods can be implemented entirely as pre/post processing steps, and do not necessitate any normative changes to the core Main10 encode/decode processes. In this proposal modification to the pre and post processing components of the anchor HDR video coding processing chain is described.

Color transformations and chroma resampling filters are in wide use in video processing applications. Most video codecs take YUV 4:2:0 as input, whereas the native representation of acquired video is typically RGB 4:4:4. Thus, this involves two processes: color transform to a YUV space, and downsampling to 4:2:0. A standard color conversion is using the Y’CbCr color transform (as tuned for BT.709, or BT.2020). While powerful, these transformations are still floating-point, built on a design that dates back decades, and may not be optimal for modern finite-precision compression technology. FastVDO has developed high performance, low-complexity integer color transforms with excellent coding gain. The integer color transforms can be implemented in a lossless manner with lifting steps, etc., rather than as matrix multiplies [1, 2]. Our new color transform is preferred over Y’CbCr for coding gain and potential integer processing benefits.

In any case, information is definitely lost in the color space resampling to YUV 4:2:0, so that irreversible conversion needs to be of the highest quality achievable. FastVDO has designed advanced resampling filters (down and up), which are applicable for both spatial and chroma resampling, which achieve optimized recovery of the original signal, whether spatially or chromatically [1, 3]. These processes can be used strictly as pre and post processes, which leave the core HEVC Main10 codec (or any suitable core codec) alone. All existing hardware and software that support this can be reused. Moreover, the output of a Main10 decoder is a viewable SDR video.

To achieve true backward compatibility, one can use the standard Y’CbCr color space; then, the output of the decoder is directly viewable as a nearly optimized SDR rendering. We study this option as well.

# System Overview

  - Pre-processing: The pre-processing step in the proposed HDR coding chain is presented in Figure 1.

Grade HDR to SDR

RGBHDR	RGBSDR

Generate HDR to SDR Grading Process

Metadata

Figure 1. Pre-Processing, which regrades HDR to SDR.

The HDR-to-SDR grading process uses as a key component a base transfer function; in our approach, this can be static or adaptive. Example base transfer functions that can be used include ST-2084 (Dolby PQ), Philips TF [4], and HLG (ARIB STD-B67). A statistical analysis of the linear light HDR RGB input signal is performed here as input to generate the grading process, and the associated metadata. The grading process is then used to grade the HDR RGB input signal to an SDR RGB signal. The SDR RGB signal is then color transformed to Y’CbCr (or Y’FbFr) signal, downsampled from 4:4:4 to 4:2:0 (using advanced downsampling filters) and encoded using HEVC reference codec (Main10 profile). The Metadata is sent as SEI data.

  - Post-processing: The post-processing step in the proposed HDR coding chain is presented in Figure 2.

Regrade SDR to HDR

RGBSDR	RGBHDR

Generate SDR to HDR Regrading Process

Metadata

Figure 2. Post-Processing, which recovers HDR from SDR.

The HEVC reference codec is used to decode the bitstream and generate the reconstructed YUV 4:2:0 signal and metadata. The YUV 4:2:0 signal is then upsampled (using co-designed upsampling filters) from 4:2:0 to 4:4:4 and color transformed from Y’CbCr (or Y’FbFr) to SDR RGB. Then the regrading process is computed using the metadata and SDR RGB signal. Finally, regrading is performed on the SDR RGB input to obtain the linear light reconstructed HDR RGB signal.

# Objective Results

The objective results obtained by the proposed system using a) Y’FbFr as the intermediate color representation and b) Y’CbCr as the intermediate color representation are shown in Figure 4 and Figure 5 respectively. The reference for these tests is the HDR10 anchor v3.2. Spreadsheets with the complete metrics are provided as evidence. The proposed coding scheme provides equal or superior visual quality to the HDR10 anchor v3.2. We plan to bring some visual evidence for potential viewing at the Geneva meeting. Moreover, in the opinion of the authors, and for a limited test set, the SDR YUV 4:2:0 sequences generated after pre-processing looks more natural, and more similar to the original HDR video than the anchor (HDR10) generated YUV 4:2:0 videos.

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR R | PSNR G | PSNR B | PSNR RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 92.6% | 68.9% | 239.5% | 120.6% | 178.6% | 138.3% | -22.9% | 28.2% | 0.0% | -52.0% | 0.0% | -17.3% |
|  | Market3Clip4000r2 | 46.9% | 47.0% | 46.6% | 46.9% | 27.0% | -27.4% | -64.7% | -0.9% | -50.6% | -46.7% | -58.8% | -52.0% |
|  | SunRise | 90.0% | 96.5% | 98.9% | 95.2% | 56.3% | -45.8% | -24.4% | 14.7% | -56.2% | -43.7% | -65.2% | -55.0% |
| class B | BikeSparklers cut 1 | 31.5% | 33.4% | 48.6% | 37.6% | 39.3% | 12.5% | -1.7% | 17.9% | -54.6% | -39.6% | -52.7% | -49.0% |
|  | BikeSparklers cut 2 | 38.8% | 37.0% | 38.5% | 24.1% | 36.7% | 7.5% | -38.9% | 23.3% | -30.0% | -20.2% | -28.5% | -26.2% |
|  | GarageExit | 20.9% | 23.2% | 23.9% | 22.7% | 22.4% | 4.4% | -83.0% | 10.8% | -32.7% | -19.0% | -42.3% | -31.3% |
| class C | ShowGirl2Teaser | 24.7% | 23.1% | 120.3% | 52.5% | 56.7% | 27.1% | 0.0% | 7.1% | -59.5% | -45.4% | -62.9% | -56.0% |
| class D | StEM_MagicHour cut 1 | 7.3% | 8.8% | 23.8% | 15.3% | 15.0% | 3.2% | -76.6% | 3.1% | -62.9% | -46.4% | -44.7% | -51.3% |
|  | StEM_MagicHour cut 2 | -37.5% | -39.8% | -55.0% | -47.0% | -49.9% | -60.2% | -83.9% | -28.9% | -57.8% | -31.5% | -51.6% | -47.0% |
|  | StEM_MagicHour cut 3 | 2.9% | 3.4% | 5.2% | 4.2% | 4.1% | 6.6% | -29.9% | -0.5% | -57.4% | -50.5% | -27.3% | -45.1% |
|  | StEM_WarmNight cut 1 | 14.6% | 13.7% | 18.4% | 15.9% | 18.3% | 10.1% | -28.8% | 2.7% | -69.4% | -48.7% | -65.7% | -61.3% |
|  | StEM_WarmNight cut 2 | 9.6% | 5.9% | 7.4% | 7.4% | 10.1% | -4.6% | 0.0% | -3.5% | -54.4% | -64.6% | -51.7% | -56.9% |
| class G | BalloonFestival | 12.4% | 18.6% | 22.1% | 18.2% | 9.7% | -26.2% | -50.8% | -7.7% | -50.3% | -40.6% | 0.0% | -30.3% |
| class H | EBU_04_Hurdles | 11.7% | 17.5% | -1.4% | 7.7% | -5.2% | -64.3% | 0.0% | 1.7% | -48.1% | -19.9% | -51.3% | -39.7% |
|  | EBU_06_Start | 46.8% | 50.0% | 46.9% | 47.9% | 32.7% | -31.1% | 0.0% | 15.5% | -45.8% | -20.5% | -44.4% | -36.9% |
|  | Overall | 27.5% | 27.1% | 45.6% | 31.3% | 30.1% | -3.3% | -33.7% | 5.6% | -48.7% | -39.3% | -43.1% | -43.7% |

Figure 4. Objective Results (using Y’FbFr). The use of this color space provides coding as well as complexity efficiency, while still providing a level of direct viewability on SDR monitors.

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR R | PSNR G | PSNR B | PSNR RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 73.1% | 51.5% | 208.0% | 99.3% | 149.1% | 115.2% | -32.7% | 21.6% | -67.6% | -42.5% | -69.9% | -60.0% |
|  | Market3Clip4000r2 | 42.2% | 42.3% | 37.3% | 40.4% | 20.4% | -33.9% | -79.2% | -3.4% | -33.0% | -30.5% | -46.0% | -36.5% |
|  | SunRise | 55.7% | 61.0% | 69.1% | 62.2% | 33.7% | -55.2% | -45.9% | 5.5% | -54.9% | -44.8% | -65.0% | -54.9% |
| class B | BikeSparklers cut 1 | 26.4% | 29.2% | 36.5% | 30.7% | 34.6% | 8.0% | -24.3% | 18.5% | -7.5% | -6.1% | -14.0% | -9.2% |
|  | BikeSparklers cut 2 | 21.6% | 25.7% | 30.3% | 25.6% | 29.3% | 0.2% | -26.3% | 18.1% | -37.5% | -31.1% | -44.1% | -37.5% |
|  | GarageExit | 13.9% | 15.8% | 16.2% | 15.3% | 14.4% | -0.6% | -82.0% | 7.1% | -68.8% | -60.3% | -77.0% | -68.7% |
| class C | ShowGirl2Teaser | 25.5% | 25.5% | 48.8% | 33.7% | 27.2% | -9.6% | -83.4% | 0.8% | -24.1% | -20.8% | -24.1% | -23.0% |
| class D | StEM_MagicHour cut 1 | 5.4% | 6.3% | 17.6% | 11.4% | 11.3% | 1.5% | -83.2% | 1.6% | -59.2% | -42.0% | -44.9% | -48.7% |
|  | StEM_MagicHour cut 2 | 6.8% | 6.3% | 16.3% | 11.0% | 11.6% | 5.1% | -40.3% | 0.1% | -17.0% | -22.8% | -46.0% | -28.6% |
|  | StEM_MagicHour cut 3 | 8.1% | 7.0% | 18.7% | 13.0% | 15.2% | 7.7% | -23.9% | -0.7% | -61.4% | -52.3% | -13.5% | -42.4% |
|  | StEM_WarmNight cut 1 | 10.3% | 10.2% | 15.2% | 12.3% | 13.5% | 4.0% | -37.2% | 0.4% | -64.0% | -40.2% | -57.8% | -54.0% |
|  | StEM_WarmNight cut 2 | 9.4% | 6.9% | 20.9% | 13.7% | 18.8% | 0.1% | 0.0% | -1.3% | -53.8% | -21.2% | -25.1% | -33.4% |
| class G | BalloonFestival | 10.1% | 15.0% | 24.7% | 17.4% | 11.6% | -21.0% | -69.3% | -8.5% | -55.0% | -46.8% | -69.9% | -57.3% |
| class H | EBU_04_Hurdles | 8.0% | 11.7% | -0.6% | 5.2% | -7.0% | -53.6% | -42.4% | -4.4% | -55.0% | -29.2% | -52.0% | -45.4% |
|  | EBU_06_Start | 34.4% | 37.5% | 31.6% | 34.4% | 19.5% | -39.3% | 0.0% | 7.9% | -65.6% | -42.3% | -62.9% | -56.9% |
|  | Overall | 23.4% | 23.5% | 39.4% | 28.4% | 26.9% | -4.8% | -44.7% | 4.2% | -48.3% | -35.5% | -47.5% | -43.8% |

Figure 5. Objective Results (using Y’CbCr and Philips TF as the base transfer function), which provides for direct viewability of the SDR video on current generation monitors. We note that the results for Y’CbCr and for YFbFr are quite similar. In particular, for RGB PSNR, the averaged results are almost identical. However, note that YFbFr outperforms Y’CbCr in the Green channel, which human vision is most sensitive, at the expense of the blue channel, to which human vision is least sensitive. Moreover, may reaching nearly 40 dB in all channels, it provides a better distribution of error in the color channels of RGB compared to Y’CbCr.

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR R | PSNR G | PSNR B | PSNR RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 50.9% | 43.3% | 107.0% | 61.6% | 75.0% | 62.7% | -33.5% | 20.4% | -70.2% | -44.5% | -69.6% | -61.5% |
|  | Market3Clip4000r2 | 42.9% | 44.1% | 31.6% | 39.0% | 18.2% | -45.0% | 0.0% | -1.5% | -43.5% | -38.9% | -53.2% | -45.2% |
|  | SunRise | 52.3% | 59.6% | 50.7% | 54.3% | 23.8% | -70.5% | -47.2% | 6.4% | -55.0% | -41.4% | -67.7% | -54.7% |
| class B | BikeSparklers cut 1 | 29.5% | 33.9% | 26.9% | 30.1% | 32.6% | -5.3% | -22.1% | 22.2% | -41.6% | -23.4% | -40.4% | -35.2% |
|  | BikeSparklers cut 2 | 24.9% | 30.6% | 22.3% | 25.8% | 28.5% | -12.0% | -28.2% | 22.2% | -34.2% | -16.5% | -28.9% | -26.5% |
|  | GarageExit | 12.5% | 17.2% | 1.4% | 9.3% | 7.0% | -25.7% | -81.1% | 10.1% | -38.5% | -27.3% | -47.6% | -37.8% |
| class C | ShowGirl2Teaser | 20.7% | 23.9% | 27.9% | 24.8% | 10.0% | -25.5% | -83.4% | 1.8% | -48.4% | -36.4% | -59.6% | -48.1% |
| class D | StEM_MagicHour cut 1 | 3.9% | 7.8% | 6.2% | 5.9% | 5.3% | -12.9% | 0.0% | 4.1% | -60.8% | -38.8% | -40.7% | -46.7% |
|  | StEM_MagicHour cut 2 | 5.0% | 6.9% | 4.4% | 5.0% | 4.2% | -10.2% | -40.8% | 2.4% | -68.9% | -47.3% | -23.3% | -46.5% |
|  | StEM_MagicHour cut 3 | 5.7% | 7.5% | 5.1% | 5.7% | 5.5% | -8.2% | -19.5% | 2.0% | -60.5% | -42.6% | -28.9% | -44.0% |
|  | StEM_WarmNight cut 1 | 6.0% | 9.0% | 3.8% | 5.8% | 5.0% | -11.8% | -33.9% | 2.1% | -66.7% | -41.3% | -61.9% | -56.7% |
|  | StEM_WarmNight cut 2 | 3.8% | 6.7% | 6.6% | 5.7% | 7.1% | -14.0% | 0.0% | 1.0% | -76.1% | -47.6% | -61.6% | -61.8% |
| class G | BalloonFestival | 8.5% | 16.3% | 9.4% | 11.0% | 3.0% | -36.5% | 0.0% | -5.2% | -45.5% | -34.6% | -64.1% | -48.0% |
| class H | EBU_04_Hurdles | 11.8% | 16.6% | -0.3% | 7.9% | -4.9% | -61.0% | -54.6% | 1.3% | -54.1% | -25.9% | -51.3% | -43.8% |
|  | EBU_06_Start | 39.5% | 45.9% | 28.9% | 37.3% | 22.5% | -47.4% | 0.0% | 16.1% | -43.6% | -16.5% | -41.7% | -33.9% |
|  | Overall | 21.2% | 24.6% | 22.1% | 21.9% | 16.2% | -21.6% | -29.6% | 7.0% | -53.8% | -34.9% | -49.4% | -46.0% |

Figure 5. Objective Results (using Y’CbCr and ST-2084 TF as the base transfer function), with direct viewability of the Main10 decoded signal on current SDR monitors.

- Conclusion

A framework is presented for effective coding of HDR/WCG video, which can be HDR10 compliant, and provide a measure of backward compatibility. When using Y’CbCr, one has direct viewability of the SDR video after Main10 decoding.  Moreover, in addition, when the intermediate processing chain (the HDR to SDR conversion) uses the ST-2084 TF, our approach is essentially compatible with HDR10 directly, and becomes a minor updating with some additional SEI information. It is suggested that the proposed technique should be further studied as a potential updating for the current anchor generation process.

- References

[1] 	Pankaj Topiwala, Wei Dai and Madhu Krishnan, “HDR CE5: Report of Experiment 5.3.2”, JCTVC-W0055, San Diego, Feb., 2016.

[2] 	Pankaj Topiwala and Chengjie Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video”, JVT-I015r7, San Diego, Sept., 2003.

[3] Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Re-sampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

[4] R. Goris, R. Brondijk, and R. van der Vleuten, “Philips Response to CFE on HDR and WCG,” ISO/IEC MPEG doc. m36266, Warsaw, PL, July, 2015.

# Patent rights declaration(s)

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).