| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, Switzerland, 26th May–1st June 2016 | Document: JCTVC-X0045 |
| --- | --- |

| Title: | A study on chroma resampling methods for HDR/WCG video signals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Wei Dai Madhu Krishnan 3097 Cortona Dr., Melbourne, FL 32940 | Tel: Email: | 321-355-7376 {pankajtva, daisy, madhupk83 @ gmail.com} |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

A number of chroma resampling techniques used in the 4:4:4 to 4:2:0 and 4:2:0 to 4:4:4 conversions are studied in this paper. The filters studied are all part of the current HDRTools package. Since the combination of downsampling and upsampling is a highly lossly operation even without compression processing, this study looks at just the impact of chroma resampling in the HDR video coding processing chain, minus the compression itself.

# Introduction

The HDR/WCG coding tool chain currently utilizes a 3-tap filter [1 6 1]/8 for 4:4:4 to 4:2:0 conversion (downsampling) and a 4-tap filter [-1 9 9 -1]/16 for 4:2:0 to 4:4:4 conversion (upsampling) in the anchor generation process (Anchor v3.2) [1]. The question arises if this pair is optimal in any way, or can it be improved upon. There are a number of other filters of interest in the HDRTools software package [4], including: (a) the [1 2 1]/4 downsampling, [-1 9 9 -1]/16 upsampling pair; (b) a 15 tap downsampling, 4 tap upsampling pair (mentioned as DF_GS, UF_GS in the HDRTools package) and (c) various adaptive filters in HDRTools-0.11-dev [6].

In any case, information is definitely lost in the color space resampling to YUV 4:2:0, so that irreversible conversion needs to be of the highest quality achievable. FastVDO has designed advanced resampling filters (down and up), which are applicable for both spatial and chroma resampling, which achieve optimized recovery of the original signal, whether spatially or chromatically [2], [3].

In this paper, a FastVDO-designed 9-tap downsampling, 4-tap upsampling pair [2] is compared against the various filters mentioned above.

# Experiment

During pre-processing, the linear light HDR RGB input signal is converted to the perceptual domain by applying the PQ EOTF. The resultant signal is color transformed to Y’CbCr 4:4:4. The chroma components are then downsampled in floating point domain using the various filters mentioned in Section 1. The Y’CbCr 4:2:0 signal generated by this process is then scaled to fixed precision (10 bits).

In post-processing the fixed precision 10 bit Y’CbCr signal is converted to a floating point representation first. Then the chroma components are upsampled using filters mentioned in Section 1. Inverse PQ EOTF and color transformation are performed, to arrive at the linear light HDR RGB signal.

Objective metrics are calculated by comparing the HDR RGB signal generated after the pre/post processing step with the original signal.

# Objective Results

The test sequences mentioned in the common test conditions document [5] were evaluated in terms of the various distortion metrics. The tables 1 through 4 show the objective results obtained for each filter

Table 1. Current Anchor v3.2 filters

|  |  | R | G | B | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE0100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 63.82 | 72.50 | 69.24 | 63.55 | 70.21 | 52.40 | 56.78 | 55.25 | 49.43 | 24.73 | 58.38 |
|  | Market3Clip4000r2 | 54.07 | 62.98 | 51.06 | 55.75 | 69.61 | 46.02 | 50.34 | 50.07 | 37.01 | 21.88 | 47.89 |
|  | SunRise | 61.00 | 68.63 | 61.37 | 60.10 | 69.61 | 51.12 | 55.32 | 55.09 | 38.40 | 24.23 | 47.94 |
| class B | BikeSparklers cut 1 | 55.43 | 64.26 | 56.65 | 47.55 | 69.76 | 37.05 | 41.45 | 39.85 | 34.21 | 21.83 | 51.47 |
|  | BikeSparklers cut 2 | 56.32 | 64.99 | 58.66 | 48.63 | 69.73 | 38.43 | 42.80 | 41.65 | 34.55 | 21.78 | 51.07 |
|  | GarageExit | 60.83 | 68.78 | 55.69 | 56.55 | 69.70 | 47.17 | 51.44 | 50.21 | 38.25 | 22.85 | 49.77 |
| class C | ShowGirl2Teaser | 65.14 | 72.20 | 66.25 | 53.53 | 69.24 | 45.03 | 49.19 | 47.98 | 39.41 | 22.52 | 52.89 |
| class D | StEM_MagicHour cut 1 | 76.42 | 83.35 | 69.20 | 54.13 | 69.81 | 40.00 | 44.60 | 42.07 | 36.60 | 24.97 | 52.15 |
|  | StEM_MagicHour cut 2 | 70.76 | 78.59 | 70.73 | 51.56 | 69.87 | 38.73 | 43.28 | 40.82 | 36.02 | 24.71 | 52.67 |
|  | StEM_MagicHour cut 3 | 75.08 | 82.02 | 67.40 | 50.95 | 69.92 | 37.52 | 42.09 | 39.67 | 36.09 | 24.29 | 52.93 |
|  | StEM_WarmNight cut 1 | 74.47 | 82.93 | 69.29 | 52.65 | 69.92 | 38.47 | 43.08 | 40.67 | 36.70 | 25.12 | 52.93 |
|  | StEM_WarmNight cut 2 | 68.89 | 76.88 | 63.36 | 54.29 | 70.16 | 39.16 | 43.79 | 41.74 | 37.08 | 23.36 | 52.80 |
| class G | BalloonFestival | 59.42 | 66.03 | 53.97 | 53.78 | 69.59 | 45.09 | 49.30 | 51.97 | 40.88 | 21.15 | 48.24 |
| class H | EBU_04_Hurdles | 50.65 | 58.94 | 53.25 | 54.25 | 69.59 | 46.74 | 50.78 | 49.31 | 36.98 | 22.24 | 47.34 |
|  | EBU_06_Start | 51.39 | 59.20 | 53.69 | 51.28 | 69.60 | 44.11 | 48.11 | 47.88 | 36.29 | 21.28 | 47.71 |
|  | Overall | 62.91 | 70.82 | 61.32 | 53.90 | 69.76 | 43.14 | 47.49 | 46.28 | 37.86 | 23.13 | 51.08 |

Table 2.  [1 2 1]/4 downsampling, [-1 9 9 -1]/16 upsampling pair

|  |  | R | G | B | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE0100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 62.68 | 71.36 | 67.65 | 62.96 | 70.21 | 51.78 | 56.17 | 54.38 | 49.03 | 24.58 | 58.38 |
|  | Market3Clip4000r2 | 53.78 | 62.80 | 50.63 | 55.33 | 69.61 | 45.55 | 49.87 | 49.18 | 36.62 | 22.06 | 47.89 |
|  | SunRise | 60.79 | 68.47 | 61.00 | 59.80 | 69.61 | 50.73 | 54.94 | 54.55 | 38.19 | 24.59 | 47.94 |
| class B | BikeSparklers cut 1 | 55.15 | 64.04 | 56.40 | 47.18 | 69.76 | 36.75 | 41.14 | 39.06 | 33.82 | 22.24 | 51.47 |
|  | BikeSparklers cut 2 | 56.03 | 64.73 | 58.63 | 48.23 | 69.73 | 38.13 | 42.50 | 40.86 | 34.16 | 22.16 | 51.07 |
|  | GarageExit | 60.54 | 68.51 | 55.33 | 56.14 | 69.70 | 46.66 | 50.95 | 49.32 | 37.83 | 23.04 | 49.77 |
| class C | ShowGirl2Teaser | 64.61 | 71.64 | 65.69 | 53.32 | 69.23 | 44.80 | 48.96 | 47.43 | 39.11 | 22.63 | 52.88 |
| class D | StEM_MagicHour cut 1 | 75.81 | 82.87 | 69.03 | 54.02 | 69.81 | 39.84 | 44.44 | 41.57 | 36.36 | 25.00 | 52.15 |
|  | StEM_MagicHour cut 2 | 70.05 | 77.93 | 70.55 | 51.46 | 69.87 | 38.58 | 43.13 | 40.33 | 35.79 | 24.85 | 52.67 |
|  | StEM_MagicHour cut 3 | 74.42 | 81.61 | 67.42 | 51.02 | 69.92 | 37.56 | 42.13 | 39.37 | 35.95 | 24.52 | 52.93 |
|  | StEM_WarmNight cut 1 | 73.60 | 82.32 | 68.99 | 52.58 | 69.92 | 38.50 | 43.10 | 40.34 | 36.53 | 25.27 | 52.93 |
|  | StEM_WarmNight cut 2 | 67.92 | 76.14 | 63.28 | 54.24 | 70.16 | 39.17 | 43.81 | 41.41 | 36.93 | 23.64 | 52.80 |
| class G | BalloonFestival | 58.44 | 65.25 | 53.09 | 52.91 | 69.58 | 44.21 | 48.42 | 50.56 | 40.16 | 21.10 | 48.24 |
| class H | EBU_04_Hurdles | 49.50 | 57.98 | 52.49 | 53.70 | 69.59 | 46.30 | 50.33 | 48.46 | 36.52 | 22.33 | 47.34 |
|  | EBU_06_Start | 50.98 | 58.92 | 53.63 | 51.03 | 69.60 | 43.95 | 47.93 | 47.21 | 35.96 | 21.73 | 47.71 |
|  | Overall | 62.29 | 70.30 | 60.92 | 53.60 | 69.75 | 42.83 | 47.19 | 45.60 | 37.53 | 23.32 | 51.08 |

Table 3. 15 tap downsampling, 4 tap upsampling pair

|  |  | R | G | B | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE0100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 64.60 | 73.29 | 69.59 | 63.92 | 70.21 | 52.84 | 57.21 | 55.46 | 49.48 | 24.87 | 58.38 |
|  | Market3Clip4000r2 | 53.81 | 62.84 | 50.84 | 56.05 | 69.61 | 46.40 | 50.71 | 49.82 | 36.86 | 22.59 | 47.89 |
|  | SunRise | 61.18 | 68.78 | 61.55 | 60.36 | 69.61 | 51.50 | 55.68 | 55.02 | 38.40 | 24.94 | 47.94 |
| class B | BikeSparklers cut 1 | 55.35 | 64.24 | 56.31 | 47.88 | 69.76 | 37.37 | 41.77 | 39.66 | 34.10 | 22.29 | 51.47 |
|  | BikeSparklers cut 2 | 56.25 | 64.99 | 58.27 | 48.96 | 69.73 | 38.76 | 43.13 | 41.46 | 34.44 | 22.14 | 51.07 |
|  | GarageExit | 60.96 | 68.94 | 55.63 | 56.93 | 69.70 | 47.58 | 51.85 | 50.03 | 38.16 | 23.46 | 49.77 |
| class C | ShowGirl2Teaser | 65.49 | 72.50 | 66.83 | 53.77 | 69.26 | 45.26 | 49.42 | 47.81 | 39.34 | 23.18 | 52.89 |
| class D | StEM_MagicHour cut 1 | 76.97 | 83.90 | 69.50 | 54.46 | 69.81 | 40.33 | 44.93 | 42.03 | 36.58 | 25.43 | 52.15 |
|  | StEM_MagicHour cut 2 | 71.28 | 79.14 | 71.04 | 51.88 | 69.87 | 39.06 | 43.60 | 40.77 | 36.00 | 25.22 | 52.67 |
|  | StEM_MagicHour cut 3 | 74.71 | 81.89 | 67.70 | 51.29 | 69.93 | 37.86 | 42.44 | 39.66 | 36.09 | 24.70 | 52.93 |
|  | StEM_WarmNight cut 1 | 75.08 | 83.51 | 69.58 | 52.98 | 69.93 | 38.81 | 43.42 | 40.63 | 36.69 | 25.58 | 52.93 |
|  | StEM_WarmNight cut 2 | 69.81 | 77.69 | 63.68 | 54.62 | 70.17 | 39.49 | 44.12 | 41.69 | 37.08 | 23.83 | 52.81 |
| class G | BalloonFestival | 59.41 | 66.02 | 53.91 | 54.23 | 69.60 | 45.50 | 49.71 | 51.45 | 40.62 | 21.17 | 48.24 |
| class H | EBU_04_Hurdles | 51.44 | 59.65 | 53.80 | 54.60 | 69.59 | 47.18 | 51.20 | 49.17 | 36.84 | 22.82 | 47.34 |
|  | EBU_06_Start | 52.16 | 59.86 | 54.05 | 51.59 | 69.60 | 44.51 | 48.49 | 47.58 | 36.12 | 22.01 | 47.71 |
|  | Overall | 63.23 | 71.15 | 61.48 | 54.24 | 69.76 | 43.50 | 47.84 | 46.15 | 37.78 | 23.61 | 51.08 |

Table 4. FastVDO-designed 9-tap downsampling, 4-tap upsampling pair

|  |  | R | G | B | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE0100 | MD100 | PSNRL100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 65.14 | 73.84 | 70.28 | 64.21 | 70.21 | 53.19 | 57.55 | 55.97 | 49.74 | 24.93 | 58.38 |
|  | Market3Clip4000r2 | 54.00 | 62.97 | 51.03 | 56.36 | 69.61 | 46.73 | 51.03 | 50.54 | 37.18 | 22.34 | 47.89 |
|  | SunRise | 61.40 | 68.92 | 61.81 | 60.59 | 69.61 | 51.77 | 55.94 | 55.46 | 38.59 | 24.64 | 47.94 |
| class B | BikeSparklers cut 1 | 55.63 | 64.46 | 56.51 | 48.19 | 69.76 | 37.65 | 42.05 | 40.32 | 34.42 | 21.97 | 51.47 |
|  | BikeSparklers cut 2 | 56.48 | 65.18 | 58.47 | 49.28 | 69.73 | 39.03 | 43.40 | 42.11 | 34.76 | 21.88 | 51.07 |
|  | GarageExit | 61.18 | 69.14 | 55.87 | 57.23 | 69.70 | 47.93 | 52.19 | 50.73 | 38.48 | 23.11 | 49.77 |
| class C | ShowGirl2Teaser | 65.86 | 72.89 | 67.10 | 54.01 | 69.28 | 45.52 | 49.68 | 48.33 | 39.61 | 22.89 | 52.89 |
| class D | StEM_MagicHour cut 1 | 77.30 | 84.17 | 69.73 | 54.65 | 69.81 | 40.55 | 45.15 | 42.48 | 36.80 | 25.28 | 52.15 |
|  | StEM_MagicHour cut 2 | 71.69 | 79.52 | 71.28 | 52.07 | 69.88 | 39.28 | 43.82 | 41.21 | 36.21 | 25.07 | 52.67 |
|  | StEM_MagicHour cut 3 | 75.48 | 82.44 | 67.86 | 51.41 | 69.93 | 38.00 | 42.58 | 39.96 | 36.23 | 24.55 | 52.93 |
|  | StEM_WarmNight cut 1 | 75.55 | 83.85 | 69.85 | 53.16 | 69.93 | 38.96 | 43.57 | 40.95 | 36.86 | 25.38 | 52.93 |
|  | StEM_WarmNight cut 2 | 70.26 | 78.05 | 63.85 | 54.79 | 70.17 | 39.65 | 44.28 | 42.01 | 37.24 | 23.69 | 52.81 |
| class G | BalloonFestival | 59.88 | 66.38 | 54.26 | 54.72 | 69.61 | 46.01 | 50.21 | 52.52 | 41.14 | 21.23 | 48.24 |
| class H | EBU_04_Hurdles | 51.88 | 60.02 | 54.13 | 54.96 | 69.59 | 47.49 | 51.52 | 49.83 | 37.22 | 22.55 | 47.34 |
|  | EBU_06_Start | 52.40 | 60.06 | 54.23 | 51.85 | 69.60 | 44.72 | 48.71 | 48.16 | 36.42 | 21.58 | 47.71 |
|  | Overall | 63.61 | 71.46 | 61.75 | 54.50 | 69.76 | 43.76 | 48.11 | 46.71 | 38.06 | 23.41 | 51.08 |

- Conclusion

From tables 1 through 4 it is asserted that the FastVDO 9 tap downsampling/ 4 tap upsampling pair outperforms the other filters tabulated above. In fact, the FastVDO filters also outperform the various adaptive filters mentioned in [6] for the common test (CTC) sequences, as can be seen by comparing our results with those in [6]. And by contrast, the adaptive filters in [6] are significantly more complex to implement than our simple fixed filters, due to the need to perform iterative testing to find an optimal solution. Our simple assertion is that among the filters tested, FastVDO filters are the highest performing resampling filters, and merit being recommended for best practice use with respect to HDR coding, for example in HDR10 applications.

- References

[1] 	J. Strom, J. Sole, Y. He, “Report on HDR CE1”, JCTVC document JCTVC-W0021, Vancouver, CA, Jan. 2016

[2] Pankaj Topiwala, Wei Dai and Madhu Krishnan “HDR CE5: Report of Experiment 5.3.2”, JCTVC-W0055, San Diego, Feb., 2016.

[3] Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Resampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

[4] HDRTools-0.10

[5] JCTVC-W1020, 19-26 February 2016, Common Test Conditions for HDR/WCG video coding experiments.

[6] Alexis Michael Tourapis, Yeping Su, David Singer, “Enhanced Filtering and Interpolation Methods for Video Signals”, JCTVC-W0051, San Diego, USA, 19–26 February 2016.

# Patent rights declaration(s)

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).