| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0049 |
| --- | --- |

| Title: | Conversion between PQ and Hybrid Log-Gamma (HLG) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories Inc. |  |  |

_____________________________

# Abstract

This information input document reports testing methodology and results of various conversion processes between PQ and HLG signals. In the primary case of study,  display-referred video graded by a human professional in a BT.HDR PQ (BT.2020 primaries) container is later automatically converted to BT.HDR Hybrid Log-Gamma (also BT.2020 primaries) by two different processes: (1) conversion to PQ by the BT.HDR Annex 2 method that “stretches” the real-value video signal [0.0, 1.0] by system gamma display peak luminance (Lw) computed in BT.HDR Note 5e to map peak real-value level 1.0 to the PQ source mastering display maximum luminance level (2) automatic re-grading the PQ source signal to a new display target of approximately 1000 nits, such that pixels substantially above 1000 nits are rolled by a tone curve into the upper range of the output signal with a system gamma that assumes a peak (Lw) around 1000 nits. If suitable monitors are available at the Geneva meeting, viewing will be offered and results will be included in an update of this document.

# Introduction

The BT.HDR [1][2] reference conversion process between PQ and HLG signals assumes that the intensity range of the input PQ signal is to be preserved in the HLG output signal. However, in the majority of modern studio environments, HDR signals are monitored on displays peaking at 1000 nits or less such as the Sony BVM-X300 OLED professional reference monitor which rely on either pre-grading process to map pixels outside its display range to its rendering range, or its externally programmable look up tables (LUT) to map the pixels. For example, the LUT can map [0.00005, 10000.0 ] nits range of PQ to the [0.0001, 1000.0 ] nits rendering range.  A more common source range for modern HDR/WCG mastering would be [0.001, 4000.0] nits within P3 color primaries.

## Reference mapping between PQ and HLG

The BT.HDR [1] reference conversion process consist of conversion of the non-linear PQ signal to a universal intermediate linear display light signal that is then remapped to the nonlinear HLG signal:

The above steps include:

- PQ EOTF

- HLG Inverse EOTF

The first (1) PQ EOTF step in Table 4 [1] is a direct map between 10-bit or 12-bit integer code levels to display intensity values (nits or cd/m2), nominally represneted in processing by 16-bit half-float or 32-bit floating point data types.

The second step, “HLG Inverse EOTF” in the figure is not directly defined in BT.HDR, but is implied to be the inverse of the “HLG Reference EOTF” in Table 5 [1].

Table 1 HLG Reference EOTF (from BT.HDR [1] Table 5)

| HLG Reference EOTF | FD=OOTFE=OOTFOETF−1E' Thus, RD=αYSγ−1RS+βGD=αYSγ−1GS+βBD=αYSγ−1BS+β where: Rs, Gs, Bs are the scene linear light signals, E, for each colour component normalised in the range [0:12]5c. E=OETF−1E'=4E0≤E'≤12expE'−ca+b12<E'YS=0.2627RS+0.6780GS+0.0593BSα=LW−LB12γβ=LB and: FD is the luminance of a displayed linear component {RD, GD, or BD}, in cd/m2. E´ is the non-linear signal {R’,G’,B’} as defined for the OETF. RD, GD, BD are the displayed light for each colour component5d, in cd/m2. The values of parameters a, b, and c are as defined for the OETF. The OOTF is defined below.  = 1.2 at the nominal display peak luminance of 1000 cd/m2. 5e, 5f LW is nominal peak luminance of the display in cd/m2. LB is the display luminance for black in cd/m2. The nominal signal range of E, Rs, Gs, Bs, and YS is [0:12]. 5g The reference display shall not display values greater than E' = 1.0. Such values should be clipped to 1.0 prior to display5h. |
| --- | --- |

Table 2  Notes accompanying BT.HDR[1] Table 5

Note 5c: If E is normalised to the range [0:1] then the equivalent equation for the E is: E=OETF−1E'=E/30≤E'≤12expE'−ca+b12<E' where a, b, and c are as defined in Note 5b.

Note 5d: This EOTF applies gamma to the luminance component of the signal, whereas some legacy displays may apply gamma separately to colour components. Such legacy displays approximate this reference OOTF.

Note 5e: For displays with nominal peak luminance (LW) greater than 1000 cd/m2, or where the effective nominal peak luminance is reduced through the use of a contrast control, the system gamma value should be adjusted according to the formula below, and may be rounded to three significant digits: γ=1.2+0.42Log10LW/1000

Note 5f: The system gamma value may be decreased for brighter background and surround conditions.

Note 5g: When E is normalised to the range [0:1] then the equation for α is: α=LW−LB

Example roll off method (2) from 4000 nits PQ to Lw = 1000 nits (10-bit narrow-range code 940) HLG with super whites to 1200 nits (10-bit narrow-range code 1019)

# References

[1] Draft New Recommendation ITU-R BT.[HDR-TV]: Image parameter values for high dynamic range television for use in production and internal programme exchange. [ also included in liaison statement to MPEG: m37788]

[2] BT.2390. High Dynamic Range television for production and international programme exchange