| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0050r1 |
| --- | --- |

| Title: | AHG13: ICTCP Compression Using HEVC Main 10 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Tao Chen, Walt Husak 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

ICTCP colour representation is specified in the ITU-R Draft New Recommendation BT.[HDR-TV]. This contribution presents the compression results of ICTCP PQ signal using common test conditions for HDR/WCG video coding experiments. The simulation uses the Anchor 3.2 HM software but with different configuration parameters which fit ICTCP colour representation. The simulation results show that ICTCP signal can be compressed well using HEVC Main 10 codec.

# Introduction

ICTCP colour representation is specified in the ITU-R Draft New Recommendation BT.[HDR-TV] [1]. An overview of ICTCP is given in [2], [3] and [7]. Compared to NCL Y'CbCr PQ, ICTCP PQ offers the benefit of constant luminance in that the chroma channels are lacking luminance information. This reduces errors that can be introduced when spatially sub-sampling the chroma components compared to conventional non-constant luminance encoding. In addition, ICTCP has advantage that the lines of constant hue are straighter, and the MacAdam's ellipses are more circular. For baseband signal, ICTCP significantly decreases visible distortions for worst case scenario thus enabling excellent colour performance with 10-bit encoding. When measuring the worst case visual colour difference quantization error between chroma channel code values (using ∆E2000) at various luminance levels, it was shown (in [2] Fig. 4) that 10-bit ICTCP provides an approximately 1.5 bit colour difference improvement over 10-bit NCL Y'CbCr.

In this contribution, we present the compression results using HEVC Main 10 codec. The simulation shows that ICTCP signal can be compressed well using HEVC Main 10 codec.

# Proposed setting in Anchor 3.2 HM software

Anchor 3.2 HM encoding is performed with two changes compared to HM16.7 [4]:

- A specific chroma QP offset process is applied to better control the chroma shifting;

- A specific control of the luma delta QP is applied to better balance the bitrate between dark and bright areas.

We configured our setting for ICTCP PQ to follow the bitrate allocation behavior of Y'CbCr PQ. Therefore, we can use the same QP as the anchor. Compared to Y'CbCr PQ 10bit signal, ICTCP PQ 10bit signal has similar variance level in luma component but higher variance in chroma components [5]. In our simulation, we keep the setting for luma delta QP untouched and only modify the configuration parameters for chroma QP offset.

The chroma QP Offset in Anchor 3.2 is computed as in Equ. 1, where the baseQP is the QP value set to compress the luma component.

(Equ. 1)

For HDR10 signal, the configuration parameters are specified in Table 1, where different parameters are set for P3 content and 709 content.

Table 1 Chroma QP offset parameters for Y'CbCr PQ signal

| Type | QpScale | QpOffset | CbQpScale | CrQpScale | CbQPoffset | CrQPoffset |
| --- | --- | --- | --- | --- | --- | --- |
| 709 | -0.46 | 9.26 | 1.14 | 1.79 | 0 | 0 |
| P3 | -0.46 | 9.26 | 1.04 | 1.39 | 0 | 0 |

Since ICTCP 10bit signal has higher energy in chroma components, the above parameters are adjusted for CTCP Components in order to reduce chroma bitrate so as to match the Y'CbCr PQ chroma bitrate given the same quantization step. The configuration parameters are for CTCP Components are specified in Table 2.

Table 2 Chroma QP offset parameters for ICTCP PQ signal

| Type | QpScale | QpOffset | CbQpScale | CrQpScale | CbQPoffset | CrQPoffset |
| --- | --- | --- | --- | --- | --- | --- |
| 709 | -0.46 | 10 | 0.9 | 1.6 | 6 | 6 |
| P3 | -0.46 | 10 | 0.5 | 0.9 | 8 | 7 |

# Simulation Results

The conversion of ICTCP PQ signal uses HDRTools [6]. Compared to Y'CbCr PQ, the modification of configuration parameters are listed in Table 3. It is noted that ClosedLoopConversion is set to 0 because ICTCP signal offers the benefit of constant luminance already. HM Anchor v3.2 software is used for compression test. The configuration parameter for luma delta QP is unchanged. The configuration parameters for chroma QP Offset are modified according to Table 2. Since VUI is enabled in Anchor v3.2, the corresponding setting for MatrixCoefficients is shown in Table 4 for correct signaling of ICTCP. All other settings follow CTC for HDR/WCG video coding [4].

Table 3 Configuration parameters for conversion in HDRTools

|  | Y'CbCr PQ | ICTCP PQ |
| --- | --- | --- |
| ColorSpace | 1 | 10 |
| ColorPrimaries | 0 | 9 |
| ClosedLoopConversion | 5 | 0 |

Table 4 VUI parameters in HM encoding

|  | Y'CbCr PQ | ICTCP PQ |
| --- | --- | --- |
| MatrixCoefficients | 9 | 14 |

Table 5 shows compression results compared to Anchor 3.2. It has been checked subjectively that with the proposed setting, ICTCP signal has comparable coding performance as the anchor.

Table 5 Compression results (BD rates) of ICTCP compared to Y'CbCr PQ

# Conclusion

This contribution justifies the compression performance of HEVC Main10 Profile codec in encoding ICTCP signal. In the experiments, the encoder settings are derived based on the rate allocation behavior in encoding Y’CbCr PQ signal, which serves as the benchmark in this evaluation. Without further optimization, the coding efficiency of encoding ICTCP signal is, comparable to that of encoding Y’CbCr PQ signal. The simulation results have proven that ICTCP signal can compress as well.

# Reference

- Rec. ITU-R BT.[HDR-TV], Image parameter values for high dynamic range television for use in production and international programme exchange.

- J. Pytlarz, “Overview of IPT-PQ Color Space”, JCTVC-W0050, March 2016.

- Report ITU-R BT. 2390-0, High dynamic range television for production and international programme exchange, 2016.

- “Common Test Conditions for HDR/WCG video coding experiments”, JCTVC-W1020, March 2016.

- J. Pytlarz, R., T. Lu, F. Pu, P. Yin, T. Chen and W. Husak, “Overview of IPT-PQ Color Space”, MPEG contribution, doc. m37266, Oct. 2015.

- HDRTools: https://gitlab.com/standards/HDRTools/

- ICTCP  white paper: http://www.dolby.com/us/en/technologies/dolby-vision/ICtCp-white-paper.pdf

# Acknowledge

We would like to thank InterDigital for helping to crosscheck this proposal.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).