| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0054 |
| --- | --- |

| Title: | AHG13: Further results for LUT-based luma sample adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V. Kolesnikov Building A, The Park Estate 5 Talavera Road Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia |  |  |

_____________________________

# Abstract

The ‘Common test conditions for HDR/WCG video coding experiments’ defines an HDR anchor that includes a method known as ‘luma sample adjustment’. This method compensates for a shift in luminance that occurs e.g. when downsampling from 4:4:4 to 4:2:0 using non-constant luminance and YCbCr. The method performs an iterative search to determine each luma sample values, which is asserted to be overly complex for real time implementations. This contribution shows a method for performing ‘luma sample adjustment’ that replaces an iterative search with a LUT-based approximation whereby a second-order model is applied to predict the adjusted luma sample, with the coefficients for the second-order model obtained from the LUT. The LUT generation process has been updated compared to the process used for the contribution JCTVC-W0056. With the updated LUT contents, tPSNR is reported as 53.53dB, 66.31dB and 43.07dB. Also shown are results of an alternative to the linearization method presented in JCTVC-W0107, with the following tPSNR results of 53.80dB, 69.70dB, and 43.08dB in X, Y, and Z components, respectively.

# Introduction

When generating Y’CbCr data from R’G’B’ using non-constant luminance and downsampling to 4:2:0, luminance after conversion back to RGB 4:4:4 is distorted compared to the input RGB luminance. This problem is described in [2].

A luma sample adjustment process, described by Ericsson in [3], was proposed to compensate for distortions introduced by these stages. Figure 1 shows how this process operates in the context of encoder-side processing, as used in the MPEG HDR/WCG CfE.

Figure 1. Luma sample adjustment process.

Within the ‘luma adjustment’ block above the following steps are performed:

- Youtput is initialised from Y’initial.

- Compute R’G’B’ from Y’outputCbCr.

- Convert R’G’B’ to RGB.

- Compute Ylinear from RGB.

- Compare Ylinear to Ytarget and adjust Youtput according to a binary search.

- While Youtput differs from Ytarget repeat steps 2-5.

- Compute and output Y’ from Youtput.

Contribution JCTVC-W0056 [4] presented an alternative method whereby a second-order model was used to derive the luma value Y from the input YlinearCbCr. The LUT contents were derived offline and involve decomposing the YlinearCbCr into an octant hierarchy to select a particular set of coefficients for the second-order model.

The LUT generation process has since been revised to produce higher tPSNR values compared to those reported in [4].

An alternative method of producing a linear approximation of the above iterative method is presented in section 4.

# Method description

Note: This section is a recap of the second-order method, as presented in [4].

The iterative method is replaced with a LUT-based method. A LUT containing multiple coefficient sets is used. A coefficient set contains coefficients needed to map Ytarget, Cb and Cr to Y’ using a second-order model, as follows:

Y’ = m0Y’target + m1Y’target2 + m2Cb + m3Cb2 + m4Cr + m5Cr2 + m6. (Eqn 1)

Where Y’target is the PQ encoding of Ytarget, quantised to 10 bits, and Cb and Cr are expressed in the native bit depth of the system, i.e. 10 bits.

First, an octant location LUT (yadjust_search_table) is used to locate a containing octant within a pre-defined octant hierarchy, using the most significant bits (MSBs) of the input Y’target, Cb and Cr values. After this a coefficient set LUT (yadjust_coeff_table) is used to obtain a coefficient set, i.e. seven predetermined coefficients (as implied by the use of a second order model) associated with a given octant.

Runtime complexity of calculation of Y' using a model is: 9 multiplications, 6 additions and 1 transfer function calculation (or 1 lookup if transfer function LUT is in use).

The octant hierarchy and associated coefficient sets are determined offline using a recursive search method described in the next section.

# Revised LUT generation process

The octant hierarchy and associated coefficient sets are determined offline using a two-stage algorithm:

- An octant hierarchy and associated approximation coefficient sets for each octant in the hierarchy are determined using recursive search (this stage only was performed in [4]).

- Then, for each octant, a refinement pass is performed for the coefficients resulting from the first stage.

## First-stage search

The first-stage search starts at the largest possible octant (for 10-bit Cb and Cr components and 10-bit PQ quantization of luminance this is a 1024x1024x1024 octant).

At a given octant, a set of sample points covering the entire octant is derived. Compared to [4], a sampling algorithm bug was fixed. The bug led to non-uniform distribution of sampling points within the octant.

Then, a curve fitting algorithm over three independent variables (Y'target, Cb, Cr) is applied to the set of sample points to derive a set of coefficient values for a 2nd order polynomial model associated with the octant. Of the tested sample points, the algorithm uses the maximum sample error as the error metric.  Sample error is computed as the difference between the Y' value derived using the iterative algorithm [3] and the Y' value calculated using the model.

The output of the algorithm is a set of seven coefficient values and an associated error metric.

If the error metric exceeds a predetermined threshold, the octant is split into eight equally sized octants and the search algorithm is applied to the resulting octants recursively until a predetermined minimum octant size of 2x2x2 is reached. Otherwise, if the maximum error value does not exceed the predetermined threshold, the search progresses to the next octant.

The presence of a minimum octant size constraint limits the complexity of the logic required to select a particular model for the Y'targetCbCr sample triplet.

The first stage algorithm completes after performing search over the space of the initial 1024x1024x1024 octant.

## Second-stage refinement pass

The second-stage coefficient refinement pass iterates over the octant hierarchy determined by the first-stage search and refines model coefficients using a minimization algorithm and negated tPSNR objective quality metric as cost estimate. The algorithm operates as follows:

At an initialization step, one original EXR frame and one YCbCr 4:2:0 frame of each test sequence are loaded to be used for tPSNR metric calculation.

Then, for each octant, a minimization algorithm is applied to the coefficients. At each iteration of the minimization algorithm, the cost estimate for given coefficients is calculated as follows:

- the coefficients are applied to all pre-loaded YCbCr 4:2:0 data,

- YCbCr to EXR conversion is performed, and

- tPSNR metric is calculated between the original EXR data and the EXR data from the previous step.

# Modified linearization method

Contribution JCTVC-W0107 [5] presented a method for deriving the luma sample value using a linear approximation.  The linear approximation was produced by testing the Y’ value and finding the derivative at this point too. Then, solving this linear approximation produced an adjusted Y’ value to be used.

This method provides a fixed complexity means to perform luma sample adjustment, however the linear approximation can give poor performance when the function being approximated has non-linear segments, i.e. at so-called ‘saturation points’ (where the transfer function saturates).

An alternative method of producing the linear approximation is to test two points, located at some offset either side of the derived Y’ value.  The tested implementation here tests two points located at luminances corresponding to one codeword to either side of the Y’ value. Other close values were also tested, producing results substantially similar to the one codeword case.

As seen in the results section, the performance of this method under the no-compression simulation is 0.07dB less than the iterative method for tPSNR_Y. The worst case loss was 0.22dB for tPSNR_Y, seen in BikeSparklers_cut1. No loss was seen in t_PSNR_X/Z.

# Results

Table 1 and Table 2 present results for the second-order model with LUT programmed for 10K cd/m2 case, PQ transfer function, using the two stage LUT generation process, with PQ implemented as a function and a look-up table, respectively. Note that these results were produced using a development version of HDRTools-0.11.

Table 3 presents results for the modified linearization method.

Table 1. Results for the second-order model with LUT programmed for 10K cd/m2 case, function implementation of PQ.

|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | Time (Sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 61.45 | 65.73 | 52.27 | 56.38 | 54.45 | 48.91 | 24.73 | 56.41 | 885.68 |
| Market3Clip4000r2 | 55.46 | 66.86 | 45.94 | 50.22 | 49.72 | 36.78 | 21.87 | 46.83 | 1753.72 |
| SunRise | 59.30 | 66.98 | 50.87 | 54.96 | 54.22 | 37.87 | 24.22 | 46.88 | 859.99 |
| BikeSparklers cut 1 | 47.52 | 66.91 | 37.04 | 41.43 | 39.81 | 34.19 | 21.82 | 50.30 | 764.25 |
| BikeSparklers cut 2 | 48.59 | 67.01 | 38.42 | 42.78 | 41.60 | 34.52 | 21.77 | 49.96 | 952.14 |
| GarageExit | 56.18 | 66.73 | 47.06 | 51.29 | 49.87 | 38.09 | 22.85 | 48.58 | 2086.96 |
| ShowGirl2Teaser | 53.13 | 64.75 | 44.89 | 48.97 | 47.57 | 39.28 | 22.51 | 51.54 | 2226.23 |
| StEM_MagicHour cut 1 | 53.91 | 66.53 | 39.98 | 44.57 | 42.01 | 36.57 | 24.96 | 50.90 | 592.08 |
| StEM_MagicHour cut 2 | 51.42 | 65.88 | 38.72 | 43.25 | 40.78 | 36.00 | 24.71 | 51.23 | 1398.99 |
| StEM_MagicHour cut 3 | 50.83 | 65.34 | 37.51 | 42.07 | 39.63 | 36.07 | 24.28 | 51.50 | 606.73 |
| StEM_WarmNight cut 1 | 52.45 | 66.00 | 38.47 | 43.06 | 40.63 | 36.67 | 25.11 | 51.70 | 1436.69 |
| StEM_WarmNight cut 2 | 53.95 | 65.59 | 39.15 | 43.77 | 41.68 | 37.05 | 23.35 | 51.40 | 1137.02 |
| BalloonFestival | 53.60 | 66.56 | 45.05 | 49.22 | 51.41 | 40.65 | 21.15 | 47.06 | 1029.36 |
| EBU_04_Start | 54.05 | 66.95 | 46.65 | 50.66 | 49.04 | 36.86 | 22.23 | 46.28 | 2189.92 |
| EBU_06_Hurdles | 51.18 | 66.79 | 44.06 | 48.04 | 47.61 | 36.11 | 21.28 | 46.57 | 2159.93 |
| Average | 53.53 | 66.31 | 43.07 | 47.38 | 46.00 | 37.71 | 23.12 | 49.81 | 1338.646 |

Table 2. Results for the second-order model with LUT programmed for 10K cd/m2 case, look-up table implementation of PQ.

|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | Time (Sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 61.45 | 65.73 | 52.27 | 56.38 | 54.45 | 48.91 | 24.73 | 56.41 | 447.88 |
| Market3Clip4000r2 | 55.46 | 66.86 | 45.94 | 50.22 | 49.72 | 36.78 | 21.87 | 46.83 | 979.26 |
| SunRise | 59.30 | 66.98 | 50.87 | 54.96 | 54.22 | 37.87 | 24.22 | 46.88 | 473.62 |
| BikeSparklers cut 1 | 47.52 | 66.91 | 37.04 | 41.43 | 39.81 | 34.19 | 21.82 | 50.30 | 350.9 |
| BikeSparklers cut 2 | 48.59 | 67.01 | 38.42 | 42.78 | 41.60 | 34.52 | 21.77 | 49.96 | 407.85 |
| GarageExit | 56.18 | 66.73 | 47.06 | 51.29 | 49.87 | 38.09 | 22.85 | 48.58 | 950.72 |
| ShowGirl2Teaser | 53.13 | 64.75 | 44.89 | 48.97 | 47.57 | 39.28 | 22.51 | 51.54 | 1046.61 |
| StEM_MagicHour cut 1 | 53.91 | 66.53 | 39.98 | 44.57 | 42.01 | 36.57 | 24.96 | 50.90 | 286.31 |
| StEM_MagicHour cut 2 | 51.42 | 65.88 | 38.72 | 43.25 | 40.78 | 36.00 | 24.71 | 51.23 | 667.26 |
| StEM_MagicHour cut 3 | 50.83 | 65.34 | 37.51 | 42.07 | 39.63 | 36.07 | 24.28 | 51.50 | 292.71 |
| StEM_WarmNight cut 1 | 52.45 | 66.00 | 38.47 | 43.06 | 40.63 | 36.67 | 25.11 | 51.70 | 693.53 |
| StEM_WarmNight cut 2 | 53.95 | 65.59 | 39.15 | 43.77 | 41.68 | 37.05 | 23.35 | 51.40 | 553.86 |
| BalloonFestival | 53.60 | 66.56 | 45.05 | 49.22 | 51.41 | 40.65 | 21.15 | 47.06 | 575.84 |
| EBU_04_Start | 54.05 | 66.95 | 46.65 | 50.66 | 49.04 | 36.86 | 22.23 | 46.28 | 1212.48 |
| EBU_06_Hurdles | 51.18 | 66.79 | 44.06 | 48.04 | 47.61 | 36.11 | 21.28 | 46.57 | 1179.88 |
| Average | 53.53 | 66.31 | 43.07 | 47.38 | 46.00 | 37.71 | 23.12 | 49.81 | 674.5807 |

Table 3. Results for the modified linearisation method.

|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | Time (Sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 63.17 | 70.18 | 52.24 | 56.60 | 55.01 | 49.15 | 24.72 | 58.36 | 740.39 |
| Market3Clip4000r2 | 55.64 | 69.53 | 45.96 | 50.27 | 49.89 | 36.79 | 21.87 | 47.88 | 1550.53 |
| SunRise | 59.69 | 69.59 | 50.91 | 55.09 | 54.57 | 37.88 | 24.22 | 47.93 | 802.48 |
| BikeSparklers cut 1 | 47.56 | 69.56 | 37.03 | 41.43 | 39.83 | 34.19 | 21.82 | 51.44 | 521.94 |
| BikeSparklers cut 2 | 48.64 | 69.56 | 38.41 | 42.78 | 41.63 | 34.52 | 21.77 | 51.04 | 589.59 |
| GarageExit | 56.38 | 69.73 | 47.09 | 51.35 | 50.05 | 38.10 | 22.85 | 49.78 | 1408.98 |
| ShowGirl2Teaser | 53.45 | 69.19 | 44.96 | 49.12 | 47.85 | 39.29 | 22.50 | 52.88 | 1520.7 |
| StEM_MagicHour cut 1 | 54.05 | 69.82 | 39.98 | 44.58 | 42.05 | 36.57 | 24.96 | 52.16 | 419.97 |
| StEM_MagicHour cut 2 | 51.53 | 69.87 | 38.72 | 43.27 | 40.81 | 36.00 | 24.71 | 52.68 | 954.08 |
| StEM_MagicHour cut 3 | 50.94 | 69.85 | 37.51 | 42.09 | 39.66 | 36.07 | 24.28 | 52.93 | 427.02 |
| StEM_WarmNight cut 1 | 52.62 | 69.85 | 38.47 | 43.07 | 40.65 | 36.67 | 25.11 | 52.93 | 989.74 |
| StEM_WarmNight cut 2 | 54.22 | 70.15 | 39.14 | 43.78 | 41.72 | 37.06 | 23.35 | 52.81 | 806.5 |
| BalloonFestival | 53.76 | 69.55 | 45.05 | 49.26 | 51.72 | 40.68 | 21.15 | 48.23 | 923.03 |
| EBU_04_Start | 54.17 | 69.56 | 46.67 | 50.71 | 49.18 | 36.87 | 22.23 | 47.34 | 1994.87 |
| EBU_06_Hurdles | 51.25 | 69.47 | 44.07 | 48.07 | 47.75 | 36.12 | 21.28 | 47.70 | 1923.02 |
| Average | 53.80 | 69.70 | 43.08 | 47.43 | 46.16 | 37.73 | 23.12 | 51.07 | 1038.189 |

# References

- E. François, P. Yin, J. Sole, “Common Test Conditions for HDR/WCG Video Coding Experiments”, Moving Pictures Experts Group, w15793, Geneva, October, 2015.

- J. Strom, “Investigation of HDR Color Subsampling”, Moving Pictures Experts Group, m35841, Geneva, February, 2015.

- J. Strom, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson’s response to CfE for HDR and WCG”, Moving Pictures Experts Group, m36184, Warsaw, June, 2015.

- C. Rosewarne, V. Kolesnikov, ”CE1-related: LUT-based luma sample adjustment”, Joint Collaborative Team on Video Coding (JCT-VC), JCTVC-W0056, San Diego, USA, February 2016.

- A. Norkin, “Closed form HDR 4:2:0 chroma subsampling (HDR CE1 and AHG5 related)”, ”, Joint Collaborative Team on Video Coding (JCT-VC), JCTVC-W0107, San Diego, USA, February 2016.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).