| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0055 |
| --- | --- |

| Title: | AHG14: On Test Plan of SHVC Main 10 for HDR/WCG Backward Compatibility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Walt Husak, Fangjun Pu, Taoran Lu, Peng Yin, Tao Chen, 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution presents multiple suggestions for SHVC Main 10 HDR/WCG backward compatibility verification test. In JCTVC reflector, Qualcomm has recently shared the updated software SHM11.0 and the test clips with their corresponding rates. It is observed that color contour artifact exists in the highest rate point selected as near transparent quality for multiple clips. The test clips in current test plan consist of four BT.709 clips and only one P3 clip. It is not clear how test condition and bitrate is defined. The contribution has the following suggestions: 1) fix the color contour artifact in near transparent rate point; 2) increase number of P3 content; 3) define test condition to be better aligned with industry practice.

# Introduction

In JCTVC reflector, Qualcomm has proposed the initial test plan for SHVC Main 10 verification test for HDR/WCG backward compatibility. The test plan includes the updated software of SHM11.0 which incorporates changes from HDR Anchor 3.2 [4]. The test plan also suggested some test clips and test conditions.

This contribution presented several suggestions about test plan on the following aspects: 1) fix color contour artifact in highest bitrate; 2) select more P3 content; 3) define test condition to better align with industry practice.

# Suggestions

SHVC Main 10 Profile was finalized in 2014 and HDR/WCG backward compatible scalability was not officially tested at that time. In [1], it was pointed out that color contour artifact can be observed at the highest bitrate where other types of the compression artifact are not visible using SHM7.0. In [2][3], several solutions have been proposed and tried to fix the artifact. In the latest updated SHM11.0 software in JCTVC reflector, it was found that the similar artifact can be still observed at highest bitrate for several clips, such as ShowGirl, AutoWelding, FireEater and MagicHour. Since it is difficult to view color artifacts on the SDR displays such as a regular PC/laptop monitor, we took a few shots of the display in our lab to illustrate how the color artifacts look like, as shown in Figure 1. The actual severeness of these artifacts on the real HDR/HDR display is more than the SDR redening can illustrate. It is highly recommended to fix the colour contour artifact especially for highest bitrate which supposes to have near transparent quality.

Figure 1 Examples of color artifacts observed in reconstructed HDR images with SHM encoding

The selected test clips have more BT.709 source content than P3 source content. Since nearly every major display manufacture has demoed models capable of DCI-P3 displays, it is suggested to increase the number of P3 content for testing. Table 1 shows the distributed content over the last two years that is part of the HDR/WCG pool of content and half of the clips are P3 content. It is also noted that for some BT. 709 clips, there are two versions of SDR content available: auto-mapped version and color graded version. The AHG needs to decide which version to be used for SDR.

Table 1 HDR/WCG Content

| seq | gamut | TF | Sequence name |
| --- | --- | --- | --- |
| S00 | BT.709 | Linear | FireEater2Clip4000r1_1920x1080p_25_hf_709_ct2020_444_xxx.exr |
| S01 | BT.709 | Linear | Tibul2Clip4000r1_1920x1080p_30_hf_709_ct2020_444_xxx.exr |
| S02 | BT.709 | Linear | Market3Clip4000r2_1920x1080p_50_hf_709_ct2020_444_xxx.exr |
| S03 | P3D65 | PQ12b | AutoWeldingClip4000_1920x1080p_24_12_P3_ct2020_xxx.tif |
| S04 | P3D65 | PQ12b | BikeSparklersClip4000_1920x1080p_24_12_P3_ct2020_xxx.tif |
| S05 | P3D65 | PQ12b | ShowGirl2TeaserClip4000_1920x1080p_25_12_P3_ct2020_xxx.tif |
| S06 | P3D65 | PQ12b | StEM_MagicHour_1920x1080p_24_12_P3_xxx.tif |
| S07 | P3D65 | PQ12b | StEM_WarmNight_1920x1080p_24_12_P3_xxx.tif |
| S08 | BT.709 | Linear | BalloonFestival_1920x1080p_24_hf_709_ct2020_444_xxx.exr |
| S09 | BT.709 | Linear | Walkpath_1920x1080p_24_hf_709_ct2020_444_xxx.exr |
| S10 | BT.709 | Linear | EBU_04_Hurdles_1920x1080p_25_hf_709_ct2020_444_xxx.exr |
| S11 | BT.709 | Linear | EBU_06_Starting_1920x1080p_25_hf_709_ct2020_444_xxx.exr |
| S12 | BT.709 | Linear | Sunrise_1920x1080p_25_hf_709_ct2020_444_xxx.exr |
| S13 | P3D65 | PQ12b | GarageExit_1920x1080p_24_12_P3_ct2020_xxx.tif |
| S14 | P3D65 | PQ12b | Telescope Interior _1920x1080p_24_12_P3_ct2020_xxx.tif |
| S15 | P3D65 | PQ12b | Telescope Nebula_1920x1080p_24_12_P3_ct2020_xxx.tif |

For test conditions, there are several questions to be answered: 1) compare against Simulcast or HDR single layer? 2) If single layer, there are two ways to verify the test: show same quality but what additional rate overhead needed; or show fixed overhead and compare quality? Which one is better to be used? How can we align with industrial practice?

# Conclusion

The contribution discussed several suggestions for SHVC Main 10 verification test for HDR/WCG backward compatibility. It recommends to: 1) fix the color contour artifact in near transparent rate point; 2) increase number of P3 content; 3) define test condition and bitrate to be better aligned with industry practice.

# Reference

- T. Lu, F. Pu, P. Yin, T. Chen, W. Husak, “Compression results of SHM 7.0 for SDR backward compatibility using color graded SDR and HDR/WCG content”, doc. m35941, Geneva Switzerland, Feb 2015.

- A. Ramasubramonian, J. Sole, M. Karczewicz, S. Lee, D. Rusanovskyy, D. Bugdayci, X. Li, “Dual layer non-normative (category 3b) response to the Call for Evidence on HDR/WCG ”, doc. m36280, Geneva Switzerland, June 2015.

- P. Bordes, C. Ying, F. Hiron and E. François, “SHVC encoder improvement ”, doc. m36376, Geneva Switzerland, June 2015.

- “Common Test Conditions for HDR/WCG video coding experiments”, JCTVC-W1020, March 2016.