| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 25 May – 1 June 2016 | Document: JCTVC-X0062 |
| --- | --- |

| Title: | Regional nesting SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Joel Sole Ye-Kui Wang Dmytro Rusanovskyy Done Bugdayci Sansli Marta Kacrzewicz Pierre Andrivon Edouard Francois Wiebe de Haan Robert Brondijk | Tel: Email: | aramasub@qti.qualcomm.com Pierre.Andrivon@technicolor.com wiebe.de.haan@philips.com |
| Source: | Qualcomm, Technicolor, Philips |  |  |

_____________________________

# Abstract

This document proposes a regional nesting SEI message that specifies rectangular regions to which one or more SEI messages would be applicable. The design is similar to the scalable nesting SEI message; a set of rectangular regions is specified and a set of SEI messages associated with all the regions is also specified. It is suggested to keep the design flexible to allow existing SEI messages to be nested, with the option of adding syntax elements that may be beneficial for some applications. The proposed SEI provides carriage means for SMPTE ST 2094-1 and is a mechanism for carrying SMPTE 2094-30 by nesting the CRI SEI.

In the r1 version of this document, an option to normatively specify certain syntax elements is added using the rns_id syntax element. It is suggested that approach to address this aspect be discussed in the meeting.

# Introduction

The HEVC Specification includes several supplemental enhancement information (SEI) messages that are not necessary (except for those related to conformance) for decoding of coded pictures, but provide additional information that may be beneficial for decoder-side entities to process the decoded picture. There are nesting SEI messages, in particular scalable nesting SEI message, that includes one or more SEI messages and indicates that the SEI message(s) is (are) applicable to one or more bitstream subsets/set of layers.

SMPTE ST 2094 standards family describes dynamic metadata that is useful for colour volume transformation of the video content, particularly when the consumer display (the “targeted” display) is expected to be of different luminance, colour volume or transfer characteristics compared to the mastering display used to create the content. Typically, metadata information sent from the encoder may be beneficial for the pictures to be displayed when the targeted display has different characteristics. Several applications have been specified in SMPTE ST 2094-related documents and different carriage of metadata is also specified to support these applications. Although applications in the context of video standards usually refer to video streaming, video broadcasting, video telephony, video conferencing and other such use cases, the word “application” in this document is also used to refer to specific conditions under which some of the colour volume transformations are performed under SMPTE ST 2094. Some such “applications” are as follows - when the content is to be displayed in smaller colour volume or lower luminance renderer, or when the HDR content is to be displayed at an SDR display. It is also anticipated that more such “applications” may be specified.

Common features in the metadata corresponding to SMPTE ST 2094-1 across various applications include targeted display characteristics (colour primaries and white point), persistence and description of regions to which the metadata are applicable. One or more metadata sets is expected to be sent for rectangular regions in these different “applications”, different target displays, etc.

# Problems

- SMPTE ST 2094-1 metadata are associated with regions to which they apply. To support different applications, more than one metadata set may be signalled for the same region. It would be inefficient to signal the regions for each individual metadata set. The number of metadata sets are also expected to be very large. In some extreme examples, it expected that there may be 3 metadata sets for each of 10 targeted displays and each of 15 applications at a given time instance. Signalling the region data for each of these metadata seems unnecessary.

- Several SEI messages are expected to have a region-specific area of application. Not having a common framework to support the different region-specific applications would result in repeated definitions of region-specific syntax elements across SEI messages.

- Existing SEI messages may be re-usable for the applications that need region-specific signalling. For example, the SMPTE ST 2094-30 uses the information that is signalled in the CRI SEI message. Avoiding a redefinition of the CRI SEI message would be beneficial. However, there are certain syntax elements in the SMPTE ST 2094-30 that are not included in the CRI SEI message. An extension to the CRI SEI message that is specific to SMPTE ST 2094 may not be a clean design.

# Proposed regional nesting SEI message

## Syntax structure

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| … |  |
| else if( payloadType = = 149 ) |  |
| regional_nesting( payloadSize ) |  |
| … |  |

| regional_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| rns_id | u(16) |
| rns_num_rect_regions | u(8) |
| for( i = 0; i < rns_num_rect_regions; i++) { |  |
| rns_rect_region_id[ i ] | u(8) |
| rns_rect_left_offset[ i ] | u(16) |
| rns_rect_right_offset[ i ] | u(16) |
| rns_rect_top_offset[ i ] | u(16) |
| rns_rect_bottom_offset[ i ] | u(16) |
| } |  |
| num_seis_in_rns_minus1 | u(8) |
| for( i = 0; i <= num_seis_in_rns_minus1; i++ ) { |  |
| sei_message( ) |  |
| rns_num_data_bytes[ i ] | u(8) |
| for( j = 0; j < rns_num_data_bytes[ i ]; j++ ) |  |
| rns_data_byte[ i ][ j ] | u(8) |
| } |  |
| } |  |

## Semantics

The regional nesting SEI message provides a mechanism to associate SEI messages with regions of the picture, which are specified in the SEI message.

A regional nesting SEI message contains one or more SEI messages. When an SEI message is nested in a regional nesting SEI message, the semantics of the SEI message are to be interpreted as to be independently applicable to each region specified in the SEI message. When a particular sample in the current picture is present in more than one region specified by regional nesting SEI messages associated with the current picture, the SEI message that is applied to the particular sample may be determined by other syntax elements in the SEI message or by means outside of this Specification.

Each nested SEI message has the same persistence scope as if the SEI message was not nested.

It is a requirement of bitstream conformance that the following restrictions apply on nesting of SEI messages in a regional nesting SEI message:

–	An SEI message that has payloadType equal to 0 (buffering period), 1 (picture timing), 2 (pan scan), 3 (filler payload), 4 (user data registered), 5 (user data unregistered), 6 (recovery point), 9 (scene information), 15 (picture snapshot), 16 (progressive refinement segment start), 17 (progressive refinement segment end), 45 (frame packing arrangement), 47 (display orientation), 56 (green metadata), 128 (structure of pictures), 129 (active parameter sets), 130 (decoding unit information), 131 (temporal sub-layer zero index) 132 (decoded picture hash), 133 (scalable nesting), 134 (region refresh), 135 (no display), 136 (time code), 137 (mastering display colour volume), 138 (segmented rectangular frame packing), 139 (temporal motion constrained tile sets), 143 (deinterlaced field), 144 (content light level), 145 (dependent RAP), 146 (coded region), 160 (layers not present), 161 (inter layer constrained tile sets), 162 (bsp nesting), 163 (bsp initial arrival time), 164 (sub bitstream property), 165 (alpha channel info), 166 (overlay info), 167 (temporal mv prediction), 168 (frame field info), 176 (three dimensions reference displays), 177 (depth representation info), 178 (multiview scene), 179 (multiview acquisition), 180 (multiview view position), 181 (alternative depth), 182 (alternative characteristics), and 183 (ambient viewing environment) shall not be nested in a regional nesting SEI message. [Ed. (AR): This currently includes all except film grain characteristics, post filter hint, tone mapping information, resampling filter hint and chroma remapping information SEI messages.]

NOTE – Unlike the scalable nesting SEI message, the SEI messages nested in the regional nesting SEI message should not be extracted and sent as a separate SEI message as the values signalled in the nested SEI message may not be applicable outside the applicable regions.

rns_id contains an identifying number that may be used to identify the purpose of the one or more regional nesting SEI messages . The value of rns_id shall be in the range of 0 to 216 − 2, inclusive.

Values of rns_id from 0 to 255 and from 512 to 215 − 1 may be used as determined by the application. Values of rns_id from 256 to 511 and from 215 to 216 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of rns_id in the range of 256 to 511, inclusive, or in the range of 215 to 216 − 2, inclusive, shall ignore it.

rns_num_rect_regions specifies the number of rectangular regions specified by the regional nesting SEI message. The value of rns_num_rect_regions shall be in the range of 1 to 255, inclusive. The value of rns_num_rect_regions equal to 0 is reserved for future use by ITU-T | ISO/IEC.

rns_rect_region_id[ i ] indicates an identifier for the i-th rectangular region specified in the regional nesting SEI message.

Unless indicated otherwise by some means not specified in this Specification, when a sample belongs to more than one rectangular region, the SEI messages that are associated with the region that has the greatest value of rns_rect_region_id[ ] and includes the sample are applicable to the sample.

NOTE - The rns_rect_region_id[ i ] may be used as a priority number that is used to indicate the metadata parameters corresponding to which region should be applied when more than one region associated with different sets of metadata is applicable for a give sample or region of the picture.

rns_rect_left_offset[ i ], rns_rect_right_offset[ i ], rns_rect_top_offset[ i ] and rns_rect_bottom_offset[ i ] specify the coordinates of the i-th rectangular region specified in the SEI message.  The offsets for the rectangular region are specified in units of luma sample positions. The i-th rectangular region contains the samples with horizontal picture coordinates from rns_rect_left_offset[ i ] to pic_width_in_luma_samples − ( rns_rect_left_offset[ i ] + 1 ), inclusive, and vertical picture coordinates from rns_rect_top_offset[ i ] to pic_height_in_luma_samples − ( rns_rect_bottom_offset[ i ] + 1 ), inclusive. The value of ( rns_rect_left_offset[ i ] + rns_rect_right_offset[ i ] ) shall be less than pic_width_in_luma_samples and the value of ( rns_rect_top_offset[ i ] + rns_rect_bottom_offset[ i ] ) shall be less than pic_height_in_luma_samples.

num_seis_in_rns_minus1 plus 1 specifies the number of nested SEI messages specified in the regional nesting SEI message. The value of num_seis_in_rns_minus1 shall be in the range of 0 to 255, inclusive.

regional_nesting_zero_bit shall be equal to 0.

rns_num_data_bytes[ i ] specifies the number of bytes of data signalled by instances of rns_data_byte[ i ][ j ] immediately after the syntax element rns_num_data_bytes[ i ]. The value of rns_num_data_bytes[ i ] shall be in the range of 0 to 255, inclusive.

rns_data_byte[ i ][ j ] may have any value. Unless specified otherwise by means not in this Specification, decoders shall ignore the value of rns_data_byte[ i ][ j ].

# Example

An application standard such as SMPTE may choose to specify the syntax and semantics of rns_data_byte[ ][ ]. This could be specified based on a particular value of rns_id. In the example below, the value of rns_num_data_bytes[ i ] will be 22.

| regional_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| rns_id | u(16) |
| rns_num_rect_regions | u(8) |
| for( i = 0; i < rns_num_rect_regions; i++) { |  |
| rns_rect_region_id[ i ] | u(8) |
| rns_rect_left_offset[ i ] | u(16) |
| rns_rect_right_offset[ i ] | u(16) |
| rns_rect_top_offset[ i ] | u(16) |
| rns_rect_bottom_offset[ i ] | u(16) |
| } |  |
| num_seis_in_rns_minus1 | u(8) |
| for( i = 0; i <= num_seis_in_rns_minus1; i++ ) { |  |
| sei_message( ) |  |
| rns_num_data_bytes[ i ] | u(8) |
| for( i = 0; i < rns_num_data_bytes[ i ]; i++ ) |  |
| rns_data_byte | u(8) |
| application_identifier[ i ] | u(8) |
| application_version[ i ] | u(8) |
| for( c = 0; c < 4; c++ ) { |  |
| targeted_system_prim_chromaticity_x[ i ][ c ] | u(16) |
| targeted_system_prim_chromaticity_y[ i ][ c ] | u(16) |
| } |  |
| targeted_display_max_luminance[ i ] | u(16) |
| targeted_display_min_luminance[ i ] | u(16) |
| } |  |
| } |  |
| } |  |

application_identifier[ i ] is an identifier that may be used to identify the i-th SEI message. application_identifier is an integer in the range of 0 to 255, inclusive.

application_version[ i ] indicates the version associated with the i-th SEI message. application_version shall be in the range of 0 to 255, inclusive.

targeted_system_prim_chromaticity_x[ i ][ c ] and targeted_system_prim_chromaticity_y[ i ][ c ] for c in the range of 0 to 2, inclusive, are used to specify the primaries of the targeted display in units of 0.0001 for the i-th SEI message.

targeted_system_prim_chromaticity_x[ i ][ 3 ] and targeted_system_prim_chromaticity_y[ i ][ 3 ] are used to specify the white point chromaticity of the targeted display in units of 0.0001 for the i-th SEI message.

targeted_display_max_luminance[ i ] is used to derive the maximum luminance of the targeted display corresponding to the i-th SEI message. The value of targeted_display_max_luminance[ i ] is specified in units of 0.01 candela per meter square.

targeted_display_min_luminance[ i ] is used to derive the minimum luminance of the targeted display corresponding to the i-th SEI message. The value of targeted_display_min_luminance[ i ] is specified in units of 0.0001 candela per meter square.

If a normative implementation is desired for the above example within JCT-VC, following syntax structure may be used. In this case, a particular value of rns_id would be used in this case.

| regional_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| rns_id | u(16) |
| rns_num_rect_regions | u(8) |
| for( i = 0; i < rns_num_rect_regions; i++) { |  |
| rns_rect_region_id[ i ] | u(8) |
| rns_rect_left_offset[ i ] | u(16) |
| rns_rect_right_offset[ i ] | u(16) |
| rns_rect_top_offset[ i ] | u(16) |
| rns_rect_bottom_offset[ i ] | u(16) |
| } |  |
| num_seis_in_rns_minus1 | u(8) |
| for( i = 0; i <= num_seis_in_rns_minus1; i++ ) { |  |
| sei_message( ) |  |
| rns_num_data_bytes[ i ] | u(8) |
| if( rns_id = = 2094 ) |  |
| for( i = 0; i < rns_num_data_bytes[ i ]; i++ ) |  |
| rns_data_byte | u(8) |
| else { |  |
| if( rns_num_data_bytes[ i ] != 0 ) { |  |
| application_identifier[ i ] | u(8) |
| application_version[ i ] | u(8) |
| for( c = 0; c < 4; c++ ) { |  |
| targeted_system_prim_chromaticity_x[ i ][ c ] | u(16) |
| targeted_system_prim_chromaticity_y[ i ][ c ] | u(16) |
| } |  |
| targeted_display_max_luminance[ i ] | u(16) |
| targeted_display_min_luminance[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

When rns_id is equal to 2094, the value of rns_num_data_bytes[ i ] shall be equal to one of 0 and 22.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Philips may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).