| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0064 MPEG Document: m38456 |
| --- | --- |

| Title: | Backward Compatible Opto-Electrical Transfer Function for HDR Video Coding Based on Rational Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Jung, Q. Lin, S. Yu Xidian University 2 South Taibai Road Xian 710071, China M. Li, P. Wu No. 6 Huashendadao Road, ZTE R&D Center, Nanjing 210012, China | Tel: Email: | +86-29-88202654 zhengzk@xidian.edu.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

# Abstract

Hybrid log gamma (HLG) opto-electronic transfer function (OETF) combines a gamma function and a logarithmic function to achieve display backward compatibility. Proposed is a backward compatible OETF for high dynamic range (HDR) video coding based on rational quantization. Uniform rational mapping function (URMF) based on rational quantization is derived to approximate the logarithmic function of HLG. The HLG OETF is then set as the combination of the HLG gamma function and UMRF. Experimental results demonstrate that compared with HLG, the proposed method gets an average bit-rate reduction of 4.59% and 5.70% for the same quality measured in tPSNR_XYZ and deltaE, respectively.

# Introduction

The MPEG Call for Evidence (CfE) on HDR XYZ wide color gamut video coding [1] has provided a framework for encoding and decoding HDR videos. Up to now, the HDR video coding solutions are classified into two main categories: backward compatible HDR coding and perception-based HDR coding. In perception-based coding, it is very critical to select an appropriate opto-electronic transfer function (OETF) for representing HDR videos. To emulate the human visual system (HVS), non-linear operations are used to quantize the sample values cause the minimal subjective distortions. Up to now, many OETFs have been proposed based on HVS: Perceptual quantizer (PQ) [2, 3], power law gamma [4], and Philips transfer function [5]. PQ transfer function proposed by Miller et al. [2] is being standardized by the Society of Motion Picture Engineers (SMPTE) ST 2084. Since PQ is based on the human contrast sensitivity model [3], it reflects human visual perception better than other OETFs. However, PQ transfer function has a limit to be generally used because it is not compatible with existing standards and infrastructures. To address this issue, an OETF of hybrid log gamma (HLG) is proposed [6]. HLG consists of two main parts: (1) A conventional gamma curve for dark regions of HDR videos and (2) a logarithmic curve for the others. As it is based on an extension to the existing ITU-R Recommendations BT.709 and BT.2020, it is compatible with existing infrastructure and distributions systems and delivers a high quality compatible image to legacy SDR displays.

(a)                                                (b)

Fig. 1 Mapping curves. (a) Logarithmic mapping curves with different  [8]. (b) URMF curves with different p. The x-coordinate indicates original input intensity, while the y-coordinate indicates mapped output intensity.

In this contribution, we propose a new backward compatible OETF for HDR video coding based on rational quantization. We provide a uniform rational mapping function (URMF) based on rational quantization to approximate a logarithmic function. And then we replace the logarithmic function in HLG with UMRF to get a new OETF. The proposed OETF has low computational complexity and experimental results demonstrate that proposed OETF produces better reconstruction results in human visual perception than HLG under the same visual quality and achieves better BD rates in terms of tPSNR_XYZ and deltaE.

# Proposed Method

URMF [7] is a fast and simple quantization technique which displays HDR contents in LDR devices. URMF approximates a logarithmic mapping function using rational quantization. [7] shows that URMF compresses HDR image and conveys the desired image content with the least distortions. URMF is defined as follows:

F=pLHDRpLHDR−LHDR+Hival                            (1)

with

p=MN⋅HivalLoval                                 (2)

where  is the HDR image;  and  are the maximum and minimum values in ; and M and N are the brightest and darkest levels of the LDR display, respectively. As shown in Fig. 1, URMF generates very similar curves like logarithmic mapping curves. Since logarithmic mapping considers the non-linear response of visualization devices and human visual perception based on HVS, URMF successfully meets the HVS characteristics. Moreover, URMF needs simple calculations including 1 division, 1 multiplication, 1 subtraction and 1 addition， and thus is more efficient than the logarithmic mapping function.

HLG transfer function was proposed by British Broadcasting Corporation (BBC) in [6] as a response to the MPEG CfE [1]. Similar to the traditional camera “knee” function, HLG combines a log curve with a traditional “gamma” transfer function. Thus, HLG is compatible with legacy LDR displays. HLG is defined as follows:

(3)

where E is proportional to the light intensity in camera colour channels, i.e. R, G, and B, which is normalized by the reference white level; E’ is the “gamma corrected” signal; r is the reference value of E’; and a, b, c are constants for HLG. The signal value is unity for a relative luminance of 12.0. The electronic-optical transfer function (EOTF) of HLG is not only an inverse function of OETF, which is defined as the inverse function of OETF concatenated with the opto-optic transfer function (OOTF). OOTF is to compensate for changes in tone perception as the eye adapts to different environments, which depends on the brightness of the surround and the peak brightness of the display [6]. URMF has a similar curve to the log curve, and satisfies human visual perception. We replace the log function of HLG with URMF as follows:

(4)

where a and b are constants for URMF-based OETF; EMax is the maximum luminance normalized by the reference white level. In (4), p is obtained by (2) and E, E’, r are the same as (3). Fig. 2 shows the proposed OETF with HLG (Green: HLG; Red: Proposed). It can be observed that the proposed OETF is a “knee” curve while the HLG is a smooth one. We use the “knee” curve to keep the characteristics of URMF since the value of the coefficient a in (4) is a constant which is nearly 1. Thus, a successfully keeps the original characteristics of URMF.

It has proven that most HDR contents do not utilize the full possible dynamic range [5]. Moreover, most pixels in HDR content belong to dark areas (we define the dark areas as the luminance smaller than 4 times of reference white level). Since the proposed curve is below HLG, the pixels in dark areas are mapped to a smaller range than HLG. In theory, mapping to a larger range will represent the original content better, while more bits are needed for encoding the quantization contents. Using a smaller range which needs fewer bits for encoding the quantization contents may introduce distortions, but it can also represent the original content well if the distortions are not perceived by human. Since the URMF meets the HVS characteristics [7] and the proposed curve is below HLG in dark areas, that is, the proposed OETF saves more bits when encoding the quantized videos. In short, compared with HLG, the proposed curve is more suitable for the compression and transmission of HDR videos because it is more consistent with the luminance distribution of HDR contents.

Fig. 2 Proposed OETF and HLG.

Fig. 3 Web fractions

To measure the subjective quality of the proposed OETF, we estimate Weber fraction. Weber fraction represents the visibility of quantization noise, and is defined as the ratio between adjacent quantization levels as follows:

(5)

where N is the number of quantization levels, which equals to 876 for 10 bit videos and 220 for 8 bit videos; L is displayed luminance; and V is the signal. The difference between adjacent quantization levels is not able to be perceptual when the Weber fraction less than 2%, i.e. this threshold increases at low luminance, according to the Schreiber curve [9]. Fig. 3 indicates that the proposed OETF has a low Weber fraction, which implies that the proposed OETF successfully satisfies the human visual perception. Since the number of quantization levels is 876 for 10 bit videos and 220 for 8 bit videos, the reduction of the bit depth from 10 to 8 increases the Weber fractions by a factor of 4 according to (5). In the 8 bit videos, i.e. LDR contents, the Weber fraction in HLG increases to2.4%, which results in visible banding artifacts in bright areas. Thus, HLG is not very compatible with common 8 bit contents. However, the Weber fraction in the proposed OETF increases only to 0.6%, which is still less than 2%. This means that the quantization noise is imperceptible. The proposed EOTF is the inverse OETF concatenated with the opto-optic transfer function (OOTF) which has the same process as HLG. The inverse EOTF is obtained as follows:

(6)

In (6), all parameters have the same meanings and values as OETF in (4).

# Experimental Results

In the experiments, we use a PC with Intel (R) Core (TM) i5 CPU (3.20GHZ) and 4.00GB RAM running a Windows 7 environment and Microsoft Visual Studio 2013. The experiments are performed on the common test conditions (CTC) as described in [10]. The test materials in the experiments are comprised of five classes with total nine sequences (as show in TABLE I) [10]. All the test sequences are RGB 4:4:4 format with the resolution of 1920×1080. Class A, G and H are in BT.2020 containers, while Class B and C are in P3D65 containers. The test materials are first pre-processed by HDRTools 1.0, and then compressed using the HEVC reference test model (HM16.7) Main 10 Profile with Random Access (RA) configuration. In (3), we set r=0.5, a=0.17883277, b= 0.28466892, and c= 0.55991073. In (3), we set a=0.98280620, b=0.32170239, and Emax=12. As evaluation metrics, we use tPSNR_XYZ and deltaE [1] to measure the quality of reconstructed videos. tPSNR_XYZ measures intensity distortions between the reconstructed videos and its original videos, while deltaE measures color distortions between them. The larger values of tPSNR_XYZ and deltaE are, the better reconstruction is. We use four quantization parameter (QP) values are used to encode each sequence according to the attached files in [10], and utilize the Bjøntegaard Delta (BD) rate in [11] to calculate the average bit-rate reduction at the same tPSNR_XYZ and deltaE. To compare with HLG, all parameters are set the same as [12], and the system gamma in OOTF is set to 1 to ignore other objective factors.

A. Quantitative Measurements

TABLE I shows the bit-rate reductions of the proposed method in comparison with HLG. From TABLE I, it can be observed that the proposed method has better performance than HLG. Under the same tPSNR_XYZ values, the proposed method achieves bit-rate reductions ranging from -2.26% to -11.01%. The average bit-rate reduction is -4.59%. Under the same deltaE value, the proposed method also achieves better performance than HLG. The only exception is the test sequence Sun. That is, the proposed method produces better HDR reconstruction results than HLG at the same bits. Since URMF has low complexity, the proposed method needs less time costs than HLG. As shown in TABLE II, for all test sequences, the average time is 30ms/frame in the proposed OETF, while HLG needs 38ms/frame.

B. Visual Comparisons

Although the proposed OETF needs smaller bits than HLG, the subjective perceptual quality by the proposed method is nearly the same as HLG because the proposed method has a lower value of Weber fraction and is more suitable for human visual perception. Fig. 4 shows that the proposed method preserves more details and structural information than HLG and needs fewer bits. In the regions of the red rectangles, the proposed method preserves details better than HLG.

# Conclusions

We have proposed a new backward compatible OETF for HDR video coding based on human visual perception. The proposed OETF is based on rational quantization, and replaces the logarithmic mapping for display backward compatibility. It is in line with the luminance distribution of HDR contents so that it is more suitable for HDR video coding. Thus, the proposed OETF achieves a low Weber fraction so that HDR contents can be conveyed with less distortion. The Weber fraction in the proposed OETF is only 0.1% between quantization levels in high dynamic ranges, which means the difference between levels is imperceptible by HVS. Experimental results demonstrate that the proposed method achieves a better performance than HLG in terms of tPSNR_XYZ and deltaE, while successfully keeping backward compatibility with lower computational complexity. Moreover, the proposed OETF effectively preserves fine details and structural information. It is recommended to consider the proposed method in HDR Anchor generation.

TABLE I

AVERAGE BD RATE SAVINGS OF THE PROPOSED METHOD IN COMPARISON WITH HLG

| Class | Sequences | tPSNR_XYZ | deltaE |
| --- | --- | --- | --- |
| A | FireEater2 | -2.75% | -2.54% |
|  | Market3 | -2.26% | -4.11% |
|  | Sun | -11.01% | 7.1% |
| B | Bike | -2.28% | -15.89 |
|  | Garage | -3.22% | -5.70% |
| C | ShowGirl2 | -2.82% | -3.10% |
| G | Balloon | -4.82% | -9.85% |
| H | Hurdles | -5.33% | -9.85% |
|  | Starting | -6.72% | -7.33% |
| Average |  | -4.59% | -5.70% |

TABLE II

COMPUTING TIME OF HLG AND THE PROPOSED METHOD (UNIT: MSEC/FRAME)

| Class | Sequences | HLG | Proposed |
| --- | --- | --- | --- |
| A | FireEater2 | 28 | 30 |
|  | Market3 | 62 | 29 |
|  | Sun | 37 | 29 |
| B | Bike | 30 | 31 |
|  | Garage | 34 | 30 |
| C | ShowGirl2 | 31 | 30 |
| G | Balloon | 39 | 30 |
| H | Hurdles | 48 | 32 |
|  | Starting | 33 | 30 |
| Average |  | 38 | 30 |

(a)                                           (c)

(b)                                          (d)

Fig. 4 Reconstruction results of the 28th frame in Hurdles when QP is 27. (a) HLG: bits = 13008, tPSNR_XYZ= 39.993dB, deltaE=33.323dB. (b) The proposed method: bits = 11200, tPSNR_XYZ= 39.765dB, deltaE=33.339dB. (c)  Enlarged regions of (a). (d) Enlarged regions of (b).

# References

[1] A. Luthra, E. François, and W. Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding,” N15083, 111th MPEG meeting, February 2015, Geneva, CH.

[2] S. Miller, M. Nezamabadi, and S. Daly. “Perceptual signal coding for more efficient usage of bit codes,” SMPTE Motion Imaging Journal, vol. 122, pp. 52-59, 2013.

[3] P. G. J. Barten, “Formula for the contrast sensitivity of the human eye,” Proc. SPIE-IS&T, vol. 5294, pp. 231–238, Jan. 2004.

[4] ITU-R BT.2020 (2012), “Parameter Values for ultra-high definition television systems for production and internal programme exchange”.

[5] A. Motra and H. Thoma, “An adaptive logluv transform for high dynamic range video compression,” Proc. IEEE ICIP, pp. 2061–2064, 2010.

[6] A. Cotton, T. Borer, M. Pindoria, S. Thompson, M. Naccari, S. Schwarz, and M. Mrak, “BBC’s response to CfE for HDR video coding,” M36249, 112th MPEG meeting, June 2015, Warsaw, Poland.

[7] C. Schlik, Quantization techniques for visualization of high dynamic range picture, Photorealistic Rendering Techniques, 1995.

[8] F. Drago, K. Myszkowski, T. Annen, and N. Chiba. “Adaptive logarithmic mapping for displaying high contrast scenes,” Comput. Graph. Forum, vol. 22, no. 3, pp. 419-426, 2003.

[9] W. F. Schreiber, Fundamentals of electronic imaging systems, Springer-Verlag, 1992, ISBN: 978-3-540-56018-0

[10] E. François, J. Sole, J. Ström, and P. Yin, “Common test conditions for HDR/WCG video coding experiments,” M38220, 114th MPEG meeting, February 2016, San Diego, USA.

[11] G. Bjøntegaard, “Calculation of average PSNR difference between RD curves,” Proc. VCEG Meeting ITU-T Q.6/SG16, VCEG-M33, Austin, TX, Apr. 2010.

[12] M. Naccari, A. Cotton, M. Pindoria and T. Borer,” Updating HDR Tools for the hybrid log gamma transfer function,” M36976, 111th MPEG meeting, October 2015, Geneva, CH.

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).