| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0069 |
| --- | --- |

| Title: | Content colour volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Done Bugdayci Sansli Joel Sole Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document proposes a content colour volume SEI message that is used to indicate the colour volume occupied by the content. It is asserted that this information is useful for display devices to map the content according to the specifications of the display.

# Introduction

In earlier JCT-VC meetings, contributions [1][2][3] have proposed an SEI message to support the indication of the colour gamut or the effective colour volume of the content. The contributions argue that signalling the colour gamut/colour volume information of the content would be beneficial in several processes associated with post-processing, display and other steps that are related to processing the content. Document [3] specifies that a number of colour primaries may be specified to indicate the colour gamut boundary, in addition to the white point. Document [2] specifies the use of multiple colour primary models (3-, 5-, and 6-primary models) to specify effective colour volume. Document [2] also proposes to signal the nominal values of the minimum and maximum values of the signals for each primary component in that particular space. It is also proposed to signal this information for different spatial regions in the picture.

This document proposes that in addition to the primary coordinates to describe the gamut/volume of the content, more information about the colour boundaries be indicated to provide more accurate information regarding the volume occupied by the content. The variation with respect to various luminance values is proposed to be signalled that can more effectively describe the colour volume. It is argued that this information is beneficial for display devices to perform several processing algorithms, including gamut mapping to fit to the specific display characteristics.

# Proposal

## Syntax structure

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| content_colour_volume_id | ue(v) |
| content_colour_volume_persistence_cancel_flag | u(1) |
| if( !content_colour_volume_persistence_cancel_flag ) { |  |
| for( c = 0; c < 3; c++ ) { |  |
| content_gamut_primary_x[ c ] | u(16) |
| content_gamut_primary_y[ c ] | u(16) |
| } |  |
| content_volume_min_lum_value | u(32) |
| content_volume_max_lum_value | u(32) |
| content_volume_info_present_flag | u(1) |
| if( content_volume_info_present_flag ) { |  |
| content_volume_num_lum_ranges_minus1 | ue(v) |
| if( content_volume_num_lum_ranges_minus1 > 0 ) |  |
| content_volume_equal_lum_ranges_idc | u(2) |
| for( i = 0; i <= ContentVolumeNumLumRanges; i++ ) { |  |
| if( !content_volume_equal_lum_ranges_idc ) |  |
| if( i != 0 ¦¦ i != ContentVolumeNumLumRanges ) |  |
| content_volume_lum_range_value[ i ] | u(32) |
| content_volume_num_chrom[ i ] | ue(v) |
| for( j = 0; j < content_volume_num_chrom[ i ]; j++ ) { |  |
| content_volume_chrom_x[ i ][ j ] | u(16) |
| content_volume_chrom_y[ i ][ j ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

The content colour volume SEI message indicates the colour volume of the content that may be utilized by display devices to map the content according to the display specifications.

content_colour_volume_id contains an identifying number that may be used to identify the purpose of the SEI message. The value of content_colour_volume_id shall be in the range of 0 to 232 − 2, inclusive.

Values of content_colour_volume_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of content_colour_volume_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of content_colour_volume_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

content_colour_volume_persistence_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous content colour volume SEI message in output order that applies to the current layer. content_colour_volume_persistence_cancel_flag equal to 0 indicates that content colour volume information follows.

content_gamut_primary_x[ c ] and content_gamut_primary_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c, for c in the range of 0 to 2, inclusive, of the content gamut in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing content gamut using red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Error: Reference source not found). The values of content_gamut_primaries_x[ c ] and content_gamut_primaries_y[ c ] shall be in the range of 0 to 50 000, inclusive.

content_volume_min_lum_value specify a minimum luminance value that is used to specify the colour volume of the content. The values of content_volume_min_lum_value are in units of 0.0001 candelas per square metre.

content_volume_max_lum_value specify a maximum luminance value that is used to specify the colour volume of the content. The values of content_volume_max_lum_value are in units of 0.0001 candelas per square metre.

content_volume_info_present_flag equal to 0 specifies that additional syntax elements specifying the content colour volume information are not present in the SEI message. content_volume_info_present_flag equal to 1 specifies that additional syntax elements specifying the content colour volume information are present in the SEI message.

content_volume_num_lum_ranges_minus1 plus 1 specifies the number of luminance ranges used to describe the content volume. The value of content_volume_num_lum_ranges_minus1 shall be in the range of 0 to 255, inclusive. When not present, the value of content_volume_num_lum_ranges_minus1 is inferred to be equal to 0.

The variable ContentVolumeNumLumRanges is set equal to content_volume_num_lum_ranges_minus1 + 1.

content_volume_equal_lum_ranges_idc equal to 0 specifies that the syntax elements content_volume_lum_value[ i ] are explicitly signalled. content_volume_equal_lum_ranges_idc equal to 1 or 2 specifies that the syntax element content_volume_lum_value[ i ] are not explicitly signalled. When content_volume_num_lum_ranges_minus1 is equal to 0, the value of content_volume_equal_lum_ranges_idc is inferred to be equal to 1. The value of content_volume_equal_lum_ranges_idc shall be in the range of 0 to 2, inclusive. The value of content_volume_equal_lum_ranges_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC.

content_volume_lum_value[ i ] is used to derive the i-th luminance value used to the specify the colour volume when content_volume_equal_lum_ranges_idc is equal to 0, in the same units as content_volume_min_lum_value and content_volume_max_lum_value.

The variable ContentVolumeLuminanceRange[ ], for i in the range of 0 to ContentVolumeNumLumRanges, inclusive, is derived as follows:

ContentVolumeLuminanceRange[ 0 ] = content_volume_min_lum_value
lumDiffVal = content_volume_max_lum_value − content_volume_min_lum_value
for( i = 1; i < ContentVolumeNumLumRanges; i++ )
	if( content_volume_equal_lum_ranges_idc  = =  0 )
		ContentVolumeLuminanceRange[ i ] = content_volume_lum_value[ i ]
	else if( content_volume_equal_lum_ranges_idc  = =  1 ) {
		diffVal = lumDiffVal  ContentVolumeNumLumRanges 
		ContentVolumeLuminanceRange[ i ] = ContentVolumeLuminanceRange[ i − 1 ] + diffVal
	}
	else if( content_volume_equal_lum_ranges_idc  = =  2 ) {
		logDiffVal = Log10( lumDiffVal )  ContentVolumeNumLumRanges
		ContentVolumeLuminanceRange[ i ] = ContentVolumeLuminanceRange[ 0 ] + 										10( i*logDiffVal )
	}
ContentVolumeLuminanceRange[ ContentVolumeNumLumRanges ] = content_volume_max_lum_value

content_volume_num_chrom[ i ] specifies the number of chromaticities associated with the i-th luminance range that is used to describe the colour volume of the content. The value of content_volume_num_chrom[ i ] shall be in the range of 0 to 15, inclusive. When not present, the value of content_volume_num_chrom[ i ] is inferred to be equal to 3.

content_volume_chrom_x[ i ][ j ] and content_volume_chrom_y[ i ][ j ] specify the normalized x and y chromaticity coordinates, respectively, of the j-th chromaticity coordinate that is used to derive an estimate of the colour volume for the i-th luminance value in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). The values of content_volume_chrom_x[ i ][ j ] and content_volume_chrom_y[ i ][ j ] shall be in the range of 0 to 50 000, inclusive.

When not present, the values of content_volume_chrom_x[ i ][ c ] and content_volume_chrom_y[ i ][ c ] are inferred to be equal to content_gamut_primary_x[ c ] and content_gamut_primary_y[ c ], respectively, for c in the range of 0 to content_volum_num_chrom[ i ] − 1, inclusive, and i in the range of 0 to ContentVolumeNumLumRanges, inclusive.

For each value of i in the range of 0 to ContentVolumeNumLumRanges, inclusive, let convexRegion[ i ] refer to the two-dimensional convex hull of the chromaticity coordinates content_volume_chrom_x[ i ][ j ] and content_volume_chrom_y[ i ][ j ] for j in the range of 0 to content_volume_num_chrom[ i ] – 1, inclusive.

The colour volume of the content, colourVolumeBoundingRegion, is defined as the union of colourVolumeRegions[ i ] for i in the range of 0 to ContentVolumeNumLumRanges − 1, inclusive, where colourVolumeRegions[ i ] is defined as the three-dimensional convex hull of the points convexRegion[ i ] and convexRegion[ i + 1 ]:

# Reference

- A.M. Tourapis, Y. Su, D. Singer, “Effective Colour Volume SEI”, JCTVC document JCTVC-V0038, Geneva, CH, Oct. 2015.

- A.M. Tourapis, Y. Su, D. Singer, “Effective Colour Volume SEI”, JCTVC document JCTVC-W0098, San Diego, USA, Feb. 2016.

- H.M. Oh, J.W. Choi, J.-Y. Suh, “Content colour gamut SEI message”, JCTVC document JCTVC-W0057, San Diego, USA, Feb. 2016.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).