| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, Switzerland, 30 May–3June 2016 | Document: JCTVC-X0070/ m38481 |
| --- | --- |

| Title: | HEVC Encoder Optimization Based on Perceptual Block Merging |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Jung, Q. Lin, S. Yu Xidian University 2 South Taibai Road Xian 710071, China M. Li, P. Wu No. 6 Huashendadao Road, ZTE R&D Center, Nanjing 210012, China | Tel: Email: | +86-29-88202654 zhengzk@xidian.edu.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

In this contribution, we propose HEVC encoder optimization for HDR video coding based on perceptual block merging. We perform perceptual block merging for HEVC Main 10 Profile-based HDR video coding. First, we generate 10bit luminance adaptation (LA) for quantized HDR videos based on Barten contrast sensitivity function (CSF) model. Then, we obtain a free-energy based just-noticeable-difference (FEJND) map in each frame to represent the disorderly concealment effect. Next, we calculate average FEJND values of each coding unit (CU) and the frame from the FEJND map to get the perceptual Lagrange multiplier for rate-distortion optimization (RDO). Finally, we achieve perceptual block merging by minimizing RD costs based on the perceptual Lagrange multiplier. Experimental results demonstrate that the proposed method produces the block merging results more correlated with human visual perception, while successfully improving the visual quality of the reconstructed HDR videos.

HEVC Encoder Optimization

The human visual system (HVS) can perceive orderly stimulus well, but tries to avoid uncertainties caused by disorderly regions, called the free-energy principle. There exists disorderly concealment effect in HVS. In [1], we have proposed a perceptual block merging method for quadtree-based partitioning in HEVC encoding based on the disorderly concealment effect. We represent the disorderly concealment effect by the free-energy based just-noticeable-difference (FEJND) model, and use it for perceptual block merging in HEVC encoder. This method conceals more distortions in disorderly regions while improving the visual quality of the reconstructed videos. The proposed HEVC encoder optimization based on perceptual block merging is illustrated in Fig. 1. As illustrated in Fig. 1, input HDR videos are firstly pre-processed to use the HEVC Main 10 Profile [2]. Next, we perform perceptual block merging for quadtree-based partitioning in HEVC as follows. We first compute the FEJND map from the 10bit frame, and then calculate average FEJND values of each CU and the frame from the FEJND map to obtain the perceptual Lagrange multiplier for RDO. Based on the perceptual Lagrange multiplier, we perform perceptual block merging by minimizing RD costs. In the decoder part, after post-processing, the reconstructed HDR videos are obtained.

(a)

(b)

Fig. 1 Block diagram of the proposed method. (a) Whole framework of the proposed HDR video compression. (b) Perceptual block merging.

We obtain FEJND maps for perceptual block merging as follows. In general, the HVS sensitivity highly depends on the adaptation and background luminance. The sensitivity is known as luminance adaptation (LA). We obtain 10bit LA for quantized HDR videos based on Barten CSF as follows:

(1)

with

(2)

(3)

(4)

Its inverse PQ-TF is obtained as follows:

(5)

where m1, m2, c1, c2, and c3 are parameters for inverse PQ-TF; min() denotes the minimum value; and max() denotes the maximum value. We use an autoregressive (AR) model to predict the orderly regions of an image [3]. The prediction residual is regarded as the disorderly regions of an image. To compute JND in the orderly part, we use spatial masking (SM) and the proposed LA model as follows:

(6)

(7)

(8)

where B(x,y) is the background luminance of a pixel (x, y); G(x,y) is the maximum edge height of its neighborhood [4]; min() denotes the minimum value; and JNDp(x,y) is JND of the orderly part at (x, y). Moreover, JND in the disorderly part, JNDc(x,y), is computed as follows:

(9)

where  is an adjustable parameter; and C is the disorderly map. Therefore, we compute FEJND as follows:

(10)

The quadtree structure allows partitioning of a video frame into blocks of variable sizes, and thus keeps the tradeoff between the distortion D and the bit-rate R. It is an optimization problem which is formulated by the optimization of a Lagrangian function as follows:

(11)

where J is the RD cost; and λ is the Lagrange multiplier that controls the tradeoff between distortion and bit-rate.

We adjust λ using FEJND maps as follows. Because FEJND refers to the minimum visibility threshold of HVS, more distortion can be tolerated in the disorderly regions, and thus disorderly regions have higher FEJND values than orderly ones. Therefore, if one pixel has a relatively greater FEJND value, its corresponding original information is highly uncertain, and of less importance to human eyes. Thus, it can be classified as a candidate pixel within disorderly regions. FEJND of each CU is defined as follows:

(12)

where CUi means the i-th CU in the frame, and Mi×Ni is the resolution of the i-th CU. We compute FEJND of each CU instead of largest CU (LCU). Moreover, FEJND of the whole video frame is obtained by the average of all pixels’ FEJND in a frame as follows:

(13)

where M×N is the resolution of the frame. Thus, if one CU has an FEJND value lager than FEJNDframe, it is classified as a disorderly CU; otherwise, it is an orderly CU. Based on the FEJND map, we calculate the perceptual weight for the Lagrange multiplier as follows:

(14)

where three coefficients a, b and c are set to 0.7, 0.6, and 4 empirically. A smaller visual weight η (smaller than 1) indicates that the CU is orderly and more sensitive to distortions. Such a CU might be perceived at a higher visual sensitivity, which cannot tolerate higher distortions. In contrast, a larger visual weight η (larger than 1) describes a disorderly CU and is less sensitive to the distortion so that it can tolerate more distortions. When η equals 1, it is not required to adjust the Lagrange multiplier. Then, we get the perceptual Lagrange multiplier for each CU as follows:

TABLE I

BD Rate Comparison between Proposed Method and HM16.7

(15)

where QP is the frame-level quantization parameter in a group of pictures (GOP) structure. Therefore, the final RD cost function in (11) is rewritten as follows:

(16)

Experimental Results

We perform experiments in the HEVC test model (HM16.7) Main 10 Profile using Random Access (RA) configuration, which uses a hierarchical B picture structure with a Group of Pictures (GOP) size of not larger than 8 frames according to Common Test Conditions (CTC) for HDR/WCG video coding experiments [5]. We compare the performance of the proposed method with that of Anchor which uses HM16.7 and the updated HDRTools 0.10 (HDRTools 0.10 with patch files). For the tests, we use a workstation with Intel Xeon E5-2640 v3 CPU (2.60GHZ and 8Cores) and 32.00GB RAM running a Windows 7 environment and Microsoft Visual Studio 2013. The test sequences are RGB 4:4:4 format with the resolution of 1920×1080. Classes A, G and, H are in BT.2020 containers, while Classes B, C and, D are in P3D65 containers. The test materials are first pre-processed by the updated HDRTools 0.10. In (5), we set m1, m2, c1, c2, and c3 to 0.1593, 78.8438, 0.8359, 18.8516, and 18.6875, respectively.

The proposed encoder optimization is integrated with HM16.7. The perceptual quality of the proposed method is better than that of HM16.7 at the same low bit-rate level due to the perceptual bit allocation in (16). Specifically, the proposed method preserves more structural information in the original HDR contents than HM16.7. As shown in Figs. 2-4 (see the red circles), details of the proposed method are clearer than those of HM16.7. HVS is more sensitive to the distortions of orderly regions than those of disorderly regions. Thus, the proposed method produces more pleasing results in HVS than HM16.7.

TABLE I lists the experimental results compared with HM16.7. As shown in TABLE I, the proposed method achieves better BD-rates than HM16.7 in most text sequences, while it achieves worse performance in FireEater2, ShowGirl2Teaser and StEM_WarmNight. The three sequences of FireEater2, ShowGirl2Teaser, and StEM_WarmNight are captured under low light environment, and thus have a narrow dynamic range with a dark tone. The proposed method classifies each frame into orderly regions and disorderly regions by FEJND (mainly, based on luminance information), and  thus most regions in FireEater2, ShowGirl2Teaser and StEM_WarmNight are regarded as orderly regions. Thus, more bits are needed for them to encode them. In summary, the proposed method achieves bertter BD-rate performance in most test sequences, but needs more bits to maintain visual quality in some test sequences captured under low light enviroment. TABLE II shows the runtime comparison between the proposed method and HM16.7. As shown in the table, the proposed method increases the runtime of 11% over HM16.7. The runtime is mainly caused by getting disorderly maps.

In our previous work [6], it has been verified that perceputal block merging achieves significant bit-savings of 8.41% with little coding loss in 8bit videos. Moreover, the proposed method can be easily extended to the encoding of 12bit, 14bit, and 16bit videos based on Barten CSF. That is, the proposed method is a general encoding technique that can be applied to various contexts. Moreover, the proposed method yields HDR reconstrction resluts which are more suitable for HVS.

Fig. 2 Visual quality comparison in the 40th frame of Market when QP is 34. Top: Proposed method. Bottom: HM16.7.

Fig. 3 Visual quality comparison in the 10th frame of GarageExit when QP is 34. Top: Proposed method. Bottom: HM16.7.

Fig. 4 Visual quality comparison in the 500th frame of Hurdles when QP is 36. Top: Proposed method. Bottom: HM16.7.

TABLE II

Runtime Comparison between Proposed Method and HM16.7

Conclusions

We have proposed perceptual block merging for HEVC Main 10 Profile-based HDR video coding. We have obtained new LA for quantized HDR videos based on Barten CSF model, and generated the FEJND map in each frame. Based on the FEJND map, we have obtained the perceptual Lagrange multiplier in each CU for RDO, and then performed perceptual block merging by minimizing RD costs. Experimental results show that the proposed method achieves better BD-rate performance than HM16.7 in most test sequences, but needs more bits to maintain visual quality in some test sequences captured under low light environment. Moreover, the proposed method is a general encoding technique, not only for HDR video coding. In the future, we will investigate JND models for HDR video coding which are fast, simple, and more suitable for human visual perception. Besides, since the proposed method adjusts λ using FEJND maps, we will perform comparative analysis with QP adjusting techniques such as luma delta QP [7] and chroma QP offset [8].

- References

[1] C. Jung, Y. Chen, Q. Lin, M. Li, and P. Wu, "Optimization of HEVC Main 10 Coding in HDR Videos Using Perceptual Block Merging," ISO/IEC JTC1/SC29/WG11 MPEG2015/M37073, Geneva, Switzerland, October 2015.

[2] C. Jung, Q. Fu, G. Yang, M. Li, and P. Wu, "CE1-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile," ISO/IEC JTC1/SC29/WG11 MPEG2016/JCTVC-W0066, San Diego, USA, 19–26 February 2016.

[3] J. Wu, G. Shi, W. Lin, A. Liu, and F. Qi, “Just noticeable difference estimation for images with free-energy principle,” IEEE Trans. Multimedia, vol. 15, no. 7, pp. 1705-1710, 2013.

[4] X. Yang, W. Lin, Z. Lu, E. Ong, and S. Yao, “Just noticeable distortion model and its applications in video coding,” Signal Processing, vol. 20, no. 7, pp. 662-680, 2005.

[5] E. François, J. Sole, J. Ström, and P. Yin, “Common test conditions for HDR/WCG video coding experiments,” ISO/IEC JTC1/SC29/WG11 MPEG2015/M38220, February 2016, San Diego, USA.

[6] C. Jung, Y. Chen, and Q Lin, “Perceptual Block Merging for Quadtree-Based Partitioning in HEVC Using Disorderly Concealment Effect,” Proc. IEEE VCIP, 2015.

[7] J. Kim, J. Lee, E.Alshina, Y. Park, “Luma delta QP adjustment based on video statistical information,” ISO/IEC JTC1/SC29/WG11 MPEG2015/JCTVC-W0039, San Diego, USA, February 2016.

[8] K. Andersson, P. Hermansson, J. Samuelsson, J. Strom, and M. Petersson, “Report for CE1.a (Chroma QP),” ISO/IEC JTC1/SC29/WG11 MPEG2015/m37179, Geneva, Switzerland, October 2015.

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).