| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X0075 |
| --- | --- |

| Title: | Indication of SMPTE ST 2094-20 metadata in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wiebe de Haan HTC 36 5656 AE Eindhoven, The Netherlands Robert Brondijk Rocco Goris Leon van de Kerkhof Rutger Nijland | Tel: Email: | +31 40 27 96248 wiebe.de.haan@philips.com robert.brondijk@philips.com rocco.goris@philips.com leon.van.de.kerkhof@philips.com rutger.nijland@philips.com |
| Source: | Philips |  |  |

_____________________________

# Abstract

This contribution proposes text changes in the HEVC specification to support carriage of metadata of the SMPTE ST 2094-20 standard: Dynamic Metadata for Color Volume Transform - Application #2. Specifically, it is proposed to add a new SEI message to Annex D for carriage of such metadata.

Note: This document is a mainly editorial update of JCTVC-W0133-v2.

# Introduction

The SMPTE ST 2094 suite of documents defines metadata for use in color volume transforms of content. The metadata are content-dependent and can vary scene-by-scene or frame-by-frame. The metadata are intended to transform High Dynamic Range and Wide Color Gamut (HDR/WCG) source content for presentation on a display having a smaller color volume than the source content’s mastering display [1].

SMPTE ST 2094 Part 20 specifies the content-dependent color volume transform metadata items for Application #2 [2]. This color volume transform uses content-dependent dynamic metadata to reproduce the artistic intent of High Dynamic Range and Wide Color Gamut content as accurately as possible on displays having a smaller color volume than that of the display with which that content was mastered. This contribution proposes a new SEI message in Annex D of the HEVC specification to carry such metadata.

# Proposed Text

## SEI payload Syntax for ST 2094-20

D.2.4x Colour volume transform information SEI message syntax

| Color_Volume_Transform_processing_data( metadataSize) { | Descriptor |
| --- | --- |
| application_identifier | u(8) |
| application_version | u(8) |
| for( c = 0 ; c < 3 ; c++ ) { |  |
| targeted_system_display_primaries_x[ c ] | u(16) |
| targeted_system_display_primaries_y[ c ] | u(16) |
| } |  |
| targeted_system_display_white_point_x | u(16) |
| targeted_system_display_white_point_y | u(16) |
| targeted_system_display_max_luminance | u(32) |
| targeted_system_display_min_luminance | u(32) |
| broadcast_flag | u(1) |
| if( broadcast_flag == 1 ) { |  |
| local_slope_adjustments( 0 ) |  |
| } |  |
| else { |  |
| nr_of_windows_minus1 | u(4) |
| for( w=0; w <= nr_of_windows_minus1; w++ ) { |  |
| if( w !=0 ) { |  |
| window_x_low[ w ] | u(12) |
| window_y_low[ w ] | u(12) |
| window_x_high[ w ] | u(12) |
| window_y_high[ w ] | u(12) |
| luminance_lower_bound[ w ] | u(12) |
| luminance_upper_bound[ w ] | u(12) |
| u_prime_chromaticity_disk_center[ w ] | u(10) |
| v_prime_chromaticity_disk_center[ w ] | u(10) |
| chromaticity_disk_radius[ w ] | u(10) |
| luminance_range_selector[ w ] | u(1) |
| chromaticity_area_selector[ w ] | u(1) |
| } |  |
| tone_mapping_input_signalweights_R[ w ] | u(8) |
| tone_mapping_input_signalweights_G[ w ] | u(8) |
| tone_mapping_input_signalweights_B[ w ] | u(8) |
| tone_mapping_input_signalweights_Y[ w ] | u(8) |
| local_slope_adjustments( w ) |  |
| } |  |
| } |  |
| } |  |
| local_slope_adjustments( w ) { |  |
| tone_mapping_input_signal_black_level_offset[ w ] | u(8) |
| tone_mapping_input_signal_white_level_offset[ w ] | u(8) |
| shadow_gain_control[ w ] | u(8) |
| highlight_gain_control[ w ] | u(8) |
| mid_tone_width_adjustment_factor[ w ] | u(8) |
| tone_mapping_output_fine_tuning_num_val[ w ] | u(4) |
| for( j = 0; j < tone_mapping_output_fine_tuning_num_val[ w ]; j++ ) { |  |
| tone_mapping_output_fine_tuning_x[ w ][ j ] | u(8) |
| tone_mapping_output_fine_tuning_y[ w ][ j ] | u(8) |
| } |  |
| saturation_gain_num_val[ w ] | u(4) |
| for( j = 0; j < saturation_gain_num_val[ w ]; j++ ) { |  |
| saturation_gain_function_x[ w ][ j ] | u(8) |
| saturation_gain_function_y[ w ][ j ] | u(8) |
| } |  |
| } |  |

## SEI payload semantics

### D3.4x Colour volume transform information SEI message semantics

This SEI message provides information to enable colour volume transformation of the reconstructed colour samples of the output pictures. The input to the indicated colour volume transform process is either in luminance-chrominance color spaces or directly on RGB color components. In both cases, the set of decoded sample values shall apply an (unspecified) upsampling conversion process to the 4:4:4 colour sampling format as necessary when chroma_format_idc is equal to 1 (4:2:0 chroma format) or 2 (4:2:2 chroma format).

The information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of Society of Motion Picture and Television Engineers ST 2094-20. Variables derived from parameter defined in this section refer to variables of the same name in ST 2094-20.

application_identifier identifies an application and its defining document in the SMPTE ST 2094 suite. For this SEI message, application_identifier shall be set to 2.

application_version specifies the application version in the application defining document in the SMPTE ST 2094 suite. For this SEI message, application_identifier shall be set to 0.

targeted_system_display_primaries_x[ c ] and targeted_system_display_primaries_y[ c ] specify the x and y chromaticity coordinates, respectively, of the colour primary component c of the targeted display in increments of 0.00002.

targeted_system_display_white_point_x and targeted_system_display_white_point_y specify the x and y chromaticity components, respectively, of the white point of the targeted display in increments of 0.00002.

targeted_system_display_max_luminance specifies the nominal maximum luminance of the targeted display, in units of 0.01 candelas per square metre.

targeted_system_display_min_luminance specifies the nominal minimum luminance of the targeted display, in units of 0.0001 candelas per square metre. targeted_system_display_min_luminance shall be less than targeted_system_display_max_luminance.

broadcast_flag indicates whether a restricted or the full metadata set is used.

nr_of_windows_minus1 plus 1 indicates the number of processing windows. The first processing window is always present (if broadcast_flag is 0) and encompasses the whole picture. Subsequent windows (if present) apply to pixel selectors. The value of nr_of_windows_minus1 shall be in the range of 0 to 2, inclusive, i.e. a maximum of 2 pixel selectors can be present.

window_x_low[ w ], window_y_low[ w ] indicates the upper left corner of the local processing window rectangle in pixels for window w. The value of window_x_low[ w ] and window_y_low[ w ] shall be in the range of 0 to 4095, inclusive.

window_x_high[ w ], window_y_high[ w ] indicates the right lower corner of the local processing window rectangle in pixels for window w. The value of window_x_high[ w ] and window_y_high[ w ] shall be in the range of 0 to 4095, inclusive. The value of window_x_high[ w ] shall be larger than the value of window_x_low[ w ]. The value of window_y_high[ w ] shall be larger than the value of window_y_low[ w ].

luminance_lower_bound[ w ] indicates the lower threshold for Luminance for window w. The value of luminance_lower_bound[ w ] shall be in the range of 0 to 4095, inclusive.

The variable LuminanceLowerBound is derived as follows

LuminanceLowerBound = 10(luminance_lower_bound ÷ 1000)-1				(D-xx)

luminance_upper_bound[ w ] indicates the upper threshold for Luminance for window w. The value of luminance_upper_bound[ w ] shall be in the range of 0 to 4095, inclusive. The value of luminance_upper_bound[ w ] shall be larger than the value of luminance_lower_bound[ w].

The variable LuminanceUpperBound is derived as follows

LuminanceUpperBound = 10(luminance_upper_bound ÷ 1000)-1				(D-xx)

u_prime_chromaticity_disk_center[ w ], v_prime_chromaticity_disk_center[ w ] indicate the center values for the chromaticity coordinates in a CIE 1976 uniform color space for window w. The value of u_prime_chromaticity_disk_center[ w ] and v_prime_chromaticity_disk_center[ w ] shall be in the range of 0 to 1023, inclusive.

The variables ChromaticityDiskCenter_u’ and ChromaticityDiskCenter_v’ are derived as follows

ChromaticityDiskCenter_u’ = u_prime_chromaticity_disk_center  ÷ 1632		(D-xx)

ChromaticityDiskCenter_v’ = v_prime_chromaticity_disk_center  ÷ 1632		(D-xx)

chromaticity_disk_radius[ w ] indicates the threshold for the distance between the threshold center values (ChromaticityDiskCenter_u’, ChromaticityDiskCenter_v’) and the actual pixel data for window w. The value of chromaticity_disk_radius[ w ] shall be in the range of 0 to 1023, inclusive.

This distance to actual pixel data (u_prime, v_prime) is measured as

Sqrt( ( u_prime - u_prime_center )2 + ( v_prime - v_prime_center )2 )			(D-xx)

The threshold center values and the threshold distance define a disk in the uv-space.

The variable ChromaticityDiskRadius is derived as follows

ChromaticityDiskRadius = chromaticity_disk_radius ÷ 1632				(D-xx)

luminance_range_selector[ w ] indicates whether the active area is within or outside the specified threshold range for luminance values for window w and corresponds to LuminanceRangeSelector. A value equal to 0 means outside. A value equal to 1 means inside.

chromaticity_area_selector[ w ] indicates whether the active area is within or outside the specified threshold range for chromaticity values for window w and corresponds to ChromaticityAreaSelector. A value equal to 0 means outside. A value equal to 1 means inside.

tone_mapping_input_signal_black_level_offset[ w ] indicates black offset for window w. The value of tone_mapping_input_signal_black_level_offset[ w ] shall be in the range of 0 to 255, inclusive.

The variable ToneMappingInputSignalBlackLevelOffset is derived as follows

ToneMappingInputSignalBlackLevelOffset =

tone_mapping_input_signal_black_level_offset ÷ 2040		(D-xx)

tone_mapping_input_signal_white_level_offset[ w ] indicates white offset for window w. The value of tone_mapping_input_signal_white_level_offset[ w ] shall be in the range of 0 to 255, inclusive.

The variable ToneMappingInputSignalWhiteLevelOffset is derived as follows

ToneMappingInputSignalWhiteLevelOffset =

tone_mapping_input_signal_white_level_offset ÷ 510 		(D-xx)

shadow_gain_control[ w ] indicates the adjustment to the shadow (darker) region of the tone mapping for window w. The value of shadow_gain_control[ w ] shall be in the range of 0 to 255, inclusive.

The variable ShadowGainControl is derived as follows

ShadowGainControl = shadow_gain_control ÷ 127.5					(D-xx)

highlight_gain_control[ w ] indicates the adjustment to the highlight (brighter) region of the tone mapping for window w. The value of highlight_gain_control[ w ] shall be in the range of 0 to 255, inclusive.

The variable HighlightGainControl is derived as follows

HighlightGainControl = highlight_gain_control ÷ 127.5				(D-xx)

mid_tone_width_adjustment_factor[ w ] indicates the adjustment to the mid-tone region of the tone mapping for window w. The value of mid_tone_width_adjustment_factor[ w ] shall be in the range of 0 to 255, inclusive.

The variable MidToneWidthAdjustmentFactor is derived as follows

MidToneWidthAdjustmentFactor = mid_tone_width_adjustment_factor ÷ 127.5	(D-xx)

tone_mapping_input_signalweights_R[ w ], tone_mapping_input_signalweights_G[ w ], tone_mapping_input_signalweights_B[ w ], tone_mapping_input_signalweights_Y[ w ] indicate the relative weights of the R, G, B, and Y components in the tone mapping for window w. The value of tone_mapping_input_signalweights_R[ w ], tone_mapping_input_signalweights_G[ w ], tone_mapping_input_signalweights_B[ w ] and tone_mapping_input_signalweights_Y[ w ] shall be in the range of 0 to 255, inclusive.

The four elements of the vector ToneMappingInputSignalWeights are derived as follows

ToneMappingInputSignalWeights_x = tone_mapping_input_signalweights_x ÷ 255	(D-xx)

with the suffix _x indicating the elements R, G, B and Y

tone_mapping_output_fine_tuning_num_val[ w ] specifies the number of fine tuning curve points for window w. The value of tone_mapping_output_fine_tuning_num_val shall be ≤ 10 and may be set to 0 to indicate that no curve points are included.

tone_mapping_output_fine_tuning_x[ w ][ j ], tone_mapping_output_fine_tuning_y[ w ][ j ] represents the j-th point for the fine tuning curve for window w.

The value of tone_mapping_output_fine_tuning_x and tone_mapping_output_fine_tuning_y shall be in the range of 0 to 255, inclusive, indicating multiples of 1/255.

The value of each pair are derived as follows

x,y = tone_mapping_output_fine_tuning_x,y ÷ 255					(D-xx)
saturation_gain_num_val[ w ] indicates the number of saturation gain curve points for window w. The value of saturation_gain_num_val shall be ≤ 6 and may be set to 0 to indicate that no saturation gain curve points are included.

saturation_gain_function_x[ w ][ j ], saturation_gain_function_y[ w ][ j ] represents the j-th point for the saturation gain curve for window w. The value of saturation_gain_function_x[ w ][ j ] and saturation_gain_function_y[ w ][ j ] shall be in the range of 0 to 255, inclusive.

The values of each pair are derived as follows

x = 2-15*(saturation_gain_function_x  255) 							(D-xx)
	y = 2-15*( saturation_gain_function_y  255)							(D-xx)

Note: the actual color volume transform method is outside the scope of this specification. A transform method using this SEI message is described in Annex B of reference [2].

# Reference

- SMPTE ST 2094-1:2016: "Dynamic Metadata for Color Volume Transform – Core Components"

- SMPTE ST 2094-20:2016: "Dynamic Metadata for Color Volume Transform – Application #2"

# Patent rights declaration(s)

Philips may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).