| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0004 |
| --- | --- |

| Title: | HEVC conformance test development (AHG4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Rajan Joshi Yan Ye Ji-Zheng Xu | Tel: Email: | teruhiko.s@sony.com rajanj@qti.qualcomm.com Yan.Ye@InterDigital.com jzxu@microsoft.com |
| Source: | AHG on HEVC conformance development |  |  |

_____________________________

# Introduction

The mandates of this AHG are:

- Study the requirements of HEVC conformance testing to ensure interoperability.

- Develop proposed improvements to the SCC conformance testing draft X1016.

- Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt, SHVC, and SCC conformance testing.

- Establish and coordinate bitstream exchange activities for HEVC.

- Identify needs for HEVC conformance bitstreams with particular characteristics.

# Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set.Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set.Activities

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG4] in the subject line for easy grouping.

## ftp site

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

## Bitstream generation guideline

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

## HEVC v.1, MV/3D-HEVC, RExt and SHVC conformance

There was no updates from the last JCTVC meeting. All known problems were resolved. The latest  bitstreams are available at the following site.

http://wftp3.itu.int/av-arch/jctvc-site/draft_conformance/

## SCC conformance

### Bitstream generation instructions

The document JCTVC-O1010 contains the general guidelines for bit-stream generation, uploading and bit-stream naming conventions.

[Ed.: Instructions specific to conformance bit-streams for Screen Content Coding extensions should be added here.]

### Test plan

HEVC Screen Content Coding extensions conformance testing will test the following profiles:

Screen-Extended Main, Screen-Extended Main 10, Screen-Extended Main 4:4:4, Screen-Extended Main 4:4:4 10, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, and Screen-Extended High Throughput 4:4:4 14.

Table 1 lists the bitstream features to be tested. Some bitstream features may be represented using multiple bitstreams.

| Chroma format | Bit depth | Category | Sub category | Bitstream feature | Volunteers | Candidates |
| --- | --- | --- | --- | --- | --- | --- |
| 4:4:4 4:2:0 | 8/10 | Palette | Predictor palette initialization (PPS/SPS/initialized to zero) |  | InterDigital |  |
|  |  |  | Palette size 0/1 |  |  |  |
| 4:4:4 4:2:0 | 8/10 | Current picture reference (CPR) | bi-prediction restriction (conversion from bi to uni) |  | Qualcomm |  |
|  |  |  | DPB | in-loop filtering enabled/disabled | MediaTek |  |
| 4:4:4 4:2:0 | 8/10 | adaptive residual transform | slice ACT QP offsets |  | Qualcomm | InterDigital |
| 4:4:4 4:2:0 | 8/10 | adaptive motion vector resolution | motion_vector_resolution_ control_idc = 0/1/2 |  | Microsoft |  |
| 4:4:4 4:2:0 | 8/10 | Intra coding | Intra boundary filtering disable |  | MediaTek |  |
| 4:4:4 4:2:0 | 8/10 | delta QP / chroma QP offset signalling | delta QP and chroma QP offsets are signalled in the palette block |  |  |  |
| 4:2:2 4:4:4 | 8/10/14 | Screen-extended high throughput profiles | Enable tiles and wavefronts in the same bitstream |  | Apple |  |

## Originally planned bitstream features (HEVC v.1)

The following bitstreams were originally planned to generate and the yellow highlighted bitstreams are not generated yet.

Table: Candidates of HEVC conformance test features

|  | Categories | Sub categories | Feature Code | Bitstream features | Volunteers |
| --- | --- | --- | --- | --- | --- |
|  | Intra coding | Intra prediction | IPRED | Functional stage: Decoding of intra PU. Purpose: Check that the decoder can properly decode all values of intraPredMode for different Luma and Chroma transform block sizes. | C.S. Lim, F. Bossen, J. Lainema, K. Sugimoto |
|  |  | Constraint intra prediction | CIP | Functional stage: Decoding of intra PU in non-intra slice when constrained_intra_pred_flag is set to one Purpose: Check that the decoder can properly perform reference sample substitution process. | C.S. Lim, K. Chono |
|  | Inter frame coding | AMVP | AMVP | Check whether motion vector prediction candidate generation and signalling perform correctly. Check the process when neighboring PUs have various partition mode, prediction mode, reference index, inter_pred_idc, etc.. Related syntax element: mvp_l0_flag, mvp_l1_flag | C. Kim, M. Zhou, Y. -W. Huang |
|  |  | MVs | MV | maximum number of motion vectors permitted per CTU |  |
|  |  |  | MV | vectors reference padded picture edges. | M. Coban |
|  |  |  | MV | clipping of motion vector range in MVP/Merge candidates | M. Coban |
|  |  |  | MV | motion vectors are random, maximum page DRAM misses / minimize cache hits (according to memory bandwidth model... e.g. JCTVC-H007) |  |
|  |  | Merge | MERGE | Merge candidate list index signaling: Check that merge candidate list can deal with every merge index (0..4) | B. Bross |
|  |  |  | MERGE | Parallel merge: test all the possible combinations of log2_parallel_merge_level_minus2 and CTB sizes. | M. Zhou, Y. -W. Huang |
|  |  |  | MERGE | number of maximum merge candidates signalling: test all the values of five_minus_max_merge_cand, i.e. 0, 1, 2, 3, 4 for CTB size of 64x64 | M. Zhou, |
|  |  | WP | WP | weighted_pred_flag and/or weighted_bipred_flag are equal to 1. | T. Chujoh |
|  |  |  | WP | various weighted prediction parameter combinations | T. Chujoh |
|  |  | mvd_l1_zero_flag | MVD | Test mvd_l1_zero_flag | F. Bossen |
|  |  | temporal_mvp | TMVP | Test slice_temporal_mvp_enable_flag | J. Xu |
|  |  | Collocated ref idx | RIDX | Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists.) |  |
|  | HL syntax | slices, PSs | PS | bitstreams with stuffed extensions (vps_extension_data_flag=1, sps_extension_data_flag=1, pps_extension_flag=1, ..) | W. Jang, M. Coban |
|  |  | dependent slice | DSLICE | Check: - dependent slices - WPP with dependent slices - tiles with dependent slices | B. Bross |
|  |  | POC | POC | POC increments > 1 | M. Coban |
|  |  | RPS | RPS |  | M. Coban |
|  |  |  | RPS |  | F. Bossen |
|  |  |  | RPS | Slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Slice header RPS with short-term and long-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures and inter-RPS prediction | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-and long-term pictures | R. Sjöberg |
|  |  | RPL | RPL |  | M. Coban |
|  |  | RAP | RAP | all 6 (currently identified) RAP types | F. Bossen |
|  |  | HRD | HRD | maximum number of pictures in DPB | TBD |
|  |  |  | HRD | bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR) | TBD |
|  |  |  | HRD | bumping DPB | R. Sjöberg |
|  |  |  | HRD | Low delay, Sub picture | K. Kazui |
|  | Transform | RQT | RQT | Check all depths of inter & intra transform tree: QuadtreeTUMaxDepthInter : 0..4 QuadtreeTUMaxDepthIntra : 0..4 | B. Bross |
|  | Quantization | dQP | DQP | all depths of CU and diff_cu_qp_delta_depth | W. Wan, T. Suzuki |
|  |  | Scaling list | SLIST | non-default scaling lists | T. Suzuki, M. Zhou |
|  | Other coding tools | AMP | AMP | Check all asymetric partition mode enable | X. Zheng, C. Kim |
|  |  | Transform skip | TSKIP |  | J. Xu |
|  |  | I_PCM | IPCM | Functional stage: Parsing of I_PCM size information at SPS and I_PCM flag at CU. Purpose: Check that the decoder can properly decode pcm_flag for different intra CU sizes. Related Syntax: log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_flag. | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Parsing and decoding of I_PCM CUs. Purpose: Check that the decoder can properly decode I_PCM CUs with different PCM bit depth. Related syntax: pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, pcm_flag | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Decoding of I_PCM CUs with loop filter skipping. Purpose: Check that the decoder can properly decode I_PCM CUs by skipping loop filter. Related syntax: pcm_loop_filter_disable_flag, pcm_flag. Note: Combination with cu_transquant_bypass_flag will be tested. | W. Wan, C.S. Lim, K. Chono |
|  |  | Lossless | LOSSLESS | transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles lossless coding. | F. Henry |
|  | Parallel processing tools | Tile | TILE | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag | A. Fuldseth, M. Zhou |
|  |  | WPP | WPP | entropy_coding_sync_enabled_flag on. Purpose: Check that the decoder handles wavefront parallel processing | F. Henry |
|  |  | WPP and slices | WPP | entropy_coding_sync_enabled_flag on, number of slice varies randomly between 2 and maximum number of slices permitted, each non-first slice is either regular or dependent in a random way. Purpose: Check that the decoder handles wavefront parallel processing and slices | F. Henry |
|  |  | Tile & WPP | WPP | entry point of WPP | Hendry |
|  | Temporal scalability |  | TSCL |  | W. Jang |
|  |  |  | TSCL | range of nested temporal_id's |  |
|  | Deblocking filter |  | DBLK | various cb, cr qp_offsets | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | various beta_offset_div2, tc_offset_div2 | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | trigger all deblocking decisions | T.Suzuki, R. Sjöberg |
|  |  |  | DBLK | deblocking + tile, etc |  |
|  | SAO |  | SAO | Check all CTU sizes with max. SAO information rate. | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random and extreme SAO offsets values | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random SAO merge left/up flag values | E. Francois, Y. -W. Huang |
|  |  | Slice- and component-level control | SAO | Pictures divided into x slices / randomly enabling SAO Y and/or SAO UV per slice | E. Francois |
|  |  | Tile- and component-level control | SAO | Pictures divided into x tiles / randomly enabling SAO Y and/or SAO UV per tile | E. Francois |
|  | Entropy coding |  |  |  |  |
|  |  | Sign data hiding | SDH | sign_data_hiding_flag on, transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles sign data hiding with lossless coding. | H. Henry |
|  |  | High through put binalizaion | HTB |  | M. Coban |
|  |  | cabac_init_idc | CAINIT |  | A. Segall |
|  |  | Max number of bins | MAXBIN |  | V. Sze |
| Corner case bitstream |  |  |  | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics | C. Fogg |
|  |  |  |  | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) | C. Fogg |
|  |  |  |  | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset | C. Fogg |
|  |  |  |  | inferred splits along non-CTU multiple picture borders |  |
|  |  |  |  | combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, cu_transquant_bypass_flag |  |
|  |  |  |  | something to test the vast universe that is: POC signalling |  |
|  | Video resolution |  |  | pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes. |  |

## Originally planned bitstream features (RExt)

Table: Candidate features of RExt conformance

| Chroma format | Bit depth | Category | Sub category | Bitstream | Volunteers | Candidates |
| --- | --- | --- | --- | --- | --- | --- |
| 4:4:4 |  | Intra coding | Intra smoothing disable | This feature is covered by other bitstreams | Fujitsu | Fujitsu ? |
| 4:2:2 |  |  | Adjustment to intra chroma prediction angle | ADJUST_IPRED_ANGLE_A_RExt_Mitsubishi_1 | A.Minezawa | Mitsubishi |
| 4:2:0, 4:2:2, 4:4:4 | 12, 16 | Inter frame coding | WP (high_precision_offsets_enabled_flag) | ?? TBD |  | Toshiba ? |
| 4:4:4 |  |  | Cross component prediction | CCP_8bit_RExt_QCOM CCP_10bit_RExt_QCOM CCP_12bit_RExt_QCOM | Qualcomm | HHI/Qualcomm |
| 4:4:4 | L/C 10/8 8/10 |  | Different bit depth for luma and chroma | Bitdepth_A_RExt_Sony_1 Bitdepth_B_RExt_Sony_1 | Sony |  |
|  |  | Transform |  | This feature is covered by other bitstreams | Sony |  |
|  |  | Quantization | Scaling list | QMATRIX_A_RExt_Sony_1 | Sony | Sony ? |
|  |  |  | Chroma_qp_adjustment | This feature is covered by other bitstreams | Black Berry | Black Berry ? Apple ? |
| 4:2:2 |  | Loop filter | Deblocking | This feature is covered by other bitstreams |  | ? |
|  |  |  | SAO | SAO_A_RExt_MediaTek_1 | Media Tek | Samsung ? Media Tek ? |
|  |  | Entropy coding | Rice parameter initialization | ?? | Qualcomm | Qualcomm, Sony ? |
|  |  |  | Persistent Rice parameter tool | PERSIST_RPARAM_A_RExt_Sony_2 | Sony |  |
|  |  |  | CABAC bypass alignment | This feature is covered by other bitstreams | Sony | Qaulcomm ? Sony ? |
| 4:4:4 | 16 | Precision | Extended precision | EXTPREC_HIGHTHROUGHPUT_444_16_INTRA_8BIT_RExt_Sony_1 EXTPREC_HIGHTHROUGHPUT_444_16_INTRA_10BIT_RExt_Sony_1 EXTPREC_HIGHTHROUGHPUT_444_16_INTRA_12BIT_RExt_Sony_1 EXTPREC_HIGHTHROUGHPUT_444_16_INTRA_16BIT_RExt_Sony_1 EXTPREC_MAIN_444_16_INTRA_8BIT_RExt_Sony_1 EXTPREC_MAIN_444_16_INTRA_10BIT_RExt_Sony_1 EXTPREC_MAIN_444_16_INTRA_12BIT_RExt_Sony_1 EXTPREC_MAIN_444_16_INTRA_16BIT_RExt_Sony_1 | Sony |  |
|  |  | HL syntax |  |  |  |  |
| 4:2:2, 4:4:4 | 10, 12, 16 | Others | PCM | IPCM_A_RExt_NEC IPCM_B_RExt_NEC | NEC | NEC |
| 4:4:4 | 8, 10, 12, 16 |  | Transform skip rotation | ?? TBD | ?? | Microsoft, Qualcomm (N0288) |
| 4:4:4 | 8, 10, 12, 16 |  | Transform skip context | TSCTX_8bit_I_RExt_SHARP_1, TSCTX_8bit_RExt_SHARP_1, TSCTX_10bit_I_RExt_SHARP_1, TSCTX_10bit_RExt_SHARP_1, TSCTX_12bit_I_RExt_SHARP_1, TSCTX_12bit_RExt_SHARP_1 | Sharp | Qualcomm (N0044) Sharp |
| 4:4:4 | 8, 10, 12, 16 |  | RDPCM (implicit & explicit) | ExplicitRdpcm_A_BBC_1 ExplicitRdpcm_B_BBC_1 | Samsung BBC(explicit) | Media Tek (P0154) |
| 4:4:4 | 8, 10, 12, 16 |  | Max transform skip block size | ?? TBD | BBC | Many. Perhaps BBC? |
| 4:2:2 | 10 |  | Various combination of tools/parameters | Main_422_10_A_RExt_Sony_1 Main_422_10_B_RExt_Sony_1 | Sony |  |
| 4:4:4 | 8/16 |  | Various combination of tools/parameters | GENERAL_8b_400_RExt_Sony_1 GENERAL_8b_420_RExt_Sony_1 GENERAL_8b_444_RExt_Sony_1 GENERAL_10b_420_RExt_Sony_1 GENERAL_10b_422_RExt_Sony_1 GENERAL_10b_424_RExt_Sony_1 GENERAL_12b_400_RExt_Sony_1 GENERAL_12b_420_RExt_Sony_1 GENERAL_12b_422_RExt_Sony_1 GENERAL_12b_444_RExt_Sony_1 GENERAL_16b_400_RExt_Sony_1 GENERAL_16b_444_RExt_Sony_1 GENERAL_16b_444_highThroughput_RExt_Sony_1 WAVETILES_RExt_Sony_1 | Sony |  |

## Originally planned bitstream features (SHVC)

|  |  |  | Replace Needed | # | LAYER DEPENDENCIES |
| --- | --- | --- | --- | --- | --- |
| Nokia | Antti Hallapuro | LAYERID | Y | 1 | gaps in layer id |
| InterDigital | Yan Ye | MVD | N | 1 | no MV dependency |
| Nokia | Antti Hallapuro | MVD | N | 1 | varying sample, MV dependencies |
| ETRI | Hahyun Lee | MAXTID | N | 3 | max_tid |
| Qualcomm | Adarsh Ramasubramonian | INACTIVE | N | 1 | inactive ref layers |
| Vidyo | Wonkap Jang | REFLAYER | Y | 4 | multiple active ref layers with various inter layer predictors and dependencies |
|  |  |  |  |  | VPS SYNTAX |
| HHI | Robert Skupin | SPLITFLAG | N | 1 | splitting_flag |
| Qualcomm | Hendry | VUI | N | 3 | VUI present |
| Qualcomm | Hendry | NONVUI | N | 3 | non VUI extension |
|  |  |  |  |  | DPB |
| Vidyo | Won Kap Jang | DPB | Y | 2 | Sub layer dbp info |
|  |  |  |  |  | RESOLUTIONS |
| Samsung | Elena Alshina | SRATIOS | N | 2 | 2 spatial layers w/ unusual ratios |
| InterDigital | Yan Ye | SNR | N | 3 | 2, 3 SNR layers |
| Vidyo | Wonkap Jang | REPFMT | Y | 3 | VPS rep formats |
| Vidyo | Wonkap Jang | RESCHANGE | Y | 1 | resolution change in SPS |
| Ericsson | Rickard Sjoberg | ADAPTRES | N | 1 | adaptive resolution |
| Sony | Ohji Nakagami | SPSREPFMT | N | 1 | SPS rep format idx |
| Vidyo | Wonkap Jang | CONFCROP | Y | 3 | conformance cropping window in VPS |
|  |  |  |  |  | OFFSETS AND PHASE ADJUSTMENTS |
| Qualcomm | Adarsh Ramasubramonian | SCREFOFF | N | 1 | scaled ref layer offsets |
| Sharp | Tomoyuki Yamamoto | REFREGOFF | N | 1 | ref region offsets |
| Samsung | Elena Alshina | RESPHASE | N | 1 | resample phase |
|  |  |  |  |  | OUTPUT LAYERS, PICS |
| Nokia | Antti Hallapuro | OLS | N | 1 | add output layer sets, non-default target output layer sets, unnecessary layers |
| Qualcomm | Hendry | DISFLAG | N | 1 | discardable flag |
|  |  |  |  |  | SCALING LIST |
| Sony | Ohji Nakagami | PPSSLIST | N | 1 | SPS scaling list infer |
| Sony | Ohji Nakagami | SPSSLIST | N | 1 | PPS scaling list infer |
|  |  |  |  |  | COLOR GAMUT SCALABILITY |
| Technicolor | Franck Hiron | CGS | Y | 9 | Colour gamut scalability 8-to-10 bits, 10-to-10 bits, spatial ratio change with CGS |
|  |  |  |  |  | ADDITIONAL EXTENSIBILITY |
| Vidyo | Wonkap Jang | PSEXT | N | 1 | SPS, PPS additional extension |
| HHI | Karsten Suehring | LAYERID63 | N | 1 | NUHs with layer_id of 63 |
|  |  |  |  |  | POC |
| Ericsson | Rickard Sjoberg | POC | N |  | unaligned POC |
|  |  |  |  |  | BASE LAYER TYPE |
| Qualcomm | Hendry | HYBRID | N | 1 | hybrid scalability |
| Nokia | Antti Hallapuro | INBLD | Y | 1 | base layer not available (INBLD) |
| InterDigital | Yan Ye | SIM | N | 2 | simulcast |
|  |  |  |  |  | LEVEL SIGNALING |
| Vidyo | Wonkap Jang | SLLEV | N | 1 | sub_layer level signaling |
|  |  |  |  |  | AUX PICS |
| BBC | Matteo Naccari | ALPHA | N | 1 | Alpha |
| Nokia | Antti Hallapuro | DEPTH | N | 1 | Depth |
|  |  |  |  |  | LAYERS |
| Qualcomm | Vadim Seregin | LAYERS | N | 1 | 8 layers |
|  |  |  |  |  | PROFILE CONFORMANCE |
| Fujitsu | Guillaume Barroux | SREXT | N | 6 | Scalable Range extension profiles |

# Recommendations

The AHG recommends

- to start to discuss work plan of SCC conformance testing and collect bitstreams