| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0024r2 |
| --- | --- |

| Title: | Centralized Texture Depth Packing SEI Message for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC – Revision of Output Document JCT3V-O1004 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang (NCKU, Taiwan) Guan-Cheng Chen We-Jong Yang | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU), 1 University Road, Tainan, Taiwan 701 |  |  |

_____________________________

# Abstract

This document is a revision of JCT3V-O1004 [1]. The purpose of the proposed “Centralized Texture Depth Packing SEI message for HEVC” is to deliver one texture view plus one depth map, or two texture views plus two depth maps, packed into the samples of a monoscopic (texture only) stream for HEVC.  In [1], we remove depth_spatial_flipping_flag and two_views_texture_packing_type syntax to further reduce syntax and packing complexities and change the linear mapping function between Cb/Cr components and the residuals of DPCM predictions of depths to a piecewise linear mapping function to improve the packing performance.

- Centralized texture depth packing SEI message syntax

- Replace specification of general_non_packed_constraint_flag semantics in clause 7.4.4 with the following:

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages nor centralized texture depth packing SEI messages present in the CVS. general_non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages present in the CVS.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages or centralized texture depth packing SEI messages.

In subclause D.2.1 (General SEI message syntax), add the following rows at the position in the table corresponding to the payloadType number X:

| else if( payloadType == X ) | Descriptor |
| --- | --- |
| centralized_texture_depth_packing( payloadSize ) /* specified in Annex D */ |  |

where “X” is an appropriate payloadType number in the sequence of defined payloadType numbers of the respective specification.

Add a new subclause D.2.X “centralized texture depth packing SEI message syntax”, where. X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text of the respective standard.

D.2.X	Centralized texture depth packing (CTDP) SEI message syntax

| centralized_texture_depth_packing( payloadSize ) { | Descriptor |
| --- | --- |
| centralized_texture_depth_packing_cancel_flag | u(1) |
| if( ! centralized_texture_depth_packing_cancel_flag ) { |  |
| centralized_texture_depth_packing_type | u(3) |
| quarter_depth_size | u(12) |
| baseline_dist_flag | u(1) |
| focal_length_flag | u(1) |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( baseline_dist_flag ) |  |
| view_syn_rep_info_element( BdistanceSign, BdistanceExp, BdistanceMantissa, BdistanceManLen ) |  |
| if( focal_length_sample_flag ) |  |
| view_syn_rep_info_element( FlengthSign, FlengthExp, FlengthMantissa, FlengthManLen ) |  |
| if( z_near_flag ) |  |
| view_syn_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| view_syn_rep_info_element ( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| view_syn_rep_info_element ( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| view_syn_rep_info_element ( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| centralized_texture_depth_packing_persistence_flag | u(1) |
| } |  |
| } |  |
| } |  |

In Table F.4, add the persistence scope for the centralized texture depth packing SEI message as follows:

| - SEI message | - Persistence scope |
| --- | --- |
| centralized texture depth packing | - Specified by the syntax of the SEI message |

Add a new subclause D.3.X “centralized texture depth packing SEI message semantics” where X will be replaced with an appropriate subclause number in the sequence of subclauses present in the text. In the following subclause D.3.X, the numbers of tables, figures, and equations started from Table D-X1, Figure D-X1, D-X1 will be also replaced with appropriate numbers in the sequence of tables, figures, and equations present in the text, respectively.

- D.3.X Centralized texture depth packing SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains a specific packing arrangement of texture and depth samples, associated with one or two views. This information can be used by a process applied after decoding to appropriately interpret the samples and process them appropriately for display of 3D video or other purposes (which are outside the scope of this Specification).

When general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer, there shall be no centralized texture depth packing SEI message applicable for any picture of the CLVS of the current layer.

When a frame packing arrangement SEI message or a segmented rectangular frame packing arrangement SEI message is applicable for any picture of the CLVS of the current layer, there shall be no centralized texture depth packing SEI messages applicable for any picture of the CLVS of the current layer.

centralized_texture_depth_packing_cancel_flag equal to 1 indicates that the centralized texture_depth packing (CTDP) arrangement SEI message cancels the persistence of any previous centralized texture depth packing SEI message in output order. centralized_texture_depth_packing_cancel_flag equal to 0 indicates that centralized texture depth packing is applicable.

centralized_texture_depth_packing_type indicates the intended interpretation of the constituent frames as specified in Table D-X1.

Table D-X1 – Definition of centralized_texture_depth_packing _type

| Value | Interpretation |
| --- | --- |
| 0 | Indicates the CTDP-1TB type with an arrangement of subsample depth top (SDT), subsample texture (ST), and subsample depth bottom (SDB) regions from top to bottom, as illustrated in Figure D-X1. |
| 1 | Indicates the CTDP-1LR type with an arrangement of subsample depth left (SDL), subsample texture (ST), and subsample depth right (SDR) regions from left to right, as illustrated in Figure D-X2. |
| 2 | Indicates the CTDP-2TB type with an arrangement of_subsample depth leftview top (SDLT), subsample texture leftview (STL), and_subsample depth leftview bottom (SDLB) regions in the left-half picture and subsample depth rightview top (SDRT), subsample texture rightview (STR), and subsample depth rightview bottom (SDRB) regions in the right-half picture from top to bottom, as illustrated in Figure D-X3. |
| 3 | Indicates the CTDP-2LR type that the CTDP packed frame is arranged in the order of subsample depth topview left (SDTL), subsample texture topview (STT), and_subsample depth topview right (SDTR) regions in the top-half picture and_subsample depth bottomview left (SDBL), subsample texture bottomview (STB), and subsample depth bottomview right (SDBR) regions in the bottom-half picture from left to right, as illustrated in Figure D-X4. |

Values of centralized_texture_depth_packing_type that do not appear in Table D-X1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore centralized texture depth packing SEI messages that contain reserved values of centralized_texture_depth_packing type. Subsequently, centralized_texture_depth_packing type is called as packing type.

quarter_depth_size multiplied by four indicates the number of lines carrying the vertically subsampled depth map if centralized_texture_depth_packing_type is equal to 0 or 2. It indicates the number of columns carrying the horizontally subsampled depth map if centralized_texture_depth_packing_type is equal to 1 or 3. The quarter_depth_size specifies the arrangements of subsample texture and subsample depth information. When centralized_texture_depth_packing_type is not equal to 0, 1, 2, or 3, quarter_depth_size is ignored and reserved for future use by ITU-T | ISO/IEC. The detailed interpretation how the subsampled texture positions relate to the original full resolution positions of the texture frame is addressed and the detailed interpretation how the subsampled depth positions relate to the original full resolution positions of the depth map is given as follows.

Let W and H be the width and height respectively expressing pic_width_in_luma_samples and pic_height_in_luma_samples of the decoded picture output from the decoder. It is assumed that the original texture pictures and depth maps are both of size WxH.

In case of packing type equal 0 or 2, the packed frame from top to bottom is horizontally divided into top subsample depth, subsample texture, and bottom subsample depth regions. The height of top and bottom subsample depth regions in units of frame luma samples is given as

HHD = 2 * quarter_depth_size,								(D-X1)

while the height of subsample texture regions in units of frame luma samples becomes:

HST = H − 4 * quarter_depth_size		(D-X2)

When packing type is equal to 0 as illustrated in Figure D-X1, the packed frame with HHD and HST is divided into SDT, ST and SDB regions. The width of SDT, ST and SDB regions is W in units of frame luma samples. When packing type is equal to 2, as illustrated in Figure D-X3, they are further vertically divided into two left and right half regions to obtain top subsample depth leftview and top subsample depth rightview, subsample texture leftview and subsample texture rightview, bottom subsample depth leftview and bottom subsample depth rightview respectively identified as SDLT and SDRT, STL and STR, and SDLB, and SDRB, whose width becomes W/2 in units of frame luma samples.

In case of packing type equal 1 or 3, the packed frame from left to right is vertically divided into left subsample depth, subsample texture, and right subsample depth regions. The width of top and bottom subsample depth regions in units of frame luma samples is given as

WHD = 2 * quarter_depth_size,								(D-X3)

while the width of subsample texture regions in units of frame luma samples becomes:

WST = W − 4 * quarter_depth_size.		(D-X4)

When packing type is equal to 1, as illustrated in Figure D-X2, the packed frame with WHD and WST is divided into SDL, ST, and SDR regions. The height of SDL, ST and SDR regions is H in units of frame luma samples. When packing type is equal to 3, as illustrated in Figure D-X4, these three regions are further horizontally divided into top and bottom half regions to obtain left subsample depth topview and left subsample depth bottomview, subsample texture topview and subsample texture bottomview, right subsample depth topview and right subsample depth bottomview respectively identified as SDTL, SDBL, STT, STB, SDTR and SDBR regions, whose height becomes H/2 in units of frame luma samples.

Figure D-X1 Arrangement of the CTDP-1TB type and its depacking steps (packing type = 0)

Figure D-X2 Arrangement of the CTDP-1LR type and its depacking steps (packing type = 1)

Figure D-X3 Arrangement of the CTDP-2TB type and its depacking steps (packing type = 2)

Figure D-X4 Arrangement of the CTDP-2LR type and its depacking steps (packing type = 3)

The paired top and bottom or left and right subsample depth regions should be merged together to become one subsample depth region.

–	If packing type is equal to 0 or 2, as shown in Figures D-X1 and D-X3, the paired top and bottom subsample depth regions, which are SDT and SDB, SDLT and SDLB or SDRT and SDRB in YCbCr representation are vertically merged to obtain SD, SDL or SDR, respectively. If the paired top and bottom subsample depth regions are respectively denoted as SDXT and SDXB, the subsample depth after merging process is obtained as:

for( x = 0; x < Wd; x++ )

for( y = 0; y < HHD; y++ ) {

SDX[x][y] = SDXT[x][ y]

SDX[x][HHD +y] = SDXB[x][ y]

}

where Wd and HHD are the width and the height of top and bottom subsample depth regions in units of frame luma samples, respectively. If packing type is equal to 0, Wd=W and SDXT and SDXB respectively denote SDT and SDB regions, the merged result SDX becomes subsample depth, SD as shown in Figure D-X1. If packing type is equal to 2, Wd=W/2 and SDXT and SDXB respectively denote SDLT and SDLB or SDRT and SDRB regions, the merged result SDX becomes subsample depth leftview, SDL or subsample depth rightview, SDR, respectively as shown in Figure D-X3. After vertical merging process, the height of the subsample depth regions, SD, SDT, and SDB becomes 2HHD, while the width of the subsample depth is unchanged.

–	If packing type is equal to 1 or 3, as shown in Figures D-X2 and D-X4, the paired left and right subsample depth regions, which are SDL and SDR, SDTL and SDTR, or SDBL and SDBR in YCbCr representation are horizontally merged to obtain SD, or SDT and SDB, respectively. If the paired left and right subsample depth regions are respectively denoted as SDXL and SDXR, the subsample depth after merging process is obtained as:

for( x = 0; x < WHD; x++ )

for( y = 0; y < Hd; y++ ) {

SDX[x][y] =SDXL[x][y]

SDX[ x+WHD ][ y ] = SDXR[ x ][ y ]

}

where WHD and Hd are the width and the height of left and right subsample depth regions in units of frame luma samples, respectively. If packing type is equal to 1, Hd=H and SDXL and SDXR respectivel denote SDL and SDR regions, the merged result SDX become the subsample depth SD as shown in Figure D-X2. if packing type is equal to 3, Hd=H/2 and SDXL and SDXR respectively denote SDTL and SDTR or SDBL and SDBR regions, the merged result SDX becomes subsample depth topview, SDT or subsample depth bottomview, SDB, respectively as shown in Figure D-X4., After horizontal merging process, the width of the subsample depth regions, SD, SDT, and SDB becomes 2WHD, while the height of the subsample depth is unchanged.

For most video systems, the color representation for the texture is of YCbCr 4:2:0. For simplification, the subsample depth regions, SD, SDR, SDL, SDT, and SDB regions are also with the color representation of YCbCr 4:2:0. As shown in Figure D-X5, YCbCr 4:2:0 components of color subsample depth are mapped to 12 spatial positions of the subsample depth, where first 3 sets of YCbCr 4:2:0 components are marked by red, blue, and black rectangulars. YCbCr 4:2:0 components in each set contain 4 Y’s, 1 Cb, and 1 Cr, which are used to carry 6 selected depth values.

- If packing type is equal to 0 or 2, subsample color depth, SD, SDL or SDR region in YCbCr 4:2:0 representation is arranged in vertically quincunx positions with 6 unknown (marked x’s) and 6 known depth values. After the horizontally-positioned arrangement of YCbCr 4:2:0 components, the height of subsample depth, SD, SDL, or SDR is expanded 3 times to become 6HHD, while the width of the subsample depth is unchanged..

- If packing type is equal to 1 or 3, subsample color depth, SD, SDT or SDB region  in YCbCr 4:2:0 representation is arranged in horizontally quincunx positions with 6 unknown (marked x’s) and 6 known depth values. After the vertically-positioned arrangement of YCbCr 4:2:0 components, the width of the subsample depth, SD, SDT or SDB is expanded 3 times to become 6WHD, while the height of the subsample depth is unchanged.

- To simplify the notations, as shown in Figure D-X6, YCbCr 4:2:0 components are spatially indexed as 4 luma components, {Yi, for i = 1, 2, 3, 4} and two chroma components Cb1 and Cr1 while the corresponding 12 depth values are spatially indexed as {Di1, Di2, Di3, for i = 1, 2, 3, 4}.

Figure D-X5 Positioning YCbCr 4:2:0 components to 6 selected positions of subsample depth values

Figure D-X6 Grouped YCbCr 4:2:0 components to 6 selected depth samples: (a) vertical depacking for packing type 0 or 1; (b) horizontal depacking for packing type = 1 or 3

To recover the known depth values, 4 luma components Y1, Y2, Y3, Y4 in range of {16, 235} are first converted to 4 selected depth samples, D11, D22, D32 and D43 in the range of {0, 255} by linear mapping PCM values as:

D11 = (11644 * (Y1 − 16) + 5000) / 10000,	(D-X5)

D22 = (11644 * (Y2 − 16) + 5000) / 10000,	(D-X6)

D32 = (11644 * (Y3 − 16) + 5000) / 10000,	(D-X7)

D43 = (11644 * (Y4 − 16) + 5000) / 10000,	(D-X8)

respectively. The chroma components, Cb1 and Cr1 are respectively transformed to 2 selected depth samples, D13 and D41 by the piecewise range mappers of residuals plus their corresponding DPCM predictions. The detailed conversions are given as:

f(C)=23364∗(10∗C−1251)+50000)100000,C−128,23364∗(10∗C−1308)+50000)100000,16≤C<123123≤C≤133133<C≤240

- For packing type equal to 0 or 2, the depth values, D13 and D41 are computed by:

D13 = f(Cb1, D11,D32); 	(D-X9)

D41 = f(Cr2, D22, D43); 	(D-X10)

where the piecewise linear mapping function, f(C, D, E) maps C from the range of {16. 240} to the range of {255, 255} plus the DPCM prediction, which is the average of D and E, is defined as:

f(C, D,E)=(23364∗(10∗C−1251)+50000∗(D+E)+50000)100000,(10*(C−128)+5∗(D+E)+5)/10,(23364∗(10∗C−1308)+50000∗(D+E)+50000)100000,16≤C<123123≤C≤133133<C≤240    (D-X11)

The piecewise linear mapping function with the slope of 1 to map {123. 133} to the residual range of {5, 5}, while the piecewise linear mapping function with the slope of 2.3364 to map the reminding ranges of {16, 123} and {133, 240} as shown in (D-X11)

- For packing type equal to 1 or 3, the depth values, D13 and D41 are computed by:

D13 = f(Cb1, D11, D22);   	(D-X12)

D41 = f(Cr2, D32, D43); 	(D-X13)

The above procedures for conversing YCbCr 4:2:0 components to depth values only recover 6 known depth values arranged in quincunx fashions as shown in Figure D-X6. There are 6 unknown depth samples (in black colour) should be interpolated by a proper interpolation method from the neighboring known depth sample values interpolation. Thus, YCbCr 4:2:0 components in each set can be depacked to 12 depth values including 6 interpolated ones.

After the above conversion of YCbCr 4:2:0 components to depth values, the height, 2HHD of subsample depth SD, SDL, and SDR regions for packing type = 0 or 2 is vertically magnified to 3 times to become 6HHD or the width, 2WHD of subsample depth SD, SDT, and SDB regions for packing type = 1 or 3 is horizontally magnified to 3 times to become 6WHD. To completely depack the CTDP frame, the final upsampling processes of subsample texture and subsample depth regions are described as follows:

- For packing type =0, the subsample texture, ST with the size of WxHST is vertically upsampled by a factor of H:HST to obtain texture (T) region while the subsample depth, SD with the size of Wx6HHD is vertically upsampled by a factor of H:6HHD to obtain depth (D) region both with the size of WxH in units of frame luma samples.

- For packing type =1, the subsample texture, ST with the size of WSTxH is horizontally upsampled by a factor of W:WST to obtain texture (T) region while the subsample depth, SD with the size of 6WHDxH is horizontally upsampled by a factor of W:6WHD to obtain depth (D) region both with the size of WxH in units of frame luma samples.

- For packing type =2, the subsample texture leftview and subsample texture rightview, STL and STR with the size of W/2xHST are first performed horizontal 2:1 line-offset upsampling and then vertical H:HST upsampling to obtain texture leftview and texture rightview, TL and TR, respectively both with the size of WxH in units of frame luma samples, where the horizontal 2:1 line-offset upsamping is defined in Figure D-X7. A proper interpolation of all the missing pixels, which are marked with black color, should be peroformed to complete line-offset 2:1 upsampling.

- Figure D-X7 Horizontal line-offset 2:1 upsampling procedures (black=missing sample

- The subsample depth leftview and subsample depth rightview, SDL and SDR with the size of W/2x6HHD are performed horizontal 2:1 upsampling and vertical H:6HHD upsampling to obtain depth leftview and depth rightview, DL and DR, with the size of WxH in units of frame luma samples, respectively.

- For packing type =3, the subsample texture topview and subsample texture bottomview, STT and STB with the size of WST xH/2 are first performed vertical 2:1 line-offset upsampling and then horizontal W:WST upsampling to obtain texture topview and texture bottomview, TT and TB, respectively both with the size of WxH in units of frame luma samples, where the vertical 2:1 line-offset upsamping is defined in Figure D-X8. A proper interpolation of all the missing pixels, which are marked with black color, should be peroformed to complete line-offset 2:1 upsampling.

Figure D-X8 Vertical 2:1 line-offset upsampling procedures (black = missing sample)

The subsample depth topview and subsample depth bottomview, SDT and SDB with the size of 6WHDxH/2 are performed vertical 2:1 upsampling and horizontal W:6WHD upsampling to obtain depth topview and depth bottomview, DT and DB, with the size of WxH in units of frame luma samples, respectively.

-

-

baseline_dist_flag equal to 0 specifies that the syntax elements specifying the baseline distance are not present in the syntax structure. baseline_dist_flag equal to 1 specifies that the syntax elements specifying the baseline distance value are present in the syntax structure.

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure. focal_length_sample_flag equal to 1 specifies that the syntax elements specifying the focal length value are present in the syntax structure.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table D-X2. In Table D-X2, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to (1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table D-X2 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[i]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

The variables in the x column of Table D-X3 are derived from the respective variables in the s, e, n and v columns of Table D-X3 as follows:

- If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s* 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table D-X3 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| Bdistance | BdistanceSign | BdistanceExp | BdistanceMantissa | BdistanceManLen |
| Flength | FlengthSign | FlengthExp | FlengthMantissa | FlengthManLen |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture.

The units for the ZNear and ZFar, when present, are identical but unspecified.

When depth_representation_type is equal to 0 or 2, the disparity D can further be obtained from the value of Z, Bdistance, and Flength.

D = Bdistance * Flength ÷ Z								(D-X20)

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_‌representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first(0, 0) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {

pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev1 = depth_nonlinear_representation_model[ k ]

pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )

dev2 = depth_nonlinear_representation_model[ k + 1 ] (G-10)

x1 = pos1 − dev1

y1 = pos1 + dev1

x2 = pos2 − dev2

y2 = pos2 + dev2

for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )

DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )

}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

centralized_texture_depth_packing_persistence_flag specifies the persistence of the centralized texture depth packing arrangement SEI message.

centralized_texture_depth_packing_persistence_flag equal to 0 specifies that the centralized texture depth packing arrangement SEI message applies to the current decoded frame only.

centralized_texture_depth_packing_persistence_flag equal to 1 specifies that the centralized texture depth packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins;

–	The bitstream ends;

–	A frame in an access unit containing a centralized texture depth packing SEI message with the same.

- D.4.X	View synthesis representation information element syntax

| view_syn_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the view synthesis representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

# Performance Evaluations for Nonlinear Mapping of Cb and Cr

In [1], the depth values of CTDP formats are linearly transformed from the PCM coded Y’s and DPCM coded Cb, Cr components. The residual after DPCM prediction of depth values is in the range of [255, 255] is linearly mapped to Cb and Cr, which is in the range of {16, 240}. In [1], the depth values are recovered by the addition of the DPCM prediction and its DPCM residual, which is linear mapping with slope of 2.2768 from Cb as

f(C)=23364∗(10∗C−1251)+50000)100000,C−128,23364∗(10∗C−1308)+50000)100000,16≤C<123123≤C≤133133<C≤240

f(Cb,D,E)= (22768 * (Cb − 128) + 5000 * (D + E) + 5000) / 10000.	             (2.3)

The residuals of DPCM predictions are exponentially distributed in the range of [255, 255]. Thus, the residuals are mostly with very small. To improve the packing performance, we replace the linear mapping function with  a piecewise linear mapping function, where the piecewise linear mapping function with the slope of 1 to map {123. 133} to the residual range of {5, 5} and with the slope of 2.3364 to map the reminding ranges of {16, 123} and {133, 240} to {6, 255} and {6, 255}, respectively. Thus, the piecewise linear mapping function is given by

f(C, D,E)=(23364∗(10∗C−1251)+50000∗(D+E)+50000)100000,(10*(C−128)+5∗(D+E)+5)/10,(23364∗(10∗C−1308)+50000∗(D+E)+50000)100000,16≤C<123123≤C≤133133<C≤240        (2.4)

To verify the packing PSNR and SSIM performances, we conduct the following simulations in the conditions of CTDP formats without HEVC coding and the CTDP formats with HEVC coding comparing to 3D-HEVC.

2.1 Performance Evaluations of CTDP Formats in YCbCr 4:2:0 Chroma Representation

Without any video coding systems, the performances of the CTDP one-view and one-depth map formats are totally affected by: 1) subsample texture; 2) subsample depth; and 3) Cb and Cr components representation. In simulations, we adopt Shark and UndoDancer sequences for performance evaluations of uncoded CTDP videos with the linear mapping and the nonlinear mapping functions. In various CTDP formats, Tables 2.1 and 2.2 show the PSNR and SSIM performances of depacked texture, depth, and their synthesized virtual views generated by VSRS rendering system. The simulation results show that Cb and Cr components with the nonlinear mapping function show much better PSNR and better SSIM performances than those with the linear mapping function. Besides, the synthesized virtual view with the nonlinear CbCr mapping also exhibits better PSNR and SSIM performances than that with the linear mapping.

Table 2.1. Objective performances for uncoded CTDP-1TB texture-5/6 packing videos

| Performances |  | Depacked Texture and Depth |  |  |  |  |  |  | Synthesized Virtual View |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Texture |  | Depth |  |  |  |  | Virtual View Generated by VSRS |  |  |  |  |
|  |  |  | 5/6_1TB |  | Linear Mapping CbCr |  | Piecewise Linear Mapping CbCr |  |  | Linear Mapping CbCr |  | Piecewise Linear Mapping CbCr |  |  |
| PSNR | 45.2634 |  |  | 45.1297 |  | 45.4699 |  | 38.6801 |  |  | 38.7427 |  |  |  |
| SSIM | 0.9922 |  |  | 0.9832 |  | 0.9939 |  | 0.9843 |  |  | 0.9846 |  |  |  |

Table 2.2. Objective performances for uncoded CTDP-1TB texture-8/9 packing videos

| Performances | Depacked Texture and Depth |  |  | Synthesized Virtual View |  |
| --- | --- | --- | --- | --- | --- |
|  | Texture | Depth |  | Virtual View Generated by VSRS |  |
|  | 8/9_1TB | Linear Mapping CbCr | Piecewise Linear Mapping CbCr | Linear Mapping CbCr | Piecewise Linear Mapping CbCr |
| PSNR | 46.1485 | 43.9897 | 44.1021 | 38.7200 | 38.8875 |
| SSIM | 0.9938 | 0.9881 | 0.9918 | 0.9853 | 0.9867 |

2.2 Performance Evaluations of CTDP-HEVCwith respect to 3D-HEVC

To compare coding performances of CTDP-HEVC and the 3D-HEVC coding system, the CTDP-1TB, CTDP-1LR formats with the subsample texture ratios of 5/6 and 8/9 are encoded by HEVC HM13.0 and compared to 3D-HEVC HTM8.0. The QP setting for both HM13.0 and HTM8.0 are 27, 32, 37, and 42. Two CG 3D video test sequences, Shark and UndoDancer (1920x1088) sequences were removed top and bottom 4 horizontal lines to become the size of 1920x1080. In simulations, the coding performances of depacked texture, the depacked depth, and the synthesized virtual view by VSRS with the linear and nonlinear mapping Cb and Cr components in depth color of CTDP formats are exhibited. With respect to 3D-HEVC, Tables 2.3 – 2.6 shows that the BDBR and BDPSNR performances of synthesized virtual views for Texture-5/6 CTDP-1TB and Texture-8/9 CTDP-1TB formats.

Table 2.3. Average BDBR (%) results for Texture-5/6 CTDP-1TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  | Depacked Depth |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Linear Mapping CbCr+ | Piecewise Linear Mapping CbCr* | Linear Mapping CbCr | Piecewise Linear Mapping CbCr* | Linear Mapping CbCr+ | Piecewise Linear Mapping CbCr* |
| ra | 21.0704 | 18.6241 | -43.7626 | -47.7598 | 25.5579 | 21.1467 |
| ldp | 16.5524 | 13.8604 | -56.0816 | -62.3414 | 15.7390 | 11.8604 |
| ai | 12.1774 | 9.7019 | -32.1844 | -39.1668 | 27.9285 | 23.9325 |
| ave | 16.6000 | 14.0621 | -44.0095 | -49.7560 | 23.0751 | 18.9799 |

+Lancos 2 and Lancos 4 filters for texture downsampling and upsampling     *Lancos 4 filters for texture downsampling nad upsampling

Table 2.4. Average BDPSNR (dB) results for Texture 5/6 CTDP-1TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  | Depacked Depth |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Linear+ Mapping CbCr | Piecewise Linear* Mapping CbCr | Linear Mapping CbCr | Piecewise Linear Mapping CbCr | Linear + Mapping CbCr | Piecewise Linear* Mapping CbCr |
| ra | -0.6299 | -0.5694 | 2.2445 | 2.6245 | -0.6718 | -0.5769 |
| ldp | -0.5127 | -0.4383 | 3.9586 | 4.7799 | -0.4522 | -0.3570 |
| ai | -0.4329 | -0.3558 | 1.6828 | 2.2510 | -0.8275 | -0.7368 |
| ave | -0.5251 | -0.4545 | 2.6286 | 3.2184 | -0.6505 | -0.5569 |

+Lancos 2 and Lancos 4 filters for texture downsampling and upsampling     *Lancos 4 filters for texture downsampling nad upsampling

Table 2.5. Average BDBR (%) results for Texture 8/9 CTDP-1TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  | Depacked Depth |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Linear+ Mapping CbCr | Nonlinear* Mapping CbCr | Linear Mapping CbCr | Nonlinear Mapping CbCr | Linear+ Mapping CbCr | Nonlinear* Mapping CbCr |
| ra | 15.7538 | 13.6254 | -23.2366 | -30.6426 | 30.0211 | 25.7829 |
| ldp | 10.6721 | 8.2266 | -47.0105 | -53.8872 | 18.0217 | 13.9024 |
| ai | 9.0084 | 6.8931 | -7.0863 | -19.7578 | 33.8656 | 29.2954 |
| ave | 11.8114 | 9.5817 | -25.7778 | -34.7625 | 27.3028 | 22.9936 |

+Lancos 2 and Lancos 4 filters for texture downsampling and upsampling     *Lancos 4 filters for texture downsampling nad upsampling

Table 2.6. Average BDPSNR (dB) results for Texture 8/9 CTDP-1TB format with HEVC with respect to 3D-HEVC

| Coding Modes | Depacked Texture |  | Depacked Depth |  | Virtual View by VSRS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Linear+ Mapping CbCr | Nonlinear* Mapping CbCr | Linear Mapping CbCr | Nonlinear Mapping CbCr | Linear+ Mapping CbCr | Nonlinear* Mapping CbCr |
| ra | -0.4791 | -0.4247 | 0.9576 | 1.3941 | -0.7665 | -0.6789 |
| ldp | -0.3410 | -0.2707 | 2.9329 | 3.6932 | -0.5036 | -0.4051 |
| ai | -0.3244 | -0.2557 | 0.1938 | 0.8799 | -0.9675 | -0.8694 |
| ave | -0.3815 | -0.3170 | 1.3614 | 1.9902 | -0.7458 | -0.6511 |

+Lancos 2 and Lancos 4 filters for texture downsampling and upsampling     *Lancos 4 filters for texture downsampling nad upsampling

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

[1] J.-F. Yang and J.-R. Ohm, “Conceptual study of potential centralized texture depth packing SEI message”, Document, no. JCT3V-O1004, May 2016.