| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0032 |
| --- | --- |

| Title: | AHG 7: On Content Colour Volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tao Chen, Peng Yin, Taoran Lu, Fangjun Pu, Walt Husak 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

In JCT-VC July 2016 meeting, several proposals were submitted on how to represent content colour volume. Among all those proposals, some concerns arise: 1) which information is most important in the display adaptation for the optimal picture quality; 2) the minimum set of metadata that the bitstream should carry; 3) the complexity of generating and using those metadata. Taken all those into consideration, this contribution proposes a source content colour volume SEI which aims at providing a compact set of metadata.

# Introduction

In last meeting, JCTVC-X0040, JCTVC-X0052, and JCTVC-X0069 were proposed on how to represent content colour volume. The metadata is mainly intended as display adaptation assistance information. In JCTVC-X0040, a content gamut SEI message is proposed to signal source content colour gamut, which describes the actual colour distribution of the video content [2]. This is different from colour_primaries in VUI, which is used to indicate the container colour gamut instead of true source colour gamut [1]. In JCTVC-X0052, multiple primary expressions and spatial regions are proposed to be associated with the identified source characteristics [3]. It provides very detailed information about source content colour volume. In JCTVC-X0069, colour gamut primaries and luminance slice based colour primaries are proposed to indicate the colour volume occupied by the content [4]. It uses an (x, y, Y) description of the colour coordinates and has slices of Y with associated polygons for each slice.

This contribution proposes a source content colour volume SEI which aims at providing a compact set of metadata, which is important for the optimal signal adaptation on target displays and meanwhile is minimal in terms of data size.

# Proposal

The first question to answer is which metadata is must to have in order to represent a colour volume. A colour volume can be simply constructed by colour primaries (indicating the largest 2D colour gamut projected from 3D colour volume), white point chromaticity, maximum luminance and minimum luminance. This presents the largest colour volume boundary which the source content cannot exceed. Since it is reasonable to assume that the white point chromaticity is the same as what is indicated in VUI for container colour gamut, only colour primaries, maximum luminance and minimum luminance need to be signalled.

However, a particular content might only cover partial of this “largest” colour volume. Therefore, in order to precisely describe a source colour volume, [3] and [4] propose to add more metadata because of the irregular shape of source colour volume. However, it is necessary to limit the amount of metadata; otherwise too much metadata may increase the bitrate overhead to carry the SEI in the bitstream and thus impair coding efficiency.  In order to preserve temporal stability, display adaptation is often consistent with a scene. It is therefore desirable to avoid unnecessary and impractical representation of content colour volume. This contribution proposes to signal the most useful information to achieve a balance between complexity and versatility.

The study [5][6] shows that that in addition to maximum and minimum luminance values, another most important luminance value is the average luminance value or so called mid-point luminance value. These three values together are sufficient to generate a well performing tone curve for display mapping. To further refine the colour volume, the colour gamut information for the maximum, minimum and average luminance is made optional. This can be made application specific.

In summary, the contribution proposes to include the following metadata in the colour volume SEI message:

- the colour primaries (source colour gamut)

- the maximum, minimum and average luminance values

- if applicable, colour gamut for those three luminance values, (or more luminance values or more spatial partitions).

It is noted that colour volume should describe colour volume of the source in its original form. For example, it should describe the source colour volume before any chroma subsampling process or bitdepth conversion because these processes will modify colour volume information. Therefore the proposed SEI message is named as source colour volume SEI message.

# Syntax and Semantics

Table 1 shows the proposed syntax. For source colour primaries, the syntax would like to reuses of the semantics of colour_primaries in Table E.3 (Colour primaries interpretation using colour_primaries syntax element) in HEVC specification. The belief is that current source content can reach P3 colour space. But it will take some time to reach BT. 2020/2010 colour. Therefore, for the source whose colour gamut is smaller or equal to P3, or equal to BT. 2020/2010 colour primaries, Table E.3 can be used. But for the source whose colour gamut is larger than P3 but smaller than BT. 2020/2010, explicit signalling of colour gamut might be required. The luminance syntax will follow the definition in mastering display colour volume SEI message. For luminance sliced colour gamut, the colour gamut might not be triangle anymore. The proposal still uses 3-primary colour coordinates to simplify the design.

| source_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| source_colour_volume_id | ue(v) |
| source_colour_volume_cancel_flag | u(1) |
| if( !source_colour_volume_cancel_flag ) { |  |
| srouce_colour_volume_persistence_flag | u(1) |
| source_colour_primaries | u(8) |
| if( source_colour_primaries == 2) { |  |
| for( c = 0; c < 3; c++ ) { |  |
| source_primaries_x[ c ] | u(16) |
| source_primaries_y[ c ] | u(16) |
| } |  |
| } |  |
| max_source_luminance | u(32) |
| min_source_luminance | u(32) |
| avg_source_luminance | u(32) |
| luminance_colour_primaries_info_present_flag | u(1) |
| if(luminance_colour_primaries_info_present_flag) { |  |
| for( i = 0; i <= 3; i++ ) { |  |
| for( c = 0; c < 3; c++ ) { |  |
| luminance_primaries_x[ i ][ c ] | u(16) |
| luminance_primaries_y[ i ][ c ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

source_colour_volume_id contains an identifying number that may be used to identify the purpose of the source colour volume. The value of source_colour_volume_id shall be in the range of 0 to 232 − 2, inclusive.

Values of source_colour_volume_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of source_colour_volume_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour remapping information SEI messages containing a value of source_colour_volume_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

source_colour_volume_cancel_flag equal to 1 indicates that the source colour volume SEI message cancels the persistence of any previous source colour volume SEI message in output order that applies to the current layer. source_colour_volume_cancel_flag equal to 0 indicates that source colour volume follows.

source_colour_volume_persistence_flag specifies the persistence of the source colour volume SEI message for the current layer.

source_colour_volume_persistence_flag equal to 0 specifies that the source colour volume information applies to the current picture only.

Let picA be the current picture. source_colour_volume_persistence_flag equal to 1 specifies that the source colour volume persists for the current layer in output order until either of the following conditions is true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a source colour volume SEI message with the same value of source_colour_volume_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

source_colour_primaries has the same semantics as specified in clause Error: Reference source not found for the colour_primaries syntax element, except colour_primaries in clause Error: Reference source not found signals the container source colour primaries and source_colour_primaries signals the colour primaries the source content truly occupies.

When the value of source_colour_primaries is equal to 2, the source_colour_primaries is explicitly specified by syntax source_primaries_x[ c ] and source_primaries_y[ c ].

source_primaries_x[ c ] and source_primaries_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the source content in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing the source content that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Error: Reference source not found). The values of source_primaries_x[ c ] and source_primaries_y[ c ] shall be in the range of 0 to 50 000, inclusive.

max_source_luminance, min_source_luminance and avg_source_luminance specify the nominal maximum, minimum and average luminance, respectively, of the source in units of 0.0001 candelas per square metre. min_source_luminance shall be less than avg_source_luminance and avg_source_luminance shall be less than max_source_luminance.

luminance_colour_primaries_info_present_flag equal to 1 specifies that syntax elements luminance_primaries_x and luminance_primaries_y are present, luminance_colour_primaries_info_present_flag equal to 0 specifies that syntax elements luminance_primaries_x and luminance_primaries_y are not present.

luminance_primaries_x[ i ][ c ] and luminance_primaries_y[ i ][ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the source content at the nominal luminance i in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing the source content luminance, the index value 0, 1 and 2 shall correspond to max_source_luminance, min_source_luminance and avg_source_luminance, respectively. For describing the source content that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Error: Reference source not found). The values of luminance_primaries_x[ i ][ c ] and luminance_primaries_y[ i ][ c ] [ i ][ c ] shall be in the range of 0 to 50 000, inclusive.

# Conclusion

This contribution proposes a source colour volume SEI aiming at providing a compact set of metadata, which is important for the optimal signal adaptation on target displays and meanwhile is minimal in terms of data size.

# References

- "HEVC Screen Content Coding Draft Text 6", JCTVC-W1005, March 2016, San Diego, USA.

- "Content colour gamut SEI message", JCTVC-X0040, May 2016, Geneva, CH.

- "Improvements to the Effective Colour Volume SEI", JCTVC-X0052, May 2016, Geneva, CH.

- "Content colour volume SEI message ", JCTVC-X0052, May 2016, Geneva, CH.

- SMPTE ST 2094-10:2016: "Dynamic Metadata for Color Volume Transform – Application #1"

- SMPTE ST 2094-20:2016: "Dynamic Metadata for Color Volume Transform – Application #2"

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).